MAMALA BAY STUDY

PLUME MODELING

PROJECT MB-4

Principal Investigator:

Philip J. W. Roberts

School of Civil and Environmental Engineering

Georgia Institute of Technology

Atlanta, GA 30332

July 15, 1995

# 1  EXECUTIVE SUMMARY

The near and far field behavior of the plumes from the Sand Island and Honouliuli outfalls were modeled mathematically.  The models directly used treatment plant flow data and oceanographic data measured by current meters and thermistor strings located near the two diffusers.

The oceanographic conditions are very variable and quite complex near the diffusers.  The currents have a strong semi-diurnal tidal component but superimposed on this is a variable low frequency drift which can also be quite strong.  The surface temperature shows little variance and changes slowly with the season.  The average temperature decreases with depth but the variance of the temperature increases markedly with depth.  This causes the density stratification profile to be very variable ranging from strongly stratified to homogeneous over a few hours.  Homogeneity occurs when the warmer surface water extends down to the diffuser.  These oscillations may be internal waves forced by the tide.  The frequency with which the water column is close to homogeneous is much higher in winter than in summer.

More than 20,000 near field simulations were run and long time series of predicted rise height, dilution, and other plume characteristics were generated from which frequency distributions of wastefield behavior were obtained.  It was found that the oceanographic variability resulted in extreme variability in wastefield behavior.  Over the whole modeling period, the frequency of surfacing of the Sand Island plume was predicted to be 22.4%.  When submerged, the average plume rise height was 34.0 m, and average dilution 627.  When the plumes surface, dilutions are very high;  the lowest dilution of a surfacing plume was 597, average 1353, and highest 5401.

The overall averages obscure the extreme seasonal variability predicted.  The frequency of surfacing was predicted to vary from 5.1% in September to 62.1% in December.  In the summer months the plume is usually very deeply submerged.  Results from Honouliuli are generally similar but with increased submergence and higher dilutions due to the lower flow per unit diffuser length.  The results are discussed in detail for the period 1 August 1994 to October 1994, and summarized by month in Tables 4.1 and 4.2.  Appendices A and B contain graphical summaries of all the data and simulation results.

Because of the importance of surfacing of the Sand Island outfall plume it was analyzed further.  It was found that a density difference over the water column of only 0.09 sigma-t units will essentially always submerge the plume.  This corresponds to a temperature difference of only about 0.3 °C.  Apparently, temperature differences are often less than this, leading to plume surfacing.

The results should be considered representative of the data period only.  Data for other years may differ, and the very frequent December surfacing may be an anomaly.  Also, the present simulation period does not extend for a full year, and the frequency of surfacing on an annual basis should be less than the 22.4% predicted above.

The outfalls appear to be well designed and working well.  The plumes are highly dilute and usually submerged.  When surfacing, plume dilutions are at least 600:1.  No change in outfall operations are recommended.

The far field behavior of surfacing plumes were also modeled to estimate the potential for shoreline impact.  Using the measured current data, predictions were made of visitation frequency, minimum expected dilutions, and harmonic average dilutions in the vicinity of the diffusers.  It was found that current motions of tidal periods are too small to bring the plume to shore.  Longer period motions may occasionally cause shoreline impactions, but they will be very infrequent, and, if they occur, dilutions will be very high.  The harmonic average dilution increases rapidly to very high levels so that little effect of the discharges would be expected beyond about 5 km from the diffusers.  Because the Trade winds blow consistently to the Southwest for much of the year, they are likely to reduce shoreline impact further.  Onshore (Kona) winds may cause occasional shoreline impacts, but these also will be infrequent and highly dilute.

# Table of Contents

- 1  EXECUTIVE SUMMARY

TABLE OF CONTENTS

LIST OF FIGURES

LIST OF TABLES

2  INTRODUCTION

2.1  Scope of work

2.2  Objectives

2.3  Project Organization

3  METHODS

3.1  Task Summary

3.2  Study Sites

3.3  Near Field Behavior

3.4  Mathematical Model

3.4.1  Introduction

3.4.2  Chosen Model

3.5  Conditions Evaluated

3.5.1  Introduction

3.5.2  Currents

3.5.3  Density Stratification

3.5.4  Effluent Flowrates

4  RESULTS

4.1  Introduction

4.2  Detail:  Sand Island

4.3  Summary Results

4.4  Plume Submergence

4.5  Comparisons With Previous Studies

4.6  Effect of Water Column Velocity Shear

5  FAR FIELD MODEL

5.1  Introduction

5.2  Methodology

5.3  Results

5.4  Comparisons with Previous Studies

6  CONCLUSIONS

7  REFERENCES

8.  APPENDICES

APPENDIX A. Near Field Simulation Summary Plots:  Sand Island

APPENDIX B. Near Field Simulation Summary Plots: Honouliuli

APPENDIX C Wind record at Honolulu Airport

APPENDIX D  Far Field Simulation Summary

# List of Figures

Figure 2.1  Bathymetry, outfalls, and oceanographic instrumentation sites.

Figure 2.2  Sand Island and Honouliuli Outfalls.

Figure 3.1  Schematic Depiction of Wastewater Behavior in Coastal Waters.

Figure 3.2  Definition of Predicted Near Field Characteristics.

Figure 3.3  Photographs of Plume Behavior from Multiport Diffusers in Density-Stratified Currents (From Roberts et al., 1989abc).

Figure 3.4  Polar scatter diagrams of near bottom currents along diffuser bathymetry, 8/1/94 - 10/15/94.

Figure 3.5  Time series of near bottom first and second principal current components at D2.  Raw and low pass filtered currents are shown.

Figure 3.6  Simultaneous thermistor string data at various depths (13-63 m) at D2.

Figure 3.7  Near surface and near bottom temperatures and temperature difference over water column at D2.

Figure 3.8  Frequency distribution of density difference over water column for data of Figures 3.6 and 3.7.

Figure 3.9  Sand Island treatment plant effluent flows used in simulations.

Figure 4.1  Sand Island near field predictions of rise height and dilution.

Figure 4.2  Frequency distributions of Sand Island near field simulations of Figure 4.1.

Figure 4.3  Surface and bottom thermistor string data at Sand Island.

Figure 4.4  Surface and bottom temperature at Sand Island for November and December.

Figure 5.1  Visitation frequencies: 12 hour (top) and 24 hour (bottom) for surfacing Sand Island plume.  Winter, no winds.

Figure 5.2  Estimates of dilution.  Minimum (top) and harmonic average (bottom) for surfacing Sand Island plume.  Winter, no winds.

Figure 5.3  Visitation frequencies: 12 hour (top) and 24 hour (bottom) for surfacing Sand Island plume.  Winter, with winds.

Figure 5.4  Estimates of dilution.  Minimum (top) and harmonic average (bottom) for surfacing Sand Island plume.  Winter, with winds.

# List of Tables

Table 2.1  Characteristics of Sand Island and Honouliuli Outfalls

Table 4.1  Summary Statistics of Wastefield Behavior: Sand Island Outfall

Table 4.2  Summary Statistics of Wastefield Behavior: Honouliuli Outfall

Table 4.3  Effect of Depth of Current Meter Bin on Dilution Simulations

Table A1  Summary of current conditions, D2 at bottom

Table B1  Summary of current conditions, B2 at bottom

# 2  INTRODUCTION

## 2.1  Scope of work

The purpose of this project is to mathematically model the behavior of the wastefields generated by the Sand Island and Honouliuli outfalls which discharge treated domestic wastewater into Mamala Bay, Hawaii.  The extensive oceanographic data gathered near both outfalls is used directly in the mathematical models to obtain detailed statistics of wastefield behavior both near and farther from the diffusers.  The near field is the region near the diffuser where plume mixing results primarily from turbulence generated by the discharge itself.  Model predictions include the plume rise height, thickness of the drifting wastefield, length of the mixing zone, and initial dilution.  The far field is the region where mixing is due primarily to oceanic turbulence.  Far field predictions include the transport and behavior of the plume in the vicinity of the diffusers.  They include the frequency with which the plume impacts any location, minimum dilutions and harmonic average dilutions.

The collection of current meter data began in January 1994 and thermistor string data in April 1994.  Due to early problems with the Acoustic Doppler Current Profilers (ADCP’s), however, simultaneous collection of current and thermistor string data did not begin until June 1994.  This report discusses modeling based on data collected from June 1994 to April 1995.  This project uses the oceanographic data obtained in Project MB-6, “Ocean Current Measurements,” and treatment plant flow data from Project MB-2, “Data Base.”  The model results are used directly in Project MB-5, “Modeling Fate and Transport of Pathogenic Organisms.”

## 2.2  Objectives

The objectives of this project are to predict the behavior of the plumes generated by the Sand Island and Honouliuli outfalls, using measured oceanographic data directly in the models.  Near field predictions will comprise long time series of wastefield behavior including rise height and dilution.  From these time series frequency distributions of plume surfacing, rise height, and initial dilution will be obtained.  The results of the near field modeling will be put in a form suitable for direct input into the water quality model, Project MB-5.  Far field transport predictions in the vicinity of the diffusers will also be made including the frequency with which the plume impacts any location, minimum dilutions and harmonic average dilutions.  The results will be interpreted and compared with previous studies which have been undertaken on the Sand Island outfall.

## 2.3  Project Organization

The Principal Investigator for this project is Philip J. W. Roberts, Professor of Civil Engineering, Georgia Institute of Technology, Atlanta, Georgia.  Dr. Roberts was responsible for model development and application, interpretation of the results and preparation of the report.  He was assisted by graduate research assistants Adrian Ferrier, Greg Daviero, and Amy Brown, whose responsibilities included data analysis and preparation and graphics.

Figure 2.1  Bathymetry, outfalls, and oceanographic instrumentation sites.

Figure 2.2  Sand Island and Honouliuli Outfalls.

Table 2.1  Characteristics of Sand Island and Honouliuli Outfalls

|  | Sand Island | Honouliuli |
| --- | --- | --- |
| Average design flow (m3/s) | 4.64 |  |
| Present average daily flow | 3.07 | 1.14 |
| Typical daily maximum flow | 3.85 | 1.60 |
| Typical daily minimum flow | 2.01 | 0.50 |
| Total outfall length (m) | 3811 | 3203 |
| Internal diameter (m) | 2.13 | 1.98 |
| Diffuser length (m) | 1031 | 533 |
| Number of ports | 285 | 148 |
| Port spacing (m) | 7.3 | 7.3 |
| Port diameter (mm) | 76 - 90 | 95 |
| Discharge depth (m) | 67 - 72 | 61 - 63 |

# 3  METHODS

## 3.1  Task Summary

The behavior of the plumes from the Sand Island and Honouliuli outfalls is predicted by mathematical modeling using measured oceanographic data directly as input to the model.  In this Chapter, the study sites and outfall characteristics are first summarized in Section 3.2.  Then mixing processes of coastal ocean outfalls are briefly discussed and the meaning and definition of the near- and far-field is given in Section 3.3.  The choice of the near field mathematical model is given in Section 3.4, and the conditions evaluated are discussed in Section 3.5.  This section includes a brief discussion of the oceanographic data relevant to near field behavior, and the treatment plant flowrates.

## 3.2  Study Sites

The topography of Mamala Bay, the oceanographic instrumentation sites, and major outfalls are shown in three dimensional relief in Figure 2.1.  The two ocean outfalls which discharge primary treated effluent into Mamala Bay at Sand Island and Honouliuli are shown in more detail in Figure 2.2.  The outfalls terminate in long multiport diffusers which discharge into quite deep water due to the steep slope of the seabed away from the shore. The major characteristics of these two outfalls and diffusers are summarized in Table 2.1.

## 3.3  Near Field Behavior

A schematic depiction of the behavior of wastewater discharged from a multiport diffuser into coastal waters is shown in Figure 3.1.  The sewage is ejected horizontally as round jets from ports which are spaced uniformly along both sides of the diffuser.  Because the density of sewage is essentially the same as fresh water, it is very buoyant in sea water.  The jets therefore quickly become plumes which merge with their neighbors as they rise towards the surface.  These plumes are turbulent and they entrain ambient sea water as they rise.  The entrained sea water mixes with the plumes and dilutes the wastewater.  Depending on the density stratification in the ocean the plumes may reach a level of neutral buoyancy, the “trapping level,” where they stop rising and begin to spread laterally.  The region near the diffuser where mixing is intense is called the “near field” or “initial mixing region.”  The spreading layer, called the established wastefield, then drifts with the ocean current and is diffused by oceanic turbulence in a region called the “far field.”

The rate of mixing in the far field is much slower than in the near field.  As the wastefield drifts some particles may separate out, and, by differential settling and turbulent diffusion, will deposit on the ocean floor.  For a full discussion of the dynamics of near field mixing see Roberts (1991).

The characteristics of the established wastefield depend on both diffuser and oceanographic parameters.  The most important diffuser parameters are the port diameter, port spacing, jet effluent velocity, and diffuser length.  The most important oceanographic parameters are the current speed and direction and the density stratification over the water column.

Figure 3.1  Schematic Depiction of Wastewater Behavior in Coastal Waters.

The objective of near field modeling is to predict the established wastefield and how it depends on these parameters.  The most important characteristics are defined in Figure 3.2.  They are the rise height to the top of the drifting wastefield, ze, its thickness he, the height to the level of minimum dilution zm, the minimum dilution Sm, and the length of the initial mixing region or near field xi.  Here the definitions of Sm, xi, etc. are those used in Roberts et al. (1989abc), and Baumgartner et al. (1994).  The choice of an appropriate mathematical model to predict these is discussed in the following section.

Some photographs, from Roberts et al. (1989 abc), of merging plumes from multiport diffusers similar to those here are shown in Figure 3.3  Figure 3.3a shows photographs of plume behavior in currents perpendicular to the diffuser at various current speeds.  The reduction in rise height and wastefield thickness with increasing current speed can be seen. Figure 3.3b shows side and overhead views of plumes in currents parallel to the diffuser.  A complex three-dimensional flowfield is evident with rapid expansion of the wastefield due to gravitational spreading.

Figure 3.2  Definition of Predicted Near Field Characteristics.

## 3.4  Mathematical Model

### 3.4.1  Introduction

Many mathematical models have been developed to predict wastefield characteristics.  Well known, for example, are those used by the EPA (Muellenhof et al., 1985):  UPLUME, UOUTPLM, UDKHDEN, UMERGE, and ULINE.  Some of these models are of the entrainment type, some are empirical, and some are based on extrapolations from unstratified experiments to stratified fluids.  These and other models are discussed in detail in Roberts (1991).  Comparisons of their predictions with experimental results were also given.  It was found that the models do not always reliably predict dilution or rise height, and large differences were found between the predictions of the various models.

Figure 3.3  Photographs of Plume Behavior from Multiport Diffusers in Density-Stratified Currents (From Roberts et al., 1989abc).

These models also have other significant deficiencies.  For example, most implicitly assume the discharge to be from one side of the diffuser only and do not account for merging of the plumes from both sides.  In addition, the models do not generally provide the type of output needed as input to far field models.  Typically predicted is the flux-average dilution and rise height.  What is needed in the far-field model is the lateral extent and thickness of the established wastefield and the spatial variation of dilution over it.  These models do not predict lateral gravitational spreading from which the initial field width is obtained.  They also do not predict the extent of the initial mixing zone where the transition to the far-field model should occur.  Except for ULINE, these models implicitly assume the current to be perpendicular to the diffuser and use only the component of current speed perpendicular to the diffuser.  Finally, this generation of models does not include the additional mixing which can occur in the horizontally spreading layer nor the possibility of re-entrainment from this layer.  Other entrainment models also typically suffer from these deficiencies.

Another model often used for diffuser calculations is CORMIX, especially CORMIX 2 (Jirka & Akar, 1991) for submerged multiport diffusers.  CORMIX is a general purpose model which employs length scale analyses and asymptotic solutions to jet and plume flows.  It attempts to cover a wide range of conditions but does not have refined codes specifically for multiport coastal water sewage diffusers in stratified flows.  It has not been validated for stratified discharges and comparisons with field observations of surfacing plumes show considerable discrepancies (Frick, 1995).  It is also only suited for individual runs and not for the thousands of simulations which will be done here.  CORMIX was therefore rejected from further consideration.

### 3.4.2  Chosen Model

A model has recently been developed which accounts for the most important hydrodynamic aspects of multiport diffuser flow discussed above.  This model, RSB, is based on the extensive experimental studies on multiport diffusers in flowing density-stratified currents described in Roberts, Snyder, and Baumgartner (1989abc).  The photographs and results discussed in Section 3.2.2 were taken from this study.  A wide range of jet momentum and buoyancy fluxes, port spacings, stratification conditions, and current speed and direction, including parallel currents, were tested.  These parameters mostly cover the range expected in Mamala Bay.  Noda (1992) used some of the results from this study, which they refer to as the "state-of-the-art" (p. 5).

RSB has been adopted by the EPA and is described in the update to the 1985 Discharge Manual (Baumgartner et al., 1994).  Because the model basis is extensively discussed in a series of three refereed journal papers, Roberts et al. (1989abc), and the model formulation is described in the EPA manual, the solution method will not be described in detail here.

RSB can be thought of as a replacement for and a significant update of ULINE.  Whereas ULINE was based on experiments in unstratified environments, RSB is based on experiments in stratified flows, and is therefore more reliable in this situation.  ULINE applies only to single slot plumes whereas RSB is based on experiments with multiport diffusers.  It therefore includes the effects of port spacing and source momentum flux, and is more realistic in that it includes discharges from both sides of the diffuser, as is the case with the Mamala Bay outfalls.

In addition to the very extensive original data set, the model has been validated against other data.  These include field tracer and laboratory experiments for the San Francisco outfall (Roberts & Wilson, 1990), and implicitly with the field results for the Whites Point, Los Angeles, outfall (Washburn et al., 1992).  Experiments for the Boston Harbor outfall (Roberts & Snyder, 1993), which has a very non-linear stratification, were also well simulated by the model.

This EPA approved model has a solid physical basis, as it is derived directly from realistic experiments on multiport diffusers which incorporate the most important hydrodynamics of ocean outfalls.  These phenomena, lacking in other models, include merging of plumes from both sides of the diffuser, re-entrainment and additional mixing in the spreading layer, direct plume impingement in parallel currents, and lateral gravitational spreading.  The experiments on which RSB is based are the only experiments reported on multiport diffusers in stratified parallel currents and it simulated the observed wastefield behavior for the Boston Harbor outfall model tests in parallel currents.  As discussed below, parallel current behavior will be of particular significance to the Mamala Bay outfalls.

Despite the belief that RSB is the best model for the Mamala Bay outfalls, its limitations should also be kept in mind.  First, it is based on experiments using only linear density stratification conditions.  Coastal water density profiles are rarely linear, however.  The solution procedure iteratively linearizes the density profile over the rise height by a method described in Baumgartner et al. (1994).  The comparisons with field and other laboratory measurements discussed above show this to be usually a good assumption.  Second, the experiments are based on a uniform current, i.e. the current is constant in speed and direction over the rise height.  This may not be the case in deep coastal waters, and the effect of such vertical shear is not well known.  The experiments are based on straight line diffusers whereas the Mamala Bay diffusers have a slight curvature.  This is unlikely to affect the results significantly.  Finally, the experiments were run with a smaller aspect ratio (the ratio of diffuser length to rise height) than actually occurs in the field.  Again, based on other experiments and field observations this is not a significant effect.  It should  be kept in mind that these limitations are also shared by the other models, which are also based ultimately on coefficients derived from small scale laboratory experiments using homogeneous and linear density stratifications.

RSB was rewritten in FORTRAN to run with long time series of input data and to provide the type of output required by the far-field model.  Its run-time is sufficiently fast that it can easily run the thousands of simulations required to simulate the statistics of the wastefield with the time series of oceanographic data.  This is similar to the approach taken in the recent modeling studies for the San Diego outfall extension, in which an earlier version of RSB was used.

## 3.5  Conditions Evaluated

### 3.5.1  Introduction

The modeling period in the present report extends from June 1994 to April 1995.  The oceanographic data are discussed in detail in Project MB-6 “Ocean Current Measurements.”  Here we discuss them only briefly to summarize the characteristics of importance in the near field analysis.  We discuss the Sand Island data for the period August 1, 1994, to October 15, 1994; the corresponding model results for Sand Island for this period are discussed in Section 4.2.  The remaining simulation results are summarized in Section 4.3, and all results for the whole modeling period and the data used are summarized graphically in Appendices A and B.  The instruments were rotated and data recovered at either three or six month intervals depending on the instrument type;  Occasional data discontinuities occur as a result of these rotations.

### 3.5.2  Currents

Two ADCP’s were positioned close to the Sand Island and Honouliuli diffusers (D2 and B2 respectively, Figure 2.2).  The instruments were set to record current speed and direction over the water column at either half or one-hour intervals.  The instruments average current velocities in two-meter wide bins over the water column, providing 32 bins at D2 and 28 bins at B2.  At D2 the readings extend from about 7 m depth to about 65 m; at B2 the readings extend from about 10 m depth to about 60 m.  During Deployment 4 (November 1994 - January 1995) the ADCP at B2 was replaced by an Interocean S4 electromagnetic current meter at 12.5 m depth and two Niskin General Oceanics Winged current meters at 37.5 and 62.5 m depth.  The ADCP was replaced for Deployment 5 (January 1995 - April 1995).

Polar scatter diagrams of the near-bottom currents are shown in Figure 3.4 for the period 8/1/94 - 10/15/94.  In general, the currents show peak velocities of approximately 100 cm/s at Diamond Head and about 50 cm/s near Barbers Point.  The currents are primarily aligned with the local bathymetry. Near the diffuser sites at B2 and D2, peak currents are somewhat slower.  At D2 the typical maxima are around 50 cm/s and at B2 around 30 cm/s.  Again, the scatter diagrams show the currents to be predominantly along the local bathymetry.  This alignment of currents is predominantly parallel to the diffusers and would be expected to produce flow patterns similar to those discussed in Chapter 3.  They also affirm the choice of the model RSB, which is the only one which reliably incorporates parallel currents.

The principal components of the currents were also computed using a method described in Koh (1978).  Briefly, the directions of the principal axes are the eigenvectors of the covariance matrix of the orthogonal current components.  The principal components are the components of the currents projected onto these axes.  This corresponds to a rotation of the coordinate axes until they maximize and minimize the energy, or variance, of the currents. For the record shown in Figure 3.4, the direction of the first principal component at B2 is 2300 (clockwise to true North) and at D2 about 2670.  These directions are almost parallel to the local bathymetric contours.

Figure 3.4  Polar scatter diagrams of near bottom currents along diffuser bathymetry, 8/1/94 - 10/15/94.

The time series of the raw and the 45-hour low-pass filtered first and second principal components at D2 are shown in Figure 3.5.  Although further analyses were done, they are not discussed in detail here.

Figure 3.5  Time series of near bottom first and second principal current components at D2.  Raw and low pass filtered currents are shown.

The currents are clearly very complex.  The first principal components show a strong semi-diurnal tidal component, reversing twice daily.  Peak tidal currents are typically around 30 cm/s.  Superimposed on this is a low-frequency drift with fluctuations of around 4 days periodicity.  This drift can be strong, with speeds up to 20 cm/s; for this period, the net drift is towards the West.  The net drift can occasionally be strong enough to cause uni-directional currents with no tidal reversal.  Finally, high frequency fluctuations are also evident which can have speeds up to about 20 cm/s.  It is not clear, however, how much of this high frequency component is due to instrument noise.  The second principal components, approximately orthogonal to the local bathymetry and therefore to the diffuser, are generally weaker than the first;  typical maximum speeds are around 15 cm/s.  Similar time series for the rest of the modeling period are shown in Appendices A and B.

### 3.5.3  Density Stratification

As discussed in Section 3.3 the density stratification over the water column plays a crucial role in the plume behavior.  It affects rise height and dilution and whether the plume surfaces.  Stratification can vary rapidly in coastal waters and in this case was measured continuously by two thermistor chains at D2 and B2.  There were 11 thermistors on each chain at 5 m depth increments extending from 13 to 63 m depth at D2 and 15 to 65 m depth at B2.  The thermistor time constant is about 1 minute and instantaneous readings were recorded at half-hour intervals.

Time series of temperatures for six of the thermistors at 10 m spacings at D2 are shown in Figure 3.6 for the period 8/1/94 to 10/14/94.  The surface water temperature shows little variance, and exhibits a gradual warming trend through the summer.  Moving down through the water column, the variance of the temperature increases with the magnitude of the fluctuations becoming as high as 5 °C at 63 m depth.  Spectra of the temperature fluctuations near the bottom are sharply peaked at the semi-diurnal tidal frequency.  The fluctuations may be due to internal waves which are tidally forced.

Here we are mostly interested in the temperature difference over the water column as this is what most affects the density stratification.  Figure 3.7 shows the surface and bottom temperatures and the temperature difference for the date of Figure 3.6.  As can be seen, the temperature differences vary widely and over short times from zero (i.e. uniform) to 5 °C.

Figure 3.6  Simultaneous thermistor string data at various depths (13-63 m) at D2.

In general, density stratification in coastal waters depends on temperature and salinity variations over depth.  The thermistor strings measure only temperature but cruises in which vertical profiles of temperature and salinity were measured by means of CTD (Conductivity, Temperature, and Depth) profilers at various locations in Mamala Bay were also done as part of Project MB-6.  These data show a strong correlation between temperature and salinity, and it was assumed that the same correlation applied to the thermistor string data.  The data show the salinity generally increasing downwards, and the following relationship was assumed:

S = 35.33 - 0.090 (T - 20)	(3.1)

where S is the salinity (PSU) and T is the temperature (oC).

Figure 3.7  Near surface and near bottom temperatures and temperature difference over water column at D2.

Based on the measured temperature with salinity computed from Eq. 3.1, the seawater density was computed for each thermistor at each time measurement.  The formulae used in this computations are those of Bigg (1967), Millero, et al. (1980), and Millero & Poisson (1981).  The density profiles over the water column for each time step were then computed. Although these profiles were used directly in the simulations, it is of interest to investigate the frequency distribution of density stratification over the water column.  This is shown in Figure 3.8 for the data of Figures 3.6 and 3.7.  It can be seen that the density difference over the water column for this period is less than 0.05 sigma-t units for 9.3% of the time, and less than 0.1 sigma-t units for 13.6% of the time.  The significance of this is discussed in Section 4.4.  The maximum density difference is about 2 sigma-t units.

Figure 3.8  Frequency distribution of density difference over water column for data of Figures 3.6 and 3.7.

### 3.5.4  Effluent Flowrates

Flows from wastewater treatment plants typically vary diurnally, with minima during the early morning and one or two maxima during the day.  For these simulations we used actual flow data obtained from the Department of Public Works, City and County of Honolulu.  The measurements were obtained at the Honouliuli and Sand Island treatment plants at hourly intervals.  At Sand Island influent and effluent flowrates were reported; the effluent flows were used in the simulations.  At Honouliuli there was a malfunction of the effluent flow meter.  According to the Department of Public Works, however, the influent flowrate can be used to estimate effluent flow, and so the influent flow data were used for the Honouliuli simulations.  The data were inspected and obvious spikes and periods of zeros were removed.  The data only extends at present to December 31, 1994.  After this date, it was assumed that the flowrates were the same as those of the previous year.  The data were shifted by one day, however, to maintain the weekly Monday to Sunday variation.

The actual flows used in the Sand Island simulations are shown in Figure 3.9 for the period 8/1/94 to 10/15/94; flowrates from the rest of the modeling period are shown in Appendices A and B.  The flows are summarized in Table 2.1

Figure 3.9  Sand Island treatment plant effluent flows used in simulations.

# 4  RESULTS

## 4.1  Introduction

The model RSB was run with a half-hour time step using the measured currents, density profiles, and treatment plant flows as direct input.  The density profiles were calculated from the thermistor string data as discussed in Section 3.5.3.  RSB assumes that the current is uniform over depth.  For this reason, only the currents from the bottom bin of the ADCP were used as these were assumed to be the currents having most influence over the plume behavior.  More than 20,000 dilution simulations were run in this way.  This is vastly greater than typically done in a study of this type.  In Section 4.2 we discuss the Sand Island results for the period 8/1/94 to 10/15/94 corresponding to the oceanographic data discussed in Section 3.5.  Results for the remaining modeling period are summarized by month in Section 4.3.  Results are summarized graphically in Appendices A and B.

## 4.2  Detail:  Sand Island

Time series of rise height and dilution are shown in Figure 4.1.  It can be seen that they are both extremely variable as a result of the variable oceanographic conditions.  Frequency distributions of rise height and dilution for this period are shown in Figure 4.2.

For this period, the plume surfaces about 11% of the time.  It is more usually deeply submerged, however.  The average rise height when submerged is 28.9 m, and 50% of the time the rise height is less than 25 m (or conversely, the plume is submerged more than 45 m below the water surface for 50% of the time).  The plume was continuously deeply submerged from around September 20 to October 4.  This included the period of the field test which was conducted during the last week in September.  The reason for this can be seen in the temperature records for this period (Figures 3.6 and 3.7) which show continuous stratification for this period.

Dilutions are similarly extremely variable.  The lowest dilution for this period was 93 and the highest was 4350.  The average minimum dilution was 542.

Figure 4.1  Sand Island near field predictions of rise height and dilution.

The extreme variability in results is due to the variable oceanographic conditions (see Section 3.5) and flowrates.  Extreme values occur when certain conditions occur simultaneously.  The highest dilutions occur when the lowest effluent flowrate coincides with weakest stratification and with the strongest current most nearly perpendicular to the diffuser.  Conversely, lowest dilution occurs when the highest flowrate coincides with strongest stratification and slack water.  It should be kept in mind that dilution and rise height are related.  In general, the dilution of a surfacing plume will be much higher than a submerged plume.  The lowest dilution of 93 occurred with a deeply submerged plume.  For this period the dilutions of the surfacing plume ranged from 601 to 4350 with an average value of 1475.

Figure 4.2  Frequency distributions of Sand Island near field simulations of Figure 4.1.

## 4.3  Summary Results

The model was run in similar fashion for all periods when simultaneous current and thermistor string data were available for both the Sand Island and Honouliuli outfalls.  This period extends from approximately June 5, 1994 to April 18, 1995, with occasional data gaps due to instrument failure and rotation.  More than 20,000 simulations were run all of which are summarized graphically in Appendices A and B.  In order to summarize the huge amount of information, the results were separated into submerged and surfacing plumes and the summarized by month.  These results are tabulated in Tables 4.1 and 4.2.

Table 4.1  Summary Statistics of Wastefield Behavior: Sand Island Outfall

| Period | Submerged |  |  |  |  |  |  |  |  | Surfacing |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency | Average rise height | Dilution |  |  |  |  | Frequency | Dilution |  |  |  |  |
|  |  |  | Lowest | Average | Highest |  | Lowest | Average | Highest |
|  | % | (m) |  |  |  | % |  |  |  |
| June ‘941 | 94.5 | 23.9 | 124 | 435 | 2340 | 5.5 | 639 | 1354 | 3220 |
| July ‘94 | 89.8 | 32.6 | 127 | 581 | 2673 | 10.2 | 605 | 1458 | 4495 |
| August ‘94 | 84.1 | 24.9 | 93 | 467 | 2674 | 15.9 | 616 | 1509 | 4347 |
| September ‘94 | 94.9 | 22.9 | 103 | 396 | 2180 | 5.1 | 625 | 1486 | 3742 |
| October ‘94 | 88.2 | 27.2 | 101 | 502 | 3753 | 11.8 | 601 | 1241 | 3815 |
| November ‘94 | 58.6 | 34.7 | 87 | 617 | 2505 | 41.4 | 624 | 1408 | 5401 |
| December ‘94 | 37.9 | 46.0 | 114 | 868 | 2797 | 62.1 | 597 | 1268 | 3436 |
| January ‘951 | 88.0 | 41.4 | 233 | 705 | 2545 | 12.0 | 645 | 1138 | 2218 |
| February ‘95 | 65.2 | 45.7 | 245 | 886 | 2602 | 34.8 | 610 | 1351 | 4409 |
| March ‘95 | 62.1 | 42.5 | 258 | 847 | 3020 | 37.9 | 646 | 1348 | 3414 |
| April ‘951 | 80.7 | 32.1 | 205 | 596 | 2655 | 19.3 | 643 | 1318 | 2969 |
| Whole period2 | 77.6 | 34.0 | 87 | 627 | 3753 | 22.4 | 597 | 1353 | 5401 |

1 Partial month’s data

2 Average of columns

Table 4.2  Summary Statistics of Wastefield Behavior: Honouliuli Outfall

| Period | Submerged |  |  |  |  |  |  |  |  | Surfacing |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency | Average rise height | Dilution |  |  |  |  | Frequency | Dilution |  |  |  |  |
|  |  |  | Lowest | Average | Highest |  | Lowest | Average | Highest |
|  | % | (m) |  |  |  | % |  |  |  |
| June ‘941 | 84.7 | 19.7 | 123 | 732 | 3825 | 15.2 | 714 | 1513 | 4830 |
| July ‘941 | 91.0 | 25.7 | 129 | 517 | 3440 | 9.0 | 683 | 1533 | 3759 |
| August ‘94 | 88.9 | 19.6 | 73 | 407 | 2437 | 11.1 | 670 | 1231 | 4200 |
| September ‘94 | 98.0 | 16.6 | 75 | 318 | 3123 | 2.0 | 706 | 1241 | 1815 |
| October ‘941 | 93.6 | 20.7 | 82 | 400 | 2181 | 6.5 | 674 | 1161 | 2707 |
| November ‘941 | 50.7 | 22.4 | 88 | 480 | 3638 | 49.3 | 652 | 1448 | 7838 |
| December ‘94 | 55.8 | 35.9 | 115 | 814 | 5103 | 44.2 | 549 | 1545 | 5647 |
| January ‘951 | 88.6 | 33.6 | 195 | 756 | 5566 | 11.4 | 650 | 1572 | 16174 |
| February ‘95 | 79.0 | 37.7 | 197 | 881 | 3870 | 21.0 | 662 | 1281 | 3936 |
| March ‘95 | 79.3 | 35.1 | 165 | 826 | 4283 | 20.7 | 691 | 1322 | 4483 |
| April ‘951 | 87.2 | 24.6 | 144 | 638 | 5015 | 12.8 | 702 | 1839 | 6181 |
| Whole period2 | 81.5 | 26.5 | 73 | 588 | 5566 | 18.5 | 549 | 1426 | 16174 |

1 Partial month’s data

2 Average of columns

The results show considerable seasonal variability.  For the Sand Island outfall, the plume is predicted to be submerged 77.6% of the time, or conversely to surface 22.4% of the time on an annual basis.  The monthly variation is considerable, however, ranging from 5.1% surfacing in September to 62.1% in December.  When submerged, the average rise height is 34.0 m, ranging from a low of 22.9 m in September to 46.0 m in December.  This variation is due to seasonal temperature variations, the higher average rise height in Winter is due to generally weaker stratification at this time.  Dilutions also show considerable variation.  Submerged plume dilutions range from 93 to 3753, and surfacing plume dilutions from 601 to 5401, with an average of 1353.  Seasonal variations of surfacing plume dilutions are much less as these only occur for unstratified conditions.  The above annual averages are based on only nine months data from June to April.  When computed on the basis of a full years data they would differ.  For example, surfacing frequency would be expected to be less than 22.4%.

Results for Honouliuli are generally similar, also showing considerable seasonal variability. The frequency of surfacing is less than at Sand Island, about 18.5% on an annual basis, with higher dilutions.  The reason for the more frequent submergence and higher dilution for Honouliuli is the lower flow per unit diffuser length compared to Sand Island (see Table 2.1).  Minimum dilutions when surfacing range from 549 to 16174.  Again, however, the results imply very frequent surfacing in December, about 49.3% of the time.

## 4.4  Plume Submergence

Because plume submergence is an important issue for the Sand Island outfall, we discuss it in more detail here.  The density stratification over the water column required for submergence depends on effluent flowrate, current speed, and current direction.  In general, higher flowrates require a stronger stratification for submergence.  The required stratification decreases with increasing current speed, being most sensitive when the current is perpendicular to the diffuser, and least sensitive when more nearly parallel (see the photographs in Figure 3.3).  Thus most stratification is required for submergence at high effluent flow and slack water.

The model RSB was run to determine the least stratification required for plume submergence.  At the peak daily flow of 4.0 m3/s, the required density difference over the water column is 0.09 sigma-t units, at the daily average flow of 3.1 m3/s it is 0.07 sigma-t units, and at the minimum daily flow of 2.1 m3/s it is 0.06 sigma-t units.  Thus, a density difference of 0.09 sigma-t units over the water column will maintain submergence at all times.  For constant salinity with depth, this corresponds to a temperature difference of only 0.3 °C!

This very small stratification required for submergence results from the small flow per unit diffuser length, and the deep water (about 70 m at the diffuser site).  Indeed, the diffuser was designed this long to minimize the frequency of surfacing and for high dilutions when the plume does surface (Fischer et al., 1979).

Because of the effects of currents and variable effluent flowrate, the frequency of surfacing is less than the frequency with which the density stratification is less than 0.09 sigma-t units.  For the period discussed in detail in Section 4.2 (8/1/94 to 10/15/94) the frequency distribution of density difference over the water column was given in Figure 3.8.  The density difference is less than 0.05 sigma-t units for about 9.3% of the time and less than 0.10 sigma-t units about 13.7% of the time.  The predicted frequency of surfacing is about 11% of the time, which is quite consistent with the above discussion.  Clearly, only very small density differences are required for submergence.  What causes these differences to disappear and how do they vary seasonally?

Typical temperature records were shown in Figures 3.6 and 3.7.  The surface water has a relatively constant temperature with low variance.  Moving down through the water column, the average temperature decreases and the variance increases substantially.  As Figure 3.7 shows, the water column becomes homogeneous when the warmer surface water extends down to the bottom (the converse, colder bottom water extending to the surface appears to rarely occur).  This may be an internal wave phenomenon, forced by the tides.  The whole record of surface and bottom temperatures at D2 is plotted in Figure 4.3.

Figure 4.3  Surface and bottom thermistor string data at Sand Island.

The surface temperature warms gradually by about 4°C from April to the end of September, and then cools by about the same amount from October to January.  It is relatively constant from January to the end of March.  The variance of the bottom temperature is clearly much higher in Summer and less in Winter when the surface water has cooled to nearer the bottom temperature.  The critical period of November and December is shown in an expanded scale in Figure 4.4.  In December the water is homogenous for extended periods of time contributing to the predicted 62% of surfacing frequency for this month (Table 4.1).

Figure 4.4  Surface and bottom temperature at Sand Island for November and December.

## 4.5  Comparisons With Previous Studies

A previous analysis of the Sand Island outfall near field plume behavior was made by Noda and Associates (1992).  They predicted frequency distributions of plume submergence and dilution.  In that report, the authors used measured daily effluent flowrates for 1990 and an average daily variation of flow to estimate hourly flowrates.  All the available density stratification data were obtained from measured density profiles and separated into months.  The number of profiles varied from 1 in June to 22 in November.  For those months with less than 22 values, additional values were generated by assuming the density differences to follow a Gaussian, or normal, distribution.  Ocean currents were obtained from a University of Hawaii study conducted near the diffuser for two periods, June 6 - September 20, 1989, and March 27 - May 29, 1990.  The authors state the quality of the 1989 data and some of the 1990 data is questionable and so they used 32 days worth of data, from March 27 - April 28, 1990.

The authors used the EPA model UMERGE to predict near field behavior.  This model uses only the current velocity component perpendicular to the diffuser.  The photographs of Figure 3.3 show this to be a very crude assumption for rapid parallel currents.  Because UMERGE predicts flux-average dilution, the authors had to make the arbitrary assumption that minimum dilution is average dilution divided by 2.  The statistics of wastefield rise height, surfacing, and initial dilutions were predicted by making model runs with the statistical distributions of flow, density stratification, and currents rather than measured data directly.  They concluded  that, on an annual basis the plume is predicted to surface 15.4% of the time.  The average dilution of the submerged plume is 662, and average rise height 45.0 m.  Average dilution of the surfacing plume is 1465.

The major differences between the study of Noda and the present one is the quality of the oceanographic and flow data and the use of a more recent and appropriate model.  Nevertheless, Noda’s overall predicted surfacing frequency of 15.5% is comparable to the present 22.4% (which is not based on a full year long data set).  However, the annual estimate of Noda obscures the huge annual variation in surfacing frequency, from 5.1% in September to 62.1% in December.  The average dilutions of the submerged plumes are also similar, 627 in the present study versus 662 for Noda.  The average rise height of submerged plumes is less: 34.0 m for the present study versus 49.0 m for Noda.  Also the present frequency distribution  of rise height shows the plume to be more frequently deeply submerged than does Noda.  The average dilution of the surfacing plume is 1353 versus 1465 for Noda.  Some of this agreement may be fortuitous given the different methods and data used;  and again the overall averages obscure the very wide seasonal variation.

The present method should be more reliable, due especially to the much improved data set, in particular, the use of stratification, currents, and effluent flowrate data.  Here, the stratification was obtained from simultaneous temperature measurements.  If obtained from CTD profiles, the stratification could change substantially over the time required to complete the profile.  The present results should be considered valid for the year of the data collection, however.  Results for other years may differ, and in particular, the frequent surfacing for December could be an anomaly.

## 4.6  Effect of Water Column Velocity Shear

The ADCP’s provide velocity measurements over the whole water column.  In the present case, measurements are averaged over 2m “bins” to provide 32 velocity measurements at D2 near the Sand Island diffuser.  These measurements (see the MB-6 project report) show that considerable shear (velocity variation with depth) can occur at times.  Most plume models assume a uniform current over the plume rise height.  There are no experimental data with which to quantify the effect of shear on plume behavior and no models available for prediction of the general problem of merging multiple plumes from a line multiport diffuser into a density-stratified shear flow.  The model used here, RSB, accounts for all of these effects except shear, i.e. it assumes uniform velocity with depth.  The velocity used in the simulations previously presented is that of the bottom ADCP bin nearest the diffuser as it is assumed that this is the velocity which most affects the plume rise and its subsequent behavior.  The question remains, however: What is the effect of the shear on plume behavior?

To investigate this, we ran the model RSB using data from other ADCP bin depths.  A typical data period from Deployment 4 which ran from 6/5/94 to 1/10/95 was chosen.  The model RSB was run for the Sand Island diffuser using the measured density stratification profiles and ADCP currents at depths of 7m, 27m, 47m, and 65m below the water surface.  The results are summarized in Table 4.3.

It can be seen that average plume properties are only slightly affected.  Frequency of surfacing, average surfacing dilution, average rise height, and average submerged dilution, vary by less than 10%.  More affected are the extremes, the maximum and minimum dilutions.  These are of little consequence, however, due to the infrequency of their occurrence.  The reason for this lack of sensitivity of mean plume behavior to current shear is presumably due to the greater importance of the density profile in determining plume behavior, and the effect of averaging over many simulations.  Although it would appear that velocity shear may have little effect on the wastefield formation, it will certainly affect the thickness of the established waste field and result in accelerated mixing of the waste field in the far field by shear dispersion processes.

Table 4.3  Effect of Depth of Current Meter Bin on Dilution Simulations

| Plume behavior | Property | Depth of current meter bin (m) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 27 | 47 | 65 |
| Surfacing | Frequency, % | 78.6 | 78.2 | 78.4 | 78.7 |
|  | Lowest dilution | 100 | 84 | 95 | 87 |
|  | Average dilution | 544 | 497 | 499 | 531 |
|  | Highest dilution | 2885 | 2580 | 3201 | 3753 |
|  | Average rise height | 29.1 | 29.6 | 29.8 | 29.4 |
| Submerged | Frequency, % | 21.4 | 21.8 | 21.6 | 21.3 |
|  | Lowest dilution | 612 | 603 | 603 | 597 |
|  | Average dilution | 1371 | 1335 | 1320 | 1347 |
|  | Highest dilution | 5174 | 5011 | 3946 | 5401 |

# 5  FAR FIELD MODEL

## 5.1  Introduction

Following the initial dilution phase, the established wastefield drifts with the ocean currents and is diffused by oceanic turbulence (see Figure 3.1).  The rate of dilution in this region, sometimes called the far field, is much slower than in the near field.  As previously discussed, the ocean currents in Mamala Bay are very complex;  they vary rapidly with time and substantially over fairly small distances.  The trajectory of the plume in the far field will therefore follow an irregular path which will vary with release time.  The plume may move up coast or down coast, on shore or off shore.  The plume may visit a location only briefly, and may wander in the vicinity of the diffuser for an extended period before being flushed away by the mean drift.

Approaches to modeling this type of behavior have been discussed by Csanady (1983), Koh (1988), and Roberts (1986, 1990).  These authors, with some variations, analyze the behavior in terms of the frequency of plume impaction at any location, and the maximum and average contaminant concentrations at any location.  Csanady and Roberts term the frequency of immersion of any point in the plume the “visitation frequency,” while Koh refers to “advective transport probability.”  In this section we apply similar analyses to estimate the far field impact of the Sand Island and Honouliuli plumes.

## 5.2  Methodology

Details of visitation frequency computation are given elsewhere (Roberts, 1986, 1989) so we only summarize them here.  The wastefield is discretized as a series of puffs released at 30-min. intervals (the current meter and thermistor sampling time).  In each time step, each puff is advected by the local current to a location:

x→=x→0+u→(x→,t)Dt	(5.1)

where x→0  is 0the initial location of the puff center, u→(x→,t)0 the local velocity at position x→ and time t, and t is the time interval.  The location after a travel time T for a puff released at time t0 is given by:

x→(t0,T)=xd→+t0t0+Tu→(x→,t)dt	(5.2)

where xd is the diffuser location.  In the present computations the local current u→(x→,t) 0is obtained by horizontal spatial interpolation of the current meters.  The method of interpolation is a weighting scheme in which the contribution of any meter is proportional to the inverse of its distance squared.  No other assumptions about the currents were made.  A puff is released at the beginning of the record and the puff location computed at each time step.  A grid is superimposed over the area, and when a puff overlays a grid node, it is counted as a “visit.”  This was repeated for thousands of simulations of releases beginning at successive times throughout the record and each puff was followed up to the maximum time horizon, T.  The number of visits by a puff of age younger than the maximum time horizon was summed and divided by the total number of releases.  In the present computation, the puff size was assumed to be constant and equal to the diffuser length, as Roberts et al. (1989) have shown that the initial waste field width is of the order of the diffuser length regardless of the current direction.  Noda & Associates (1992) present similar computations in which the probability of travel of the plume centerline into a grid square is computed.  The two methods are essentially identical.

The advantage of using the recorded current meter data directly is that all of the current complexities are automatically included.  The disadvantage is that we are spatially extrapolating Lagrangian currents from fixed Eulerian measurements. The reliability of such a scheme depends on the quality and spatial coverage of the current meter data.  Clearly, the reliability of the results decreases with distance from the area covered by the meters.  Furthermore, even with regularly varying tidal flows, Zimmerman (1986) has pointed out that irregular topography can produce “lagrangian chaos,” that is, unpredictable plume trajectories.  For these reasons individual trajectories should not be inferred, and only statistical inferences of the pattern and scale of the dispersion should be made.

As the plume moves away from the diffuser it is diffused by oceanic turbulence.  It is commonly assumed that this process follows the “4/3 power law” in which the decay of peak concentration for a continuous line source in a steady current is given by Brooks’ (1960) solution to the diffusion equation.  This solution can be expressed in terms of travel time from the source as:

Sf = c0cm = erf321+8Î0tL23−112−1	(5.3)

where Sf is the far-field dilution, co the effluent concentration at the completion of near field mixing, and cm the centerline concentration after a travel time t.  The initial value of the horizontal diffusion coefficient is  Î0=αL43 0where  is a constant and L the initial puff size.  We assume that Eq. 5.3 also applies in the present unsteady case.

Eq. 5.3 is the solution for a continuous line source whose concentration is reduced by lateral diffusion.  For an isolated puff growing in two-dimensions the diffusion rate will be greater and the plume dilution will increase away from the puff center.  For the present, we will neglect these effects and use the conservative assumption that the dilution in the puff is given by the continuous solution and is constant across the puff.  The puff size w is allowed to grow by diffusion, however, and is given by:

w = L1+12syL2	(5.4)

where sy is the standard deviation of the puff size:

sy = L121+8e0tL23−1	(5.5)

The actual dilution at any location is the product of the initial dilution Sm and the far-field dilution:

(5.6)

where Sm is assumed to be the minimum dilution predicted by the near-field model RSB.  The maximum tracer concentration at any distant location is given by:

(5.7)

where c00 is the tracer concentration in the effluent leaving the treatment plant.

The concentration estimated by Eq. 5.7 is the maximum which might be expected at any location.  It is useful for estimating acute effects of the discharge.  It may occur only infrequently, however, and time-average concentrations will be much lower.  The time-average concentration is given by:

c = c00Sh	(5.8)

where Sh  is the harmonic average dilution given by:

Sh = 11nåni=11Si	(5.9)

where Si  is the dilution at any location at time step i.  The significance of the harmonic average is that it gives an estimate of time-average contaminant concentrations by Eq. 5.8 which are relevant to chronic effects of the discharge.  The harmonic average is therefore more useful than a simple time-average of dilutions.  For the present Sh is approximated by:

Shx→ = Sminx→vx→	(5.10)

where Sminx→ 0and vx→ 0are the lowest value of the dilution computed by Eq. 5.5 and the visitation frequency at location x→0.  Eq. 5.10 assumes that the dilution in the plume at any location when it is present is always the minimum value ever experienced there.  It will usually be higher than this, however, so Eq. 5.10 is a conservative estimate of the actual harmonic average dilution at any location.

The only remaining parameter of choice is the coefficient    in the equation   =  L4/3 .  According to Fischer et al. (1979) the results of oceanic diffusion experiments are bracketed by   0.01 <   < 0.006 cm2/3/s .  Here we choose an “average” value   = 0.012 cm2/3/s.   Runs were made to test the sensitivity of the results to changes in .  Increasing  results in an increase in minimum dilutions at any location, but a decrease in harmonic average dilutions.  The latter is caused by the increased probability of diffusive transport to any location.

Finally, estimates were made of the effect of winds on wastefield transport.  Hourly wind data measured at Honolulu International Airport were obtained and interpolated to calculate values at each half-hour to agree with the present time step.  The surface current was then assumed to be the sum of that computed by the interpolated current meters plus a wind component whose magnitude is assumed to be 3% of the wind speed.  The wind record used is shown in Appendix C as a polar scatter diagram and time series of speed and direction.

## 5.3  Results

Simulations of surfacing plumes using the above methods were run for the Sand Island and Honouliuli outfalls.  Surfacing plumes were emphasized because submerged plumes are likely to remain submerged and are unlikely to impact the shoreline.  The far-field model was coupled to the near-field model, RSB.  If the plume surfaced, it was advected and diffused as discussed previously.  Two periods were run for Sand Island:  a summer period 6/5/94 to 8/31/94 and an early winter period 10/15/94 to 1/10/95.  For the summer period the plume was surfacing about 11% of the time and for winter about 45% of the time.  The currents were assumed to be those recorded nearest the surface.  For Summer, these were  D2 (Sand Island Diffuser ADCP), D1 (an Interocean S4 Electromagnetic current meter further inshore), and B1 (Honouliuli ADCP) (See Figures 2.1 and 2.2).  These were also used for winter with the addition of an ADCP at C0 offshore of the Ala Wai Canal.  For Honouliuli, the meters at B1, B2, D1, and D2 were used. The Honouliuli simulation was from 11/19/94 to 1/11/95.  Only transport of up to 12 and 24 hours was considered in order to cover the most important range of tidal currents.  Results for the Sand Island plume for winter conditions are shown in Figures 5.1 to 5.4.  The results for the Sand Island plume in the summer and for the Honouliuli outfall are given in Appendix D.

Figure 5.1 shows visitation frequencies without winds for 12- and 24-hour time horizons for the surfacing Sand Island plume.  It can be seen that visitation frequency drops off rapidly with distance from the diffuser to the outer contour.  This outer contour is zero, in other words no advective transport is expected beyond this contour for the maximum travel times given.  These outer contours do not intersect the Waikiki beaches implying that the probability of impaction there is small.  The tidal excursion is too small to bring the wastefield to shore.  Even for Sand Island and other beaches closer to the diffuser impaction is likely to be very small.  Figure 5.2, showing minimum and harmonic average dilutions, indicate that, if impaction does occur, minimum dilutions will be high, on the order of 2500.  It should also be noted that this method overestimates shoreward transport as the currents do not in reality continue to move onshore but eventually must turn in a longshore direction.  Harmonic average dilutions increase very rapidly away from the diffuser, due mainly to the decrease in visitation frequency.  They approach 106 within about 10 km.  Thus, there would be little measurable effect of the discharges expected beyond a few kilometers away from the diffusers.

Even these small effects are diminished further by the wind, Figures 5.3 and 5.4.  The predominant winds are the Trade Winds which blow consistently to the Southwest from around April to December (Appendix C).  From December to April they are more variable with some onshore (Kona) winds.  The effect of these winds is to blow the visitation frequency contours offshore.  Occasional onshore excursions may occur, but dilutions will be very high.  (The dilution contours extend further than the visitation frequency contours due to diffusion.)  The oceanographic studies suggest little correlation between the winds and currents and imply that wind is not an important driving mechanism.  Actual transport is therefore more likely to be closer to that computed without wind effects.  In either case beach impacts are likely to be very small.

The results for Summer (Appendix D) are similar.  The Honouliuli simulations suggest that shoreline impacts from this outfall are also extremely small due to the coastline shape and the predilection of currents to run parallel to shore with only a small onshore component.

For the reasons discussed previously, this analysis should be regarded as only and approximation. Nevertheless, it is clear that shoreline impactions of the outfall discharges will be extremely infrequent and if they occur, dilutions will be high.  The combination of initial dilution, far field dilution, and mortality will likely reduce bacteria to very low levels.

## 5.4  Comparisons with Previous Studies

As discussed in Section 4.5, analyses of the Sand Island plume were previously reported by Noda & Associates (1992).  Using more limited oceanographic and meteorological data, they made similar analyses to those presented here of near field plume behavior, advective transport probability, minimum representative dilutions, and average representative dilutions.  Currents recorded at a single location and depth (near the diffuser) were used, and the far- and near-field models were not directly coupled with simultaneous measurements as in the present study.  The conclusions from Noda’s study was that “....impacts of the plume on the near shore environment are considered to be very minimal and generally insignificant.”  The minimum and average dilution estimates are comparable to those obtained here.  Thus, despite the differences in methodology and data, the findings of the present study confirm those of Noda & Associates that shoreline impacts are rare, and if they occur, plume dilutions are very high.

Figure 5.1  Visitation frequencies: 12 hour (top) and 24 hour (bottom) for surfacing Sand Island plume.  Winter, no winds.

Figure 5.2  Estimates of dilution.  Minimum (top) and harmonic average (bottom) for surfacing Sand Island plume.  Winter, no winds.

Figure 5.3  Visitation frequencies: 12 hour (top) and 24 hour (bottom) for surfacing Sand Island plume.  Winter, with winds.

Figure 5.4  Estimates of dilution.  Minimum (top) and harmonic average (bottom) for surfacing Sand Island plume.  Winter, with winds.

# 6  CONCLUSIONS

The near- and far-field behavior of the plumes from the Sand Island and Honouliuli outfalls discharging into Mamala Bay was modeled mathematically.  The models directly used oceanographic data measured by current meters and thermistor strings located in the vicinity of the two diffusers.  More than 20,000 simulations of near field predictions including rise height, dilution, and other plume characteristics were run.  Far field predictions provided estimates of the visitation frequency, minimum dilution and harmonic average dilution in the vicinity of the diffusers.

The oceanographic conditions are very variable and quite complex near the diffuser sites.  Principal current axes are approximately parallel to the local bathymetry.  The currents have a strong semi-diurnal tidal component but superimposed on this is a variable low frequency drift which can also be quite strong.  Peak currents are typically around 50 cm/s near the Sand Island diffuser and 30 cm/s near the Honouliuli.  The diffusers are aligned with the local bathymetry so currents frequently flow parallel to them.  The mathematical model used is appropriate for this condition.  The oceanic temperature structure is also quite complex.  The surface temperature shows little variance and changes slowly with the season.  It gradually warms by about 4°C from April to the end of September then cools by about the same amount by January.  From January to April there is little change.  The average temperature decreases with depth but the variance of the temperature increases markedly with depth.  This causes the density stratification profile to be very variable ranging from strongly stratified to homogeneous over a few hours.  Homogeneity occurs when the warmer surface water extends down to the diffuser.  These oscillations may be internal waves forced by the tide.  The frequency with which the water column is close to homogeneous is much higher in winter than in summer.

The near field modeling used all of the available simultaneous current and stratification data at the two outfall sites.  The modeling period extended from June 1994 to April 1995.  Data from the thermistor strings were converted to density profiles and used as input to the model together with current meter data and measured flows from the treatment plants.  Long time series of predicted wastefield behavior were generated from which frequency distributions of wastefield characteristics were obtained.

The oceanographic variability resulted in extreme variability in wastefield behavior.  Over the whole modeling period, the frequency of surfacing of the Sand Island plume was predicted to be 22.4%.  When submerged, the average plume rise height was 34.0 m, and average dilution 627.  The lowest submerged dilution in all the simulations was 87, and the highest 3753.  The low dilution occurs when the plume is deeply submerged.  The lowest dilution of a surfacing plume was 597, average 1353, and highest 5401.  The overall averages obscure the extreme seasonal variability predicted.  The frequency of surfacing was predicted to vary from 5.1% in September to 62.1% in December.  In the summer months the plume is usually very deeply submerged.  Results for the plume discharged from the Honouliuli outfall are generally similar but with increased submergence and higher dilutions due to the lower flow per unit diffuser length.  The results are discussed in detail for the period 1 August 1994 to 15 October 1994, and summarized by month in Tables 4.1 and 4.2.  Appendices A and B contain graphical summaries of all the data and simulation results.

Because of the importance of surfacing of the plume from Sand Island outfall it was analyzed further.  It was found that a density difference over the water column of only 0.09 sigma-t units will essentially always submerge the plume.  This corresponds to a temperature difference of only about 0.3°C.  Apparently, temperature differences are often less than this, however, leading to plume surfacing.

The far field behavior of surfacing plumes was also modeled to estimate the potential for shoreline impact.  Using measured current data, predictions were made of visitation frequency, minimum expected dilutions, and harmonic average dilutions in the vicinity of the diffusers.  It was found that current motions of tidal periods are too small to bring the plume to shore.  Longer period motions may occasionally cause shoreline impactions, but they will be very infrequent, and, if they occur, dilutions will be very high.  The harmonic average dilution increases rapidly to very high levels so that little effect of the discharges would be expected beyond about 5 km from the diffusers.  Because the Trade winds blow consistently to the Southwest for much of the year, they are likely to further reduce shoreline impact.  Onshore (Kona) winds may cause occasional shoreline impacts, but these also will be infrequent and highly dilute.

# 7  REFERENCES

Baumgartner, D.J., Frick, W.E., and Roberts, P.J.W. (1994). "Dilution Models for Effluent Discharges (Third Edition)." U.S. Environmental Protection Agency, Office of Research and Development, Washington, DC.   EPA/600/R-94/086

Bigg, P.H. (1967). "." British Journal of Applied Physics, 8, 521-537.

Csanady, G.T. (1983). "Dispersal by Randomly Varying Currents." Journal of Fluid Mechanics, 132, 375-394.

Fischer, H.B., List, E.J., Koh, R.C.Y., Imberger, J., and Brooks, N.H. (1979): Mixing in Inland and Coastal Waters. Academic Press, New York.

Frick, W. (1995).  Private Communication,  U.S. Environmental Protection Agency, Newport, Oregon.

Jirka, G.H., and Akar, P.J. (1991). "Hydrodynamic Classification of Submerged Multiport Diffuser Discharges." Journal of Hydraulic Engineering, ASCE, 117(9), 1113-1128.

Koh, R.C.Y. (1977). "Analysis of Multiple Times Series by Principal Components with Application to Ocean Currents off San Francisco." W.M. Keck Lab. Tech Memo 77-4

Koh, R.C.Y. (1988). "Shoreline impact from ocean waste discharges." Journal of Hydraulic Engineering, ASCE, 114, 361-376.

Millero (1980). "." Deep-Sea Research, 27A, 255-264.

Millero, and Poisson (1981). "." Deep-Sea Research, 28A, 625-629.

Muellenhoff, W.P., Soldate, A.M., Baumgartner, D.J., Schuldt, M.D., Davis, L.R., and Frick, W.E. (1985). "Initial Mixing Characteristics of Municipal Ocean Discharges." U.S. Environmental Protection Agency, Report No. EPA-600/3-85-073a

Noda, E.K. (1992).  “Near-Field and Far-Field Plume Analysis Sand Island Outfall System,” Edward K. Noda and Associates, Inc., Honolulu, Hawaii.

Roberts, P.J.W. (1986). "Engineering of Ocean Outfalls." The Role of the Oceans as a Waste Disposal Option, G. Kullenberg, ed., NATO ASI Series C, 172, pp. 73-109

Roberts, P.J.W., Snyder, W.H., and Baumgartner, D.J. (1989c). "Ocean Outfalls. III: Effect of Diffuser Design on Submerged Wastefield." Journal of Hydraulic Engineering, ASCE, 115(1), 49-70.

Roberts, P.J.W., Snyder, W.H., and Baumgartner, D.J. (1989a). "Ocean Outfalls. I: Submerged Wastefield Formation." Journal of Hydraulic Engineering, ASCE, 115(1), 1-25.

Roberts, P.J.W., Snyder, W.H., and Baumgartner, D.J. (1989b). "Ocean Outfalls. II: Spatial Evolution of Submerged Wastefield." Journal of Hydraulic Engineering, ASCE, 115(1), 26-48.

Roberts, P.J.W. (1990): Outfall Design Considerations. In: The Sea, Ocean Engineering Science. Vol. 9. (eds.: Le Mehaute, B., and Hanes, D.M.) Wiley Interscience, New York, .

Roberts, P.J.W., and Wilson, D. (1990). "Field and Model Studies of Ocean Outfalls." National .Conference on Hydraulic Engineering, San Diego, California, July 30-August 3, 1990

Roberts, P.J.W. (1991). "Ocean Outfalls." Critical Reviews in Environmental Control, 20(5&6), 311-339.

Roberts, P.J.W., and Snyder, W.H. (1993). "Hydraulic Model Study for the Boston Outfall. I:  Riser Configuration." Journal of Hydraulic Engineering, ASCE, 119(9), 970-987.

Roberts, P.J.W., and Snyder, W.H. (1993). "Hydraulic Model Study for the Boston Outfall. II:  Environmental Performance." Journal of Hydraulic Engineering, ASCE, 119(9), 988-1002.

Washburn, L., Jones, B.H., Bratkovich, A., Dickey, T.D., and Chen, M.-S. (1992). "Mixing, Dispersion, and Resuspension in Vicinity of Ocean Wastewater Plume." Journal of Hydraulic Engineering, ASCE, 118(1), 38-58.

Zimmerman, J.T.F. (1986). "The tidal whirlpool:  a review of horizontal dispersion by tidal and residual currents." Netherlands Journal of Sea Research, 20, 133-156.

# 8.  APPENDICES

These appendices contain summaries of all the near field simulations reported in Chapter 4.  The summaries are presented as plots, each showing two months of simulation results and data used.  Input data includes currents, density stratification, and effluent flowrates.  The top two plots in the near field simulation summaries show the measured and low-pass filtered first and second principal current components.  The third plot shows near-surface and near-bottom temperatures recorded by the thermistor strings.  Effluent flowrate is shown in the fourth plot.  Results are shown in the bottom two plots as plume rise height to the top of the wastefield, ze, and minimum dilution, Sm.  The actual output data files, which contain more information, are available to those interested.  Note that all dates and times are Greenwich Mean Time;  Honolulu local time is 10 hours earlier.

## APPENDIX A.
Near Field Simulation Summary Plots:  Sand Island

Table A1  Summary of current conditions, D2 at bottom

| Period | First principal component |  |  |  |  | Second principal component |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Direction1 | Mean speed | Standard deviation | Direction1 | Mean speed | Standard deviation |
|  | (Deg) | (cm/s) | (cm/s) | (Deg) | (cm/s) | (cm/s) |
| 6/5/94 - 1/19/95 | 266.8 | 5.0 | 14.5 | 176.8 | 0.4 | 7.1 |
| 1/20/95 - 4/18/95 | 264.2 | 2.2 | 14.4 | 174.2 | -0.7 | 6.4 |

1 Clockwise to true North

Near field simulation summary: Sand Island

Near field simulation summary: Sand Island

Near field simulation summary: Sand Island

Near field simulation summary: Sand Island

Near field simulation summary: Sand Island

Near field simulation summary: Sand Island

## APPENDIX B.
Near Field Simulation Summary Plots: Honouliuli

Table B1  Summary of current conditions, B2 at bottom

| Period | First principal component |  |  |  |  | Second principal component |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Direction1 | Mean speed | Standard deviation | Direction1 | Mean speed | Standard deviation |
|  | (Deg) | (cm/s) | (cm/s) | (Deg) | (cm/s) | (cm/s) |
| 6/14/94 - 7/24/94 | 227.2 | 1.8 | 7.4 | 137.2 | 0.3 | 6.5 |
| 7/29/94 - 10/17/94 | 234.0 | 2.4 | 7.4 | 144.0 | 0.2 | 6.3 |
| 11/19/94 - 1/17/95 | 237.6 | 3.4 | 8.9 | 147.6 | 0.0 | 6.1 |
| 1/20/95 - 4/17/95 | 239.3 | 1.0 | 7.2 | 149.3 | 0.4 | 6.0 |

1 Clockwise to true North

Near field simulation summary: Honouliuli

Near field simulation summary: Honouliuli

Near field simulation summary: Honouliuli

Near field simulation summary: Honouliuli

Near field simulation summary: Honouliuli

Near field simulation summary: Honouliuli

## APPENDIX C
Wind record at Honolulu Airport

## APPENDIX D 
Far Field Simulation Summary

Visitation frequencies: 12 hour (top) and 24 hour (bottom)
for surfacing Sand Island plume. Summer, no winds.

Estimates of dilution. Minimum (top) and harmonic average (bottom)
for surfacing Sand Island plume. Summer, no winds.

Visitation frequencies: 12 hour (top) and 24 hour (bottom)
for surfacing Sand Island plume. Summer, with winds.

Estimates of dilution. Minimum (top) and harmonic average (bottom)
for surfacing Sand Island plume. Summer, with winds.

Visitation frequencies: 12 hour (top) and 24 hour (bottom)
for surfacing Honouliuli plume. Winter, no winds.

Estimates of dilution. Minimum (top) and harmonic average (bottom)
for surfacing Honouliuli plume. Winter, no winds.

Visitation frequencies: 12 hour (top) and 24 hour (bottom)
for surfacing Honouliuli plume. Winter, with winds.

Estimates of dilution. Minimum (top) and harmonic average (bottom)
for surfacing Honouliuli plume. Winter, with winds.