3.0  Task Methodology

3.1  Introduction

Measurements in MB-6 focused primarily on documenting the dynamic pressure, velocity and density fields in the ocean waters of the study area.  This is in keeping with  the goal of both deducing key circulation patterns and forces that create them, and providing appropriate information for use by the several numerical modeling tasks in the Mamala Bay Program.

3.2  Field Measurements

3.2.1  In-Situ Observations

Instruments to measure local currents were deployed at 11 Mamala Bay sites situated along the four transects (A, B, D, E) shown in Figure 2.1.  With the nomenclature used in this study, a measurement site was defined by a letter indicating the transect and a number indicating the approximate water depth (Figure 3.1).  In this scheme, sites with numerical identifiers of "1" (e.g.  B1) were in approximately 17m of water, with "2" in approximately 75m of water, "3" in 250m, and "4" in 500m.

On all moorings, including those in water depths greater than 100m,  measurements were made at depths that would resolve adequately the expected vertical changes of the current velocity and temperature profiles(See schematics in Figure 3.2).  With this arrangement, more closely spaced measurements were made in those regions where vertical velocity/temperature gradients might be expected to be greatest.  As shown in Figure 3.1, at sites A2, B2, D2 and E2, located on approximately the 75m isobath, all current measurements were made with bottom-mounted Acoustic Doppler Current Profilers (ADCPs) which provided closely spaced (every two meters) estimates of the horizontal velocity as well as an estimate of bottom water temperature.  At D3 during most deployment periods, an ADCP was placed on a taut mooring near the bottom to provide velocity estimates every eight meters in the vertical.  At a very shallow site (C0), just at the mouth of the Ala Wai Canal, an ADCP was used to estimate an average velocity every half meter in the vertical.  At the beginning of the program, current profiles measured by the ADCPs on the 75m isobath were transmitted via cellular phone from a surface buoy at the site to a computer onshore where they were stored and available for review.  These data were also stored internally at each ADCP.  The rationale for quasi-real time access to the data was to verify instrument function and to have current data available to support field studies, such as the plume tracking project, which were dependent on knowing where the diffuser plume was and might be expected to go.  As the program progressed, a sequence of accidents befell these buoys.  A decision was made not to maintain them at each of the four stations,

Figure 3.1	Locations and types of observations made in the Mamala Bay study area

Figure 3.2	Schematics of the various instrument placements and configurations.

rather, an effort was made to maintain an operational telemetry buoy  at D2 (Sand Island diffuser) which would be the focus of the plume studies.

Other variables were measured at locations in the study area and around Oahu.  At B2 and D2, located close to the Honouliuli and Sand Island diffusers respectively, detailed temperature measurements were made every five meters between approximately 10 and 60m above the local bottom.  These observations were supplemented by bottom temperature time series from the adjacent ADCPs.  At A2 and E2, located on the west and east boundaries of the study area, SeaBird Model 26 wave tide gauges provided accurate records of changes in water level and temperature.  These instruments were supplemented by water level/temperature measurements near Pearl Harbor (Hickum Field) and Haleiwa Harbor on the north shore of Oahu.  Supplemental observations made by NOAA at Honolulu Harbor and Kaneohe Bay on the windward side of Oahu were obtained and used, as needed, to describe the pattern of tidal water level fluctuations around Oahu and in the study area(Figure 2.1).  Observations at B1 and D1, just shoreward of B2 and D2, were made with single InterOcean S4 vector measuring current meters placed at approximately mid-depth (8m) in the water column and also recorded temperature and conductivity.  Primary wind data used during the study was measured at the National Weather Service coastal stations near Barbers Point (BBP), the Honolulu Airport (HNL)  and Kaneohe Bay.  These records were continuous throughout the year of oceanographic measurements .  A summary of observations, instrumentation by location is shown in Table 3.1.

Intervals for which usable data were taken are shown by the time lines in Figure 3.3.  Also shown in this figure are the types and depths of instruments used to make the  physical oceanographic field measurements that began in January 1994 and are expected to continue through July 1995.  The absence of current observations along the 75m isobath and at the Ala Wai Canal during the first 4 to 5 months of measurements resulted from a manufacture's firmware problem which caused the observations which were taken to be in error and unrecoverable.  Resolving this problem and verifying the accuracy of subsequent data required considerable addition field operations and deployed instruments (e.g. mooring B9).  Brief data gaps occurring on three-month intervals were associated with time required for instrument maintenance and refurbishment.  Schedules were selected to reduce the time instruments were "out of the water" while efficiently using personnel and vessels.

In April 1995, mooring B1 was moved to a location just seaward of C0 to help resolve details of the circulation near the mouth of the Ala Wai Canal.  Using existing mooring hardware and equipment, an additional mooring (F3) was placed in water depth of approximately 320m at a locations approximately half way between transects A and B.  The resulting current data should help resolve where the strong, low frequency currents seen on

Table 3.1  Landscape table showing mooring locations, instruments, and variables.

Figure 3.3  Time line showing when, where and what kind of instruments were making time series measurements during this year of observations.

transect A actually go.  An additional water level gauge was placed on the leeward side of the island in the Waianae Harbor to better resolve the movement and phase of tidal and other waves which appear to move around the island.  These additions and adjustments in mooring/instrument placement resulted from unanswered or poorly resolved questions that arose during data analysis and synthesis.

The instrument placement described above provided the regional hydrodynamic model with detailed current and temperature data along the boundary of the study area and water levels at the two end transects.  At sites interior to the study area, independent observations of currents, temperature and water levels were available to verify model results.  At the two diffusers, the near field (initial mixing) model had available vertically detailed information on the temperature and velocity structure, two of the primary factors affecting the vertical rise, mixing and transport of material coming from the Honouliuli and Sand Island diffusers.

All instruments used during this study were maintained with manufacturer recommended procedures conducted by experienced ocean scientists and engineers.  The types of instruments for this study had been used and maintained previously by the task field personnel.

3.2.2  Surveys

Quarterly hydrographic surveys were made over the study area.  Typically these were conducted soon after a quarterly mooring rotation cruise.  The location of stations where vertical profiles of temperature, salinity and hence density were measured on these cruises is shown in Figure 3.4.  Table 3.2 shows the nominal dates of these cruises.

3.3  Data Processing

All oceanographic data were processed using fully tested and verified procedures and algorithms which were developed and implemented by SAIC during 18 years of conducting ocean research programs.  A key step in all processing were thorough quality control procedures which  result in an assurance that all data used for this study has been thoroughly examined using specialized software and evaluated by several oceanographers prior to being included in the task and program data base.  The comprehensive and proven nature of these procedures provides assurance of the quality of data used to develop an understanding of circulation patterns in Mamala Bay.  After QA, all data were entered in SAIC's physical oceanographic data management and analysis system for further processing.  This data base system and interactively linked analysis and graphics routines form the basis for all subsequent ocean data analysis and presentation.

Because currents tend to flow in the direction of the general trend of the bottom contours (along isobath), the current velocity data used in these analyses was rotated so that

Figure 3.4	Map showing the nominal location of stations at which hydrographic profiles were measured during the cruises shown in Table 3.1.

|  | Dates |  |  |
| --- | --- | --- | --- |
| Survey Number | Begin | End | No. of Stations |
| MB9401 | 02/11/94 | 02/12/94 | 36 |
| MB9402 | 05/07/94 | 05/08/94 | 36 |
| MB9403 | 08/14/94 | 08/15/94 | 36 |
| MB9501 | 01/24/95 | 01/25/95 | 37 |

Table 3.2	Dates of the various hydrographic cruises taken during this field measurement program

the v-component of velocity was directed along isobath and the u-component of velocity was directed across or normal to the isobaths.  Because the isobaths in this study area are curved, the coordinate frame used (the amount the coordinate frame is rotated relative to north) may differ from one mooring to another.  Where appropriate, the magnitude of this rotation is usually indicated on the appropriate figures.  This angle is indicated in the following form:  "R85" which indicates that the coordinate frame was rotated 85° clockwise from north so the v-component is directed along isobath.

Routines for computer analysis of project data have been used during many prior studies and are fully verified.  Those responsible for data QA and analytical processing have considerable prior experience with measurements resulting from all the instruments used in this study.  Hence, they have experience and software to handle all data formats as well as any special processing requirements which may be appropriate.  Display and graphics routines exist for presentation of all data types and analyses used in this study.

Following data QA, basic processing of time series, such as components of current velocity,  involved all or some of the following processing:  low pass filtering to suppress higher frequency fluctuations.  3 hour low pass (HLP) filters suppress fluctuations having periods of 3 hours or less.  40-HLP filtering suppresses fluctuations with periods less than approximately 40 hours, hence semidiurnal and diurnal tidal oscillations would be eliminated from the time series.  Spectral analysis divides the variance of a time series according to the frequency at which the variance occurs.  Hence, spectral analysis helps resolve the relative contribution which different periodic fluctuations contribute to the measurement record.  Coherence and phase analysis identifies how well correlated fluctuations at a given period in one time series are with fluctuations at that same period in another time series.  If this correlation is statistically significant, then phase information quantifies what time lag may exist at a specific periodic fluctuation between two records.  A comprehensive statistical analysis was conducted which identifies for each time series the maximum and minimum values, the mean, the 3 and 40-HLP variance, and the principal axis.  This latter is a quantitative method of determining a preferred orientation or direction for vector data.  In addition to the above and as described in subsequent sections, several special analytical methods (e.g. tidal analyses) were used to help isolate and resolve the circulation patterns and processes in Mamala Bay.

Hydrographic survey data also underwent a detailed QA procedure during which any calibration adjustments were applied.  Once verified, these data were entered in the hydrographic portion of the data management system.  Using temperature and salinity, density profiles were computed and stored.  Using the profiles taken over the grid shown in Figure 3.4, alongshore and shore normal vertical sections were contoured and plotted to display clearly the distribution of the variable as a function of depth and along or across shelf location.  Similarly, values of any measured or computed variable at the stations were contoured horizontally.  Contoured variables could be such things as the depth of a given isotherm over the station grid, or the distribution of temperature at a given depth.  Using the computed density field during a hydrographic survey, circulation and transport produced by that density field were computed.  Techniques for this latter were tailored to the special requirements of making these computations over a sloping bottom and in relatively shallow water.

Fully verified data from moorings and hydrographic surveys were supplied to the Principal Investigators of MB-4 and MB-5 within two to three weeks of each instrument servicing cruise or survey.  The modeling studies were the principal users of these data.  Copies of the data were also provided to MB-2, the program data manager.