MAMALA BAY STUDY

CHARACTERIZATION OF THE MICROBIOLOGICAL

QUALITY OF WATER IN MAMALA BAY

PROJECT MB-7

Principal Investigator:

Roger S. Fujioka

Water Resources Research Center

2540 Dole St., Holmes Hall 283

Honolulu, Hawaii 96822

Co-Principal Investigator:

Philip C. Loh

Department of Microbiology

2538 The Mall, Snyder Hall 319

Honolulu, Hawaii 96822

May 31, 1995

# 1  EXECUTIVE  SUMMARY

The MB-7 Indicator Team was charged with hosting the other MB-7 teams to meet the MB-7 goal of determining the microbial quality of waters in Mamala Bay and the contributions of point versus non-point sources of fecal indicators and fecal pathogens.  The specific task for the MB-7 Indicator Team was to analyze selected water and sediment samples from Mamala Bay for concentrations of standard fecal indicators (fecal coliform, E. coli, enterococci) and alternative fecal indicators (C. perfringens, FRNA phage).  The results of this study documented that the two major point sources of pollution for Mamala Bay (sewage from Sand Island WWTP and Honouliuli WWTP) are discharging high concentrations of all fecal indicators into Mamala Bay.  However, the monitoring evidence indicates that the primary treated and non-disinfected sewage from these two ocean outfalls are rapidly diluted by ocean water and currents predominately transport the sewage away from the land. Thus, the fecal indicator concentrations at the shoreline sites were not measurably increased as a result of these two ocean outfalls. Sewage from the third ocean outfall undergoes secondary treatment plus chlorination before it is discharged at the mouth of Pearl Harbor channel.  The contribution of fecal indicators by this outfall appears to have a minimal effect on the quality of water in Mamala Bay.

The Ala Wai Canal was the most obvious non-point source of pollution entering Mamala Bay.  The major source of fecal indicators in the Ala Wai Canal were three streams which discharged directly into the Ala Wai Canal. These stream waters contained high concentrations of fecal indicators and tended to float on the surface of the marine waters in Ala Wai Canal.  The Ala Wai Canal was a constant non-point source of pollution entering Mamala Bay.  Keehi Lagoon was another non-point source of pollution contributing indicator bacteria into Mamala Bay.  On the other hand, the contribution of fecal indicators into Mamala Bay by Pearl Harbor and Honolulu Harbor was barely measurable on dry days.  However, during heavy rains, the concentrations of fecal indicators from all in-land water sources increased dramatically and impacted the quality of water in Mamala Bay.

Most of the swimming beaches in Mamala Bay maintained good water quality and met Hawaii’s strict marine recreational standard of 7 enterococci/100 ml.  Although swimmers were shown to shed fecal indicators when they entered marine waters, the concentrations of fecal indicators at swimming beaches did not show an obvious increase when the numbers of swimmers at the beach increased.  Thus, direct evidence that swimmers contribute enough fecal indicators in excess of the recreational water quality standards was not obtained.  The sediments at beaches contained minimal levels of fecal indicators.  In contrast, sediments from sources where the water consistently contained high levels of fecal indicators (streams, canals, storm drains, harbors, zone of mixing at ocean outfall site) also contained high concentrations of fecal indicators.  The concentrations of fecal indicators in sediments reflected their stability: C. perfringens>enterococci> fecal coliform> E. coli.

The fecal indicators (fecal coliform, E. coli, enterococci) used by USEPA to establish recreational water quality standards are not reliable indicators of fecal contamination because these same bacteria are present in high concentrations in many land-based sources (soil, streams, storm drain, puddles, run-off from road and roofs). Moreover, they are less stable in marine waters than poliovirus.  The results of this study substantiated early findings that sunlight is the predominant natural agent controlling the survival of fecal indicators in marine waters.  However, sunlight-inactivated (loss of culture) fecal indicators still retained their structural integrity and ability to respire.  Fecal bacteria attached to sediments were very resistant to sunlight inactivation.  C. perfringens and FRNA were determined to be more stable in marine waters than poliovirus and therefore may be better indicators for the presence and absence of pathogens.  C. perfringens may be the best indicator of water quality because it is not found in high concentrations in soil and because it survives longer in marine waters than pathogens such as poliovirus.  Moreover, the method for the detection of C. perfringens is simple, reliable and amenable to processing large volumes of water.

# TABLE OF CONTENTS

- 1  EXECUTIVE  SUMMARY

TABLE OF CONTENTS

LIST OF FIGURES

LIST OF TABLES

2  INTRODUCTION

2.1  Scope of Work

2.2  Study Objectives

2.3  Project Organization and Project Reports

3  METHODS

3.1  Task Summary

3.2  Task Methodology

3.3  Collection of  Samples and Sampling Sites

3.4  Study Approach and Design

4  RESULTS

4.1  Establishment and Assessment of Control Stations in Mamala Bay

4.2  Impact of Discharges of Sand Island Sewage and Honolulu Harbor on Water Quality in Mamala Bay

4.3  Impact of Discharges of Honouliuli Sewage on Water Quality in Mamala Bay

4.4  Impact of Pearl Harbor Discharge on Water Quality in Mamala Bay

4.5  Impact of Ala Wai Canal on Quality of Water in Mamala Bay

4.5.1  Sources of fecal indicators in Ala Wai Canal

4.5.2  Concentrations of fecal indicators in the Ala Wai Canal

4.5.3  Pollution loading into Mamala Bay

4.5.4  Other sources of fecal indicators polluting Mamala Bay

4.6  Fecal Indicators at Popular Recreational  Beaches

4.7  People as Sources of Fecal Indicators

4.8  Impact of Wet Weather on Inland Water Sites

4.9 Concentrations of Fecal Indicators in Sediments

4.9.1  Fecal indicators in sediments from recreational beaches

4.9.2  Fecal indicators in sediments from land run-off sites

4.9.3  Fecal indicators in offshore sampling stations

4.9.4  Fecal indicators in sediments near Barbers Point ocean outfall

4.10  Survival and Fate of Microbial Fecal Indicators

4.10.1 Fate of microbial fecal indicators in sewage treatment plants

4.10.2 Survival of fecal bacterial indicators in ocean waters

4.10.3  Survival FRNA and poliovirus in ocean water

5  CONCLUSIONS

6  RECOMMENDATIONS

7  REFERENCES

A SUPPLEMENTAL STUDY TO MB-7 SUBPROJECT:  CHARACTERIZATION OF THE MICROBIOLOGICAL QUALITY OF WATER IN MAMALA BAY

# LIST of Figures

Figure 1. Mamala Bay study (MB-7):  microbial sampling sites

Figure 2. Mamala Bay study (MB-7):  Waikiki 24 hour study beach sample sites

Figure 3. Mamala Bay study (MB-7): bay sediment sampling sites

Figure 4. Mamala Bay study (MB-7):  inland water sample sites

Figure 5. Mamala Bay study (MB-7):  Barbers Point sediment sampling sites

Figure 6. Stability of E.coli and S. faecalis (CFU/100 mL) Suspended in Phosphate Buffer and Exposed to Sunlight.

Figure 7. Stability of Fecal Coliform (MPN/g) in Soil at Various Dilutions in Phosphate Buffer and Exposed to Sunlight.

Figure 8. Stability of Enterococcus (MPN/g) in Soil at Various Dilutions in Phosphate Buffer and Exposed to Sunlight.

Figure 9.  Survivability of FRNA phages and human poliovirus in non-filtered marine water

Figure 10. Survivability of E. coli, FRNA phages and human poliovirus in non-filtered marine water against sunlight exposure. (----- : taken to the lab, incubated in the dark).

# LIST of Tables

Table 1.  Mamala Bay Study Monitoring Sample Sites

Table 2.  Sand Island Primary Effluent (S1) Indicator Bacteria and Phage Geometric Means (GM) and Parameter Averages (AVG)

Table 3.  Bay Site Indicator Bacteria and Phage Geometric Means (GM) and Parameter Averages (AVG)

Table 4.  Canal/Stream Site Indicator Bacteria Geometric Means (GM) and Parameter Averages (AVG)

Table 5.  Land-based Non-Point Sources of Fecal Indicator Bacteria

Table 6.  Beach Site Indicator Bacteria and Phage Geometric Means (GM) and Parameter Averages (AVG)

Table 7.  Mamala Bay Geometric Mean (GM) and Ranges of Nutrient Concentrations.

Table 8.  Bacteria and Phage Geometric Means (GM) and Parameter Averages (AVG) of Beach and Inland Site Samples taken during Wet vs. Dry Weather Conditions

Table 9.  Bacteria, Phage and Parameters for Waikiki 24 Hour Study (2/6/95 - 2/7/95)

Table 11.  Geometric mean density of fecal coliform, enterococcus, and C. perfringens recovered from sediments from Ala Wai canal, Ilikai Harbor, and selected stream sites of Oahu. (The range of bacterial indicator concentration is indicated in parentheses).

Table 12.  Geometric mean density of fecal coliform, enterococcus, and C. perfringens recovered from sediments at offshore/deep ocean sites of Oahu. (The range of bacterial indicator concentration is indicated in parentheses).

Table 13.  Concentration of fecal coliform, enterococcus, and C. perfringens recovered from sediments from Barbers Point ocean outfall sites.

Table 14.  Geometric means of concentration of indicator bacteria and FRNA phages in sewage.

# 2  INTRODUCTION

## 2.1  Scope of Work

The charge of the MB-7 project was to use the most appropriate microbiological techniques to assess the concentrations of sewage borne microorganisms (indicators and pathogens) in various locations in Mamala Bay for the purpose of determining the quality of water in Mamala Bay and the contributions of these sewage borne microorganisms from point and non-point sources.

Five separate teams were selected for the MB-7 project. Two teams were from the University of Hawaii and had facilities at the study location. One team (Dobbs/Landry) completed most of their experiments early and did not interact with the rest of the MB-7 teams. My team can be descriptively called the “Hawaii Indicator Team” because we are based at the University of Hawaii and our team was tasked with monitoring the water samples for various “microbial fecal indicators” of sewage contamination.  It should be pointed out that the concentrations of specific microbial fecal indicators are used to establish enforceable marine recreational water quality standards.  My team was also charged with providing laboratory space and for coordinating all activities when the three  other MB-7 teams (Arizona, Maryland, Florida) came to Hawaii to complete their study objectives.  Since fecal indicators are the most feasible markers of sewage contamination of water, our team was also charged with analyzing water samples to meet the goals of other MB projects headed by HydroQual, Kinnetic Laboratories, and USC Ocean Physics Group and undertaking special studies under the direction of the Mamala Bay Commissioners.

## 2.2  Study Objectives

The goals of the MB-7 project were to assess the microbiological (sewage borne indicators and pathogens) quality of water within Mamala Bay, to determine the sources and contribution of these sewage borne microorganisms from point and non-point sources and to determine the health risk to users of Mamala Bay.  To assist in achieving this goal as well as the overall goal of this Mamala Bay Study, our team was assigned the following objectives:

1. Determine appropriate sampling sites within Mamala Bay and appropriate sampling designs to measure point and non-point sources of pollution contaminating Mamala Bay.

2. Analyze the selected samples for traditional indicators of fecal/sewage contamination (fecal coliform, enterococci, E. coli) which are currently used by USEPA to establish recreational water quality standards.  Analyze these same water samples for alternative indicators of fecal contamination such as Clostridium perfringens, and FRNA bacteriophage which have been proposed to be superior to the traditional indicators.

3.  Analyze water samples for selected physical parameters (salinity, turbidity, pH) and chemical parameters (phosphate, nitrate, ammonia, silica, dissolved organic phosphorus, dissolved organic nitrogen, total phosphorus, total nitrogen) to obtain independent measurements of the quality of these water samples.

4.  Provide the MB-7 teams from Arizona, Maryland, and Florida with laboratory space and all necessary support services during their sampling operations in Hawaii. This involved planning and coordinating for quarterly samples and for arranging for boats as well as the collection of samples to the satisfaction of all teams.  In addition, analyze water samples for fecal indicators and selected physical and chemical parameters so these teams can correlate their findings to our standard water quality measurements.

5.  Collect and ship water samples to the Arizona Team on a monthly basis so they can analyze the water samples for pathogens on a monthly basis in Arizona. Our team analyzed these same water samples for fecal indicators, physical and chemical parameters and provided these data to the Arizona Team.

6. Analyze water samples under conditions and time required by Mr. Marty Stevenson to meet the goals of project MB-3 to assess the pollutant loading of Ala Wai Canal and Pearl Harbor.  This involved analyzing water samples every two hours over two separate 24 hour periods and twice a day sampling (Ebb and Flow Tide) over 30 days.

7.  Analyze water samples under conditions and time required by Drs. Alan Blumberg and John Connolly of HydroQual to meet the goal of MB-5 which is to provide a model for the movement of water from point and non-point sources within Mamala Bay. This involved sampling water at additional sites and time.  Moreover, sample ocean water sites under trade and Kona weather conditions.

8.  Analyze water samples under conditions and time required by Dr. Burt Jones to meet the goal of tracking movement of water from the Sand Island sewage and from Ala Wai Canal into Mamala Bay.

9. Analyze water samples under conditions and time requested by the Mamala Bay Commissioners.  Thus, two 24 hour sampling of water at swimming beaches were conducted to assess the impact of contribution of swimmers to indicator bacteria.

10. Determine the survival and fate of fecal indicators and pathogens when exposed to conditions in Mamala Bay.

## 2.3  Project Organization and Project Reports

Dr. Charles Gerba served as the Project Leader for the MB-7 project which was comprised of five separate teams headed by the following principal investigators: 1) Gerba (Arizona), 2) Fujioka (Hawaii), 3) Paul/Rose (Florida), 4) Hill (Maryland) and 5) Dobbs/Landry (Hawaii).  Kimberly Roll served as coordinator for the entire MB-7 team and was under the direction of Chuck Gerba and Roger Fujioka. She was responsible for coordinating and ensuring that all studies to be conducted in Hawaii were completed on schedule.  This included keeping everyone informed and planning for the quarterly sampling periods when the Hawaii, Arizona, Florida and Maryland Teams of MB-7 worked together in Hawaii.  She was responsible for collecting and sending all samples to the Arizona team on a monthly basis.  She was also responsible for supervising the experiments and the personnel on the Hawaii team on a daily basis and for writing the quarterly reports for the Hawaii team.

The principal investigator for the Hawaii Team was Dr. Roger Fujioka. He was responsible for all the experimental designs, for the completion of all the objectives of the Hawaii Indicator Team and for the writing of this final report.  Dr. Philip Loh served as co-investigator to the Hawaii Team.  His contribution to this project was in the design and implementation of experiments involved with human poliovirus.  This primarily involved the use of his tissue culture and virology laboratory.  Chun Mei Wu was the graduate research assistant who conducted the poliovirus experiments.  Bunnie Yoneyama assisted Kimberly Roll in supervising the experiments and the personnel to complete all the experiments for the Hawaii Team.  Tua Unutoa and Arvin Wu served as graduate research assistants and then as research associates on this project.  Kartini Luther served as graduate research assistant and will earn her master’s degree from the Department of Microbiology using this work for her thesis project.  Nikolai Sintsov served as post doctoral fellow. Richard Takaba and Pauline Lucas completed their masters degree project working on two phases of the Mamala Bay Study.  Michael Okura served as student help and Mrs. Young as laboratory assistant.

The following people were assigned to write a separate, detailed report on the following phases of this project:

1. Kimberly Roll: Monitoring the Waters of Mamala Bay for Fecal Indicators.

2. Kartini Luther: Assessing the Concentrations and Survival of FRNA Phages in 			Mamala Bay.

3. Tua Unutoa: Concentrations and Survival of Sediment Bound Fecal Indicators.

4. Arvin Wu: Sources of nutrients and fecal indicators in and around Mamala Bay.

5. Nikolai Sintsov: Assessing the Concentrations of Fecal Indicators in Waikiki 			Aquarium’s Effluent Discharge into Mamala Bay.

6. Richard Takaba: Assessment of Bacterial Indicators of Fecal Contamination at 			Magic Island Beach.

7. Pauline Lucas: Determination of sunlight’s effect on the cell integrity and 			metabolic activity of bacteria in recreational water.

# 3  METHODS

## 3.1  Task Summary

The tasks for our team included the development of a scientifically acceptable study design, selection of best sampling sites, collection of samples at designated times using acceptable methods, storage and transportation of samples to laboratory under acceptable conditions and the analysis of samples in timely manner using documented, reliable methods.

## 3.2  Task Methodology

The membrane filtration method as described in Standard Methods (APHA, 1992) was used to analyze water samples for fecal coliform, enterococci, and E. coli.  C. perfringens was analyzed by the method of Bisson and Cabelli (1979) while FRNA was analyzed by the method as described by Debartolomeis and Cabelli (1991).  The three tube MPN method as described in Standard Methods (1992) was used to determine concentrations of indicators in sediments.  Water samples were also analyzed for pH (Orion Research microprocessor), salinity (Reichert-Jung refractometer), temperature and turbidity (Laboratory Turbidimeter Model No. 2100A).  Samples for nutrients (orthophosphate, nitrate+nitrite, ammonium, silica) were analyzed by Analytical Services Laboratory at the University of Hawaii using the Technicon Continuous Flow Auto Analyzer.

## 3.3  Collection of  Samples and Sampling Sites

All samples were collected using recommended techniques.  For grab samples, samples were collected aseptically into sterile containers, properly stored and transported to the laboratory.  Many samples were subsurface samples collected from bridges or boats.  Depth sampling devices were used to collect these samples. 	 The sampling sites are shown in Figures 1, 2, 3, 4 and 5.  The sampling sites are described in Table 1.

Quarterly samples were done in collaboration with the visits to Hawaii by the Arizona, Maryland and Florida teams of MB-7.  Over the course of the year, four quarterly sampling sessions were conducted during October 25-29, 1993; February 14-18, 1994; June 20-24, 1994; and November 14-18, 1994.  These quarterly sampling consisted of five days of intensive water and sediment sampling at the designated sites within and around Mamala Bay.  The number of samples taken during each quarterly sampling period increased from 21 to 23, to 55 and to 58.

## 3.4  Study Approach and Design

For this study, the quality of water was primarily assessed based on ambient or background concentrations of microbial fecal indicators as well as concentrations of these fecal indicators used to establish water quality standards.  In the absence of water quality standards, proposed guidelines for fecal indicators were used.  In this study, the quality of water was assessed primarily on Hawaii’s marine recreational water standard (DOH, 1990) of 7 enterococci/100 ml as a geometric mean of all samples analyzed at any given site.  This level is five times more restrictive than the USEPA recommended standard (EPA, 1986) of 35 enterococci/100 ml.  Both standards as well as inland water quality standards were used in assessing the quality of water.  However, it should be noted that for enforceable water quality standards, USEPA requires sampling at a given site every 6 days for five weeks to determine the geometric mean.  Since the objective of our present study was not to determine the quality of the water to meet standards, this regimen was not followed.  However, the geometric mean for all indicators was determined at each site based on the total number of samples taken and the quality of that water compared to existing and proposed water quality standards and guidelines.  For single samples, a level above the standard should warrant concern although it is recognized that single water samples at a given site can reach high levels and still meet the water quality standard when the other four samples have low counts.

Interpretations in the concentrations of other fecal indicators were based on ambient level and existing recreational water quality standards such as 200 fecal coliform/100 ml, 126 E. coli/100 ml and 33 enterococci/100 ml for inland waters.  For C. perfringens the following guidelines proposed by Fujioka et al, 1992 were used: 50 CFU/100 ml in inland waters, 5 CFU/100 ml at coastal waters used for swimming and 2 CFU/100 ml in open ocean waters. In the absence of proposed guidelines for FRNA phage, the same numerical standard for enterococci was used for FRNA phage.

# 4  RESULTS

## 4.1  Establishment and Assessment of Control Stations in Mamala Bay

In assessing the impact of point and non-point sources of pollution in Mamala Bay, only a limited number of sampling sites can be selected.  Most of the sampling sites were selected near identified sources of point and non-point sources of pollution.  However, the values detected at these sites must be compared with sites which are not impacted, called ‘control sites’.  For this study, two deep ocean sites were designated as control sites since they represented deep ocean sites at the boundaries (Diamond Head, Barbers Point) of what is designated as Mamala Bay.  The first site was off Diamond Head and designated Station E4 while the second site was off Barbers Point and designated Station BP1 (Figure 1).  From Station E4, a total of 9 surface, mid-depth and bottom samples were obtained and analyzed for fecal indicators.  The results (Table 3) show that the geometric mean concentrations of all fecal indicators (enterococci, E. coli, fecal coliform, C. perfringens, FRNA phage) were <1 CFU/100 ml.  However, all fecal indicators were detected at elevated concentrations in some individual samples.  Thus, peaks counts (CFU/100 ml) of 60 enterococci, 38 E. coli, 48 fecal coliform, 4 C. perfringens  and 19 PFU/100 ml of FRNA were recovered from individual samples. These results were unexpected and lead to two interpretations.  First, that station E4 represent a good control station and every site within Mamala Bay can be expected to contain occasional peaks of fecal indicator bacteria.  The second and probably more correct interpretation is that Station E4 is not a good control station and is impacted by  point (septic tanks  discharges) or non-point sources (storm drains) which discharge into Mamala Bay near Diamond Head Point and these discharges are rapidly transported out to Station E4 area by the strong currents detected in that area.

At the Barbers Point Station (BP1) only 3 surface, mid-depth and bottom samples were analyzed for four bacterial fecal indicators.  The results (Table 3) show that the surface samples were least impacted with undetectable levels to only 1 CFU/100 ml recovered from surface samples.  However, peak counts (CFU/100 ml) of  3 enterococci, 5 E. coli, 13 fecal coliform and 3 C. perfringens were detected in individual samples obtained from bottom and mid-depth samples at this site. The most plausible explanation for these results is that the sewage from the Barbers Point outfall is predominantly submerged and being transported to station BP1.

In summary, two stations located at the two boundaries of Mamala Bay (Diamond Head, Barbers Point) were established as control stations to be used as comparative data for the many samples to be taken within Mamala Bay. The results were unexpected in that evidence of fecal indicators were detected at these two control stations.  The results suggest that these two stations are not good control or unimpacted stations.

## 4.2  Impact of Discharges of Sand Island Sewage and Honolulu Harbor on Water Quality in Mamala Bay

The major identified point source of pollution into Mamala Bay is the 78 MGD of non-disinfected, primary treated sewage effluent from the Sand Island Wastewater Treatment Plant.  To characterize the fecal indicator content of this effluent, 13 samples from this plant were analyzed for indicator microorganisms.  The results (Table 2) show that this effluent contained geometric mean concentrations (CFU/100 ml) of  16,901,763 fecal coliform, 14,748,853 E. coli, 983,002 enterococci, 22,344 C. perfringens and 104,726 PFU/100 ml of FRNA phages.  These results are similar to the values obtained in our previous study (Fujioka et al, 1992).  However, the present study for the first time determines the level of FRNA phage in sewage.  The effluent was also shown to have the following average values: pH of 7.08, salinity of 5.1 ppt, temperature of 26.2°C and turbidity of 34.8 NTU; and contained high concentrations (Table 7) of phosphate (57mM), nitrate (2.7 mM), ammonia (984 mM) and silica (556 mM).

A major concern is whether the sewage from Sand Island Ocean sewage outfall is being transported to the shorelines and is creating an unacceptable public health risk to people who use these coastal waters for recreation. Since the closest land area to the ocean outfall site is Sand Island, sampling stations between the sewage ocean outfall and Sand Island were established.  However, Sand Island is an off-shore island fronting a protected inland body of marine water (Honolulu Harbor, Keehi Lagoon) which is not flushed out effectively by tidal exchanges.  This inland body of water receives non-point discharges such as streams and run-off from the industrial activities which line this coastal area.  The flow of water from the western and eastern end of Sand Island represents water discharging into Mamala Bay but also ocean water entering this inland water (see Figure 1).

To determine the contribution of point source (Sand Island Sewage Outfall) and non-point source (Keehi Lagoon) into Mamala Bay, four stations were established as a straight line transect from the zone of dilution site over the Sand Island Outfall to Keehi Lagoon (Figure 1).  Station DO is a representative site considered to receive all the non-point discharges from the Keehi Lagoon area.  It is located near Keehi boat harbor.  Station D1 is a representative nearshore water site which receives water exiting from Keehi Lagoon into Mamala Bay.  It is located 2.5 mile south or seaward of DO. Station D2 represents the ocean site designated for the mixing of sewage from the Sand Island Outfall into Mamala Bay.  It is located 0.75 mile south or seaward of Station D1.  Station D3 represents the ocean site seaward of the ocean outfall.  It is located 0.75 miles seaward or south of Station D2.  Surface, mid-depth and bottom samples from each of these sites were obtained, analyzed for fecal indicator concentrations, and the geometric mean concentrations (CFU/100 ml, PFU/100 ml) summarized in Table 3.  The geometric mean concentrations of nutrients are summarized in Table 7.

At Station D2 or in the zone of dilution area of the sewage ocean outfall, bottom (50 m depth), mid-depth (35 m depth) and surface water  samples were analyzed for fecal indicators.  The results (Table 3) show that based on 7-9 samples, the geometric mean concentrations (CFU/100 ml) of fecal coliforms were 161 in the bottom samples, 4 in mid-depth samples and 2 in surface samples.  For E. coli, the concentrations were 93 in bottom samples, 3 in mid-depth samples and 1 in surface samples while concentrations for enterococci were 29 in bottom samples, 4 in mid-depth samples, and 1 in surface samples.  For C. perfringens the concentrations were 7 in bottom samples, 1 in mid-depth samples and <1 in surface samples while the concentrations for phages were 12 for bottom samples, 2 for mid-depth samples and 1 for surface samples. These results show higher concentrations of all fecal indicators in bottom samples and reflect the fact that the sewage plume is more concentrated at this level.  This was substantiated by the detection of nutrients above background concentrations in only the bottom samples (Table 7). However, even at this depth, there was a great variation in counts ranging from 0 to 32,000 CFU/100 ml providing evidence that the location of the submerged sewage plume changed from day to day.  Much lower concentrations of fecal indicators were recovered from mid-depth samples and even lower concentrations from surface water samples. These results support the concept that as the sewage effluent rises, there is greater dilution of the sewage and greater chance of inactivation by sunlight.  It should be noted that the zone of mixing allows the sewage to mix with and be diluted with ocean water. Thus, indicator bacteria above the marine recreational water quality standard is allowed within the zone of mixing.

To determine whether fecal indicators can be recovered from sites south or seaward from the ocean outfall site, 3-5 samples were obtained from Station D3.  The results (Table 3) show that the geometric mean concentrations for fecal coliform were 5 in bottom samples, 4 in mid-depth samples and 2 in surface samples.  The concentrations of  E. coli were 9 in bottom samples, 3 in mid-depth samples and 1 in surface samples while the concentrations of enterococci were 2 in bottom samples, 3 in mid-depth samples, and 1 in surface samples.  The concentrations of C. perfringens were <1 to 1 in bottom, mid-depth and surface samples while the concentrations for phages were 12 in bottom samples, 0 in mid-depth samples and 8 in surface samples.  The concentrations of nutrients in these samples approximated background levels (Table 7).  These results provide evidence that the sewage effluent from the ocean outfall was being diluted and transported south or seaward from the ocean outfall site. Based on the low concentrations of enterococci, water at Station D3 met Hawaii’s recreational water quality standard.

To determine the contribution of fecal indicators from the Keehi Lagoon area, 3 bottom and surface water samples from Station DO were analyzed.  The results (Table 3) show that the geometric mean concentrations for fecal coliform were 21 in the bottom sample and 29 in the surface sample.  The concentrations of E. coli were 18 in the bottom sample and 24 in the surface sample while the concentrations of enterococci were 9 in the bottom sample and 13 in the surface sample.  The concentrations for C. perfringens were 6 in bottom samples and 12 in surface samples while the concentrations of FRNA phages were 0 PFU/100 ml from two bottom samples and 2 PFU/100 ml from five surface samples.  These results show that water at Station DO was clearly contaminated with fecal indicator bacteria and failed to meet Hawaii’s marine recreational standard. The relatively high turbidity and increased concentrations of nutrients including silica in the DO samples indicate that the source of the fecal indicators at this site are land based run-offs (streams, activities from the Keehi Boat Harbor).

To determine the concentrations of fecal indicators at Station D1, 3-5 samples of bottom, mid-depth and surface water samples were obtained.  The results (Table 3) show that the geometric mean concentrations for fecal coliform were 6 in the bottom sample, 1 in the mid-depth sample and 5 in the surface sample.  For E. coli, the concentrations were 5 in the bottom sample, 0.6 in the mid-depth sample and 9 in the surface sample.  For enterococci, the concentrations were 9 in the bottom sample, 3 in the mid-depth sample, and 8 in the surface sample.  Thus, fecal indicators were present at all depths at this station and based on enterococci concentrations, the water quality at this site does not meet Hawaii’s recreational water quality standard.  The nutrient concentrations in the D1 samples were slightly elevated.  Station D1 is between DO and D2.  The characteristic of the water at D1 is slightly closer to DO suggesting that, the source of fecal indicators at Station D1 is most likely from water transported from Station DO (Keehi Lagoon). However, the results do not exclude the possibility that fecal indicator were transported to Station D1 from D2 (the ocean outfall).

Station SB1 was established because this is the closest swimming beach to the ocean outfall.  Although approved for swimming, this shoreline site has rough waters and few people use this beach for swimming.  A total of 19 water samples were obtained from this site, and the results (Table 6) show that the geometric mean concentrations (CFU/100 ml) for enterococci were 2 and <1 for all other fecal indicators.  Thus, the quality of water at this beach site is excellent and meets Hawaii’s marine recreational standard.  These results also indicate that the fecal indicators from the ocean outfall are not being transported to this beach.  The concentrations of nutrients in water samples from this site were slightly elevated (Table 7) when compared to open ocean sites indicating some contributing impact from land (eg. groundwater).

Station (HH1) was established to determine whether the ocean outfall is being transported landward, toward Waikiki Beach area as well as to determine the contribution of fecal indicator exiting the Honolulu Harbor Channel via the eastern end of Sand Island. A total of 3-4 samples obtained from bottom, mid-depth and surface water samples were obtained from this site.  The results (Table 3) show that the geometric mean concentrations (CFU/100 ml) for all fecal indicators were very low (0-1 CFU/100 ml) and that the quality of water at this site meets Hawaii’s marine recreational standard.  The concentrations of nutrients in these samples were only slightly elevated (Table 7).  These results provide evidence that the sewage from the ocean sewage outfall as well as the discharge of water from Honolulu Harbor area into Mamala Bay do not have an impact the quality of water at this nearshore site.

## 4.3  Impact of Discharges of Honouliuli Sewage on Water Quality in Mamala Bay

The Honouliuli Wastewater Treatment Plant discharges 25 MGD of primary treated, non-disinfected sewage into Mamala Bay and is the second most significant point source of pollution into Mamala Bay.  The MB-7 team had made a decision early in the project that with limited resources, the impact of the Honouliuli sewage outfall into Mamala Bay would not be a primary objective.  Thus, only few samples were collected from and near this ocean outfall.  Moreover, the sewage effluent from the Honouliuli wastewater treatment plant was not monitored.  A recent study by Fujioka et al (1992) documented that based on 20 samples taken in 1990, the effluent from Honouliuli Wastewater Treatment Plant contained geometric mean concentrations (CFU/100 ml) of 9,930,000 E. coli, 396,000 enterococci and 4,240 C. perfringens.

Two off shore stations were established to determine the transport of sewage from the ocean outfall in a landward direction.  One sampling station (HU1) was located one mile offshore and within the ocean outfall zone of mix area. The second sampling Station (HU2) was located shoreward from Station HU1 and 0.5 mile offshore.  Only 3 bottom, mid-depth and surface samples from these two stations were analyzed for fecal indicators.

The results (Table 3) for Station HU1 show that the geometric mean concentrations (CFU/100 ml) of fecal coliform were 14 in bottom samples and 0 from mid-depth and surface samples.  The concentrations of E. coli were 14 in bottom samples, 1 in mid-depth samples and 0 in surface samples while the concentrations of enterococci were 2 in bottom and mid-depth samples and 0 in surface samples.  The concentrations of C. perfringens were 7 in bottom samples, 3 in mid-depth samples and <1 from surface samples.  The concentrations of nutrients in these samples were generally low to slightly above background levels (Table 7).  These results show that highest concentrations of fecal indicators were recovered from bottom samples but at low concentrations indicating that the plume was not directly sampled even in the bottom samples.  The very low recoveries of fecal indicators in mid-depth and surface samples provide evidence that surfacing of the sewage in the zone of mixing was not occurring during these 3 sampling periods.

The results (Table 3) for Station HU2 show that the geometric mean concentrations (CFU/100 ml) of all fecal indicators in bottom, mid-depth and surface samples were usually <1.  The only exception was in the surface sample where 2 CFU/100 of C. perfringens were measured.  The concentrations of nutrients in these samples were low but with slightly elevated concentrations of silica (Table 7).  These results provide evidence that the transport of sewage from the Barbers Point ocean outfall in a shoreward direction to site HU2 is minimal.

Three shoreline stations were established at the Ewa Beach area, the land closest to the Barbers Point Outfall.  Station EW1 was located nearest the mouth of Pearl Harbor and this site was not a designated swimming beach. The two other stations represented swimming beaches;  Station EW2 at Ewa Beach Park and Station EW3 at Oneula Beach Park (Figure 1). Sixteen water samples obtained from Stations EW1 and EW2 and 4 water samples obtained from Stations EW3 were used to calculate the geometric mean concentrations of fecal indicators and the results summarized in Table 6.

The geometric mean concentrations of all fecal indicators at these three shoreline stations (EW1, EW2, EW3) were <1 to 1 with the exception of 2 CFU/100 ml of enterococci at Station EW1.  These results provide evidence that the quality of the shoreline waters near the Ewa Beach area is good and is not impacted by the Barbers Point sewage ocean outfall or the Navy sewage discharged into Pearl Harbor.  Despite these low levels of fecal indicators, all of these samples contained elevated concentrations of silica as well as phosphate and nitrate (Table 7) indicating presence of freshwater.  At station EW1, the freshwater source may be the sewage outfall discharged into Pearl Harbor or freshwater exiting Pearl Harbor.  The area around EW2 and EW3 is very dry and there are no streams discharging into this area.  Thus, the elevated concentrations of silica at these swimming beach sites must represent land run-off such as groundwater or storm drain discharges from the nearby communities.

## 4.4  Impact of Pearl Harbor Discharge on Water Quality in Mamala Bay

Pearl Harbor is an enclosed body of marine water and receives discharges from many streams as well as sewage from the third Wastewater Treatment Plant (WWTP) known to discharge its waste into Mamala Bay via an ocean outfall. The Fort Kamehameha WWTP is a Navy facility which processes 7 MGD of sewage by a) preliminary treatment, b) primary treatment, c) secondary treatment and d) chlorination and e) dechlorination before it is discharged into the Pearl Harbor Entrance Channel.  The NPDES for this plant requires that the monthly average based on geometric mean of five samples does not exceed 133 CFU/100 ml of enterococci.  In a preliminary study conducted earlier by Rijal and Fujioka (1994), the final chlorinated effluent from this plant was effective in reducing the concentrations of enterococci to a geometric mean of approximately 12 CFU/100 ml indicating that most of the fecal indicators from this sewage effluent were disinfected before the effluent was discharged into Pearl Harbor.. For the Mamala Bay Study, the impact of all the water leaving Pearl Harbor was assessed, which included the discharge from Fort Kamehameha WWTP.  In the current study, the fecal indicator concentrations of the effluent from the Fort Kamehameha WWTP were not analyzed separately.

To determine the levels of fecal indicators entering Mamala Bay from Pearl Harbor, three stations were established (see Figure 1) as a straight line transect from within Pearl Harbor (station C1), at the mouth of Pearl Harbor (C2) and an off-shore site just outside the mouth of Pearl Harbor (C3).  To determine the contribution of fecal indicators from Pearl Harbor, 2 samples of water were obtained from Station C1 as representative of general water quality within Pearl Harbor.  The results (Table 3) show that the geometric mean concentrations (CFU/100 ml) of all fecal indicators obtained from the bottom sample (12 m depth) and mid-depth (7 meter depth) samples were very low (0-1 CFU/100 ml) providing evidence that the quality of the submerged water was good.  The submerged water samples were determined to have a salinity of 35 ppt; close to open ocean water.  However, these submerged water samples from Pearl Harbor contain elevated concentrations of nutrients including silica demonstrating that the quality of water within Pearl Harbor differs considerably from open ocean water.  The surface water sample from Station C1 had an average salinity of 30 ppt and silica of 30 mM providing strong evidence that fresh water from streams tends to float at the surface of marine water.  Surface water samples at this station contained geometric mean concentrations (CFU/100 ml) of 9 fecal coliform, 6 E. coli, 4 enterococci, 2 C. perfringens as well as FRNA phage.

The quality of water at the mouth of Pearl Harbor was determined by analyzing two water samples  from Station C2.  The results (Table 3, Table 7) show that the quality of water at the mouth of Pearl Harbor was similar to the quality of water within Pearl Harbor.  Thus, the geometric mean concentrations (CFU/100 ml) of all fecal indicators obtained from the bottom (20 m) and mid-depth (10 m) samples were very low (0-1 CFU/100 ml).  These submerged water samples had salinity of 35 ppt and moderate levels of nutrients including silica (2.6-3.3 mM).  Surface water samples at this site contained geometric mean concentrations (CFU/100 ml) of 6 fecal coliform, 4 E. coli, 3 enterococci,  and <1 C. perfringens as well as FRNA phage.  Surface water samples were characterized by average of 33 ppt of salinity and elevated levels of silica (21 mM).

These results indicate the net transport of water from within Pearl Harbor to Mamala Bay via the mouth of Pearl Harbor.  The data do not show the impact of the discharge of the treated and disinfected sewage effluent from the Fort Kamehameha WWTP into the vicinity of the mouth of Pearl Harbor.

To determine the quality of water in nearshore waters of Mamala Bay just outside the mouth of Pearl Harbor, 7-9 water samples were obtained from Station C3.  The results (Table 3) show that the geometric mean concentrations (CFU/100 ml) in the bottom and the surface water samples contained low concentrations (0 to 1 CFU/100 ml) of all the fecal indicators.  The salinity of surface and bottom water at this site was approximately 34 ppt.  However, surface water samples at this site contained elevated concentrations of silica (9 mM) as compared to only 3.5 mM in bottom water samples.  These results indicate that water from Pearl Harbor was being transported to the offshore station C3 but the concentrations of fecal indicators being discharged into Mamala Bay via Pearl Harbor is relatively low and under most conditions does not seriously impact on the quality of water in Mamala Bay.

The effect of wet weather conditions on the quality of water in Pearl Harbor was assessed by analyzing and isolating the data collected under wet weather conditions during the February 1994 quarterly sampling period.  During this wet weather sampling day, surface water samples at Station C1 had low salinity (28 ppt) clearly indicating the predominance of fresh water at the surface.  These surface water samples contained   elevated concentrations (CFU/100 ml) of 62 fecal coliform, 51 E. coli, and 55 enterococci.  At Station C2, the surface water was determined to have a salinity of 34 ppt but similar concentrations (CFU/100 ml) of 52 fecal coliform, 44 E. coli, and 53 enterococci.  At Station C3 the surface water had  salinity of 35 ppt and slightly lower concentrations (CFU/100 ml) of 6 fecal coliform, 7 E. coli, and 8 enterococci.  The nutrient data collected on this day also showed results similar to those obtained for fecal indicators.  Thus, highest levels of nutrients including silica were detected in surface water samples at Stations C1, C2 with some reductions of these constituents in surface water samples obtained from Station C3.  Thus, the data collected on this wet weather day provides evidence that when it rains, the fresh water content of surface waters in Pearl Harbor increases and is transported out into Mamala Bay carrying with it increased concentrations of fecal indicators and nutrients.  These results demonstrate that under wet conditions, the discharge of water from Pearl Harbor measurably impacts the quality of water in Mamala Bay.

## 4.5  Impact of Ala Wai Canal on Quality of Water in Mamala Bay

The Ala Wai Canal is a major body of in-land water discharging near the popular swimming beaches in Mamala Bay.  It has been previously documented that the quality of this body of water is poor and contains high concentrations of fecal indicators, often exceeding the in-land recreational water quality standard of 200 FC/100 ml.  As a result, the Hawaii Department of Health has determined that this body of water is unsuitable for swimming and has posted ‘no swimming’ signs.  At the outset, the Mamala Bay Study identified the Ala Wai Canal as a major known non-point source of pollution.  Thus, one of the major tasks of this study was to fully characterize the quality of water in the Ala Wai Canal and to quantify the impact of this water on the quality of water throughout Mamala Bay. The task of our team was to characterize the quality of the water in the Ala Wai Canal based on concentrations of several fecal indicators, to determine the sources of the fecal indicators recovered from this canal and the extent to which the Ala Wai Canal contributed to the concentrations of fecal indicators recovered from other areas within Mamala Bay.

### 4.5.1  Sources of fecal indicators in Ala Wai Canal

Three streams (Makiki, Palolo, and Manoa) are the most identifiable sources of non-point, land-based discharges entering the Ala Wai Canal.  Of these three streams, Manoa Stream has the largest flow and therefore, a sampling station (MS1) was established at this stream near the University of Hawaii (Figure 1). At this sampling station 10 samples were analyzed for fecal bacteria and 4 samples were analyzed for FRNA phage. The results (Table 4) show that Manoa Stream water contains geometric mean concentrations (CFU/100 ml) of 1,718 fecal coliform, 2,358 E. coli, 695 enterococci, 51 C. perfringens and 255 FRNA phage.  All ten water samples contained fecal indicators exceeding the various recreational water quality standards such as 200 FC/100 ml, 126 E. coli/100 ml or 33 enterococci/100 ml.  These results confirm our earlier study (Hardina and Fujioka, 1991) which documented that Manoa Stream consistently contains high concentrations of all fecal indicators.  That study provided evidence that soil was a natural land based source of these fecal indicators.  The present study for the first time documents that FRNA can be consistently recovered from streams in Hawaii.

The more than 65 storm drains which line Ala Wai Canal are less visible sources of pollution.  However, we (Fujioka et al, 1988; Roll & Fujioka, 1994) previously reported that storm drains in Hawaii contain high concentrations of fecal indicators.  In this current study, we determined that high concentrations of fecal indicators were present in the Ala Wai Canal near the ends of most of these storm drains. These results provide evidence that storm drains are another source of fecal indicators entering the Ala Wai Canal.

Two other sources may be contributing fecal indicators to the Ala Wai Canal.  The first is sewage leaking from sewage pipes.  Whether this is a real or significant source cannot be determined.  It is a possible source because breaks in sewage lines do occur and it is very plausible that sewage leaks will occur for some time before breaks in sewage lines are identified.  The second source is from live-in boats which collect sewage in their tanks and which are moored at the Ala Wai Boat Harbor.  In a previous study, we (Loh et al, 1979) recovered human viruses from the Ala Wai Canal and concluded that the most likely source of these human viruses was feces released from the boats at the Ala Wai Boat Harbor.

### 4.5.2  Concentrations of fecal indicators in the Ala Wai Canal

To determine the concentrations of fecal indicators in the Ala Wai Canal, a sampling station was established near the ocean exit of the Ala Wai Canal.  This sampling station (AW1) was located over the Ala Moana Bridge (Figure 1) and was established as the single monitoring station for stream water leaving the canal as well as ocean water entering the canal.  A total of 30-44 surface water samples were obtained from this site over a year long period and analyzed for fecal indicators.  The results (Table 4) show that the geometric mean concentrations (CFU/100 ml) determined were 403 fecal coliform, 206 E. coli, 117 enterococci, 16 C. perfringens, and 7 FRNA phage. These results  show that concentrations of fecal indicators in the Ala Wai Canal exceeded all of the recreational water quality standards.  The concentrations of fecal indicators in the canal were lower than in Manoa Stream reflecting the dilution of the stream water with ocean water (salinity of 24.4 ppt) as well as inactivation of fecal indicators in the Ala Wai Canal, especially due to sunlight exposure.  It should be noted that the salinity of individual samples ranged from 3 to 36 ppt indicating the variable mixtures of fresh water (stream, stormdrain, rainwater) and marine waters.  These results are correlated with the variable concentrations of fecal indicators such as fecal coliform which ranged from 21 to 42,000 CFU/100 ml.  Water samples from station AW1 were characterized by high concentrations of nutrients and silica (Table 7) reflecting the concentrations of these parameters in stream water.

### 4.5.3  Pollution loading into Mamala Bay

The impact of Ala Wai Canal discharge into Mamala Bay can be assessed by determining the concentrations of fecal indicator in the water exiting the canal and the concentrations of these indicator bacteria in Mamala Bay.  As a first attempt to address this question, a monthly monitoring station was established 0.25 mile from the Ala Wai sampling site (AW1) and positioned in nearshore waters of Mamala Bay to receive the discharge exiting the Ala Wai Canal.  Only 3-5 water samples were obtained from this site (AW2) and the geometric mean concentrations of fecal indicators determined.  The results (Table 3) show that <1 CFU/100 ml of all fecal indicators were recovered from bottom and mid-depth water samples from this site.  However, surface water samples from this site contained 3 fecal coliform, 4 E. coli, 3 enterococci, 1 C. perfringens and 0 for FRNA phage.  The salinity of the bottom, mid-depth and surface water samples averaged 34-35 ppt.  These results indicate that by the time the water from the Ala Wai Canal reaches Station AW2, it is thoroughly mixed and diluted with ocean water. However, some of the residual fresh water including some fecal indicators are still floating at the surface of the water.  It should be noted that due to the limited number of samples taken, no quantitative conclusion could be drawn as to the impact of the discharge of Ala Wai Canal into Mamala Bay.

To quantitatively determine the expected impact of the Ala Wai Canal discharge into Mamala Bay, a study to assess the loading of pollutants from the Ala Wai Canal into Mamala Bay was initiated based on a study design developed by John Connolly (MB5) and Marty Stevenson (MB3).  Marty Stevenson was the project leader for this phase of the study.  Our team analyzed water samples for fecal indicators from AW1 station every 2 hours for 24 hour periods during wet weather conditions (March 25-26, 1994) and during dry weather conditions (July 6-7, 1994).  A supplemental study involved sampling at AW1 station twice a day (ebb tide, flow tide) over a 30 day period.  The results of this phase of the study, including the data from all measurements are included in the MB-5 report.  In summary, the surface water samples collected during the 24 hour wet weather sampling day measured only 0.5 to 5.6 ppt salinity and these surface water samples consistently contained high concentrations of fecal coliform (Range: 228 to 13,600 CFU/100 ml) and enterococci (Range: 25 to 7,200 CFU/100 ml).  Under these conditions, all 39 water samples exceeded the recreational standard of 200 FC/100 ml and 7 enterococci/100 ml.  During this same sampling period, the bottom water samples had salinity of 35 ppt and contained low concentrations of fecal indicators.  Thus under heavy rain conditions, evidence was obtained that stream water is floating on marine water and is being transported directly out to Mamala Bay.

A second 24 hour sampling study was conducted under dry weather conditions and the results showed that the salinities of surface water samples ranged from 22.8 to 31.4 ppt, indicating extensive mixing of the fresh (stream) water with ocean water. Under these conditions, the surface water samples contained lower levels of fecal coliform (Range: 1 to 1,880 CFU/100 ml) and enterococci (Range: 0 to 1080 CFU/100 ml).  Thus, under dry weather conditions 21% (8/39) of the samples exceeded 200 FC/100 ml while 51% (20/39) exceeded 7 enterococci/100 ml.  These results document that the loading of fecal indicators from the Ala Wai Canal into Mamala Bay is dependent on wet or dry conditions.

### 4.5.4  Other sources of fecal indicators polluting Mamala Bay

The Ala Wai Canal is the most obvious non-point source of pollution entering Mamala Bay.  Three streams (Manoa, Palolo, Makiki) as well as storm drains were determined to be major sources of fecal indicators in the Ala Wai Canal.  Thus, any stream or storm drain which discharge into Mamala Bay must be suspected as sources of fecal indicators.  To address this question, four other streams (Kalihi, Nuuanu, Moanalua, Kapalama) which flow through Honolulu and discharge into Mamala Bay (Figure 4) were assayed for fecal bacteria on May 5, 1995.  The results showed that highest concentrations of fecal bacteria were recovered when water samples were taken directly from inland streams which were determined to be predominantly fresh water, based on low salinity measurements (2 ppt).  Under these conditions, water samples from Kalihi and Nuuanu Streams were determined to contain elevated concentrations (CFU/100 ml) of enterococci (1,520/1,320),  E. coli (3360/728),  fecal coliform (5,200/344) and C.  perfringens (400/228).  Streams eventually flow into and are diluted by receiving ocean water.  When stream water samples are obtained from these sites, the extent of dilution by ocean water can be determined by measuring the salinity of the water samples.  Under these conditions, water samples from Kapalama (30 ppt) and Moanalua (24 ppt) Streams were determined to contain reduced concentrations (CFU/100 ml) of enterococci (1/400), E. coli (3/38), fecal coliform (4/64) and C. perfringens (20/34).  These results show that other streams which discharge into Mamala Bay are sources of high concentrations of fecal indicators and that the concentrations of fecal indicators in streams are highest in stream water samples with low salinity.  The concentrations of fecal indicators decrease when streams are diluted by ocean water.

Another source of fecal indicators are marinas which are characterized by many boats tied up in waters which undergo poor circulation with ocean water.  Three marinas (Ilikai, Kewalo, Keehi) were identified in the Honolulu area of Mamala Bay.  On November 16 of 1993, single water samples were obtained from these three sites to estimate the expected concentrations of fecal bacteria in these marinas. The results showed that the concentrations of enterococci in these three marina water samples ranged from a low of 0 CFU/100 ml at Ilikai Marina to a high of 124 CFU/100 ml at Keehi Lagoon Marina while the concentrations of E. coli ranged from a low of 26 CFU/100 ml at the Ilikai Marina to a high of 160 CFU/100 ml at Kewalo Marina Site 2.  These same water samples contained concentrations of fecal coliform which ranged from a low of 24 CFU/100 ml at the Ilikai Marina to a high of 250 CFU/100 ml at the Kewalo Marina Site 2 while the concentrations of C. perfringens ranged from 1 to 2 CFU/100 ml in all marina water samples.  These results provide evidence that waters in marinas are sources of fecal indicators but the concentrations of indicators are lower than observations in the Ala Wai Canal which is directly impacted by stream run-off.  The Hilton Lagoon is another source of elevated concentrations of fecal indicators.  Water from the Hilton Lagoon is also discharged into Mamala Bay.

In addition to these non-point sources of discharges, there are other point source discharges which are regulated by NPDES permit and which discharge into Mamala Bay. For example, the effluent from the Waikiki Aquarium discharges into shallow water in Mamala Bay next to the Natatorium.  This shallow area is used extensively for swimming and wading.  A major source of fecal indicators at the Aquarium are Monk seals which are mammals.  The Aquarium recently underwent major renovation including the use of ozone to treat its effluent.  To address the effectiveness of ozone as a disinfectant, our team completed a short study working with the personnel at the Waikiki Aquarium.  The results of this study (Sintsov, 1994) showed that feces of monk seals contained high concentrations of fecal coliform, enterococci and C. perfringens which are shed into the monk seal pool.  The ozone treatment effectively disinfects fecal coliform and enterococci but not C. perfringens.  However, the design of the Aquarium disinfection system does not ensure that all sources of fecal indicators are disinfected before they are discharged into Mamala Bay.  Thus, some levels of fecal indicators are discharged into Mamala Bay via the Aquarium effluent.  In this regard, the health risks of fecal indicators from monk seal feces for humans should be determined.  We are currently working with the Aquarium to implement a design to ensure that effluent from the Aquarium is reliably disinfected of all fecal indicators.

Soil in Hawaii has been previously reported to be a natural habitat of fecal indicators (Hardina and Fujioka, 1991).  Since soil is everywhere, many other forms of land run-off can be expected to contain high concentrations of fecal indicators.  To verify this hypothesis,  we analyzed other sources of run-off which discharge into storm drains and streams.  The results (Table 5) show that puddles of water, run-off from road and even run-off from roofs are sources of high concentrations of enterococci (1,760 to 208,000 CFU/100 ml), E. coli, (21 to 176,000 CFU/100 ml), fecal coliform (64 to 804,000 CFU/100 ml) and C. perfringens (5 to 4,080 CFU/100 ml). Based on these results, it can be concluded that many sources of land-based run-off are major sources of fecal indicators and these sources can pollute waters in Mamala Bay via many routes.

Although we have documented that surface water run-off contain high concentrations of fecal indicators and these surface water samples are discharged into Mamala Bay, the quality and quantity of subsurface water entering Mamala Bay have not been assessed in this study.  For example, large volumes of subsurface water are known to be entering the coastal waters of Hawaii, including Mamala Bay.  These groundwater sources could represent the flow of surface water which has percolated to some shallow depth and is then transported to coastal water as submerged sources of groundwater.  The quality and quantity of these waters in terms of their fecal indicator and nutrient content have not been determined.  Leaks from sewage lines and from discharges from cesspools or from septic tanks into groundwater can be expected to contain elevated concentrations of fecal indicators.  The extent or significance of these sources of pollution has not been determined.  However, the cumulative effects of these unmeasureable subsurface water sources do not appear to be significant since the quality of most coastal water samples in Mamala Bay contain low levels of fecal indicators.

In conclusion, there are many other sources of fecal indicators which are most likely entering Mamala Bay.  Some of these sources can be measured.  Others are not visible and are difficult to document.  Nevertheless, in the assessment of the impact of water quality in Mamala Bay, all sources of pollution should be considered.

## 4.6  Fecal Indicators at Popular Recreational  Beaches

More people come in intimate contact with beach water than any other type of marine waters.  Beach water contaminated with feces or sewage greatly increase the risk of water borne diseases to swimmers.  USEPA has established recreational water quality standards using concentrations of fecal indicators to assess the suitability of beach water for swimming.  Seven beaches within Mamala Bay and one beach outside of Mamala Bay were monitored for concentrations of fecal indicators.  More water samples (19 to 40) were obtained from the three most popular beaches which are Ala Moana (AM1), Queens (Q1) and Waikiki (W1).  The geometric mean concentrations of all fecal indicators at these three beach sites were low and met the most stringent standard of 7 enterococci/100 ml (Table 6).  However, there were days when the quality of water at these beaches exceeded the standard.  For example the 7 enterococci/100 ml standard was exceeded in  13% of the samples from Queen’s Beach, in 38% of the samples from Waikiki Beach and in 28% of the samples from Ala Moana Beach.  The single beach at Sand Island (SB1) as well as the three beaches in Ewa (EW1, EW2, EW3) contained even lower concentrations of fecal indicators.  The highest concentrations of fecal indicators were recovered from waters obtained from Hanauma Bay Beach which was selected as a control beach outside of Mamala Bay.  At this site, the geometric mean concentration of 6 enterococci/100 ml was still within the recreational standard of 7 enterococci/100 ml, although 46% of the samples exceeded 7 enterococci/100 ml.  The chronically elevated concentrations of fecal indicators in the waters at Hanauma Bay has been previously studied by our laboratory (Fujioka & Roll, 1993).

## 4.7  People as Sources of Fecal Indicators

In assessing the concentrations of fecal indicators recovered from beaches, great emphasis is given to sewage such as ocean outfall and to non-point sources such as streams, canals and storm drains.  However, people are also sources of fecal indicators and may be contributing to the levels of fecal indicators at the recreational beach site. This hypothesis was supported by the results of our previous study in which we (Charoenca and Fujioka, 1993) reported that the concentrations of staphylococcus in beach waters increased with the number of swimmers at the beach.  In a simulated study, Smith and Dufour (1993) reported that after 15 minutes of full immersion in bath tub filled with water, 7,400,000 staphylococcus bacteria was shed per human body as compared to 123,000 E. coli and 66,000 enterococci.  In a slightly different experimental design, we simply measured the concentrations of fecal indicator in beach water after allowing children to sit (lower body immersion) in an inflatable pool filled with 22.8 liters of seawater for 30 seconds before they entered the ocean water for the first time. Although fecal indicators were documented to be shed from this short immersion experiment (Takaba, 1995), the concentrations shed were much lower than the full immersion, unclad subjects in the Smith and Dufour (1993) study.  In this study, (Takaba, 1995) the quality of water at the Magic Island site usually met the Hawaii state standard of 7 enterococci/100 ml.  However, higher concentrations of fecal indicators were consistently recovered from the shoreline areas where most of the swimmers and children were as compared to samples obtained from deeper waters within Magic Island.  Based on these results, it was concluded that swimmers are sources of fecal indicators in recreational beaches. However, the contribution of fecal indicator per swimmer is variable and generally low.  At Magic Island, the quality of water at the beach site generally met the restrictive 7 enterococci/100 ml marine recreational water quality standard indicating that the impact of swimmers on water quality at this site was minimal.

In an independent study, the Dobbs team of MB-7 reported that fecal indicator bacteria concentrations increased in Waikiki beach water samples as the number of people at the beach increased.  These results indicated that people are significant sources of fecal indicators at swimming beaches.  To substantiate the results of the Dobbs team, the Mamala Bay Commissioners requested that our team work with MB5 to design a study to sample water from two beaches every 2 hours over a 24 hour sampling period and to correlate the recovered concentrations of fecal indicators from the beach waters with the number of people at the beach.  The two primary beaches were Royal Hawaiian Beach, (R1), the same site monitored by the Dobbs team, and Waikiki Beach (W1), the standard monitoring station used in our current study and the same site (Tavern Beach) used by the Hawaii Department of Health for their routine monitoring of Waikiki Beach. A third beach site in front of the Sheraton Waikiki Hotel was also selected as a monitoring site so three beaches in a row could be monitored (see Figure 2). However, due to limitations on our resources, the Sheraton Beach site was monitored every 6 hours instead of every 2 hours.  This sampling design allowed us to determine the concentrations of fecal indicators under spatial and temporal conditions.

The 24 hour sampling study at the beach site with good water circulation was conducted on two separate days.  The results (Tables 9, 10) show no obvious correlation between the increase in the numbers of swimmers in the water and the concentrations of fecal indicators in the beach water samples.  Thus, data to support the hypothesis that people are significant sources of fecal indicators in the water was not obtained.  A similar conclusion was obtained from an earlier study conducted at Kuhio Beach, a beach with poor water circulation.  The data we obtained in that study, (Roll & Fujioka, 1994), also did not show a direct increase in concentrations of fecal indictors in beach water samples as the swimmers at the beach increased.  Thus, although it can be shown that people shed fecal indicators when they enter a body of water, it is also known that these fecal indicators are diluted by the ocean water, and are transported away from that site by currents and are inactivated.  Thus, many factors determine the concentrations of fecal indicators recovered from beach water samples. We speculate that under some conditions, one can expect substantial increases in fecal indicator bacteria in beach water samples as a result of increased numbers of swimmers. However, the data we obtained did not support this hypothesis.

## 4.8  Impact of Wet Weather on Inland Water Sites

Non-point sources of pollution are generally identified as run-off from land such as streams and storm drains.  In Hawaii, these sources contain high concentrations of fecal indicators and have been shown to contribute to the fecal indicators recovered from swimming beaches.  Rain is one of the major events known to increase the levels of fecal indicators in inland and coastal waters such as beaches.  To assess the impact of wet weather on concentrations of fecal indicators, the data collected at seven beach sites (AM1, EW1, EW2, HB1, Q1, SB1, W1) as well as at Manoa Stream (MS1) and Ala Wai Canal (AW1) were separately tabulated under wet and dry conditions.  The results (Table 8) show that wet weather conditions cause an increase in the levels of fecal indicators at Manoa Stream and Ala Wai Canal.  This is not surprising since rain run-off from land is discharged directly into these two bodies of water.  However, streams, storm drains or other non-point sources of pollution do not directly discharge into the seven recreational beaches.  Thus, even under wet weather conditions, the waters at these seven beaches did not exceed the recreational standard of 7 enterococci/100 ml.  Of these seven beaches, Ala Moana Beach appears to be most susceptible to increases in fecal indicator concentrations during wet weather conditions.  It is plausible that some of the water from Ala Wai Canal is being transported to Ala Moana Beach.

## 4.9 Concentrations of Fecal Indicators in Sediments

To assess the quality of water, water quality standards are based only on determining the concentrations of fecal indicators in  the water column.  However, fecal indicators have been reported to be concentrated at higher levels in the sediments and to be resuspended into the water column from time to time.  Thus, sediments were obtained from several selected monitoring stations and assayed for fecal indicators to correlate the quality of water in the column and in the sediment at the same sampling sites.  The three tube MPN method was used to assess the concentrations of fecal indicators in sediments. Geometric mean concentrations of these fecal indicators were determined from multiple samples at any given site.

### 4.9.1  Fecal indicators in sediments from recreational beaches

Based on 3-7 samples, the geometric mean concentrations (MPN/g) of fecal coliform, enterococcus and C. perfringens in sediments obtained from Hanauma Bay Beach (HB1), Queens Beach (QB1), Waikiki Beach (W1), Ala Moana Beach (AM1) and Sand Island Beach (SB1) were generally <1 MPN/g.  Sediments from Hanauma Bay had the highest counts with geometric mean of 3 MPN/g of enterococcus and 2 MPN/g of C. perfringens.  These results show that the sediments at the popular swimming beaches within Mamala Bay contain negligible concentrations of fecal indicators.  Hanauma Bay Beach, located outside of Mamala Bay and which was previously determined to contain elevated concentrations of fecal indicators in the water contained low but detectable levels of fecal indicators.  Thus, at recreational beaches, sediments in beach waters do not appear to be a  significant source of fecal indicators.

### 4.9.2  Fecal indicators in sediments from land run-off sites

Land run-off sites are defined as streams, canals, harbors which were known to be receiving run-off from land and to be discharging into Mamala Bay.  Manoa Stream and the Ala Wai Canal were previously identified as significant sources of fecal indicators. Therefore, multiple samples and/or multiple sampling sites were established for these bodies of water.  The results (Table 11) show that sites within Ala Wai Canal (AW1, IK1, AW-IOL, AW-MC) contained relatively low concentrations of fecal coliform (4-37 MPN/g), consistently higher concentrations of enterococci (146-439 MPN/g) and highest concentrations of C. perfringens (972-3,108 MPN/g).  The comparative concentrations of these fecal indicators in sediment reflect the relative stability of these fecal indicators (C. perfringens > enterococci > fecal coliform) in environmental conditions.

Due to the constant gravity flow of water in Manoa Stream, sediments within Manoa Stream are quickly transported away and do not accumulate in the stream bed. Thus, the physical properties of sediments obtained from Manoa Stream differ from that in the Ala Wai Canal.  Four sediment samples from Manoa Stream Station (MS1) revealed higher concentrations of fecal coliform (609 MPN/g), but similar concentrations of enterococci (494 MPN/g) and much lower concentrations of C. perfringens (172 MPN/g) as compared to the Ala Wai Canal sediments.  The relative concentrations of these three fecal indicators in Manoa Stream are similar to those found in soil, supporting the hypothesis that soil is the source of fecal indicators in Manoa Stream.  Similar results were obtained when occasional samples of sediments were analyzed from other stream sites.

### 4.9.3  Fecal indicators in offshore sampling stations

Sediments were obtained from the same offshore sampling stations used to assess the quality of water within Mamala Bay.  The  results (Table 12) show very low levels of all fecal indicators in sites located offshore of the Ala Wai Canal (AW2) and Waikiki Beach (W2).  Thus, fecal indicators are not accumulating in sediments at these sites. Sediment samples from the three transect sampling stations from Pearl Harbor (C1, C2, C3) revealed only low concentrations of fecal indicators although there was a definite increase in the concentrations of C. perfringens.  The low concentrations of fecal indicators in the sediments at station C1 indicate that the sediments within Pearl Harbor is not a significant source of fecal indicators.  It should be noted that sewage is no longer discharged into Pearl Harbor.  The highest concentrations of C. perfringens was observed in sediment from station C2.  Since this sampling station is near the discharge of sewage from the Navy plant into Pearl Harbor, it was concluded that sewage discharge was the source of fecal indicators in the sediment at this site.  However, there does not appear to be a significant accumulation of fecal indicators at this site and less in sediment from station C3.

Sediment sampling from the Sand Island transect revealed that highest concentrations of fecal indicators were recovered from the sediment at the ocean outfall site (D2).  Moderate levels of fecal indicators were observed in the sediment from station DO and low levels from sediment station D1 (Table 12).  These results indicate that the sewage outfall is a point source of fecal indicators and that sediments in the Keehi Lagoon area is another source of fecal indicators. The lower concentrations of fecal indicators at station D1 indicate that this site simply receives the residual sediments transported either from Kewalo Lagoon (DO) or from the sewage outfall (D2). A plausible reason for the non-accumulation of fecal indicators in the sediment at station D1 would be the excellent circulation of water at this offshore site.

### 4.9.4  Fecal indicators in sediments near Barbers Point ocean outfall

As stated earlier, the sampling design for the MB-7 projects excluded establishing monitoring stations near the Barbers Point ocean outfall.  Since this was the second ocean outfall within Mamala Bay, we requested the assistance of the City and County of Honolulu to collect additional sediment samples for our analysis during their yearly monitoring of the Barbers Point ocean outfall. The benthic sampling stations were established by the City and County of Honolulu for their long term monitoring (Figure 5).  Only one set of sediment samples was obtained and five of the sediment stations were within the zone of dilution (ZID), in the area near the outfall pipes.  Two sediment stations were outside of the ZID.  The results (Table 13) show elevated concentrations of all fecal indicators in sediment samples obtained from stations (HZ, HB2, HB3, HB4, HB6) within the ZID and much lower levels of these fecal indicators in sediment obtained from stations (HB1, HB7) away from the ZID.  This single set of sediment samples obtained from the Barbers Point ocean outfall site show that fecal indicators can be recovered from sediments near the sewage outfall pipes and some of these bacteria can still be recovered in lower concentrations outside of the ZID.  Analysis of more sediment samples from sites all around the ZID is required to determine the extent to which the sediments are contaminated with fecal indicators and whether these fecal indicators are significantly accumulating in the sediment.

In conclusion, sediments are sources of fecal indicators.  The extent and concentrations of fecal indicators in sediments provide information as to the history of water pollution and the circulation or water transport capacity at that given site.  The concentrations of fecal indicators recovered from sediment reflect the comparative stability of these indicators.  In this regard, the stability and the accumulative characteristics of C. perfringens in the sediments of waterways have been previously well documented (Cabelli et al, 1984;  Hill, et al, 1993) and have been used as an index to the history of contamination at that site.

## 4.10  Survival and Fate of Microbial Fecal Indicators

Determination of the concentrations of the microbial fecal indicators in various types of water is important in establishing water quality standards. However, to assess their public health significance, the survival and fate of these fecal indicators must be known.  Several experimental studies were completed to determine the survival and fate of fecal indicators under conditions deemed important to predict their survival when released into Mamala Bay.

### 4.10.1 Fate of microbial fecal indicators in sewage treatment plants

The concentrations of the various fecal indicators in the primary treated sewage effluent from Sand Island WWTP was previously reported.  The objective of this phase of the study was to determine the concentrations of these fecal indicators in sewage as the raw sewage undergoes primary treatment, secondary treatment and chlorination.  Raw and primary treated sewage samples were obtained from Sand Island WWTP while the secondary and chlorinated sewage effluent was obtained from the Wahiawa WWTP.  The geometric mean concentrations (CFU or PFU/100 ml) of all fecal indicators were determined based on 4-10 samples.  The results (Table 14) show that in raw sewage the relative concentrations of the fecal indicators were as follows:  fecal coliform > E. coli > FRNA phage > enterococci > C. perfringens.  Primary treatment did not reduce the concentrations of these fecal indicators while secondary treatment removed 90-99% of the concentrations of these fecal indicators.  After chlorination, the concentrations of fecal indicators (fecal coliform, E. coli, enterococci) used in establishing water quality standards were drastically reduced.  In contrast, the concentrations of C. perfringens and FRNA phage were only marginally reduced by chlorination.  These results demonstrate the expected concentrations of fecal indicators as they undergo typical wastewater treatment.  The results clearly show that the two alternative indicators (C. perfringens, FRNA phage) used in this study are much more resistant to the wastewater treatment processes, including chlorination, than the currently acceptable fecal indicators.  These results allow one to predict the concentrations of fecal indicators in the various treatment phases of the wastewater treatment plant and the impact these may have after being discharged into the ocean.

### 4.10.2 Survival of fecal bacterial indicators in ocean waters

Sunlight was previously reported to be the primary factor controlling the survival of fecal indicators in marine environments (Fujioka et al, 1981; Fujioka & Siwak, 1985). To substantiate the inactivation capabilities of sunlight, fecal indicators were diluted in phosphate buffer and exposed to sunlight while maintaining the temperature at 25 °C. The results plotted in Figure 6 show that when E. coli and S. faecalis are freely suspended in buffer and kept in the absence of sunlight, they remain stable for 6 hours.  However, when these same two fecal indicator preparations were exposed to sunlight, a reduction by six orders of magnitude was observed in E. coli after one hour exposure to sunlight whereas three hours were required to inactivate S. faecalis to the same extent.  Although these populations of E. coli were determined to be inactivated by culture on standard bacteriological media, Lucas (1995) determined that sunlight had not destroyed the cells’ structural integrity based on staining with acridine orange and that these inactivated cells were still capable of respiration based on INT staining.  The fate and survival of fecal and pathogenic bacteria in marine waters are being addressed in greater detail in the report by Dr. Russell Hill.

As previously determined, fecal indicators are present in high concentrations in  sediments and these sediments are known to be resuspended into the water column.  To determine the expected fate of these sediment-bound fecal indicators to sunlight inactivation, soil was diluted with phosphate buffer and this mixture exposed to sunlight. The results plotted in Figure 7 show that fecal coliform concentrations in soil diluted 1:10, 1:100, 1:1000 or 1:10,000 with phosphate buffer were stable to sunlight inactivation.  The results plotted in Figure 8 show similar results with enterococci. These results show that when fecal indicators are freely suspended in water they are very sensitive to sunlight inactivation.  However, when they are attached to sediment, they become resistant to sunlight inactivation.

### 4.10.3  Survival FRNA and poliovirus in ocean water

The sewage pathogens of concern with regards to transmission of water borne diseases via recreational use of water have been concluded to be human enteric viruses (Cabelli et al, 1982).  Monitoring ocean water for human enteric viruses is not feasible. However, monitoring ocean water for bacterial virus is feasible.  FRNA phages are the most highly recommended bacterial virus to use to monitoring environmental waters for human viruses (Havelaar et al, 1986).  Monitoring ocean waters for FRNA phages as a reliable means to determine the presence or absence of human enteric viruses is dependent on the survival patterns of these two classes of virus in marine waters.  To address this question, the survival of FRNA phage and poliovirus in water samples obtained from Mamala Bay was determined in the absence and presence of sunlight.

The first experiment was conducted to determine the comparative survival of FRNA phage and poliovirus in marine waters in the absence of sunlight.  For this study, the first experiment evaluated the affect of natural ocean water on the survival of prototype strain (MS2) and a natural environmentally recovered strain (SW3-6) of FRNA phage as compared to poliovirus.  The samples were kept under laboratory conditions (25 C) in the absence of sunlight and sampled periodically for viable concentrations of viruses.  The results showed that the two strains of FRNA viruses were stable in buffer but were inactivated to the same extent in unfiltered and filtered seawater. These results indicate that FRNA phages are unstable in marine waters and 99% inactivation of FRNA phages can be expected in marine waters after 3 days.  Under similar conditions, poliovirus was stable in buffer and in filtered seawater but had similar stability as FRNA virus in natural, unfiltered seawater (Figure 9).  The inactivation of poliovirus by marine bacteria in seawater which are removed by filtration has been previously reported  (Fujioka et al, 1990).  These results indicate that despite the fact that poliovirus and FRNA have the same gross morphology, there still appears to be some basic difference in the structural integrity of these two classes of viruses.  The FRNA phages were similar to poliovirus, but slightly more stable in natural (unfiltered) seawater.  These results support the monitoring of FRNA phage in marine waters for the presence and absence of poliovirus in the absence of sunlight.

The second experiment was conducted to determine the comparative stability of FRNA phage and poliovirus in marine waters in the presence of sunlight.  As a control, the experiment was conducted to show the stability of  E. coli under the same conditions. The results plotted in Figure 10 show that  E. coli was most sensitive to sunlight inactivation and the concentration was reduced by approximately eight orders of magnitude within 2 hours of  exposure to sunlight.  During the same day and after six hours of  exposure to sunlight, poliovirus was inactivated by approximately three orders of magnitude.  Under the same conditions, the two strains of FRNA viruses were inactivated by less than one order of magnitude.  These results show that E. coli suspended in seawater and exposed to sunlight can be expected to be rapidly inactivated. However, under these same conditions,  sewage borne pathogens such as enteric viruses (eg. poliovirus) are much more resistant.  Thus, monitoring ocean water for bacterial indicators used in establishing water quality standards does not appear to be a reliable means to determine whether the water is free of pathogenic human viruses.  The results of this study show that FRNA phages are more stable in ocean water conditions in the presence and absence of sunlight than poliovirus.  Thus, monitoring water for FRNA phages appears to be a reliable and economic way of monitoring ocean water to ensure that the water is free of pathogenic human enteric viruses such as poliovirus.

# 5  CONCLUSIONS

1.  The two point sources of sewage (Sand Island WWTP, Honouliuli WWTP) contain very high concentrations of all fecal bacteria as well as FRNA phage. These two point sources of pollution are being constantly discharged in large volumes into Mamala Bay.  These results are consistent with the fact that sewage effluents from these two ocean outfalls have only undergone primary treatment and have not been disinfected. The monitoring evidence obtained indicates that these two point sources of pollution are being diluted by the ocean and are being preferentially transported away from the land.  The fecal indicators from these two ocean outfalls do not appear to be measurably increasing the concentrations of fecal indicator bacteria concentrations in shoreline waters.

2.  Two deep ocean sites which bordered the ends of Mamala Bay were selected as control sites.  Fecal indicators were recovered from both of these two sites.  The source of fecal bacteria from the site on the western end was concluded to come from the Barbers Point sewage outfall.  The source of fecal bacteria and FRNA phage from the eastern site (Diamond Head) was speculated to originate from Diamond Head Point. Recovery of indicator bacteria from these two control sites was unexpected and indicates that these two sites are impacted by pollution.

3.  The Fort Kamehameha sewage outfall is the third point source of sewage discharged into Mamala Bay. However, this sewage undergoes primary and secondary treatment as well as disinfection before it is discharged into the mouth of Pearl Harbor. Based on monitoring data, the impact of this outfall as well as all of the water from Pearl Harbor on the quality of water within Mamala Bay is minimal during dry days.  However, under heavy rain conditions, fresh water and elevated concentrations of fecal indicators are transported out of Pearl Harbor into Mamala Bay.

4.  Water exiting Honolulu Harbor appears to have a minimal impact on the fecal indicator quality of Mamala Bay.  On the other hand, water exiting Keehi Lagoon appears to be a constant and measurable source of fecal indicators entering Mamala Bay.  Heavy rain increased the concentrations of fecal indicator being transported from Keehi Lagoon into Mamala Bay.

5.  The Ala Wai Canal was documented to contain high concentrations of fecal indicator and the three streams (Manoa, Palolo, Makiki) which discharge into Ala Wai Canal were documented as primary sources of indicator bacteria.  The freshwater from these streams tended to float on the surface of the heavier salt water in the Ala Wai Canal. Thus, concentrations of fecal indicators were highest in surface water samples obtained in Ala Wai Canal as compared to subsurface water samples in the canal.  Under heavy rain conditions, the stream water stayed on the surface of the Ala Wai Canal and was quickly transported out to Mamala Bay.  Under dry weather conditions the impact of fecal indicators from Ala Wai Canal to Mamala Bay was measurable but minimal due to dilution and presumably, inactivation.  However, under heavy rain conditions, elevated concentrations of fecal indicator were recovered from sites within Mamala Bay.  The amount of pollution contributed by the Ala Wai Canal was determined by sampling every two hours over a 24 hour period.

6.  Many land based non-point sources of fecal indicators were identified.  These sources included streams, storm drains, soil, puddles of water on land, run-off from roads and run-off from roofs.  Thus, any surface run-off must be considered a source of fecal indicators entering coastal waters.  The contribution of subsurface water being discharged into Mamala Bay was not determined.  However, since the quality of most coastal water is good, the contribution of subsurface water on the quality of water in Mamala Bay must not be substantial.

7.  The water at most beaches within Mamala Bay had low concentrations of fecal indicators and generally met the strict Hawaii marine recreational standard of 7 enterococci/100 ml.  Evidence was obtained that water samples from Ewa beach where no streams are discharging into Mamala Bay nevertheless was impacted by contribution from land.  It was speculated that groundwater run-off including storm water might be a source of these land based input at this site.  Hanauma Bay beach was selected as the control beach outside of Mamala Bay because it could not be impacted by an ocean outfall in Mamala Bay.  However, water from this beach contained higher concentrations of fecal indicator bacteria than beaches within Mamala Bay.  In a previous study we reported that soil, sand and pigeons are sources of fecal indicator in Hanauma Bay.

8.  People were determined to shed fecal indicators after entering beach water.  However, based on intensive monitoring of beach water, the concentrations of fecal indicator in beach water did not increase in a predictable manner with an increase in the number of people at the beach.  Thus, the hypothesis that people are sources of elevated concentrations of fecal indicator in beach water was not substantiated.

9. Fecal bacterial indicators were detected in sediments and the concentrations of fecal indicators in sediments reflected the chronic level of fecal indicators in water samples from that site.  Thus, fecal indicator in sediments from beaches and in open ocean sites were minimal.  However, sediments from streams, Ala Wai Canal, and from near the ocean outfall sites contained elevated concentrations of fecal indicators.  The concentrations of fecal indicators in sediments reflected their stability.  Thus, C. perfringens was recovered in highest concentrations in sediments followed by enterococci with lower concentrations of fecal coliform and E. coli.  Fecal indicators attached to sediments were resistant to the inactivating effect of sunlight.

10. The expected survival of fecal indicators in seawater in the presence of sunlight are as follows: C. perfringens> FRNA phage>poliovirus>enterococci>fecal coliform>E. coli.  Thus, when sewage pollutes seawater, the three fecal indicators (enterococci, fecal coliform, E. coli) used in establishing recreational water quality standards can be expected to be inactivated before the pathogens (viruses, protozoans).  It should be noted that although E. coli was determined to be inactivated by cultural methods, the inactivated E. coli cells maintained their cellular integrity and appeared to be able to respire.  These results suggests that these inactivated bacteria are not dead. These kinds of information are not new.  The basic reasons for selecting these fecal bacterial indicators to establish water quality standards by USEPA are the ease of assaying for these bacteria and the correlation of gastrointestinal diseases among swimmers and concentrations of some of these fecal bacteria in water.

11. Monitoring water for physical parameters such as salinity and some chemical parameters such as nitrates, phosphates, ammonia and silica as well as fecal indicators proved to be very useful in terms of characterizing the composition of that water sample.  The concentrations of salinity and silica were especially helpful in determining the ratio of fresh water to seawater in a given sample.

# 6  RECOMMENDATIONS

1.  In Hawaii the applicability of the nationally established recreational water quality standards based on concentrations of fecal coliform, E. coli, and enterococci should be re-evaluated because these three indicators are readily found in the natural environments (soil, streams, storm drain, puddles, run-off from roads and roofs) of Hawaii in the absence of sewage contamination.  In the establishment of recreational water quality standards, it is assumed that the fecal indicators used do not have a significant environmental source.  The data used to establish the national standard by USEPA were obtained in temperate, continental conditions.  A risk assessment approach should be used in the evaluation of appropriate recreational water quality indicators.  This may include the correlation with fecal indicator concentrations in environmental samples with presence of pathogens as compared to concentrations of fecal indicator in sewage and presence of pathogens.

2.  In the monitoring of marine waters in Hawaii, some guidelines should be established for interpreting the health risk when the state’s enterococci standard of 7 enterococci/100 ml is exceeded because the source of the enterococci may be environmental rather than sewage.  Analyzing the water for concentrations of C. perfringens may prove to be very useful because sewage always contains a constant ratio of C. perfringens to enterococci.  On the other hand, soil in Hawaii contains high concentrations of enterococci but not C. perfringens.  Although, C. perfringens is highly correlated with sewage, this fecal bacteria has been recovered from storm drains, presumably due to feces of pets.

3.  Monitoring of recreational waters in Hawaii should include assays for C. perfringens because the method is very practical and large volumes of water can be processed.  Moreover, the concentrations of this fecal bacteria in the coastal waters of Hawaii are generally very low and easier to interpret than the fecal indicators used by USEPA to establish water quality standards.  Monitoring recreational water for FRNA virus should be encouraged since this virus appears to behave in a manner similar to most human enteric viruses.  However, the method for analyzing water for FRNA virus still does not meet the criteria for a monitoring method.  The method is cumbersome, the cells used must be maintained in a certain state, only a limited volume of water can be analyzed by the standard assay method, and the results are less reproducible.  The assay method for FRNA virus must be improved before wastewater laboratories can be expected to adopt this assay.  The assay for C. perfringens and FRNA virus also has the advantage of not entering a viable but non-culturable state which appears to be inherent in gram negative bacteria such as fecal coliform and E. coli and may apply as well to enterococci.

4.  Monitoring sediments for fecal indicators, pathogens and toxic chemicals should be included in assessing the quality of a body of water.  Analysis of sediments for unstable fecal indicators such as fecal coliform and E. coli can be used to determine the recent source of pollution.  Analysis of sediments for stable fecal indicators such as C. perfringens and FRNA virus can be used to determine the long term effects of pollution events.  If sediments do not contain indicators, it can be safely concluded that the site is not being significantly impacted.  Sampling of sediments near and around sewage ocean outfalls can prove to be very informative as to the settling of sewage near the ocean outfall.

5.  Use of gene probe tests to monitor recreational waters and sediments for pathogens should be encouraged since it is a feasible way to measure for the presence of pathogens.  However, guidelines on the interpretations of results obtained by these tests should be developed.  A major disadvantage in the use of gene probe tests is that it does not differentiate between dead or live cells.  Live cells present a public health risk whereas dead cells do not.  Moreover, gene probe tests can be counterproductive.  For example, effluents are usually disinfected to inactivate indicators and pathogens.  These samples when assayed by the gene probe tests may measure these inactivated cells as positive.  Thus, tests must be developed to determine whether cells which are positive by gene probe tests are viable cells and to differentiate these cells from those which had been disinfected or killed by other means.

6.  The biological and physical factors in marine waters which affect the stability of fecal indicator and pathogens should be determined.  Nutrient-rich waters (Pearl Harbor) were shown to be more active in inactivating poliovirus than nutrient-poor open ocean water.  Marine bacteria which grow in higher concentrations in nutrient-rich water were determined to be the cause of poliovirus inactivation in marine waters.  However, the specific marine bacteria and the conditions which enhance their concentrations and activity need to be identified.  The affect of this polio-inactivating bacteria on the virus structure and genes of polio virus as compared to other human pathogenic viruses must be determined.  The stability of FRNA virus in filtered marine waters differed from that of poliovirus.  The basis for this difference should be determined before a recommendation is made to use FRNA virus as a surrogate for human enteric viruses.

# 7  REFERENCES

American Public Health Association, American Water Works Association, Water Pollution Control Federation. 1992. Standard Methods for the Examination of Water and Wastewater, 18 edition. American Public Health Association, Washington D.C.

Bisson, J.W. and V.J. Cabelli. 1979. Membrane filtration enumeration method for Clostridium perfringens. Applied and Environmental Microbiology 37:55-66.

Cabelli, V.J., A.P. Dufour, L.J. McCabe, and M.A. Levin. 1982.  Swimming-associated gastroenteritis and water quality.   American Journal of Epidemiology 115:606-616.

Cabelli, V.J., R.L. Duncanson, J.L. Gaines, D. Pedersen, R.R. Sims, and R.F. Shokes. 1984. Chemical and microbiological tracers for the deposition and movement of sewage particulates. p. 272-277. Abstract of Oceans 1984 Conference and Exposition, Washington, D.C.

Charoenca, N. and R. Fujioka. 1991. Assessment of staphylococcus bacteria in Hawaii’s marine recreational waters. Water Science and Technology 27:283-289.

Debartolomeis, J. and V.J. Cabelli. 1991. Evaluation of an Escherichia coli host strain for enumeration of F male-specific bacteriophages. Journal of Applied and Environmental Microbiology 57:1301-1305.

DOH, State of Hawaii. 1990.  Chapter 11-54 Hawaii Administrative Rules on Water Quality Rules.

Environmental Protection Agency. 1986.  Ambient water quality criteria for bacteria.  EPA 440/5-84-002.

Fujioka, R., C. Fujioka, and R. Oshiro. 1992.  Application of Clostridium perfringens to assess the quality of environmental and recreational waters.  WRRC project completion report to Department of Public Works, City and County of Honolulu. May 1992.

Fujioka, R.S., H.H. Hashimoto, E.B. Siwak, and R.H.F. Young. 1981.  Effect of Sunlight on survival of indicator bacteria in seawater.  Applied and Environmental Microbiology 41:690-696.

Fujioka, R.S. and K.P. Roll. 1993. Assessing the effects of natural events and management practices on the microbial quality of recreational waters Project Completion Report 04:08:93 to the Department of Health, State of Hawaii.

Fujioka, R.S. and E.B. Siwak. 1985.  The cidal effect of sunlight on alternative microbial indicators of water quality.  AWWA Water Quality Technology Conference, Proceedings Number 13, pp. 495-507.

Fujioka, R., K. Tenno, and S. Kansako. 1988.  Naturally occurring fecal coliforms and fecal streptococci in Hawaii's freshwater streams. Toxicity Assessment. 3:613-630.

Fujioka., R., P.C. Loh, and L.S. Lau. 1980. Survival of human enteroviruses in the Hawaiian ocean environment: Evidence for virus inactivating microorganisms. Appl. Environ. Microbiol. 39:1105-1110.

Hardina, C.M. and R.S. Fujioka. 1991.  Soil: The environmental source of E. coli and enterococci in Hawaii's streams.  Environmental Toxicology and Water Quality. 6:185-195.

Havelaar, A.H., K. Furuse, and W.H. Hogeboom. 1986.  Bacteriophages and indicator bacteria in human and animal faeces.  Journal of Applied Bacteriology 60:255-262.

Hill, R.T., I.T. Knight, M.S. Anileis, and R.R. Colwell. 1993. Benthic distribution of sewage sludge indicated by clostridium perfringens at a deep ocean dump site. Applied Environmental Microbiology 59:47-51.

Loh, P.C., R.S. Fujioka, and L.S. Lau. 1979. Recovery, dissemination and survival of human enteric viruses in the Hawaiian ocean environment. Water, Air, and Soil Pollution 12:197-217.

Lucas, P. 1995. Determination of sunlight’s effect on the cell integrity and metabolic activity of bacteria in recreational water. MPH Field Research Study.

Rijal, G. and R. Fujioka, 1994. Evaluation of UV disinfection system in the inactivation of various indicator organisms in wastewater effluents. Abstract of the 94th Annual Meeting of the American Society for Microbiology. pg. 388.

Roll, K.K. and R.S. Fujioka. 1994. Microbiological characterization of the water and sediment in Kapahulu storm drain system and at Kuhio Beach Project Completion Report KSDS-4 for the Department of Health, State of Hawaii. March 1994.

Takaba, R. 1995. Assessment of bacterial indicators of fecal contamination at Magic Island. Masters in Public Health Thesis.

Smith, B.G. and A.P. Dufour. 1993. Effects of the microbiological quality of recreational waters: A simulation study. Abstract of the 93rd Annual Meeting of the American Society for Microbiology, pg. 406 Poster Q328.

Figure 1. Mamala Bay study (MB-7):  microbial sampling sites

Figure 2. Mamala Bay study (MB-7):  Waikiki 24 hour study beach sample sites

Figure 3. Mamala Bay study (MB-7): bay sediment sampling sites

Figure 4. Mamala Bay study (MB-7):  inland water sample sites

Figure 5. Mamala Bay study (MB-7):  Barbers Point sediment sampling sites

Figure 6. Stability of E.coli and S. faecalis (CFU/100 mL) Suspended in Phosphate Buffer and Exposed to Sunlight.

Figure 7. Stability of Fecal Coliform (MPN/g) in Soil at Various Dilutions in Phosphate Buffer and Exposed to Sunlight.

Figure 8. Stability of Enterococcus (MPN/g) in Soil at Various Dilutions in Phosphate Buffer and Exposed to Sunlight.

Figure 9.  Survivability of FRNA phages and human poliovirus in non-filtered marine water

Figure 10. Survivability of E. coli, FRNA phages and human poliovirus in non-filtered marine water against sunlight exposure. (----- : taken to the lab, incubated in the dark).

|  | Site | Description |  | Site Depth |  | Code |  | Sample Depth |  |  | *Sample Times |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Outfall Transect |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sand Island Outfall- | Keehi Lagoon |  | 5.5 m |  | D0S |  | 1 m |  |  | Q |  |  |
|  | shoreline |  |  |  |  | D0B |  | 5 m |  |  | Q |  |  |
|  | Sand Island Outfall- | Lat. - 21°17.15 |  | 20 m |  | D1S |  | 1 m |  |  | Q |  |  |
|  | inshore | Long. - 157°54.35 |  |  |  | D1M |  | 9 m |  |  | Q |  |  |
|  |  |  |  |  |  | D1B |  | 18 m |  |  | Q |  |  |
|  | Sand Island Outfall | Lat. - 21°16.82 |  | 70 m |  | D2S |  | 1 m |  |  | M |  |  |
|  |  | Long. - 157°54.43 |  |  |  | D2M |  | 35 m |  |  | M |  |  |
|  |  |  |  |  |  | D2B |  | 50 m |  |  | M |  |  |
|  | Sand Island Outfall- | Lat. - 21°16.40 |  | 350 m |  | D3S |  | 1 m |  |  | Q |  |  |
|  | beyond | Long. - 157°54.56 |  |  |  | D3M |  | 35 m |  |  | Q |  |  |
|  |  |  |  |  |  | D3B |  | 72 m |  |  | Q |  |  |
|  | Honouliuli -inshore | Lat. - 21°17.66 |  | 10 m |  | HU2S |  | 1 m |  |  | Q |  |  |
|  |  | Long. - 158°01.88 |  |  |  | HU2M |  | 3 m |  |  | Q |  |  |
|  |  |  |  |  |  | HU2B |  | 9 m |  |  | Q |  |  |
|  | Honouliuli Outfall | Lat. - 21°17.00 |  | 60 m |  | HU1S |  | 3 m |  |  | Q |  |  |
|  |  | Long. - 158°01.50 |  |  |  | HU1M |  | 35 m |  |  | Q |  |  |
|  |  |  |  |  |  | HU1B |  | 55 m |  |  | Q |  |  |
| Pearl Harbor Transect |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mid Pearl Harbor | Bishop’s Pt |  | 25 m |  | C2S |  | 1 m |  |  | Q |  |  |
|  |  |  |  |  |  | C2M |  | 10 m |  |  | Q |  |  |
|  |  |  |  |  |  | C2B |  | 20 m |  |  | Q |  |  |
|  | Mouth of Pearl Harbor | Green Bouy #5 |  | 5 m |  | C3S |  | 1 m |  |  | M |  |  |
|  |  |  |  |  |  | C3B |  | 5 m |  |  | M |  |  |
|  | Upper Pearl Harbor | Waipio Pt |  | 13 m |  | C1S |  | 1 m |  |  | Q |  |  |
|  |  | (10/93-Hospital Pt.) |  |  |  | C1M |  | 7 m |  |  | Q |  |  |
|  |  |  |  |  |  | C1B |  | 12 m |  |  | Q |  |  |
| Nearshore sites |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Honolulu Harbor | Lat.-21°17.31 |  | 10 - 12 m |  | HH1S |  | 1 m |  |  | Q |  |  |
|  |  | Long.-157°52.27 |  |  |  | HH1M |  | 5 m |  |  | Q |  |  |
|  |  |  |  |  |  | HH1B |  | 10 m |  |  | Q |  |  |
|  | Ala Wai Offshore | Green Channel marker #3 |  | 10 - 12 m |  | AW2S |  | 1 m |  |  | Q |  |  |
|  |  | Lat.-21°16.80 |  |  |  | AW2M |  | 5 m |  |  | Q |  |  |
|  |  | Long.-157°50.20 |  |  |  | AW2B |  | 10 m |  |  | Q |  |  |
|  | Waikiki Offshore | Lat. - 21°16.25 |  | 10 - 12 m |  | W2S |  | 1 m |  |  | Q |  |  |
|  |  | Long. - 157°50.2 |  |  |  | W2M |  | 5 m |  |  | Q |  |  |
|  |  |  |  |  |  | W2B |  | 10 m |  |  | Q |  |  |
| Deep Ocean Sites |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Barbers Point | Lat. - 21°16.50 |  | 60 m |  | BP1S |  | 3 m |  |  | Q |  |  |
|  |  | Long. - 158°06.40 |  |  |  | BP1M |  | 35 m |  |  | Q |  |  |
|  |  |  |  |  |  | BP1B |  | 45 m |  |  | Q |  |  |
|  | Diamond Head | Lat. - 21°13.26 |  | 500 m |  | E4S |  | 1 m |  |  | M |  |  |
|  |  | Long. - 157°48.58 |  |  |  | E4M |  | 35 m |  |  | M |  |  |
|  |  |  |  |  |  | E4B |  | 70 m |  |  | M |  |  |

*Q-Sampled Quarterly	M-Sampled Monthly	S-Sampled Infrequently

NA-Not Applicable

Table 1.  Mamala Bay Study Monitoring Sample Sites

|  | Site | Description |  | Site Depth |  | Code |  |  | Sample Depth |  |  | *Sample Times |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Canal/Stream |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ala Wai Canal | Mouth of canal, Ala Moana Bridge |  | 5 m |  | AW1 |  |  | Surface |  |  | M |  |  |
|  |  | McCully bridge |  |  |  | AW-MC |  |  | Surface |  |  | S |  |  |
|  |  | End of University Ave., near Iolani School |  | 1m |  | AW-IOL |  |  | Surface |  |  | S |  |  |
|  | Manoa Stream | University of Hawaii, behind EWC |  | 0 - 1 m |  | MS1 |  |  | Surface |  |  | Q |  |  |
| Beach Sites |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ala Moana Beach | East end |  | 0 - 2 m |  | AM1 |  |  | Surface |  |  | M |  |  |
|  | Ewa Beach | Iroquois Beach |  | 0 - 2 m |  | EW1 |  |  | Surface |  |  | M |  |  |
|  |  | Ewa Beach Park |  | 0 - 2 m |  | EW2 |  |  | Surface |  |  | M |  |  |
|  |  | Oneula Beach Park |  | 0 - 2 m |  | EW3 |  |  | Surface |  |  | S |  |  |
|  | Hanauma Bay Beach Park |  |  | 0 - 2 m |  | HB1 |  |  | Surface |  |  | M |  |  |
|  | Sand Island Beach Park |  |  | 0 - 2 m |  | SB1 |  |  | Surface |  |  | M |  |  |
|  | Queen’s Surf Beach |  |  | 0 - 2 m |  | Q1 |  |  | Surface |  |  | M |  |  |
|  | Waikiki | Waikiki police station |  | 0 - 2 m |  | W1 |  |  | Surface |  |  | M |  |  |
| Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Primary Sewage | Sand Island WWTP |  | NA |  | S1 |  |  | NA |  |  | M |  |  |

*Q-Sampled Quarterly	M-Sampled Monthly	S-Sampled Infrequently

NA-Not Applicable

Table 1 (cont.).  Mamala Bay Study Monitoring Sample Sites

|  | Enterococci | E. coli | Fecal Coliform | C. perfringens | Phage |
| --- | --- | --- | --- | --- | --- |
|  | CFU/100ml |  |  |  |  |  |  | PFU/100ml |
| GM | 983,002 | 14,748,853 | 16,901,763 | 22,344 | 104,726 |
| Range | 300,000 - 2,520,000 | 7,600,000 - 26,000,000 | 9,200,000 - 27,200,000 | 810 - 308,000 | 550 - 2,000,000 |
| N | 13 | 13 | 13 | 13 | 13 |
|  | pH | Salinity | Temperature | Turbidity |  |
|  |  | ppt | °C | ntu |  |
| AVG | 7.08 | 5.1 | 26.2 | 34.8 |  |
| Range | 6.93 - 7.29 | 1.0 - 8 | 24 - 28 | 6.0 - 55.0 |  |
| N | 10 | 10 | 10 | 11 |  |

Table 2.  Sand Island Primary Effluent (S1) Indicator Bacteria and Phage Geometric Means (GM) and Parameter Averages (AVG)

|  |  |  | Enterococci | E. coli | Fecal Coliform | C. perfringens | Phage | pH | Salinity | Temp. | Turbidity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SITE |  | CFU/100ml |  |  |  |  |  |  | PFU/100ml |  | ppt | °C | ntu |
| Outfall | D0B | GM/AVG | 9.0 | 17.5 | 20.8 | 6.1 | 0 | 8.34 | 34.3 | 27.0 | 4.2 |
| Transects |  | Range | 2 - 46 | 9 - 52 | 10 - 71 | 3 - 12 | 0 | 8.22 - 8.57 | 34, 35 | 27 | 2.8 - 6.5 |
|  |  | N | 3 | 3 | 3 | 3 | 2 | 3 | 3 | 1 | 3 |
|  | D0S | GM/AVG | 13.3 | 24.0 | 28.8 | 11.9 | 2.1 | 8.27 | 34.0 | 25.9 | 3.4 |
|  |  | Range | 0 - 188 | 1 - 240 | 0 - 264 | 2.0 - 28 | 0 - 31.6 | 8.04 - 8.60 | 32 - 36 | 24 - 27 | 1.9 - 4.8 |
|  |  | N | 5 | 5 | 5 | 5 | 4 | 4 | 5 | 4 | 5 |
|  | D1B | GM/AVG | 9.2 | 4.6 | 5.7 | 1.1 | 7.2 | 8.25 | 34.4 | 27.1 | 1.4 |
|  |  | Range | 0 - 90 | 0 - 45 | 0 - 160 | 0 - 12 | 0 - 312 | 7.89 - 8.58 | 34, 35 | 25 - 28 | 0.4 - 3.5 |
|  |  | N | 5 | 5 | 5 | 5 | 3 | 4 | 5 | 4 | 5 |
|  | D1M | GM/AVG | 3.1 | 0.6 | 1.2 | 0.6 | 4.2 | 8.36 | 34.7 | 28.0 | 1.2 |
|  |  | Range | 0, 66 | 0, 3 | 0, 9 | 0, 3 | 0, 26 | 8.218.57 | 34, 35 | 28 | 0.3 - 1.7 |
|  |  | N | 3 | 3 | 3 | 3 | 2 | 3 | 3 | 1 | 3 |
|  | D1S | GM/AVG | 8.2 | 8.6 | 4.5 | 0.3 | 13.9 | 8.20 | 34.8 | 26.3 | 1.0 |
|  |  | Range | 0 - 73 | 0 - 8 | 0 - 14 | 0 - 1 | 0, 166 | 7.82 - 8.55 | 34 - 36 | 24 - 28 | 0.4 - 2.5 |
|  |  | N | 5 | 5 | 5 | 5 | 2 | 4 | 5 | 5 | 5 |
|  | D2B | GM/AVG | 29.3 | 92.7 | 161.1 | 7.1 | 11.8 | 8.21 | 34.7 | 27.2 | 1.4 |
|  |  | Range | 0 - 1,630 | 0 - 25,200 | 0 - 32,200 | 0 - 272 | 0 - 887 | 7.60 - 8.58 | 34, 35 | 24 - 29 | 0.3 - 2.8 |
|  |  | N | 9 | 9 | 9 | 9 | 8 | 9 | 9 | 6 | 9 |
|  | D2M | GM/AVG | 3.8 | 2.6 | 4.2 | 1.4 | 1.9 | 8.29 | 34.8 | 26.7 | 1.6 |
|  |  | Range | 0 - 520 | 0 - 3,720 | 0 - 14,800 | 0 - 28 | 0 - 308 | 8.03 - 8.58 | 34 - 36 | 24 - 28 | 0.3 - 2.9 |
|  |  | N | 8 | 8 | 8 | 8 | 7 | 8 | 8 | 5 | 8 |
|  | D2S | GM/AVG | 1.0 | 0.9 | 2.4 | 0.3 | 1.1 | 8.33 | 34.7 | 26.4 | 1.5 |
|  |  | Range | 0 - 16 | 0 - 92 | 0 - 296 | 0 - 1 | 0 - 12.5 | 8.06 - 8.72 | 34 - 36 | 24 - 28 | 0.5 - 2.8 |
|  |  | N | 9 | 9 | 9 | 9 | 8 | 8 | 9 | 8 | 9 |
|  | D3B | GM/AVG | 1.5 | 8.7 | 4.8 | 0.1 | 12.3 | 8.29 | 34.4 | 26.3 | 1.5 |
|  |  | Range | 0 - 52 | 0 - 180 | 0 - 218 | 0 | 0 - 1,930 | 8.06 - 8.57 | 34, 35 | 24 - 28 | 0.4 - 3.1 |
|  |  | N | 5 | 5 | 5 | 5 | 4 | 4 | 5 | 4 | 5 |
|  | D3M | GM/AVG | 3 | 2.6 | 3.9 | 1 | 0 | 8.37 | 35.0 | 27.3 | 1.2 |
|  |  | Range | 0 - 52 | 0 - 11 | 0 - 38 | 0 - 9 | 0 | 8.22 - 8.58 | 35 | 27, 28 | 1.4 - 3.3 |
|  |  | N | 3 | 3 | 3 | 3 | 2 | 3 | 3 | 2 | 3 |
|  | D3S | GM/AVG | 1.0 | 1.3 | 2.4 | 0.5 | 8.0 | 8.29 | 34.2 | 25.9 | 1.2 |
|  |  | Range | 0 - 16 | 0 - 32 | 0 - 156 | 0 - 3 | 0 - 2,860 | 8.03 - 8.57 | 33 - 35 | 24 - 27 | 0.2 - 2.2 |
|  |  | N | 5 | 5 | 5 | 5 | 4 | 4 | 5 | 5 | 5 |
|  | HU1B | GM/AVG | 2 | 13.7 | 13.6 | 6.9 | ND | 8.40 | 35.0 | 24.5 | 0.7 |
|  |  | Range | 0 - 28 | 4 - 44 | 7- 29 | 1 - 40 |  | 8.28 - 8.61 | 35 | 25 | 0.3 - 1.2 |
|  |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 1 | 3 |
|  | HU1M | GM/AVG | 2 | 0.8 | 0 | 2.7 | ND | 8.42 | 35.0 | 24.5 | 1.0 |
|  |  | Range | 0 - 20 | 0, 5 | 0 | 0 - 9 |  | 8.31 - 8.60 | 35 | 25 | 0.4 - 2.0 |
|  |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 1 | 3 |
|  | HU1S | GM/AVG | 0 | 0 | 0 | 0.3 | ND | 8.42 | 34.7 | 24.8 | 1.0 |
|  |  | Range | 0 | 0 | 0 | 0, 1 |  | 8.30 - 8.61 | 33 - 36 | 25 | 0.3 - 2.0 |
|  |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 1 | 3 |

ND-Not Done

Table 3.  Bay Site Indicator Bacteria and Phage Geometric Means (GM) and Parameter Averages (AVG)

|  |  |  | Enterococci | E. coli | Fecal Coliform | C. perfringens | Phage | pH | Salinity | Temp. | Turbidity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SITE |  | CFU/100ml |  |  |  |  |  |  | PFU/100ml |  | ppt | °C | ntu |
| Outfall | HU2B | GM/AVG | 0.8 | 0 | 0.3 | 0.6 | ND | 8.39 | 35.0 | 25.0 | 1.3 |
| Transects |  | Range | 0, 5 | 0 | 0, 1 | 0, 3 |  | 8.28 - 8.58 | 35 | 25 | 0.7 - 1.9 |
| (cont.) |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 1 | 3 |
|  | HU2M | GM/AVG | 0.3 | 0 | 0.3 | 0.3 | ND | 8.39 | 35.0 | 25.0 | 1.0 |
|  |  | Range | 0, 1 | 0 | 0, 1 | 1, 0 |  | 8.26 - 8.60 | 35 | 25 | 0.6 - 1.7 |
|  |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 1 | 3 |
|  | HU2S | GM/AVG | 0.3 | 0 | 0 | 1.9 | ND | 8.37 | 35.0 | 25.5 | 1.1 |
|  |  | Range | 0, 1 | 0 | 0 | 1 - 3 |  | 8.26 - 8.57 | 35 | 25, 26 | 0.5 - 2.2 |
|  |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 2 | 3 |
| Pearl | C1B | GM/AVG | 1.0 | 0.4 | 1.0 | 0 | 0 | 8.21 | 33.5 | 27.0 | 2.9 |
| Harbor |  | Range | 0, 3 | 0, 1 | 1 | 0 | 0 | 8.23, 8.19 | 32, 35 | 26, 28 | 2.6, 3.1 |
| Transect |  | N | 2 | 2 | 2 | 22 | 2 | 2 | 2 | 2 | 2 |
|  | C1M | GM/AVG | 1.2 | 1.4 | 0.7 | 0 | 0 | 8.23 | 35.0 | 28.0 | 1.6 |
|  |  | Range | 0, 4 | 0, 5 | 0, 2 | 0 | 0 | 8.22, 8.24 | 35 | 28 | 1.3, 1.9 |
|  |  | N | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 1 | 2 |
|  | C1S | GM/AVG | 4.3 | 6.1 | 9.3 | 1.6 | 1.9 | 7.88 | 30.5 | 26.3 | 2.9 |
|  |  | Range | 1, 55 | 1, 51 | 1, 62 | 1, 5 | 0, 69.2 | 7.88 | 28, 33 | 24, 27 | 0.7, 3.5 |
|  |  | N | 2 | 2 | 2 | 2 | 2 | 1 | 2 | 2 | 2 |
|  | C2B | GM/AVG | 0 | 1.2 | 1.0 | 0.7 | 0 | 8.24 | 35.0 | 27.0 | 2.7 |
|  |  | Range | 0 | 0, 4 | 0, 3 | 0, 2 | 0 | 8.23, 8.24 | 34, 36 | 26, 28 | 2.5, 2.9 |
|  |  | N | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  | C2M | GM/AVG | 0 | 0.4 | 1.4 | 0 | 0 | 8.23 | 34.5 | 27.5 | 2.3 |
|  |  | Range | 0 | 0, 1 | 0, 5 | 0 | 0 | 8.22, 8.24 | 34, 35 | 27, 28 | 2.2, 2.3 |
|  |  | N | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  | C2S | GM/AVG | 3.2 | 4.2 | 5.6 | 0.4 | 0.7 | 7.94 | 33.2 | 26.1 | 1.6 |
|  |  | Range | 0 - 53 | 0 - 44 | 0 - 52 | 0 - 1 | 0 - 6.6 | 7.40 - 8.20 | 31 - 34 | 23 - 28 | 0.7 - 1.9 |
|  |  | N | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
|  | C3B | GM/AVG | 0 | 0.4 | 0.4 | 0 | 0.1 | 8.31 | 34.7 | 27.3 | 2.3 |
|  |  | Range | 0 | 0 - 2 | 0 - 4 | 0 | 0 - 1 | 8.20 - 8.60 | 34 - 35 | 27 - 28 | 1.4 - 5.7 |
|  |  | N | 7 | 7 | 7 | 7 | 6 | 7 | 7 | 5 | 7 |
|  | C3S | GM/AVG | 0.4 | 0.4 | 0.4 | 0.3 | 0.2 | 8.28 | 34.3 | 26.8 | 1.6 |
|  |  | Range | 0 - 8 | 0 - 7 | 0 - 6 | 0 - 3 | 0 - 3.3 | 8.03 - 8.62 | 33 - 36 | 24 - 28 | 0.7 - 2.9 |
|  |  | N | 9 | 9 | 9 | 9 | 8 | 8 | 9 | 7 | 9 |
| Near | AW2B | GM/AVG | 0 | 0 | 0.3 | 0.3 | 0 | 8.34 | 34.3 | 27.0 | 1.4 |
| Shore |  | Range | 0 | 0 | 0, 1 | 0, 1 | 0 | 8.22 - 8.59 | 34, 35 | 26 - 28 | 0.3 - 3.3 |
| Sites |  | N | 3 | 3 | 3 | 3 | 1 | 3 | 3 | 3 | 3 |
|  | AW2M | GM/AVG | 0.3 | 0.3 | 0.6 | 0 | 0 | 8.34 | 35.0 | 27.0 | 0.8 |
|  |  | Range | 0, 1 | 0, 1 | 0, 1 | 0 | 0 | 8.21 - 8.60 | 35 | 26 - 28 | .3 - 1.5 |
|  |  | N | 3 | 3 | 3 | 3 | 1 | 3 | 3 | 3 | 3 |

ND-Not Done

Table 3 (cont.).  Bay Site Indicator Bacteria and Phage Geometric Means (GM) and 			  Parameter  Averages (AVG)

|  |  |  | Enterococci | E. coli | Fecal Coliform | C. perfringens | Phage | pH | Salinity | Temp. | Turbidity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SITE |  | CFU/100ml |  |  |  |  |  |  | PFU/100ml |  | ppt | °C | ntu |
| Near | AW2S | GM/AVG | 2.8 | 3.9 | 3.2 | 0.7 | 0 | 8.25 | 34.3 | 26.1 | 1.2 |
| Shore |  | Range | 0 - 19 | 0 - 29 | 0 - 48 | 0 - 2 | 0 | 8.01 - 8.56 | 34 - 35 | 24 - 28 | 0.3 - 2.4 |
| Sites |  | N | 5 | 5 | 5 | 5 | 3 | 5 | 5 | 5 | 5 |
| (cont.) | HH1B | GM/AVG | 0 | 0 | 1.3 | 0.8 | ND | 8.36 | 35.0 | 26.3 | 1.0 |
|  |  | Range | 0 | 0 | 1, 2 | 0, 5 |  | 8.23 - 8.59 | 35 | 25, 28 | 0.6 - 1.5 |
|  |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 2 | 3 |
|  | HH1M | GM/AVG | 0 | 0.3 | 0 | 0.4 | ND | 8.34 | 34.7 | 26.3 | 1.0 |
|  |  | Range | 0 | 0, 1 | 0 | 0, 2 |  | 8.20 - 8.59 | 34, 35 | 25, 28 | 0.5 - 1.8 |
|  |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 2 | 3 |
|  | HH1S | GM/AVG | 0 | 0 | 0.2 | 0.2 | ND | 8.33 | 34.8 | 26.0 | 0.7 |
|  |  | Range | 0 | 0 | 0, 1 | 0, 1 |  | 8.23 - 8.58 | 34, 35 | 25 - 28 | 0.5 - 1.2 |
|  |  | N | 4 | 4 | 4 | 4 |  | 4 | 4 | 4 | 4 |
|  | W2B | GM/AVG | 0 | 0 | 0 | 0.6 | 0 | 8.17 | 34.3 | 26.1 | 3.2 |
|  |  | Range | 0 | 0 | 0 | 0, 1 | 0 | 8.25 - 8.58 | 35 | 26, 28 | 0.8 - 2.3 |
|  |  | N | 3 | 3 | 3 | 3 | 1 | 3 | 3 | 3 | 3 |
|  | W2M | GM/AVG | 0.3 | 0 | 0 | 0.3 | 0 | 8.37 | 35.0 | 26.8 | 1.1 |
|  |  | Range | 0, 1 | 0 | 0 | 0, 1 | 0 | 8.27 - 8.57 | 35 | 26, 28 | 0.5 - 2.2 |
|  |  | N | 3 | 3 | 3 | 3 | 1 | 3 | 3 | 3 | 3 |
|  | W2S | GM/AVG | 0.5 | 0.3 | 0.7 | 0.3 | 0 | 8.27 | 34.8 | 26.6 | 1.2 |
|  |  | Range | 0 , 1 | 0 , 1 | 0 , 3 | 0 , 1 | 0 | 8.03 - 8.56 | 34, 35 | 26 - 28 | 0.3 - 2.6 |
|  |  | N | 6 | 6 | 6 | 6 | 4 | 6 | 6 | 5 | 6 |
| Deep | BP1B | GM/AVG | 0.6 | 0.8 | 1.4 | 1.5 | ND | 8.39 | 35.0 | 25.0 | 0.8 |
| Ocean |  | Range | 0, 3 | 0, 5 | 0, 13 | 1, 3 |  | 8.25 - 8.60 | 35 | 25 | .4 - 1.4 |
| Sites |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 1 | 3 |
|  | BP1M | GM/AVG | 0 | 0.7 | 1.1 | 0.6 | ND | 8.40 | 35.0 | 25.0 | 0.8 |
|  |  | Range | 0 | 0, 4 | 0, 8 | 0, 1 |  | 8.29 - 8.59 | 35 | 25 | 0.5 - 1.3 |
|  |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 1 | 3 |
|  | BP1S | GM/AVG | 0 | 0.3 | 0 | 0.3 | ND | 8.38 | 34.3 | 25.5 | 0.9 |
|  |  | Range | 0 | 0, 1 | 0 | 0, 1 |  | 8.28 - 8.56 | 33, 35 | 25, 26 | 0.5 - 1.2 |
|  |  | N | 3 | 3 | 3 | 3 |  | 3 | 3 | 2 | 3 |
|  | E4B | GM/AVG | 0.6 | 0.5 | 0.5 | 0.4 | 0 | 8.28 | 35.0 | 26.5 | 1.8 |
|  |  | Range | 0 - 60 | 0 - 38 | 0 - 48 | 0 - 1 | 0 | 8.05 - 8.52 | 34 - 36 | 25 - 28 | 0.3 - 4.0 |
|  |  | N | 9 | 9 | 9 | 9 | 8 | 8 | 7 | 7 | 9 |
|  | E4M | GM/AVG | 0.5 | 0 | 0.2 | 0.1 | 1.0 | 8.27 | 35.0 | 26.8 | 1.6 |
|  |  | Range | 0 - 10 | 0 | 0 - 4 | 0, 1 | 0 - 19 | 7.87 - 8.60 | 34 - 37 | 25 - 28 | 0.6 - 3.2 |
|  |  | N | 9 | 9 | 9 | 9 | 8 | 9 | 9 | 7 | 9 |
|  | E4S | GM/AVG | 0.5 | 0.2 | 0.1 | 0.3 | 0 | 8.16 | 35.1 | 26.1 | 1.2 |
|  |  | Range | 0 - 19 | 0 - 4 | 0 - 2 | 0 - 4 | 0 | 7.40 - 8.42 | 34 - 37 | 24 - 28 | 0.4 - 2.2 |
|  |  | N | 9 | 9 | 9 | 9 | 8 | 9 | 9 | 8 | 9 |

ND-Not Done

Table 3 (cont.).  Bay Site Indicator Bacteria and Phage Geometric Means (GM) and 			   Parameter  Averages (AVG)

|  |  | Enterococci | E. coli | Fecal Coliform | C. perfringens | Phage | pH | Salinity | Temp. | Turbidity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SITE |  | CFU/100ml |  |  |  |  |  |  | PFU/100ml |  | ppt | °C | ntu |
| AW1 | GM/AVG | 116.6 | 206.3 | 403.3 | 15.9 | 7.0 | 8.01 | 24.4 | 26.1 | 4.4 |
|  | Range | 0 - 88,000 | 8 - 16,400 | 21 - 42,000 | 1 - 192 | 0 - 1,080 | 6.00 - 8.39 | 3.0 - 36 | 22 - 30 | 1.2 - 26.0 |
|  | N | 44 | 44 | 44 | 42 | 30 | 44 | 44 | 35 | 36 |
| MS1 | GM/AVG | 695.3 | 2,358 | 1,718 | 50.8 | 255.4 | 7.75 | 0.2 | 22.9 | 15.4 |
|  | Range | 56 - 18,400 | 508 - 10,400 | 440 - 4,080 | 14 - 320 | 16 - 3,300 | 6.80 - 8.22 | 0 - 2 | 21 - 26 | 1.4 - 47.0 |
|  | N | 10 | 10 | 10 | 10 | 5 | 10 | 10 | 4 | 6 |

Table 4.  Canal/Stream Site Indicator Bacteria Geometric Means (GM) and Parameter Averages (AVG)

| Sample |  |  |  |  | CFU/100ml |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Site | Type | Date | Time | Enterococci | E. coli | Fecal coliform | C. perfringens | pH | Turbidity (NTU) |
| Puddle | Punahou School | School | 12/9/94 | 8:15 | 136,000 | 12,000 | 212,000 | 4,080 | 7.99 | 81 |
|  | Metcalf St. | Residential | 12/9/94 | 8:30 | 208,000 | 16,000 | 804,000 | 1,640 | 7.47 | 60 |
|  | UH (Biomedical Bldg.) | School | 12/9/94 | 9:50 | 16,800 | 12,600 | 332 | 160 | 7.56 | 54 |
|  | Woodlawn Ave | Urban | 12/9/94 | 10:25 | 21,200 | 20,400 | 64 | 400 | 7.99 | 140 |
|  | Manoa Parking Lot | Shopping Center | 12/9/94 | 10:20 | 16,800 | 1,200 | 44,800 | 140 | 7.47 | 23 |
|  | Waimanu St/ Cummins St. | Industrial | 4/3/95 | 14:50 | 1,760 | 15,200 | 34,000 | 3,800 | ND | ND |
|  | Cummins St./Queen St. | Industrial | 4/3/95 | 14:55 | 12,800 | 176,000 | 196,000 | 3,600 | ND | ND |
|  | Kawaihao St./ Kamani St. | Industrial | 4/3/95 | 15:00 | 8,400 | 13,200 | 24,000 | 1,240 | ND | ND |
| Road | Manoa Mkt. Place | Shopping Center | 12/9/94 | 10:10 | 16,800 | 43,200 | 76,000 | 572 | 7.61 | 44 |
| Runoff | Woodlawn Ave | Residential | 12/9/94 | 10:30 | 1,160 | 8,040 | 7,840 | 31 | 9.38 | 21 |
|  | Upper Manoa Rd | Residential | 12/9/94 | 10:40 | 4,800 | 5,360 | 7,600 | 112 | 9.20 | 22 |
|  | *Lower Oahu Ave | Residential | 12/9/94 | 10:50 | 188,000 | 124,000 | 496,000 | 1,400 | 8.39 | 4,300 |
| Roof | UH (Krauss Hall) | School | 12/9/94 | 8:35 | 38,800 | 120 | 2,440 | 8 | 7.82 | 8 |
| Runoff | Residential Roof | Residential | 12/9/94 | 7:30 | 3,520 | 21 | 308 | 5 | 7.70 | 1 |

*near construction site

Table 5.  Land-based Non-Point Sources of Fecal Indicator Bacteria

|  |  | Enterococci | E. coli | Fecal Coliform | C. perfringens | Phage | pH | Salinity | Temp. | Turbidity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SITE |  | CFU/100ml |  |  |  |  |  |  | PFU/100ml |  | ppt | °C | ntu |
| AM1 | GM/AVG | 3.4 | 2.9 | 3.5 | 0.3 | 0.3 | 8.15 | 34.2 | 26.1 | 5.6 |
|  | Range | 0 - 160 | 0 - 660 | 0 - 624 | 0 - 5 | 0 - 100 | 7.40 - 8.54 | 27 - 36 | 20 - 30 | 1.5 - 37.0 |
|  | N | 39 | 39 | 39 | 39 | 27 | 39 | 38 | 38 | 38 |
| EW1 | GM/AVG | 2.2 | 0.8 | 1.1 | 0.5 | 0 | 8.25 | 33.6 | 26.5 | 9.5 |
|  | Range | 0 - 56 | 0 - 7 | 0 - 17 | 0 - 3 | 0 | 8.05 - 8.62 | 32 - 36 | 25 - 29 | 1.5 - 17.0 |
|  | N | 16 | 16 | 16 | 16 | 8 | 16 | 16 | 16 | 16 |
| EW2 | GM/AVG | 1.3 | 0.5 | 1.2 | 1.0 | 0 | 8.27 | 34.1 | 26.6 | 12.9 |
|  | Range | 0 - 39 | 0 - 3 | 0 - 11 | 0 - 16 | 0 | 8.13 - 8.64 | 32 - 36 | 25 - 29 | 1.6 - 39 |
|  | N | 16 | 16 | 16 | 16 | 8 | 16 | 16 | 16 | 16 |
| EW3 | GM/AVG | 0.3 | 0.2 | 0.6 | 0.7 | 0 | 8.00 | 33.8 | 23.0 | 4.5 |
|  | Range | 0 - 2 | 0 - 1 | 0 - 5 | 0 - 3 | 0 | 7.70 - 8.30 | 32 - 35 | 22, 24 | 4.3 - 4.8 |
|  | N | 4 | 4 | 4 | 4 | 1 | 4 | 4 | 2 | 4 |
| HB1 | GM/AVG | 6.1 | 3.3 | 5.9 | 0.6 | 0 | 8.04 | 34.1 | 25.5 | 2.7 |
|  | Range | 0 - 58 | 0 - 23 | 0 - 90 | 0 - 6 | 0 | 7.40 - 8.22 | 32 - 37 | 22 - 28 | 0.7 - 7.4 |
|  | N | 13 | 13 | 13 | 13 | 13 | 12 | 12 | 12 | 12 |
| Q1 | GM/AVG | 2.2 | 1.8 | 2.4 | 0.6 | 0 | 8.15 | 34.5 | 26.3 | 3.9 |
|  | Range | 0 - 55 | 0 - 38 | 0 - 81 | 0 - 9 | 0 | 7.40 - 8.55 | 32 - 36 | 23 - 29 | 1.4 - 10.5 |
|  | N | 32 | 32 | 32 | 32 | 19 | 31 | 31 | 30 | 31 |
| SB1 | GM/AVG | 1.6 | 0.3 | 0.6 | 0.4 | 0.1 | 8.21 | 34.6 | 26.2 | 6.6 |
|  | Range | 0 - 82 | 0 - 4 | 0 - 8 | 0 - 2 | 0, 1 | 7.70 - 8.57 | 33 - 36 | 23 - 29 | 1.2 - 17.0 |
|  | N | 19 | 19 | 19 | 19 | 14 | 18 | 18 | 16 | 18 |
| W1 | GM/AVG | 3.9 | 2.8 | 2.8 | 0.3 | 0.1 | 8.15 | 34.3 | 26.1 | 3.4 |
|  | Range | 0 - 27 | 0 - 88 | 0 - 188 | 0 - 4 | 0, 1 | 7.40 - 8.57 | 32 - 36 | 22 - 29 | 1.2 - 7.9 |
|  | N | 40 | 40 | 40 | 40 | 28 | 38 | 39 | 38 | 39 |

Table 6.  Beach Site Indicator Bacteria and Phage Geometric Means (GM) and Parameter Averages (AVG)

|  | Site | N |  | PO4 (µM) | NO3 (µM) | NH4 (µM) | Si (µM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Outfall Transect | D0B | 2 | GM | 0.12 | 0.12 | 0.23 | 4.03 |
| Sites |  |  | Range | 0.12, 0.12 | 0.08, 0.18 | 0.14, 0.37 | 3.44, 4.71 |
|  | D0S | 4 | GM | 0.17 | 0.36 | 0.59 | 6.93 |
|  |  |  | Range | 0.13-0.2 | 0.14-1.29 | 0.15-1.5 | 3.77-11.3 |
|  | D1B | 4 | GM | 0.10 | 0.10 | 0.12 | 1.96 |
|  |  |  | Range | 0.09-0.11 | 0.07-0.13 | 0.07-0.22 | 1.52-2.53 |
|  | D1M | 2 | GM | 0.08 | 0.01 | 0.09 | 1.55 |
|  |  |  | Range | 0.08, 0.09 | 0.00, 0.01 | 0.04, 0.22 | 1.15, 2.09 |
|  | D1S | 4 | GM | 0.09 | 0.03 | 0.20 | 1.96 |
|  |  |  | Range | 0.08-0.12 | 0.00-0.13 | 0.07-0.48 | 1.16-2.78 |
|  | D2B | 7 | GM | 0.12 | 0.04 | 0.28 | 2.49 |
|  |  |  | Range | 0.04-0.49 | 0.01-0.31 | 0.04-5.31 | 1.49-6.81 |
|  | D2M | 6 | GM | 0.07 | 0.03 | 0.06 | 1.76 |
|  |  |  | Range | 0.04-0.11 | 0.01-0.18 | 0.01-0.18 | 1.16-3.87 |
|  | D2S | 7 | GM | 0.07 | 0.02 | 0.05 | 1.60 |
|  |  |  | Range | 0.05-0.1 | 0.00-0.19 | 0.01-0.22 | 1.09-2.3 |
|  | D3B | 4 | GM | 0.07 | 0.03 | 0.13 | 1.41 |
|  |  |  | Range | 0.05-0.1 | 0.01-0.05 | 0.03-0.51 | 1.17-1.73 |
|  | D3M | 2 | GM | 0.08 | 0.02 | 0.02 | 1.43 |
|  |  |  | Range | 0.08, 0.08 | 0.01, 0.03 | 0.01, 0.06 | 1.16, 1.77 |
|  | D3S | 4 | GM | 0.09 | 0.02 | 0.10 | 1.77 |
|  |  |  | Range | 0.07-0.1 | 0.00-0.27 | 0.01-1.25 | 1.10-2.67 |
|  | HU1B | 2 | GM | 0.06 | 0.02 | 0.07 | 1.56 |
|  |  |  | Range | 0.06, 0.06 | 0.01, 0.06 | 0.05, 0.11 | 1.21, 2.02 |
|  | HU1M | 2 | GM | 0.07 | 0.02 | 0.11 | 1.51 |
|  |  |  | Range | 0.05, 0.09 | 0.01, 0.05 | 0.08, 0.16 | 1.22, 1.86 |
|  | HU1S | 2 | GM | 0.10 | 0.10 | 0.15 | 2.78 |
|  |  |  | Range | 0.08, 0.12 | 0.03, 0.33 | 0.10, 0.24 | 1.98, 3.91 |
|  | HU2B | 2 | GM | 0.08 | 0.05 | 0.11 | 2.04 |
|  |  |  | Range | 0.07, 0.1 | 0.04, 0.06 | 0.11, 0.12 | 1.98, 2.10 |
|  | HU2M | 2 | GM | 0.10 | 0.06 | 0.07 | 2.28 |
|  |  |  | Range | 0.09, 0.11 | 0.04, 0.08 | 0.03, 0.17 | 2.26, 2.31 |
|  | HU2S | 2 | GM | 0.08 | 0.05 | 0.05 | 2.36 |
|  |  |  | Range | 0.08, 0.09 | 0.05, 0.06 | 0.04, 0.06 | 2.28, 2.44 |
| Pearl Harbor | C1B | 2 | GM | 0.12 | 0.19 | 0.29 | 3.54 |
| Transect Sites |  |  | Range | 0.11, 0.14 | 0.15, 0.23 | 0.22, 0.37 | 3.41, 3.68 |
|  | C1M | 2 | GM | 0.12 | 0.05 | 0.09 | 5.00 |
|  |  |  | Range | 0.11, 0.13 | 0.03, 0.08 | 0.09, 0.1 | 4.64, 5.38 |
|  | C1S | 4 | GM | 0.19 | 0.07 | 0.11 | 30.13 |
|  |  |  | Range | 0.14-0.30 | 0.02-1.14 | 0.08-0.17 | 18.70-50.06 |
|  | C2B | 2 | GM | 0.12 | 0.18 | 0.19 | 3.31 |
|  |  |  | Range | 0.11, 0.13 | 0.14, 0.22 | 0.17, 0.22 | 2.49, 4.40 |
|  | C2M | 2 | GM | 0.09 | 0.18 | 0.20 | 2.59 |
|  |  |  | Range | 0.09, 0.10 | 0.13, 0.24 | 0.13, 0.30 | 2.33, 2.87 |
|  | C2S | 4 | GM | 0.17 | 0.25 | 0.19 | 21.00 |
|  |  |  | Range | 0.13-0.25 | 0.05-2.39 | 0.12-0.58 | 15.42-28.96 |
|  | C3B | 5 | GM | 0.11 | 0.31 | 0.24 | 3.55 |
|  |  |  | Range | 0.09-0.13 | 0.21-0.72 | 0.13-0.47 | 2.73-5.12 |
|  | C3S | 7 | GM | 0.16 | 0.56 | 0.13 | 8.99 |
|  |  |  | Range | 0.08-0.31 | 0.15-1.84 | 0.10-0.20 | 2.20-15.96 |
| Nearshore Sites | AW2B | 2 | GM | 0.08 | 0.13 | 0.09 | 1.83 |
|  |  |  | Range | 0.08, 0.09 | 0.12, 0.14 | 0.04, 0.19 | 1.55, 2.17 |
|  | AW2M | 2 | GM | 0.09 | 0.17 | 0.08 | 2.12 |
|  |  |  | Range | 0.08, 0.11 | 0.16, 0.18 | 0.07, 0.10 | 2.00, 2.24 |
|  | AW2S | 5 | GM | 0.13 | 0.53 | 0.19 | 5.32 |
|  |  |  | Range | 0.08-0.17 | 0.10-1.53 | 0.06-0.50 | 1.80-14.23 |
|  | HH1B | 2 | GM | 0.09 | 0.04 | 0.04 | 1.49 |
|  |  |  | Range | 0.08, 0.10 | 0.04, 0.04 | 0.04, 0.05 | 1.07, 2.07 |

Table 7.  Mamala Bay Geometric Mean (GM) and Ranges of Nutrient Concentrations.

|  | Site | N |  | PO4 (µM) | NO3 (µM) | NH4 (µM) | Si (µM) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nearshore Sites | HH1M | 2 | GM | 0.12 | 0.02 | 0.02 | 1.42 |
| (cont.) |  |  | Range | 0.10, 0.14 | 0.01, 0.03 | 0.02, 0.03 | 1.03, 1.96 |
|  | HH1S | 3 | GM | 0.10 | 0.03 | 0.06 | 1.91 |
|  |  |  | Range | 0.07-0.14 | 0.02-0.08 | 0.03-0.12 | 1.14-2.99 |
|  | W2B | 2 | GM | 0.10 | 0.09 | 0.10 | 1.57 |
|  |  |  | Range | 0.07, 0.13 | 0.06, 0.15 | 0.10, 0.10 | 1.33, 1.86 |
|  | W2M | 2 | GM | 0.10 | 0.12 | 0.12 | 1.71 |
|  |  |  | Range | 0.08, 0.13 | 0.04, 0.39 | 0.11, 0.13 | 1.51, 1.94 |
|  | W2S | 5 | GM | 0.11 | 0.13 | 0.17 | 2.11 |
|  |  |  | Range | 0.09-0.14 | 0.05-0.23 | 0.05-0.97 | 1.22-3.65 |
| Deep Ocean | BP1B | 2 | GM | 0.07 | 0.02 | 0.09 | 1.44 |
| Sites |  |  | Range | 0.07, 0.07 | 0.02, 0.03 | 0.09, 0.09 | 1.07, 1.93 |
|  | BP1M | 2 | GM | 0.08 | 0.02 | 0.07 | 1.48 |
|  |  |  | Range | 0.08, 0.08 | 0.02, 0.03 | 0.04, 0.11 | 1.11, 1.98 |
|  | BP1S | 2 | GM | 0.08 | 0.03 | 0.06 | 1.52 |
|  |  |  | Range | 0.07, 0.09 | 0.02, 0.04 | 0.04, 0.09 | 1.15, 2.02 |
|  | E4B | 7 | GM | 0.06 | 0.04 | 0.12 | 1.52 |
|  |  |  | Range | 0.03-0.12 | 0.02-0.27 | 0.05-0.52 | 1.19-2.54 |
|  | E4M | 7 | GM | 0.06 | 0.01 | 0.11 | 1.56 |
|  |  |  | Range | 0.02-0.11 | 0.00-0.03 | 0.05-0.34 | 1.11-3.55 |
|  | E4S | 7 | GM | 0.07 | 0.03 | 0.12 | 1.71 |
|  |  |  | Range | 0.03-0.11 | 0.01-0.11 | 0.05-2.11 | 1.15-4.22 |
| Sewage | S1 | 9 | GM | 57.03 | 2.68 | 983.88 | 555.64 |
|  |  |  | Range | 39.50-93.50 | 0.50-50.00 | 679.00-1397.50 | 484.50-620.00 |
| Canal/Stream | MS1 | 4 | GM | 0.39 | 17.51 | 0.21 | 254.22 |
| Sites |  |  | Range | 0.13-1.63 | 10.97-40.82 | 0.06-1.22 | 178.09-300.54 |
|  | AW1 | 27 | GM | 0.57 | 11.78 | 3.12 | 91.35 |
|  |  |  | Range | 0.23-2.51 | 5.29-55.12 | 1.48-8.35 | 29.92-161.32 |
|  | AW-IOL | 2 | GM | 0.66 | 23.99 | 2.87 | 128.46 |
|  |  |  | Range | 0.55, 0.80 | 23.50, 24.50 | 2.64, 3.11 | 113.09, 145.93 |
|  | AW-MC | 2 | GM | 0.65 | 21.59 | 2.47 | 137.05 |
|  |  |  | Range | 0.55, 0.77 | 14.23, 32.76 | 2.13, 2.87 | 124.41, 150.98 |
| Beach Sites | AM1 | 26 | GM | 0.10 | 0.15 | 0.19 | 2.77 |
|  |  |  | Range | 0.02-0.21 | 0.01-1.54 | 0.02-1.16 | 0.31-4.72 |
|  | EW1 | 11 | GM | 0.19 | 0.37 | 0.25 | 17.54 |
|  |  |  | Range | 0.07-2.72 | 0.01-1.19 | 0.11-1.09 | 9.76-37.48 |
|  | EW2 | 11 | GM | 0.30 | 0.22 | 0.31 | 10.26 |
|  |  |  | Range | 0.13-0.99 | 0.06-0.98 | 0.14-1.97 | 3.33-41.25 |
|  | EW3 | 2 | GM | 0.24 | 1.37 | 0.04 | 6.00 |
|  |  |  | Range | 0.16, 0.35 | 0.59, 3.19 | 0.02, 0.10 | 5.60, 6.42 |
|  | HB1 | 11 | GM | 0.17 | 1.64 | 0.44 | 3.42 |
|  |  |  | Range | 0.08-0.24 | 0.44-8.19 | 0.16-1.13 | 1.78-7.07 |
|  | Q1 | 20 | GM | 0.08 | 0.20 | 0.17 | 1.68 |
|  |  |  | Range | 0.04-0.14 | 0.08-0.50 | 0.04-1.38 | 0.94-6.79 |
|  | SB1 | 13 | GM | 0.12 | 0.28 | 0.12 | 2.93 |
|  |  |  | Range | 0.07-0.17 | 0.14-0.55 | 0.03-0.31 | 1.51-5.19 |
|  | W1 | 30 | GM | 0.09 | 0.44 | 0.12 | 1.78 |
|  |  |  | Range | 0.03-0.14 | 0.11-2.23 | 0.00-0.81 | 0.60-4.79 |

Table 7 (cont.). Mamala Bay Geometric Mean (GM) and Ranges of Nutrient 				Concentrations.

|  |  | CFU/100ml |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PFU/100ml |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Enterococci |  |  | E. coli |  |  | Fecal Coliform |  |  |  | C. perfringens |  |  |  |  | Phage |  |  |  |
| SITE | Conditions | GM | N | GM | N | GM | N |  | GM |  | N |  | GM |  | N |
| AM1 | Wet | 6.3 | 7 | 9.5 | 7 | 13.8 | 7 |  | 0.4 |  | 7 |  | 0.1 |  | 4 |
|  | Dry | 3.0 | 32 | 2.1 | 32 | 2.4 | 32 |  | 0.3 |  | 32 |  | 0.4 |  | 21 |
| AW1 | Wet | 439.1 | 10 | 600.9 | 10 | 1400.4 | 10 |  | 20.7 |  | 10 |  | 18.8 |  | 7 |
|  | Dry | 78.8 | 34 | 150.5 | 34 | 279.5 | 34 |  | 14.7 |  | 32 |  | 6.2 |  | 21 |
| EW1 | Wet | 0.7 | 4 | 0.6 | 4 | 0.6 | 4 |  | 0.7 |  | 4 |  | ND |  | ND |
|  | Dry | 3.0 | 13 | 0.9 | 13 | 1.3 | 13 |  | 0.5 |  | 13 |  | 0 |  | 6 |
| EW2 | Wet | 0.0 | 4 | 0.2 | 4 | 0.2 | 4 |  | 0.7 |  | 4 |  | ND |  | ND |
|  | Dry | 2.0 | 13 | 0.6 | 13 | 1.7 | 13 |  | 1.1 |  | 13 |  | 0 |  | 6 |
| HB1 | Wet | 5.9 | 3 | 2.0 | 3 | 10.1 | 3 |  | 1.8 |  | 3 |  | 0 |  | 3 |
|  | Dry | 6.2 | 10 | 3.8 | 10 | 5.0 | 10 |  | 0.4 |  | 10 |  | 0 |  | 13 |
| MS1 | Wet | 3910.4 | 3 | 4144.9 | 3 | 1844.5 | 3 |  | 83.4 |  | 3 |  | 785.5 |  | 3 |
|  | Dry | 129.5 | 6 | 1851.1 | 7 | 1665.9 | 7 |  | 41.0 |  | 7 |  | 46.7 |  | 2 |
| Q1 | Wet | 1.4 | 4 | 1.1 | 4 | 0.7 | 4 |  | 0.2 |  | 4 |  | 0 |  | 1 |
|  | Dry | 2.3 | 29 | 1.9 | 29 | 2.7 | 29 |  | 0.6 |  | 29 |  | 0 |  | 17 |
| SB1 | Wet | 1.2 | 5 | 0.4 | 5 | 0.8 | 5 |  | 0.6 |  | 5 |  | 0 |  | 3 |
|  | Dry | 1.7 | 17 | 0.2 | 17 | 0.5 | 17 |  | 0.3 |  | 17 |  | 0.1 |  | 9 |
| W1 | Wet | 5.0 | 5 | 5.4 | 5 | 3.3 | 5 |  | 0.1 |  | 5 |  | 0 |  | 2 |
|  | Dry | 3.7 | 36 | 2.6 | 36 | 2.7 | 36 |  | 0.3 |  | 36 |  | 0.1 |  | 24 |
|  |  | pH |  |  | Salinity (ppt) |  |  | Temperature (C) |  |  |  |  | Turbidity (ntu) |  |  |  |  |  |  |
| SITE | Conditions | AVG | N | AVG | N | AVG |  | N |  | AVG |  | N |  |  |  |
| AM1 | Wet | 8.00 | 7 | 32.3 | 6 | 25.3 |  | 6 |  | 8.9 |  | 7 |  |  |  |
|  | Dry | 8.19 | 32 | 34.5 | 32 | 26.2 |  | 32 |  | 4.9 |  | 32 |  |  |  |
| AW1 | Wet | 7.81 | 10 | 18.8 | 10 | 25.3 |  | 9 |  | 6.0 |  | 9 |  |  |  |
|  | Dry | 8.08 | 34 | 26.1 | 34 | 26.3 |  | 29 |  | 4.0 |  | 30 |  |  |  |
| EW1 | Wet | 8.16 | 4 | 33.5 | 4 | 26.3 |  | 4 |  | 7.1 |  | 4 |  |  |  |
|  | Dry | 8.28 | 13 | 33.7 | 13 | 26.6 |  | 13 |  | 10.3 |  | 13 |  |  |  |
| EW2 | Wet | 8.21 | 4 | 33.5 | 4 | 26.4 |  | 4 |  | 10.8 |  | 4 |  |  |  |
|  | Dry | 8.29 | 13 | 34.2 | 13 | 26.7 |  | 13 |  | 13.6 |  | 13 |  |  |  |
| HB1 | Wet | 8.06 | 3 | 34.3 | 3 | 25.7 |  | 3 |  | 3.5 |  | 3 |  |  |  |
|  | Dry | 8.04 | 9 | 34.1 | 9 | 25.4 |  | 9 |  | 2.4 |  | 9 |  |  |  |
| MS1 | Wet | 7.54 | 3 | 0.0 | 3 | 21.8 |  | 2 |  | 28.1 |  | 3 |  |  |  |
|  | Dry | 7.84 | 7 | 0.3 | 7 | 24.0 |  | 2 |  | 2.7 |  | 3 |  |  |  |
| Q1 | Wet | 8.13 | 4 | 34.8 | 4 | 27.0 |  | 4 |  | 3.5 |  | 4 |  |  |  |
|  | Dry | 8.15 | 28 | 34.4 | 28 | 26.1 |  | 27 |  | 3.9 |  | 28 |  |  |  |
| SB1 | Wet | 8.22 | 4 | 34.0 | 4 | 26.5 |  | 4 |  | 6.9 |  | 4 |  |  |  |
|  | Dry | 8.21 | 17 | 34.8 | 17 | 26.1 |  | 15 |  | 6.6 |  | 17 |  |  |  |
| W1 | Wet | 8.06 | 5 | 34.6 | 5 | 26.7 |  | 5 |  | 3.9 |  | 5 |  |  |  |
|  | Dry | 8.16 | 34 | 34.2 | 35 | 26.0 |  | 34 |  | 3.4 |  | 35 |  |  |  |

Table 8.  Bacteria and Phage Geometric Means (GM) and Parameter Averages (AVG) of Beach and Inland Site Samples taken during Wet vs. Dry Weather Conditions

| W1-Waikiki Police Station |  |  |  |  |  |  | # people |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Enterococcus | Fecal Coliform | C. perfringens | Phage | Beach | Water | Temp. | pH | Turbidity |
| Time | CFU/100ml |  |  |  |  | PFU/100ml |  |  | °C |  | ntu |
| 8:00 | 21 | 13 | 3 | 0 | 18 | 6 | 20 | 8.16 | 2.4 |
| 10:00 | 7 | 18 | 1 | ND | 120 | 17 | 21 | 8.14 | 1.8 |
| 12:00 | 3 | 4 | 1 | ND | 437 | 119 | 22 | 8.18 | 2.0 |
| 14:00 | 2 | 1 | 2 | 0 | 494 | 70 | 23 | 8.17 | 2.0 |
| 16:15 | 4 | 1 | 2 | ND | 200 | 32 | 23 | 8.16 | 2.5 |
| 18:10 | 1 | 1 | 1 | ND | 56 | 2 | 22 | 8.29 | 3.5 |
| 20:05 | 3 | 1 | 1 | 0 | 6 | 0 | 21 | 8.23 | 4.8 |
| 22:20 | 1 | 2 | 1 | ND | 4 | 0 | 21 | 8.21 | 7.9 |
| 0:00 | 7 | 3 | 2 | ND | 4 | 0 | 23 | 8.20 | 2.8 |
| 2:00 | 2 | 2 | 1 | 0 | 1 | 0 | 22 | 8.18 | 2.2 |
| 4:00 | 4 | 1 | 1 | ND | 0 | 0 | 22 | 8.14 | 3.2 |
| 6:05 | 2 | 0 | 1 | ND | 2 | 0 | 22 | 8.13 | 1.9 |
| 7:45 | 0 | 1 | 1 | 0 | 10 | 9 | 20 | 8.14 | 1.1 |
| R1-Royal Hawaiian |  |  |  |  |  | # people |  |  |  |  |  |
|  | Enterococcus | Fecal Coliform | C. perfringens | Phage | Beach | Water | Temp. | pH | Turbidity |
| Time | CFU/100ml |  |  |  |  | PFU/100ml |  |  | °C |  | ntu |
| 8:05 | 4 | 2 | 2 | 0 | 16 | 0 | 20 | 8.13 | 4.7 |
| 10:05 | 3 | 3 | 2 | ND | 298 | 9 | 21 | 8.12 | 5.0 |
| 12:10 | 3 | 0 | 1 | ND | 875 | 88 | 22 | 8.16 | 7.3 |
| 14:05 | 2 | 0 | 0 | 0 | 896 | 71 | 23 | 8.10 | 4.6 |
| 16:20 | 5 | 10 | 0 | ND | 600 | 31 | 23 | 8.20 | 4.5 |
| 18:15 | 9 | 4 | 1 | ND | 60 | 5 | 22 | 8.23 | 5.9 |
| 20:10 | 7 | 2 | 0 | 0 | 8 | 4 | 22 | 8.21 | 5.9 |
| 22:25 | 7 | 3 | 2 | ND | 9 | 0 | 21 | 8.20 | 3.2 |
| 0:05 | 6 | 4 | 1 | ND | 10 | 0 | 23 | 8.21 | 5.9 |
| 2:10 | 4 | 3 | 1 | 0 | 1 | 0 | 23 | 8.17 | 5.4 |
| 4:05 | 3 | 2 | 1 | ND | 1 | 0 | 22 | 8.14 | 6.9 |
| 6:00 | 4 | 2 | 1 | ND | 0 | 0 | 22 | 8.14 | 4.9 |
| 7:55 | 4 | 2 | 1 | 0 | 25 | 0 | 20 | 8.12 | 3.9 |
| SH1-Sheraton |  |  |  |  |  | # people |  |  |  |  |  |
|  | Enterococcus | Fecal Coliform | C. perfringens | Phage | Beach | Water | Temp. | pH | Turbidity |
| Time | CFU/100ml |  |  |  |  | PFU/100ml |  |  | °C |  | ntu |
| 8:10 | 5 | 4 | 2 | 0 | 2 | 0 | 20 | 8.17 | 5.2 |
| 14:15 | 1 | 0 | 0 | 0 | 173 | 23 | 23 | 8.15 | 5.6 |
| 20:15 | 2 | 1 | 2 | 0 | 5 | 21 | 21 | 8.20 | 3.8 |
| 2:05 | 4 | 6 | 1 | 0 | 23 | 23 | 23 | 8.16 | 5.5 |
| 8:00 | 1 | 2 | 1 | 0 | 20 | 20 | 20 | 8.12 | 3.4 |

ND-Not Done

Table 9.  Bacteria, Phage and Parameters for Waikiki 24 Hour Study (2/6/95 - 2/7/95)

| W1-Waikiki Police Station |  |  |  |  |  |  | # people |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Enterococcus | Fecal Coliform | C. perfringens | Phage | Beach | Water | Temp. | pH | Turbidity |
| Time | CFU/100ml |  |  |  |  | PFU/100ml |  |  | °C |  | ntu |
| 8:00 | 5 | 5 | 1 | 0 | 25 | 17 | 24 | 8.02 | 3.4 |
| 9:50 | 3 | 1 | 1 | ND | 237 | 58 | 25 | 8.04 | 3.1 |
| 11:55 | 1 | 1 | 1 | ND | 348 | 111 | 26 | 8.08 | 2.7 |
| 14:05 | 2 | 2 | 0 | 0 | 360 | 92 | 26 | 8.05 | 3.9 |
| 16:45 | 0 | 1 | 1 | ND | 330 | 62 | 26 | 8.10 | 4.5 |
| 18:20 | 0 | 8 | 0 | ND | 150 | 16 | 26 | 8.11 | 4.4 |
| 20:25 | 1 | 1 | 1 | 0 | 9 | 1 | 25 | 8.07 | 4.4 |
| 22:25 | 1 | 2 | 1 | ND | 9 | 0 | 25 | 8.07 | 3.9 |
| 0:05 | 3 | 0 | 0 | ND | 17 | 3 | 25 | 8.05 | 4.5 |
| 2:15 | 12 | 10 | 0 | 0 | 4 | 0 | 24 | 8.04 | 3.9 |
| 4:00 | 6 | 2 | 0 | ND | 2 | 0 | 24 | 8.04 | 4.3 |
| 5:35 | 5 | 3 | 0 | ND | 9 | 1 | 24 | 8.04 | 4.6 |
| 7:45 | 4 | 5 | 0 | 0 | 13 | 8 | 24 | 8.02 | 2.1 |
| R1-Royal Hawaiian |  |  |  |  |  | # people |  |  |  |  |  |
|  | Enterococcus | Fecal Coliform | C. perfringens | Phage | Beach | Water | Temp. | pH | Turbidity |
| Time | CFU/100ml |  |  |  |  | PFU/100ml |  |  | °C |  | ntu |
| 8:15 | 18 | 8 | 1 | 0 | 12 | 1 | 24 | 8.05 | 7.2 |
| 10:10 | 3 | 3 | 1 | ND | 316 | 102 | 25 | 8.04 | 3.7 |
| 12:15 | 5 | 34 | 1 | ND | 485 | 123 | 26 | 8.06 | 4.9 |
| 14:05 | 1 | 5 | 0 | 0 | 441 | 146 | 26 | 8.09 | 4.6 |
| 16:35 | 2 | 4 | 1 | ND | 436 | 76 | 27 | 8.08 | 7.0 |
| 18:20 | 3 | 5 | 0 | ND | 106 | 18 | 26 | 8.12 | 7.2 |
| 20:25 | 8 | 3 | 0 | 0 | 19 | 0 | 25 | 8.09 | 4.8 |
| 22:20 | 5 | 1 | 1 | ND | 10 | 0 | 25 | 8.08 | 5.8 |
| 0:10 | 5 | 5 | 0 | ND | 16 | 0 | 25 | 8.10 | 5.0 |
| 2:10 | 12 | 7 | 1 | 0 | 3 | 0 | 24 | 8.08 | 7.3 |
| 3:55 | 27 | 20 | 1 | ND | 2 | 0 | 24 | 8.06 | 12.0 |
| 5:30 | 17 | 11 | 0 | ND | 3 | 1 | 24 | 8.06 | 8.8 |
| 7:55 | 11 | 4 | 0 | 0 | 3 | 2 | 24 | 8.00 | 5.3 |
| SH1-Sheraton |  |  |  |  |  | # people |  |  |  |  |  |
|  | Enterococcus | Fecal Coliform | C. perfringens | Phage | Beach | Water | Temp. | pH | Turbidity |
| Time | CFU/100ml |  |  |  |  | PFU/100ml |  |  | °C |  | ntu |
| 8:05 | 52 | 17 | 1 | 0 | 38 | 9 | 24 | 8.03 | 4.1 |
| 14:10 | 7 | 6 | 0 | 0 | 119 | 32 | 27 | 8.08 | 7.5 |
| 20:20 | 8 | 2 | 0 | 0 | 0 | 0 | 26 | 8.10 | 6.5 |
| 2:05 | 36 | 16 | 0 | 0 | 0 | 0 | 23 | 8.06 | 12.0 |
| 7:50 | 16 | 5 | 0 | 0 | 22 | 2 | 24 | 8.02 | 4.4 |

ND-Not Done

Table 10.  Bacteria, Phage and Parameters for Waikiki 24 Hour Study (3/14/95 - 3/15/95)

MPN/g

SITE	              n		 Fecal coliform                   Enterococcus                   C. perfringens

AW1	              8		           	27  		          439 	       	         3108

(Canal)			     (<1 - 43000)		  (21 - 120000)		(430 - 210000)

IKI		3		 	4		          209		         972

(2 - 9)	 	     (43 - 930)		  (430 - 2300)

AW-IOL	2			37		          314		         1462

(15 - 93) 		    (230 - 430)		  (930 - 2300)

AW-MC	              2			9		          146		         994

(4 - 23)		     (93 - 230)		  (430 - 2300)

MS1	        	4			609		          494		         172

(93 - 4300)		    (43 - 4300)		   (43 - 2100)

KAL	        	1			230		          230		         9300

KAP	        	1			<1		          230		         230

KL1	        	1			<1		            4		         	         2300

KW1	        	1			<1		            7		         	         150

KW2	        	1			<1		            9	  	        	          43

MOA	1			2		            93		          230

NUU	        	1			15		            43		          930

n = number of samples collected.

Table 11.  Geometric mean density of fecal coliform, enterococcus, and C. perfringens recovered from sediments from Ala Wai canal, Ilikai Harbor, and selected stream sites of Oahu. (The range of bacterial indicator concentration is indicated in parentheses).

MPN/g

SITE	        	        n			Fecal coliform	             Enterococcus          C. perfringens

W2	        	        3			          <1		          1			1

(<1)		    (<1 - 3)	        (<1 - 4)

AW2	 	        4			            1		          	          3			1

(<1 -12)		    (<1 - 43)	        (<1 - 4)

D0		        4			            1		          	         42		              178

(<1 - 4)		    (4 - 460)	        (23 - 640)

D1		        3			            6		          	         9			13						       (2 - 12)		     (1 - 43)	          (9 - 21)

D2 	        	        3			            78		        185			234

(23 - 240)	                  (23 - 930)               (15 - 1860)

C1		        3			          <1		        25			20

(<1)	                   (2 - 930)	         (4 - 230)

C2		        2			          <1		        21			101

(<1)	                    (2- 240)	         (93 - 110)

C3		        1			          <1		       <1			43

n = number of samples collected.

Table 12.  Geometric mean density of fecal coliform, enterococcus, and C. perfringens recovered from sediments at offshore/deep ocean sites of Oahu. (The range of bacterial indicator concentration is indicated in parentheses).

MPN/g

SITE	                      n			Fecal coliform	             Enterococcus          C. perfringens

HB1        	        1				6		         62			28

HB2	        	        1				28		         28			1533

HB3	        	        1				153		         15			153

HB4	       	        1				28		         62			1000

HB6	        	        1				50		         28			286

HB7	        	        1				<1		         <1			28

HZ		        1				50		         28			140

n = number of samples collected.

Table 13.  Concentration of fecal coliform, enterococcus, and C. perfringens recovered from sediments from Barbers Point ocean outfall sites.

| Sample type | Bacteria (CFU/100ml) |  |  |  | FRNA Phage |
| --- | --- | --- | --- | --- | --- |
|  | Fecal coliforms | E. coli | Enterococci | C. perfringens | (PFU/100ml) |
| Raw | 3.20x107 | 2.72x107 | 2.77x105 | 7.14x104 | 4.32x106 |
| Primary treated | 3.19x107 | 1.52x107 | 2.96x105 | 7.96x104 | 1.57x106 |
| Secondary treated | 1.39x105 | 1.39x105 | 1.66x104 | 5.75x102 | 8.51x103 |
| Post-chlorinated | 8 | 7 | 11 | 1.78x102 | 2.03x103 |

Table 14.  Geometric means of concentration of indicator bacteria and FRNA phages in sewage.

MAMALA BAY STUDY

# A SUPPLEMENTAL STUDY TO MB-7 SUBPROJECT:
 CHARACTERIZATION OF THE MICROBIOLOGICAL QUALITY
OF WATER IN MAMALA BAY

Principal Investigator:

Roger S. Fujioka

Water Resources Research Center

2540 Dole St., Holmes Hall 283

Honolulu, Hawaii 96822

September 14, 1995

I. PURPOSE OF SUPPLEMENTAL STUDY

The Mamala Bay Study Plan called for the completion and submission of all project reports as of May 31, 1995. In response to this request, I submitted my project report entitled, “Characterization of the microbiological quality of water in Mamala Bay”. This Final Report was dated May 31, 1995. Some of the objectives of the MB-7 team  could not be completed before May 31, 1995. Additional studies were conducted after May 31, 1995 to complete these unfinished tasks. The results of these additional studies conducted by the Hawaii Team of MB-7 are reported in this report identified as  “Supplemental Study”.

II. Concentrations of fecal indicators in sewage from Honouliuli WWTP

A. Need and objective for this study. The Sand Island and Honouliuli Wastewater Treatment Plants (WWTP) are the two plants discharging sewage into Mamala Bay. Emphasis and resources were placed on characterizing the effects of sewage from the Sand Island WWTP. Thus, the concentrations of fecal indicators in the primary effluent from Sand Island WWTP were determined and reported in the May 31, 1995 Final Report. However, the concentrations of fecal indicators in the sewage from Honouliuli WWTP were not determined. There is a need to document the concentrations of fecal indicators in the sewage being discharged into Mamala Bay from the Honouliuli WWTP. This is the objective of this portion of the “Supplemental Study”.

B. Methodology and study design. Raw and primary treated sewage from Honouliuli WWTP were obtained on five different days in June of 1995. The concentrations of four of the fecal indicators used in this study were determined using the same methods as described in the Final Report.

C. Results. The results of analyzing each of the samples as well as the calculated arithmetic and geometric means for all samples are summarized in Tables 1 and 2. The geometric means of the four fecal indicators measured (fecal coliform, enterococci, Clostridium perfringens, FRNA phage) in the raw sewage and in the primary treated sewage effluent from Honouliuli WWTP are plotted in Figures 1 and 2. The data (Table 1,2, Figure 1,2) show that the concentrations of all fecal indicators in the raw sewage and in the primary effluent from Honouliuli WWTP are very similar. In the primary effluent which is being discharged into Mamala Bay, the geometric mean concentrations (CFU or PFU/100 ml)  is highest for fecal coliform (25,060,802), followed by FRNA phage (243,852), followed by enterococci (63,134) and lowest for C. perfringens (17,832).

D. Conclusions. Primary treatment essentially separates the settleable solids and the material floating on sewage from the water portion of the raw sewage. In the analysis for fecal indicators, it is the water portion of the raw and primary sewage effluent which is tested. These results show that concentrations of fecal indicators from the water portion of the raw and primary treated sewage are similar. Moreover, the total and relative concentrations of the four fecal indicators in the primary effluent from Honouliuli WWTP are very similar to those obtained from the primary effluent from Sand Island WWTP. Of these four fecal indicators, fecal coliform were found at highest concentrations followed by FRNA phage followed by enterococci and  C. perfringens was at the lowest, relative concentration. For water quality monitoring, these results must be tempered with the findings in the previous Final Report which showed that under Mamala Bay conditions, C. perfringens can be expected to be the most stable followed by FRNA phage followed by enterococci with fecal coliforms being the most unstable or to be inactivated most quickly. These results document the concentrations of fecal indicators being discharged into Mamala Bay from the Honouliuli WWTP via the Barbers Point ocean outfall.These results for the first time document the concentrations of FRNA phage in the sewage effluent from Honouliuli WWTP.

III. Survival of fecal indicators in marine waters under sunlight and dark conditions

A. Need and objective for this study. Data on the survival of fecal indicators in marine water samples from Mamala Bay in the presence and absence of sunlight were reported in the Final Report.  In that study, the Hawaii Team reported that inactivation of fecal indicators in marine waters in the dark was measured in days while most of the fecal bacterial indicators were inactivated within a few hours in the presence of sunlight. It was therefore concluded that sunlight is the natural physical agent most effective in inactivating fecal indicators suspended in marine waters. The relative sensitivity of the fecal indicators  to sunlight inactivation were as follows: coliforms>enterococci>poliovirus>FRNA phage>C. perfringens Meanwhile, other members of the MB-7 teams were able to recover a  variety of bacterial (Salmonella spp., Shigella spp.), viral (human enteric viruses, adenovirus) and protozoan (Giardia spp., Cryptosporidium spp.) pathogens from some water and sediment samples obtained from within and outside of Mamala Bay. The survival of these pathogens as compared to that of fecal indicators under Mamala Bay conditions had not been determined and is an identified need. The objectives of this portion of the Supplemental Study are two fold. First, to determine the survival rate of  E. coli, S. faecalis (enterococcus) and FRNA phage in marine water from Mamala Bay in the absence and presence of sunlight under experimental conditions established by the Hawaii Team previously.  Second, to host MB-7 teams from Florida and Arizona for the purpose of determining the survival rate of various pathogens in marine water samples from Mamala Bay under the same experimental conditions established by the Hawaii Team.

B. Methodology and experimental design. The same methodology and the experimental design as described in the May 31, 1995 Final Report were used to determine the survival of fecal indicators and pathogens. Initially, marine water samples from Mamala Bay were sent to Arizona and Florida so the survival times of their respective pathogens suspended in Mamala Bay water were determined under their laboratory conditions utilizing their established methods.

Upon completion of their preliminary testing at their home laboratories, representatives from each of the laboratories were sent to Hawaii and the survival studies conducted utilizing the same water samples and experimental set up established by the Hawaii Team for fecal indicators. Survival studies were conducted utilizing three types of suspending medium: 1. phosphate buffer as control, 2. clean ocean water (Black Point Beach), 3.  estuary water (Ala Wai Canal). These three types of water samples were placed into 1-2 liter flasks with appropriate concentrations of the fecal indicators or pathogens. For dark experiments, the vessels were kept under laboratory conditions in the absence of sunlight in Holmes Hall at constant temperature of about 24 +/- 2°C, mixing occasionally. Another set of vessels were taken to the roof of Holmes Hall and exposed to sunlight for approximately six to eight hours per day. These vessels were constantly mixed and bathed in cold water bath to prevent the temperature of water in the vessel from exceeding 27 °C. The reaction beakers exposed to sunlight were brought back to the laboratory during non-sunlight hours and brought back into sunlight the following day. Samples were taken from all of these reaction vessels periodically and these samples assayed for concentrations of fecal indicators by the Hawaii Team and for pathogens by members of the Arizona and Florida Teams.

Dr. Ian Pepper from the Arizona Team was responsible for survival of viruses (poliovirus, adenovirus) and salmonella. The Hawaii Team provided Dr. Pepper with the salmonella culture and set up the survival experiments for both the virus and the salmonella experiments. The Hawaii Team also collected the samples for salmonella and virus. Collection of the salmonella samples covered only four days and the samples were assayed  by Dr. Pepper. The collection of samples for virus was demanding as the samples had to be exposed to sunlight conditions (6 hours/day) on a daily basis and then returned to the laboratory conditions the rest of day for a 21 day period. Samples were taken at various intervals but every other day during the last two weeks. The virus samples were frozen and sent to Dr. Gerba’s laboratory for virus assays.

Dana Johnson from the Arizona Team was responsible for survival of giardia. Dana collected her own samples and did her own assays utilizing the phase contrast microscope provided by our laboratory at the University of Hawaii. In addition to the survival studies conducted at the University of Hawaii, Dana conducted an in-situ study at a beach site on Oahu.

Tracie Davis from the Florida Team of Joan Rose was responsible for survival of cryptosporidium. Tracie collected her own samples and did her own assays but required a DIC microscope which was not available at the University of Hawaii. The Hawaii Team obtained an agreement with the Hawaii State Department of Health to allow Tracie Davis to use their facilities and their DIC microscope in exchange for learning  some of the techniques from Tracie.

Considerable resources and effort were provided by the Hawaii Team to ensure the completion of this joint survival study conducted by the MB-7 Teams from Hawaii, Arizona and Florida. The following Hawaii Team members contributed to completing this study: Tuamasaga Unutoa, Arvin Wu, Bunnie Yoneyama, Audrey Asahina, Geeta Rijal, Mike Okura and Mrs. Young.

C. Results. The survival study experiments began on the morning of August 8, 1995  which happened to be an unusually cloudy day. However, since all arrangements were made with all concerned, exposure of samples to sunlight conditions on Day 1 represented a very cloudy day. However, exposure of samples to sunlight conditions on Day 2 and Day 3 were typically sunny days.  As a control on the experimental set up and the conditions of the day, the Hawaii Team completed a survival study by adding E. coli (ATCC 25922), S. faecalis (ATCC 29212) and MS2 phage (prototype FRNA virus)  to marine water samples obtained from Black Point Beach. A portion of this mixture was kept in a glass flask under laboratory conditions in the absence of sunlight while another portion of this mixture was exposed to sunlight conditions approximately six - eight hours a day. The temperature of these mixtures were kept below 27° C.

The results of the survival experiment for fecal bacterial indicators conducted by the Hawaii Team are summarized in Tables 3 and 4 and plotted in Figures 3 and 4. The results show that populations of both E. coli and S. faecalis which were suspended in marine waters and incubated under laboratory conditions (24 °C) in the absence of sunlight were stable through 32 hours. However, when these same samples were exposed to sunlight, the initial culturable population (1.88 X 108 CFU/100 ml of  E. coli on mTEC medium was reduced by approximately 5 logs after only 30 minutes of exposure to sunlight and to undetectable levels (0 CFU/100 ml) after 4 hours of exposure to sunlight. In comparison,  the initial cultural population (8.8 X 107 CFU/100 ml) of S. faecalis was only reduced by approximately 68% after 30 minutes of exposure to sunlight but was reduced to undetectable levels (0 CFU/100 ml) after 4 hours of exposure to sunlight. These results show that S. faecalis, an enterococcus and the current fecal indicator used to establish marine recreational water quality standard is more stable to sunlight inactivation than E. coli, a fecal coliform which is the fecal indicator used in the older recreational water quality standard. However, the results show that both kinds of fecal bacterial indicators are still highly susceptible to the inactivating effect of sunlight and greater than 99.99% of the populations of these fecal bacterial indicators can be expected to be inactivated after 1 to 4 hours of exposure to sunlight.

Under similar conditions, the Hawaii Team determined that populations of MS-2 phage was inactivated at a steady rate in marine waters kept in the absence of sunlight but underwent a much more rapid rate of inactivation when the MS-2 phage was alternatively exposed to sunlight and dark conditions. Under these conditions the MS-2 virus was much more rapidly inactivated during periods of exposure to sunlight showing that  sunlight also has an inactivating effect on MS-2, a FRNA phage. Under the same 4 hours of exposure to sunlight when more than 99.99% of the fecal bacterial indicators (E. coli, S. faecalis) were inactivated, only 90% of the population of fecal viral indicator (MS-2) was inactivated.

D. Conclusions. Human enteric viruses are believed to be the primary etiological agent responsible for the transmission of disease via recreational use of marine water. More recent data suggest that pathogenic protozoans such as giardia or cryptosporidia may also be transmitted via the recreational water route to infect humans. Pathogens such as human enteric viruses and protozoans are known to be much more stable in marine waters than the fecal bacterial indicators such as fecal coliform, E. coli  or enterococci. The results of this study support the proposal that the more stable fecal indicators such as FRNA phage or C. perfringens are more likely to predict the presence of pathogens such as human enteric viruses or protozoans in marine recreational waters. The data presented in this supplemental study document the inactivating conditions of Mamala Bay marine waters in the presence and absence of sunlight to fecal bacterial (E. coli, S. faecalis) and viral indicator (MS-2 phage). The Arizona and Florida Teams determined the survival of pathogens (salmonella, poliovirus, adenovirus, giardia, cryptosporidium) under the same set of conditions. Thus, the results of the inactivation rate for these pathogens can be directly compared to inactivation rates of fecal bacteria and fecal viral indicators under Mamala Bay conditions.

Table 1

Concentrations of Fecal Coliform (FC), enterococci (EN), C.  perfringens  (CP), 
and F-Phage (FP)  in Raw Sewage Samples from 
Honouliuli Wastewater Treatment Plant

¬¾¾¾¾¾¾¾¾¾CFU or PFU/100 ml¾¾¾¾¾¾¾¾®

Dates		      FC		      EN		  CP		FP

6/13/95		65,000,000	2,960,000	4,800	        592,000

6/14/95		88,000,000	148,000	94,000	370,000

6/15/95	72,000,000	296,000	13,200	          280,000

6/19/95	56,000,000	720,000	88,000	180,000

6/20/95	48,000,000	256,000	38,400	340,000

Arithmetic Mean	65, 800, 000	876,000	47,680	             352,400

Geometric Mean   64, 391, 947	473,896	28,890 	             327,255

Table 2
	Concentrations of Fecal Coliform (FC), enterococci (EN), 
C. perfringens  (CP), and F-Phage (FP)  in Primary Treated Sewage from Honouliuli Wastewater Treatment Plant

¬¾¾¾¾¾¾¾¾¾CFU or FU/100ml¾¾¾¾¾¾¾¾¾®

Dates                             FC	                             EN	                     CP	                         FP

6/13/95	32,000,000	460,000	1,840	        450,000

6/14/95	34,000,000	28,800	24,000	104,000

6/15/95	26,000,000	52,000	22,000	470,000

6/19/95		22,400,000	14,000	46,400	140,000

6/20/95			15,600,000	104,000	40,000	280,000

Arithmetic Mean	26,000,000	131,760	26,848	       288,800

Geometric Mean	25,060,802	63,134	17,832	       243,852

Table 3. Stability of E. coli and S. faecalis in marine water in the absence (dark control) and presence of sunlight.

|  |  | CFU/100 ml |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SUNLIGHT |  | DARK CONTROL |  |
| DATE | TIME (hr) | E. coli | S. faecalis | E. coli | S. faecalis |
| 8/8/95 | 0 | 1.88 x 108 | 8.80 x 107 | 2.24 x 108 | 8.00 x 107 |
| Day 1: cloudy | 0.5 | 2.08 x 103 | 2.84 x 107 | ND | ND |
|  | 1 | 82 | 46 | ND | ND |
|  | 2 | 22 | 12 | ND | ND |
|  | 4 | 0 | 0 | ND | ND |
|  | 6 | 0 | 0 | 8.08 x 107 | 1.28 x 108 |
| 8/9/95 | 24 | ND | ND | 1.20 x 108 | 7.60 x 107 |
| Day 2: sunny; few clouds | 32 | ND | ND | 4.80 x 107 | 8.00 x 107 |

NOTE: ND = not done.

Table 4. Stability of MS2 bacteriophage in marine water in the absence (dark control) and presence of sunlight.

|  |  | PFU/100 ml |  |
| --- | --- | --- | --- |
| DATE | TIME (hr) | SUNLIGHT | DARK CONTROL |
| 8/7/95 | 0 | 6.80 x 1010 | 4.34 x 1010 |
| Day 1: cloudy | 1 | 4.28 x 1010 | ND |
|  | 2 | 4.22 x 1010 | ND |
|  | 4 | 5.35 x 1010 | ND |
|  | 6 | 4.20 x 109 | ND |
| 8/8/95 | 24 | 2.11 x 109 | 2.57 x 109 |
| Day 2: sunny; few | 26 | 5.90 x 108 | ND |
| clouds | 28 | 6.40 x 107 | ND |
|  | 30 | 8.23 x 106 | ND |
| 8/9/95 | 48 | 2.53 x 106 | 7.90 x 106 |
| Day 3: sunny; few | 50 | 1.85 x 105 | ND |
| clouds | 52 | 9.00 x 104 | ND |
|  | 54 | 5.8 x 102 | ND |
| 8/10/95 | 75 | 0 | 2.11 x 105 |
| Day 4: sunny |  |  |  |

NOTE: ND = not done.