Table 2.

Concentration of presumptive staph bacteria, salinity, turbidity,  temperature and pH

in selected offshore and deep ocean sites, including Sand Island ocean outfall.

| Site | Code | N | CFU/100 ml | Temperature (0C) | Salinity (ppt) | Turbidity (NTU) | pH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Diamond | E4S | 1 | .6 | 28 | 34 | 0.25 | 8.35 |
| Head | E4M | 1 | 0 | 28 | 35 | 0.30 | 8.39 |
|  | E4B | 1 | 5.4 | 28 | 35 | 0.32 | 8.33 |
| Waikiki - | W2S | 1 | 3.9 | 29 | 35 | 0.65 | 8.30 |
| Offshore | W2B | 1 | .1 | 29 | 35 | 0.70 | 8.32 |
| Ala Wai - | AW2S | 1 | .1 | 31 | 34 | 0.77 | 8.28 |
| Offshore | AW2M | 1 | .8 | 30 | 35 | 0.55 | 8.32 |
|  | AW2B | 1 | .2 | 31 | 35 | 0.51 | 8.29 |
| Sand Island | D1S | 1 | 1.4 | 30 | 35 | 0.71 | 8.28 |
| Outfall - | DIM | 1 | 6.9 | 29 | 35 | 0.64 | 8.38 |
| inshore | D1B | 1 | 6.7 | 29 | 35 | 0.52 | 8.43 |
| Sand Island | D2S | 1 | 4.7 | 29 | 36 | 0.59 | 8.46 |
| Outfall | D2M | 1 | .3 | 28 | 36 | 0.49 | 8.45 |
|  | D2B | 1 | 5.1 | 27 | 36 | 0.47 | 8.43 |
| Sand Island | D3S | 1 | 0 | 28 | 36 | 0.42 | 8.56 |
| Outfall - | D3M | 1 | .5 | 27 | 36 | 0.65 | 8.55 |
| beyond | D3B | 1 | .3 | 26 | 36 | 0.40 | 8.58 |
| Upper - | C1S | 1 | .8 | 29 | 36 | 0.83 | 8.26 |
| Pearl Harbor | C1M | 1 | .7 | 28.5 | 36 | 1.10 | 8.28 |
|  | C1B | 1 | 8 | 28 | 35 | 4.90 | 8.26 |
| Mid - | C2S | 1 | .9 | 29 | 35 | 0.95 | 8.24 |
| Pearl Harbor | C2B | 1 | .4 | 28.5 | 35 | 0.89 | 8.24 |
| Mouth of | C3S | 1 | 39 | 29 | 35 | 1.00 | 8.22 |
| Pearl Harbor | C3B | 1 | .6 | 28 | 36 | 0.84 | 8.18 |
| Keehi | D0S | 1 | 0 | 30 | 35 | 2.20 | 8.21 |
| Lagoon | D0B | 1 | 0 | 29 | 35 | 2.40 | 8.22 |

NOTE:	S  = surface

M = middle

B  = bottom

N = number of samples collected.