MAMALA BAY STUDY

INFECTIOUS DISEASE PUBLIC HEALTH RISK ASSESSMENT

PROJECT MB-10

Principal Investigators:

Robert C. Cooper, Ph.D.

Adam W. Olivieri, Dr. P.H., P.E.

EOA, Inc.

1410 Jackson Street

Oakland, California 94612

AUGUST 31, 1995

(Revision of report dated June 12, 1995)

# EXECUTIVE SUMMARY

The Mamala Bay Study Commission is conducting a comprehensive study of the sources and effects of point and non-point pollution in Mamala Bay. The study will result in recommendations for strategies to reduce pollution levels in Mamala Bay to protect human health and the marine environment. EOA, Inc. (EOA) was retained by PRC Environmental Management, Inc. (PRC) to perform an assessment of the public health risk associated with accidental exposure to microbial pathogens during recreational use of Mamala Bay waters.

The primary objectives of this project were to: 1) apply an existing quantitative microbial risk assessment model to estimate the level of microbial risk associated with recreational exposure to Mamala Bay waters; 2) evaluate how public health risk could change with order of magnitude variations in contribution of pathogen to the swimming/surfing area from sources other than shedding by swimmers/surfers; 3) identify important parameters that impact the risk assessment results.

The risk assessment model used for this project is based closely on models used in infectious disease epidemiology. Advantages of this type of model include that it can be used to integrate and organize diverse data bearing on disease risk, account for immunity to disease, model aspects of the transmission dynamics of the agent in the environment and explicitly acknowledge the uncertainty and variability in the many parameter values characteristic of comprehensive models.

Four microbial pathogens (Giardia lamblia, Cryptosporidium spp., Salmonella spp. and enteroviruses) were selected for inclusion in the risk assessment. A literature search was performed on the pathogens to help parameterize the model. For use as input to the risk assessment model, modeled pathogen concentrations in the recreational use water were provided by the Mamala Bay Study group responsible for fate and transport modeling, HydroQual, Inc. Data from Mamala Bay Study monitoring groups were used to define the rate of direct pathogen shedding by swimmers/surfers. Two Mamala Bay beaches (Ala Moana and Waikiki) were included in the risk assessment. Data on attendance and the numbers of swimmers and surfers using these two beaches were provided to EOA by PRC and used for model parameterization.

Average daily prevalences modeled for disease associated with exposure to the selected pathogens at the two Mamala Bay beaches did not vary significantly from the modeled background prevalences in the population. (Background prevalence is the expected prevalence in the population when exposure to recreational water is not the vehicle of disease transmission).

Increasing the concentration of pathogen from sources other than direct shedding by swimmers and surfers by an order of magnitude did not significantly increase the prevalence of disease in the population above background. The risk assessment modeling results, based on available water quality pathogen monitoring and modeling data, therefore suggest that water quality management strategies designed to prevent additional pathogens from point and non-point sources from reaching the beach would not appear to affect the disease prevalences associated with the selected pathogens. It should be noted that this conclusion is based on the assumption that the uncertainty in the water quality modeling results used as input to the risk assessment model is less than approximately an order of magnitude.

# TABLE OF CONTENTS

EXECUTIVE SUMMARY

TABLE OF CONTENTS

LIST OF FIGURES

LIST OF TABLES

1.0 INTRODUCTION

1.1 Scope of Work

1.2 Objectives

1.3 Project Organization

1.4 Background for Modeling Approach

1.5 Report Organization

Chapter 1.0 - References

2.0 METHODS

2.1 Conceptual Description of Model

2.2 Literature Search for Selected Pathogens

2.3 Analysis and Simulation Approach

2.3.1 Giardia and Ala Moana Beach

2.3.2 Other Microorganism and Beach Combinations

Chapter 2.0 - References

3.0 RESULTS

3.1 Giardia and Ala Moana Beach

3.2 Other Pathogen and Beach Combinations

4.0 CONCLUSIONS

APPENDIX A:  MODEL PARAMETERIZATION

APPENDIX C: BM -- FRACTION OF POPULATION THAT VISITS THE BEACH EACH DAY DURING A GIVEN MONTH OF THE YEAR

APPENDIX D: SM -- FRACTION OF BEACHGOERS THAT SWIM OR SURF EACH DAY DURING A GIVEN MONTH OF THE YEAR

APPENDIX E: ALA MOANA BEACH TIME-VARYING PARAMETERS FROM HYDROQUAL DATA

APPENDIX F: WAIKIKI BEACH TIME-VARYING PARAMETERS FROM HYDROQUAL DATA

# List of Figures

2.1	Model Structure

2.2	Beach and Lifeguard Station Locations

3.1a	Effect of Increasing Water Flow Rate Out of Swimming/Surfing Area (FD) on Prevalence

3.1b	Effect of Increasing Water Flow Rate Out of Swimming/Surfing Area (FD) on Pathogen Concentration

3.2	Average Daily Prevalence for Ala Moana Beach

3.3	Average Daily Prevalence for Waikiki Beach

3.4	Pathogen Concentrations for Ala Moana Beach

3.5	Pathogen Concentrations for Waikiki Beach

A.1	Ala Moana -- Giardia, Rate of Shedding vs Pathogen Concentration

A.2	Ala Moana -- Cryptosporidium, Rate of Shedding vs Pathogen Concentration

A.3	Ala Moana -- Salmonella, Rate of Shedding vs Pathogen Concentration

A.4	Ala Moana -- Enteroviruses, Rate of Shedding vs Pathogen Concentration

A.5	Sampling Range for BM  The Fraction of the Population that Visits the Beach Each Day During a Given Month of the Year  Ala Moana Beach

A.6	Sampling Range for BM  The Fraction of the Population that Visits the Beach Each Day During a Given Month of the Year  Waikiki Beach

A.7	Sampling Range for SM  The Fraction of the Beachgoers that Swim or Surf Each Day During a Given Month of the Year  Ala Moana Beach

A.8	Sampling Range for SM  The Fraction of the Beachgoers that Swim or Surf Each Day During a Given Month of the Year  Waikiki Beach

A.9	WNS  Average Daily Giardia Concentration of Pathogen from Sources Other than Shedding at Ala Moana Beach

A.10	WNS  Average Daily Cryptosporidium Concentration of Pathogen from Sources Other than Shedding at Ala Moana Beach

A.11	WNS  Average Daily Salmonella Concentration of Pathogen from Sources Other than Shedding at Ala Moana Beach

A.12	WNS  Average Daily Enteroviruses Concentration of Pathogen from Sources Other than Shedding at Ala Moana Beach

A.13	WNS  Average Daily Giardia Concentration of Pathogen from Sources Other than Shedding at Waikiki Beach

A.14	WNS  Average Daily Cryptosporidium Concentration of Pathogen from Sources Other than Shedding at Waikiki Beach

A.15	WNS  Average Daily Salmonella Concentration of Pathogen from Sources Other than Shedding at Waikiki Beach

A.16	WNS  Average Daily Enteroviruses Concentration of Pathogen from Sources Other than Shedding at Waikiki Beach

List of Figures (continued)

A.17	FD  Average Daily Flow Rate Out  Ala Moana Beach

A.18	FD  Average Daily Flow Rate Out  Waikiki Beach

A.19	VD  Average Daily Volume  Ala Moana Beach

A.20	VD  Average Daily Volume  Waikiki Beach

# List of Tables

1.1	Project Organization

2.1	Equations, Variables and Parameters

2.2	Parameterization for Giardia lamblia and Ala Moana Beach

2.3	List of Dependent Parameters and Functional Dependence on the Sampled Parameters

2.4	Microorganism-Dependent Parameters for Giardia lamblia

2.5	Microorganism-Dependent Parameters for Cryptosporidium spp.

2.6	Microorganism-Dependent Parameters for Salmonella spp.

2.7	Microorganism-Dependent Parameters for Enteroviruses

3.1	Average Daily Prevalence per 100,000 Statistics for Six Scenarios for Giardia and Ala Moana Beach

3.2	Average Daily Prevalence per 100,000 Statistics for Three Scenarios, Four Pathogens and Two Beaches

A.1	Dose-Response for Giardia lamblia

A.2	Dose-Response for Cryptosporidium

A.3	Dose-Response for Salmonella typhi

A.4	Dose-Response for Rotavirus

A.5	Beaches and Corresponding Lifeguard Stations

# 1.0	INTRODUCTION

The Mamala Bay Study Commission is conducting a comprehensive study of the sources and effects of point and non-point pollution in Mamala Bay. The study will result in recommendations for strategies to reduce pollution levels in Mamala Bay to protect human health and the marine environment. It consists of ten individual projects referred to as MB-1 through MB-10. This study is part of project MB-10. MB-10 involves reviewing existing data and new data collected by other Mamala Bay Study project teams, conducting ecological and human health risk assessments and identifying and ranking alternative water quality management strategies based on the risks to the ecosystem and human health.  One principal concern of project MB-10 is the public health risk associated with accidental exposure to microbial pathogens during recreational use of Mamala Bay waters. EOA, Inc. (EOA) was retained by PRC Environmental Management, Inc. (PRC) to address this concern by performing a microbial risk assessment for Mamala Bay. This report documents the results of the microbial risk assessment.

The work performed for this study is closely tied to work performed by other Mamala Bay Study groups. Part of the input to the microbial risk assessment model was provided by the group responsible for fate and transport modeling, HydroQual, Inc. (HydroQual) of Mamala Bay Study group MB-5. HydroQual provided predicted pathogen concentrations at the beaches over a nine-month modeling simulation period. These estimated concentrations were for pathogens from sources other than shedding by swimmers and surfers in the waters used for recreation. In addition, data from Mamala Bay Study monitoring groups (MB-7) were used to define the rate of direct pathogen shedding by swimmers/surfers. It should also be noted that the microbial risk assessment results will be used in conjunction with the results of the chemical risk assessment performed by PRC as part of project MB-10 to estimate the overall public health risk associated with recreational exposure to Mamala Bay waters and, if necessary, to help develop water quality management strategies.

## 1.1	Scope of Work

EOA's scope of work was comprised of the five major tasks described below.

Task 1:	Literature Search

EOA conducted a literature search for selected pathogens (Giardia lamblia, Cryptosporidium spp., Salmonella spp. and enteroviruses)

Task 2:	Assessment of How to Apply Available Data to Microbial Risk Assessment Model

EOA assessed how to apply data provided by other Mamala Bay study groups as input to a risk assessment model. This included human exposure data from local lifeguard beach surveys and water quality and hydraulic modeling data.

Task 3:	Apply Model

EOA estimated the public health risk for the selected microorganisms and beaches and evaluated how risk could change with order of magnitude variations in contribution of pathogen to the swimming/surfing area from sources other than shedding by swimmers/surfers.

Task 4:	Uncertainty Analysis

EOA performed an uncertainty analysis. Parameters that impact the risk assessment results were identified.

Task 5:	Report

EOA prepared this report which documents the work performed and presents our conclusions and recommendations.

1.2	Objectives

The primary objectives of this project were to: 1) apply an existing quantitative microbial risk assessment model to estimate the level of microbial risk associated with recreational exposure to Mamala Bay waters; 2) evaluate how public health risk could change with order of magnitude variations in contribution of pathogen to the swimming/surfing area from sources other than shedding by swimmers/surfers; 3) identify parameters that impact the risk assessment results.

1.3	Project Organization

Table 1.1 presents the members of the project team and their affiliations and briefly describes each member's role in the project.

## 1.4	Background for Modeling Approach

To perform this study, it was necessary to select a methodology for estimating the risk of waterborne disease transmission. As current exposures to environmental pathogens are generally quite low in industrialized countries, field epidemiology may not produce sufficiently sensitive information for assessing risks associated with exposure to pathogens during recreational use of water.  An alternative to the epidemiological approach is the quantitative estimation of the intensity of human exposure and the probability of human response from this exposure.  This approach is highly developed in assessing cancer risks arising from environmental exposures to chemical agents, and has resulted in a field of study called quantitative risk assessment.

Because environmental risk assessment is subject to a variety of uncertainties, the process is often cast in probabilistic terms.  Moreover, field data are frequently unavailable to quantify some elements of the process, and mathematical modeling is used to bridge these data gaps.  The principal advantage of mathematical modeling in risk assessment applications is that it makes assumptions explicit, including structural mechanisms relating human exposure to pathogens and the public health outcome and quantitative assumptions such as the dose-response relationship.  A mathematical model organizes data and assumptions in a framework leading to quantitative predictions and can be an indispensable tool for decision making.  However, the model itself brings no new data or information to the process.  Thus the biological significance of a model's output is completely dependent on the appropriateness and accuracy of the assumptions used to build the model.

Past attempts to provide a quantitative framework for the assessment of human health risks associated with the ingestion of waterborne pathogens have generally focussed on the probability of individual infection or disease as a result of a single exposure event.  Most models described in the literature are of the same generic form.1-4  They give a point estimate of the probability of a particular exposure leading to infection or disease in a single individual and, except for Dudley's work1, carry little or no information about the uncertainty or variability in this estimate. Much of quantitative risk assessment, in particular, focuses on a point estimate of the response probability, often using worst-case assumptions for exposure and other parameters.  From a public health perspective, the probable number of people infected in an exposed population is more meaningful than the probability of individual infection.  In the past, the probability of individual infection (using worst-case assumptions) has sometimes been multiplied by the population number in an attempt to predict the disease incidence in the population. This may lead to unrealistically high risk forecasts.

The project team took a somewhat different point of view in a risk analysis of waterborne disease carried out for the U.S. Army.5-7  In this work a population perspective was taken and the analysis was carried beyond the risk of infection to an individual by estimating the probability distribution of the number of infected/diseased people in the exposed population.  One feature of the Army model was its probabilistic treatment of dose-response data (i.e., data which provide a quantitative linkage between the number of organisms ingested and the probability of infection or overt disease).  From this model's population perspective, each member of the population received a different dose and also had a different probability of responding to this dose.  The combination of these two factors resulted in each member of the population carrying a different probability of becoming infected or diseased.

In general, the above models assume that the populations are homogeneous and the disease transmission processes static. The risk assessment model used for this project takes advantage of a large literature describing the use of deterministic and stochastic dynamic population models in the study of epidemics.8  These epidemiological models emphasize the importance of the changing immune status of a population over time and are therefore dynamic, requiring a subdivision of the population by susceptibility status.  Thus an epidemiological risk assessment model that accounts for immunity and the transmission dynamics of the system was used for this project.

One central issue in biological risk assessment is how to extract information from biological data, which tends to be highly uncertain and variable.  In particular, the uncertainty and variability of factors affecting infectious disease transmission limit the usefulness of traditional curve-fitting techniques.  An alternative goodness-of-fit procedure that explicitly acknowledged these uncertainties and variabilities was therefore used for this project.  The approach consisted of assigning probability distributions to each parameter, sampling these distributions during Monte Carlo simulations, and using a binary classification to assess the output of each simulation.

## 1.5	Report Organization

Chapter 2.0 presents a conceptual description of the risk assessment model and the methods used to apply the model to examine the risk associated with ingestion of waterborne pathogens during recreational use of Mamala Bay.  The methodology used for addressing the uncertainties in the process is given.  Chapter 3.0 presents the results of the study. The estimated health risks associated with four microorganisms and two beaches are compared. An evaluation of how health risk could change with order of magnitude variations in contribution of pathogen to the swimming/surfing area from sources other than shedding by swimmers/surfers is presented. Using the results of the uncertainty analysis, important parameters that impact the risk assessment results are identified. Chapter 4.0 presents our conclusions.

## Chapter 1.0 - References

1.	Dudely, R.H., K.K. Hekimain, and B.J. Mechalas, "A Scientific Basis for Determining Recreational Water Quality Criteria," Journal of the Water Pollution Control Federation, 48, 2761-2777 (1976).

2.	Fuhs, G.W., "A Probabilistic Model of Bathing Beach Safety," The Science of the Total Environment, 4, 165-175 (1975).

3.	Hass, C.N., "Estimation of Risk Due to Low Doses of Microorganisms: A Comparison of Alternative Methodologies," American Journal of Epidemiology, 55, 573-82 (1983).

4.	Regli, S., J.B. Rose, C.N. Haas, and C.P. Gerba, "Modeling the Risk from Giardia and Viruses in Drinking Water," Journal of the American Water Works Association, 83(11), 76-84 (1991).

5.	Cooper, R.C., A.W. Olivieri, R.E. Danielson, P.G. Badger, R.C. Spear, and S. Selvin, Evaluation of Military Field-Water Quality, Volume 5: Infectious Organisms of Military Concern Associated With Consumption: Assessment of Health Risks and Recommendations for Establishing Related Standards, (Lawrence Livermore National Laboratory, 1986).

6.	Olivieri, A.W. et al., "Risk Assessment of Waterborne Infectious Agents," Proceedings of the International Conference on Development and Application of Computer Techniques to Environmental Studies, Los Angeles (1986).

7.	Olivieri, A.W. et al., "Risk of Waterborne Infectious Illness Associated with Diving in the Point Loma Kelp Beds, San Diego, CA," Proceedings of the ASCE 1989 Specialty Conference on Environmental Engineering, Austin, Texas (1989).

8.	Anderson, R.M., and R. May, Infectious Diseases of Human Dynamics and Control, (Oxford University Press, New York, 1991).

TABLE 1.1
PROJECT ORGANIZATION

| Robert C. Cooper, Ph.D. | Emeritus Professor, University of California at Berkeley, School of Public Health | Principal Investigator |
| --- | --- | --- |
| Adam W. Olivieri, Dr. P.H., P.E. | EOA, Inc. | Project Manager |
| Robert C. Spear, Ph.D. | Professor, University of California at Berkeley, School of Public Health | Technical Advisor |
| Joseph Eisenberg, Ph.D. | University of California at Berkeley, School of Public Health | Project Staff |
| Jonathan I. Konnan, M.S. | EOA, Inc. | Project Staff |
| Edmund Seto, M.S. | University of California at Berkeley, School of Public Health and EOA, Inc. | Project Staff |

# 2.0	METHODS

This chapter describes the methods used to perform the risk assessment.

## 2.1	Conceptual Description of Model

The structure of the risk assessment model is illustrated in Figure 2.1.  The model is composed of five state variables, one output variable and 15 parameters, as summarized in Table 2.1.  Four of the state variables represent the human population, which is divided into four epidemiological groups:

X - susceptible individuals

Y - infectious/asymptomatic individuals

Z - non-infectious/asymptomatic individuals

D - infectious/symptomatic individuals

Individuals in state X are susceptible to infection. For the remaining groups, the terms infectious or non-infectious define whether or not individuals sheds pathogen in their stool, and the terms symptomatic and asymptomatic define whether or not an individual exhibits symptoms of disease. The state variables X, Y, Z and D keep track of the population levels in each group.  The remaining state variable, WS, keeps track of the concentration of pathogen in the water to which the population is exposed associated with direct shedding of pathogen by swimmers and surfers. The movement of individuals from one state to another and the concentration of pathogen are governed by the set of five differential equations shown in Table 2.1.

The rate at which members of the population move from state X to state Y is governed by two factors.  One is the background rate of infection, which accounts for non-outbreak transmission due to exposure routes other than ingestion of water during recreational use.  The second is a dose-response term specific to the scenario under evaluation, which is dependent on the pathogen concentration in recreational water and the amount of water ingested.

Once in state Y, an individual can move in any given time step to either state D or state Z. The rates of these two transitions, represented respectively by the parameters  and , are dependent on each other, i.e., at any given time step an individual in state Y will, with probability of 1, either stay in this state, move to state D, or move to state Z.

Individuals in state D, who show symptoms of disease and shed pathogen, move to state Z at a rate of . Individuals in state Z are asymptomatic and do not shed pathogen. The parameter  is defined as the rate at which symptoms of disease disappear as an individual recovers, i.e., the reciprocal of the duration of symptoms. This definition was chosen because state D is used to calculate average daily prevalence in the population, which is the model output used to assess risk. To minimize the number of state variables, it is assumed that an infectious/symptomatic individual will transition directly to the non-infectious/asymptomatic state.

Individuals in state Z revert back to state X at a rate of . By definition,  is the reciprocal of the period of time for an immune individual to become susceptible, i.e., the rate of immunity loss. Thus it is assumed that non-infectious/asymptomatic individuals in state Z are immune.

In addition to movement of individuals among the epidemiological states, the model also describes the concentration of the waterborne pathogen at the exposure site.  The pathogen may arrive at the exposure site in two ways.  First, individuals in state Y directly shed pathogen into the water used for swimming and surfing at a rate of . Second, pathogen from ocean wastewater outfalls and non-point sources such as the Ala Wai canal may migrate to the recreational waters. The concentration of pathogen in the recreational waters from these sources is given by the parameter WNS and was estimated using water quality modeling data provided by HydroQual.

Assumptions made by the model include the following:

	The period of time that an individual is asymptomatic and infectious is short relative to the duration of the symptomatic and infectious period.

	Background disease transmission occurs independently of the water recreation scenario under study.

	Exposure to pathogen occurs via ingestion of recreational water containing pathogen.

	The population is homogeneous with respect to susceptibility to disease.

To describe the 15 model parameters, 20 pieces of data were required.  Therefore 20 sampling parameters were established, five of which varied with time.  With the exception of the three time-varying sampling parameters that were inputted for each day of the model simulation (WNS, FD and VD), lower and upper bounds were selected for each parameter to account for the variability of the data used to parameterize the model. The ranges for the two other time-varying sampling parameters (BM and SM) were varied for each month of the simulation.

The 20 parameters were sampled from uniform distributions, except for values that spanned three or more orders of magnitude, in which case log uniform sampling was used.  Tables 2.2 lists the 20 sampling parameters and classifies the parameters as biological-, community- or water quality/flow-based parameters.  Four of the 15 model parameters are dependent on sampling parameters. Table 2.3 shows the relationship between the four dependent parameters and the appropriate sampling parameters. Appendix A describes the model parameterization in detail. Appendix B contains pathogen monitoring data provided by Mamala Bay Study group MB-7 used to define the rate of shedding of pathogen by swimmers and surfers (see Appendix A). Appendices C and D give the respective ranges that BM and SM were sampled from for each month of the simulation. Appendices E and F give the values used for WNS, FD and VD for each month of the simulation for each of the two beaches included in the risk assessment.

## 2.2	Literature Search for Selected Pathogens

Four waterborne pathogens were selected to be included in the risk assessment. Giardia lamblia  and Cryptosporidium spp. were selected to represent the protozoan pathogens, Salmonella spp. was selected to represent the bacterial pathogens and enteroviruses were selected to represent the viral pathogens. A literature review was performed on the selected microorganisms to establish ranges of values for appropriate model parameters.  The first step in the search was to review a literature search performed previously as part of the microbial risk assessment for the U.S. Army described in Chapter 1.0. The Army review was then updated by collecting new relevant data.  The emphasis was on recent literature (1980 to the present).  An information retrieval service was used to access four literature data bases: Medline, Water Resources Abstracts, SciSearch and Ei Compendex.  These data bases were selected following a review of readily accessible data bases and the relevant journals which they include.  Medline was the primary data base used; the other three data bases were used to search for articles in two journals not included in Medline but deemed important to this study (Journal of the Water Pollution Control Federation and the American Water Works Association Journal).  Selected relevant key words and the microorganism's name were searched for in titles and author's key word lists, and a list of titles was generated.  The selected key words were:

| virulence | dose | outbreak | mortality |
| --- | --- | --- | --- |
| persistence | pathogenicity | latency | review |
| immune response | indicator organism | morbidity | prevalence |
| shedding | infectivity | epidemiology | vaccine |
| infection | occurrence | incubation | risk assessment |

Relevant abstracts were selected for review from the titles.  The selected abstracts were reviewed and selected articles were then obtained.  The articles were read and data relevant to performing the risk assessment were recorded and summarized.

Ranges for appropriate model parameters were selected for each microorganism using the available data found during the literature review and, when the data were unavailable or incomplete, professional judgement.  The selected ranges are presented in Tables 2.4 through 2.7

Data on the background incidence of disease due to the enteroviruses were not found during the literature search. However, a recent edition of the Federal Register1 gives estimated numbers of cases of disease from foodborne pathogens in the United States during 1992. Most of the cases are associated with foodborne transmission. This report estimates that approximately 4,000,000 cases of bacterial gastrointestinal disease occurred in the United States in 1992. Using 250,000,000 for the population of the United States, this equates to a annual incidence of 1,600 per 100,000. Since Shigella and Yersinia are not included in the Federal Register estimate, we assumed that the total annual incidence of bacterial gastrointestinal disease would be about twice as high, or 3,200 per 100,000. Britton2 states that fifty percent of gastrointestinal disease is due to viruses. We therefore assumed that the annual incidence of viral gastrointestinal disease would also be approximately 3,200 per 100,000. A range of 2,000 to 4,000 was selected around this estimate and used as the background incidence for disease associated with enteroviruses (Table 2.7).

Data on the background incidence of cryptosporidiasis were also not found during the literature search. It was assumed to be the same as the range used for giardiasis, the other disease included in this study caused by a protozoan parasite.

## 2.3	Analysis and Simulation Approach

This study included the four selected pathogens described earlier (Giardia lamblia, Cryptosporidium spp., Salmonella spp. and enteroviruses). Two Mamala Bay beaches were selected for inclusion in the risk assessment: Ala Moana and Waikiki. These were the beaches for which water quality modeling, exposure and pathogen modeling data were available from other Mamala Bay study groups. Figure 2.2 shows the approximate locations provided by HydroQual of the water quality modeling segments which were assumed to comprise the swimming surfing area for their adjacent beaches. Figure 2.2 also shows the approximate locations provided by PRC of lifeguard stations located at the two beaches. Data from these stations were used to estimate the numbers of swimmers and surfers using the beaches for recreation (see Appendix A).

### 2.3.1	Giardia and Ala Moana Beach

One pathogen and beach combination, Giardia and Ala Moana beach, was selected for in-depth exploration. This section describes the analysis and simulation approach used for this combination.

The approach was designed to address the uncertainty and variability in the data used to parameterize the model. In general, biological systems have large variability due to both genetic differences among individuals and environmental factors that are not explicitly modeled.  Standard analytical tools, such as curve-fitting techniques and sensitivity analysis, become less useful when data such as that produced from surveillance of infectious diseases are so variable.  Traditionally, a sensitivity analysis procedure involves selecting a point in the parameter space and perturbing the parameter values about this point.  Unfortunately, in many biological models there is sufficient uncertainty in parameter values to make the selection of any particular parameter set about which to conduct the sensitivity analysis a questionable procedure.  This is particularly true with infectious disease data, which are often hard to quantify. To address this problem, a technique termed Regional Sensitivity Analysis (RSA) was used for this project.

RSA involves describing, a priori, the uncertainty and variability in each model parameter by a probability distribution function.  Multiple simulations called Monte Carlo simulations are run and for each simulation a different set of parameter values is used.  The parameter values are chosen by randomly sampling each parameter from its distribution.  Assigning a bounded uniform distribution to each parameter allowed us to take into account data from various literature sources without bias toward one value or another.

A binary classification algorithm was then applied to each simulation output, in which the simulation output either passes or fails a set of criteria.  The multivariate parameter distribution associated with a pass classification can be analyzed through a variety of statistical procedures to assess parameter sensitivity.  The binary classification is basically a goodness-of-fit criterion based on whether or not the output is representative of the data.  The strength of this approach is that it acknowledges both the uncertainty and variability in parameter values in a structured fashion.  The RSA procedure has now been applied to a variety of problems.3-6

Due to the nature of this study, the approach used was slightly different from previous applications of RSA.  The simulation approach consisted of a six-scenario comparative study, in which the first scenario (the background scenario) used the same binary classification scheme as RSA.  The remaining scenarios then used the calibrated parameter sets obtained from the first scenario to generate a distribution of prevalence levels. The six scenarios will be described later in this section.

Specifically, a classification scheme was used to identify the ten parameter values that describe the background scenario of the model, in which exposure to recreational water is not the vehicle of disease transmission.  Each simulation was classified as acceptable if its output was consistent with available disease incidence data for non-outbreak conditions.  For other scenarios, which will be discussed later in this section, the appropriate remaining parameters that describe human exposure to recreational water were sampled, combined with the valid parameter sets from the background scenario, and used as a model representative of a community exposed to pathogens via recreational water use.  The outputs generated by running the model with this complete parameter set were statistically analyzed to identify parameters whose values strongly influence the magnitude of risk.

For Giardia transmission, surveillance data from non-outbreak conditions in Vermont7 were used to obtain baseline values for the ten of the parameters not associated with recreational water transmission.  The Vermont study found that between 1983 and 1986 the annual incidence rate was 45 cases per 100,000 per year.  Selecting a range around this value, the incidence rate criterion was set at 20 - 60 cases per 100,000 per year.  To calculate the incidence rate from the simulation runs, the following equation was used:

I =   Y365  365 / N

where I is the annual incidence rate,  is the fraction of individuals in state Y who move to state D per day, Y365 is the number of individuals in state Y at day 365, and N is the total population.  This equation assumes that the system is at steady state, a good approximation for these non-outbreak simulations.

Scenario 1:	Background

Using the above criterion, Scenario 1 simulations were performed until 1,000 sets of parameter values were produced consistent with non-outbreak conditions in Vermont. The number 1,000 was selected to produce a body of data sufficiently large for meaningful statistical analysis without making the simulation process unreasonably time-consuming. Since none of the parameters related to exposure to recreational water was required for this scenario, these simulations used only ten of the 20 sampling parameters (X0, T, P, Rand, a, , , , 0 and ). Four of the remaining 11 sampling parameters (WNS, SM, BM and F) were set to zero, which removes their effect on the output of the model and results in the rest of these 11 sampling parameters being mathematically canceled.

Once established, the parameter sets for which a Scenario 1 simulation resulted in an annual incidence of 20-60 were used as a basis to run Scenarios 2 through 6.  Therefore, in the remaining scenarios the ten parameter values were predetermined while the remaining 11 parameters values were obtained by randomly sampling the parameter distributions. Scenarios 2 through 6 are described below and the model parameterization is described in detail in Appendix A.

Scenario 2:	Shedding of Pathogen by Swimmers/Surfers, No Pathogen from Non-Shedding Sources

For this scenario, the only source of pathogen in the surfing/swimming area was direct shedding by swimmers and surfers. WNS, the concentration of pathogen in the swimming/surfing water from sources other than shedding by swimmers and surfers, was therefore set to zero.

Scenario 3:	No Shedding of Pathogen by Swimmers/Surfers, Pathogen from Non-Shedding Sources

For this scenario, pathogen in the swimming/surfing water is only from sources other than shedding by swimmers and surfers. The parameter F, the rate of pathogen shedding per infectious swimmer, was therefore set to zero.

Scenario 4:	Shedding of Pathogen by Swimmers/Surfers, Pathogen from Non-Shedding Sources

For this scenario, pathogen in the swimming/surfing water is from shedding by swimmers and surfers and non-shedding sources. All the model parameters were therefore sampled.

Scenario 5:	Shedding of Pathogen by Swimmers/Surfers, Order of Magnitude Increase in Pathogen from Non-Shedding Sources

For this scenario, pathogen in the swimming/surfing water is from shedding by swimmers and surfers and non-shedding sources, with an order of magnitude increase in pathogen from non-shedding sources. All values of the input parameter WNS, the concentration of pathogen in the swimming/surfing water from sources other than shedding by swimmers and surfers, were therefore multiplied by a factor of ten.

Scenario 6:	Shedding of Pathogen by Swimmers/Surfers, Order of Magnitude Decrease in Pathogen from Non-Shedding Sources

For this scenario, pathogen in the swimming/surfing water is from shedding by swimmers and surfers and non-shedding sources, with an order of magnitude decrease in pathogen from non-shedding sources. All values of the input parameter WNS, the concentration of pathogen in the swimming/surfing water from sources other than shedding by swimmers and surfers, were therefore divided by ten.

Nine-month periods were simulated on a Sun Sparc Station using the MCSim simulation software package.8  The output variable used in the analysis was average daily prevalence, which was defined as the proportion of population that was symptomatic (in state D) calculated for each day of the simulation averaged over the nine-month simulation period. Average prevalence incorporates both the number of cases and the duration of the disease, resulting in a measure of disease intensity, whereas incidence accounts for the number of cases but not the duration of disease.  Average prevalence can be compared with incidence by the following approximation:

P  I  d

where I is the incidence and d is the duration of the disease.

### 2.3.2	Other Microorganism and Beach Combinations

The four selected pathogens and two beaches included in the risk assessment result in eight pathogen/beach combinations. In addition to Giardia and Ala Moana beach simulation approach described above, simulations were performed for the seven other microorganism and beach combinations. The simulation approach was the same as that described above, except that only three of the previously described six scenarios were performed. The three scenarios were Scenario 1 (the background scenario in which recreational use of Mamala Bay waters was not the exposure vehicle), Scenario 4 (pathogen from shedding by swimmers and surfers and from non-shedding sources) and Scenario 5 (shedding of pathogen by swimmers/surfers, order of magnitude increase in pathogen from non-shedding sources).

Microorganism-dependent parameters used in the simulations are given in Tables 2.4 through 2.7. For each of the two beaches, time-varying parameters related to human exposure to the recreational water and water quality associated with non-shedding sources are described in Appendix A.

## Chapter 2.0 - References

1.	Federal Register, Vol. 59, No. 59 / Rules and Regulations, March 28, 1994.

2.	Introduction to Environmental Virology, Britton, G. (John Wiley and Sons, 1980).

3.	Auslander, D.A., "Spatial Effects on the Stability of a Food Limited Moth Population," Journal of the Franklin Institute, 314, 347-365 (1982).

4.	Bois, F.Y., M.T. Smith, and R.C. Spear, "Mechanisms of Benzene Carcinogenesis: Application of a Physiological Model of Benzene Pharmacokinetics and Metabolism," Toxicology Letters, 56 (1991).

5.	Spear, R.C., "Control of DO Level in a River Under Uncertainty." Water Resources Research, 19, 1266-1270 (1983).

6.	Tsai, K.C., and D.M. Auslander, "A Statistical Methodology for the Design of Robust Process Controllers," Transactions of the ASME, Journal of Dynamic Systems, Measurements, and Control, 110, (1988).

7.	Birkhead, G., and R.L. Vogt, "Epidemiologic Surveillance for Endemic Giardia lamblia Infection in Vermont," American Journal of Epidemiology, 129:4, 762-768 (1989).

8.	Maszle, D.R. and F.Y. Bois, "MCSIM: A Monte Carlo Simulation Program - User's Guide" (1993).

9.	Risk Assessment Guidance for Superfund, Volume 1, Human Health Evaluation Manual (Part A), EPA/540/1-89/002, (Office of Emergency and Remedial Response, US Environmental Protection Agency, Washington, DC, 1989).

10.	Superfund Exposure Assessment Manual, EPA/540/1-88/001 (Office of Remedial Response, US Environmental Protection Agency, Washington, DC, 1988).

11.	Jokipii, L., and A.M. Jokipii, "Giardiasis in Travelers: A Prospective Study," Journal of Infectious Disease, 130, 295-299, (1974).

12.	Nash T.E., D.A. Herrington, G.A. Losonsky, and M.M. Levine, "Experimental Human Infections With Giardia lamblia," Journal of Infectious Disease, 156:(6), 974-84 (1987).

13.	Dupont H.L., and P.S. Sullivan, "Giardiasis: The Clinical Spectrum, Diagnosis and Therapy," Pediatric Infectious Disease Journal,  5:1, S31-8 (1986).

14.	Craun, G.F., "Waterborne Outbreaks of Giardiasis" in Waterborne Transmission of Giardiasis, W. Jakubowski and J. C. Hoff, Eds., (U.S. Environmental Protection Agency, Environmental Research Center, Cincinnati, OH, 1978).

15.	Jokipii, A.M., and L. Jokipii, "Prepatency of Giardiasis," Lancet, 1(8021), 1095-1096 (1977).

16.	Wolfe, M.S., "Giardiasis," Clinical Microbiology Review, 5(1), 93-100 (1992).

17.	Greensmith C.T., R.S. Stanwick, B.E. Elliot, M.V. Fast, "Giardiasis Associated With the Use of a Water Slide," Pediatric Infectious Disease Journal, 7(2), 91-4 (1988).

18.	Flanagan, P.A., "Giardia--Diagnosis, Clinical Course and Epidemiology: A Review," Epidemiology of Infection, 109, 1-22 (1992).

19.	Lopez, C.E., A.C. Dykes, D.D. Juranek, S.P. Sinclair, J.M. Conn, R.W. Christie, E.C. Lippy, M.G. Schultz, and M.H. Mires, "Waterborne Giardiasis: A Community-Wide Outbreak of Disease and a High Rate of Asymptomatic Infection," American Journal of Epidemiology, 112(4), 495-507 (1980).

20.	Boreham, P.F., J.A. Upcroft, and P. Upcroft, "Changing Approaches to the Study of Giardia Epidemiology: 1681-2000," International Journal for Parasitology, 20(4), 479-87 (1990).

21.	Ruach, A.M. et al., "Longitudinal Study of Giardia lamblia Infection In a Day Care Center Population,"  Pediatric Infectious Disease Journal, 9(3), 186-189 (1990).

22.	Rose, J.B., C.N. Haas, and S. Regli, "Risk Assessment and Control of Waterborne Giardiasis," American Journal of Public Health, 81(6), 709-713 (1991).

23.	Basic Clinical Parasitology, H.W. Brown and F.A. Neva (Appleton-Century-Crofts, Norwalk, CT, 1983), 5th ed.

24.	Barbout, A.G., C.R. Nichols and T. Fukushima, "An Outbreak of Giardiasis in a Group of Campers," American Journal of Tropical Medical Hygiene, 25, 384-389 (1976).

25.	Adam, R.D., "The Biology of Giardia spp.," Microbiological Reviews, 55:4, 706-32 (1991).

26.	Moore, G.T., W.M. Cross, D. McGuire, C.S. Mollohan, N.N. Gleason, G.R. Healy, and L.H. Newton, "Epidemic Giardiasis at a Ski Resort," New England Journal of Medicine, 298, 402-407 (1969).

27.	Rendtorff, R.C., "The Experimental Transmission of Giardia lamblia among Volunteer Subjects," Waterborne Transmission of Giardiasis, W. Jakubowski and J.C. Hoff, Eds. (U.S. Environmental Protection Agency, Environmental Research Center, Cincinnati, OH, 1978).

28.	Taylor G.D., and W.M. Wenman, "Human Immune Response to Giardia lamblia Infection," Journal of Infectious Disease, 155:1, 137-40 (1987).

29.	Control of Communicable Diseases in Man, A.S. Benenson, Ed. (American Public Health Association, Washington, D.C., 1990), 15th ed.

30.	Jarroll, E.L., A.K. Bingham, and E.A. Meyer, "Effect of Chlorine on Giardia lamblia Cyst Viability," Applied Environmental Microbiology, 41, 483-487 (1981).

31.	Rendtorff, R.C., and C.J. Holt, "The Experimental Transmission of Human Intestinal Protozoan Parasites, IV. Attempts to Transmit Entamoeba Coli and Giardia lamblia Cysts by Water," American Journal of Hygiene, 60, 327-338 (1954).

32.	Preliminary Risk Assessment for Parasites in Municipal Sewage Sludge Applied to Land, Hadden, C.T. et al., (U.S. Environmental Protection Agency, Environmental Criteria and Assessment Office, Cincinnati, OH, 1991).

33.	Regli, S., J.B. Rose, C.N. Haas, and C.P. Gerba, "Modeling the Risk from Giardia and Viruses in Drinking Water," Journal of the American Water Works Association, 83(11), 76-84 (1991).

34.	Rendtorff, R.C., "The Experimental Transmission of Human Intestinal Protozoan Parasites, II. Giardia lamblia Cysts Given in Capsules," American Journal of Hygiene, 59, 209-220 (1954).

35.	Zu, S.X., and R.L. Guerrant, "Cryptosporidiosis," Journal of Tropical Pediatrics, 39:3, 132-6 (1993).

36.	Robertson, L.J., and H.V. Smith, "Cryptosporidium and Cryptosporidiasis, Part I: Current Perspective and Present Technologies," European Microbiology, 20-29 (1992).

37.	Flanigan, T.P., and R. Soave, "Cryptosporidiosis," Progress in Clinical Parasitology, 3, 1-20 (1993).

38.	Lacroix, C. et al., "Cryptosporidium Oocysts in Immunocompetent Children: Epidemiologic Investigations in the Day-Care Centers of Poitiers, France," European Journal of Epidemiology, 3:4, 381-5 (1987).

39.	Crawford, F.G. et al., "Asymptomatic Cryptosporidiosis in a New York City Day Care Center," Pediatric Infectious Disease Journal, 7:11, 806-7 (1988).

40.	Diers, J. and G.L. McCallister, "Occurrence of Cryptosporidium in Home Daycare Centers in West-Central Colorado," Journal of Parasitology, 75:4, 637-8 (1989).

41.	Stehr-Green, J.K. et al., "Shedding of Oocysts in Immunocompetent Individuals Infected With Cryptosporidium," American Journal of Tropical Medicine and Hygiene, 36:2, 338-42 (1987).

42.	Current, W.L., and L.S. Garcia, "Cryptosporidiosis," Clinics in Laboratory Medicine, 11:4, 873-97 (1991).

43.	Dupont, Herbert L. et al, "The Infectivity of Cryptosporidium parvum in Healthy Volunteers," New England Journal of Medicine, 332(13), 855-9 (1995).

44.	Sprinz, H., E.Z. Gangarosa, M. Williams, R.B. Hornick, and T.E. Woodward, "Histopathology of the Upper Small Intestines in Typhoid Fever," American Journal of Digestive Disease, 11, 615-624 (1966).

45.	Blaser, M.J., and L.S. Newman, "A Review of Human Salmonellosis: I. Infective Dose," Review of Infectious Disease, 4, 1096-1106 (1982).

46.	Naylor, G.R.E., "Incubation Period and Other Features of Foodborne and Waterborne Outbreaks of Typhoid Fever in Relation to Pathogenesis and Genetics of Resistance," Lancet, 1:8329, 864-866 (1983).

47.	Chalker, R.B. and M.J. Blaser, "A Review of Human Salmonellosis, III: Magnitude of Salmonella Infection in the United States," Review of Infectious Diseases, 10, 111-124 (1988).

48.	Kantele, A., J.M. Kantele, H. Arvilommi and P.H. Makela, "Active Immunity is Seen as a Reduction in the Cell Response to Oral Live Vaccine," Vaccine, 9, 428-431 (1991).

49.	Sanitation and Disease: Health Aspects of Excreta and Wastewater Management, Feacham, R.G. et al. (John Wiley and Sons, New York, N.Y., 1983).

50.	A Collaborative Report, "A Waterborne Epidemic of Salmonellosis in Riverside, California, 1965, Epidemiologic Aspects," American Journal of Epidemiology, 93, 33-48 (1971).

51.	Feldman, R.E., W.B. Baine, J.L. Nitzkin, M.S. Saslaw, and R.A. Pollard, "Epidemiology of Salmonella typhi Infection in a Migrant Labor Camp in Dade County, Florida," Journal of Infectious Disease, 130, 334-342 (1974).

52.	Gamble, D.R., "Viruses in Drinking Water: Reconsideration of Evidence for Postulated Health Hazard and Proposals for Virological Standards of Purity," Lancet, 8113, 425-428 (1979).

53.	Lo, S., J. Gilbert, and F. Hetriclc, "Stability of Human Enteroviruses in Estuarine and Marine Waters," Applied Environmental Microbiology, 32, 245-249 (1976).

54.	Katzenelson, E., "Survival of Viruses," in Indicators of Viruses in Water and Food, G. Berg, Ed. (Ann Arbor Publishers, Inc., Ann Arbor, MI, 1978).

55.	Fujioka, R.S., P.C. Loh and L.S. Lau, "Survival of Human Enteroviruses in the Hawaiian Ocean Environment: Evidence for Virus-Inactivating Microorganisms," Applied and Environmental Microbiology, 39:6, 1105-1110 (1980).

56.	Ward, R.L., D.I. Bernstein, E.C. Young, J.R. Sherwood, D.R. Knowlton and G.M. Schiff, "Human Rotavirus Studies in Volunteers: Determination of Infectious Dose and Serological Response to Infection," Journal of Infectious Disease, 154, 871-880 (1986).

2.1	Model Structure

Table 2.1

Equations, Variables and Parameters

Equations

:

State Variables:

X	Number of susceptible individuals

Y	Number of infectious/asymptomatic individuals

Z	Number of non-infectious/asymptomatic individuals

D	Number of infectious/symptomatic individuals

WS	Concentration of pathogen in swimming/surfing water due to shedding by swimmers and surfers

Output Variable:

W	Concentration of pathogen in swimming/surfing water

Parameters:

	Fraction of individuals in state Y who move to state D per day (day-1)

	Fraction of individuals in state Y who move to state Z per day (day-1)

	Fraction of individuals in state D who move to state Z per day (day-1)

	Fraction of individuals in state Z who move to state X per day (day-1)

	Fraction of individuals in state D who die due to modeled disease per day (day-1)

	Fraction of individuals who die from natural causes per day (day-1)

	Number of pathogen shed per liter of water in swimming/surfing area per day per infectious/asymptomatic individual (day-1  liter-1)

0	Baseline transmission rate (day-1)

	Infection rate due to ingestion of pathogen in recreational use water (day-1)

	Fraction of pathogen in recreational use water that become non-viable per day (day-1)

a	Number of new susceptible individuals who migrate into population per day (day-1)

X0	Initial susceptible population and total population

FD	Flow rate out of water from swimming/surfing area (meters3/day)

WNS	Concentration of pathogen in swimming surfing/water from sources other than shedding by swimmers and surfers (pathogen/liter)

VD	Volume of swimming/surfing area (liters)

Table 2.2  
Parameterization for Giardia lamblia
and Ala Moana Beach

| Sampled Parameter | Definition | Range of Sampled Parameter | Units of Sampled Parameter | Basis of Sampled Parameter | Dependent Parameter |
| --- | --- | --- | --- | --- | --- |
| Biological Parameters: |  |  |  |  |  |  |  |  |  |  |
| T | Incubation period | 3 - 60 | days | 3-60 day incubation period | ,  |
| P | Fraction of state Y that moves to state D | 0.2 -0.7 |  | 20 - 70% infected develop symptoms | ,  |
| Rand | Fraction of state Y that does not move to state D that moves to state Z per day | 0 - 1 |  | Randomly generated number from 0 to 1 |  |
|  | Rate of movement from state Z to state X | 5.6e-3 - 0.033 | day-1 | Reciprocal of estimated time for an immune person to become susceptible (1-6 months) |  |
|  | Rate of movement from state D to state Z | 0.01 -0.2 | day-1 | Reciprocal of duration of symptoms (5-100 days) |  |
|  | Fraction in state D who die due to disease per day | 0 | day-1 | 0% case-fatality due to disease |  |
| a | Rate of migration of new susceptible individuals into population | 6.85e-5 - 9.59e-5 | day-1 | Birth rate |  |
|  | Rate of death due to natural causes | 1.37e-5 - 4.11e-5 | day-1 | Death rate |  |
| F | Rate of pathogen shedding per infectious swimmer | 1e5 - 1e8 | pathogen/ hour | See Appendix A |  |
| o | Background transmission rate | 0 - 0.00021 | day-1 | Calibration simulations using yearly incidence of disease of 20-60 per 100,000 |  |
| Exp | Disease transmission function parameter | 0.008 - 0.04 |  | Result of fitting transmission function to dose response data (see Appendix A) |  |
|  | Rate of pathogen die-off | 4.2e-3 - 0.01 | day-1 | Reciprocal of estimated persistence of cysts in ocean water (1-30 days) |  |
| Community Parameters: |  |  |  |  |  |  |  |  |  |  |
| Xo | Initial number of individuals in state X | 800,000 - 932,100 |  | Population of Honolulu |  |
| BM | Fraction of population that visits the beach each day during a given month of the year | Time Varying (see Figure A.5) |  | Lifeguard station data | ,  |
| SM | Fraction of beachgoers that swim or surf each day during a given month of the year | Time Varying (see Figure A.7) |  | Lifeguard station data | ,  |
| l | Rate of water ingestion during swimming | 0.03 - 0.05 | liters/hour | Assumption that 30-50 ml/hour of water is ingested during swimming9 |  |
| Ts | Number of hours swimming or surfing per day | 2 - 4 | hours/day | National average of 2.6 hours swimming per day10 | ,  |
| Water Quality/Flow Parameters: |  |  |  |  |  |  |  |  |  |  |
| WNS | Average daily concentration of pathogen in the water in the swimming/surfing area from sources other than shedding by swimmers/surfers | Time-Varying (see Figure A.9) | pathogen/ liter | Water quality modeling data from HydroQual |  |
| FD | Flow rate out of water from the swimming/surfing area | Time-Varying (see Figure A.17) | liters/day | Hydraulic modeling data from HydroQual |  |
| VD | Volume of swimming/surfing area | Time-Varying (see Figure A.19) | liters | Hydraulic modeling data from HydroQual |  |

Table 2.3
  List of Dependent Parameters and the Functional Dependence on the Sampled Parameters

| Dependent Parameter | Description | Relation to Sampled Parameters |
| --- | --- | --- |
| r | Fraction of individuals in state Y who move to state D per day | r = rP / rT |
| a | Fraction of individuals in state Y who move to state Z per day | a = aRand × (1 - r) |
| b | Infection rate due to ingestion of pathogen in recreational use water | b = [1 - exp(d × -bExp)]BM × SM where d = W × bI × bTs |
| l | Number of pathogen shed per liter of water in swimming/surfing area per day per infectious/asymptomatic individual | l = (lF × BM × SM × bTs) / VD |

Table 2.4

Microorganism-Dependent Parameters for Giardia lamblia

| Sampled Parameter | Definition | Range of Sampled Parameter | Units of Sampled Parameter | Basis of Sampled Parameter | Dependent Parameter | References |
| --- | --- | --- | --- | --- | --- | --- |
| rT | Incubation period | 3 - 60 | days | 3-60 day incubation period | r | 11-17 |
| rP | Fraction of state Y that moves to state D | 0.2 - 0.7 |  | 20 - 70% infected develop symptoms | r | 13, 18-22 |
| g | Rate of movement from state D to state X | 5.6e-3 - 0.033 | day-1 | Reciprocal of estimated time for an immune person to become susceptible (1-6 months) | g | Professional Judgement |
| s | Rate of movement from state D to state Z | 0.01 - 0.2 | day-1 | Reciprocal of duration of symptoms (5-100 days) | s | 11, 18, 23-28 |
| d | Fraction in state D who die due to disease per day | 0 | day-1 | 0% case-fatality due to disease (immunocompetent individuals) | d | 29 |
| z | Rate of cyst die-off | 0.033 - 1 | day-1 | Reciprocal of persistence of cysts in ocean water (1-30 days) | z | 30-32 |
| b0 | Background transmission rate | 0 - 0.00021 | day-1 | Calibration simulations using yearly incidence of disease of 20-60 per 100,000 | b0 | 7 |
| bExp | Disease transmission function parameter | 0.008 - 0.04 |  | Result of fitting transmission function to dose response data | b | 31, 33, 34 |
| lF | Rate of cyst shedding per infectious swimmer | 1e5 - 1e8 | cysts/hour | See Appendix A | l |  |

Table 2.5

Microorganism-Dependent Parameters for Cryptosporidium spp.

| Sampled Parameter | Definition | Range of Sampled Parameter | Units of Sampled Parameter | Basis of Sampled Parameter | Dependent Parameter | References |
| --- | --- | --- | --- | --- | --- | --- |
| rT | Incubation period | 2-14 | days | 2-14 day incubation period | r | 35-37 |
| rP | Fraction of state Y that moves to state D | 0.8 - 1 |  | 80 - 100% infected develop symptoms | r | 38-41 |
| g | Rate of movement from state D to state X | 5.6e-3 - 0.033 | day-1 | Reciprocal of estimated time for an immune person to become susceptible (1-6 months) | g | Professional Judgement |
| s | Rate of movement from state D to state Z | 0.033 - 0.5 | day-1 | Reciprocal of duration of symptoms (2-30 days) | s | 36, 37, 42 |
| d | Fraction in state D who die due to disease per day | 0 | day-1 | 0% case-fatality due to disease (immunocompetent individuals) | d | 35 |
| z | Rate of oocyst die-off | 5.6e-3 - 1 | day-1 | Reciprocal of estimated persistence of oocysts in ocean water (1 day - 6 months) | z | 29, 32 |
| b0 | Background transmission rate | 0 - 3e-5 | day-1 | Calibration simulations using yearly incidence of disease of 20-60 per 100,000 | b0 | 7 |
| bExp | Disease transmission function parameter | 2.1e-3 - 7.6e-3 |  | Result of fitting transmission function to dose response data | b | 33, 43 |
| lF | Rate of oocyst shedding per infectious swimmer | 10e6 - 10e9 | oocysts/ hour | See Appendix A | l |  |

Table 2.6

Microorganism-Dependent Parameters for Salmonella spp.

| Sampled Parameter | Definition | Range of Sampled Parameter | Units of Sampled Parameter | Basis of Sampled Parameter | Dependent Parameter | References |
| --- | --- | --- | --- | --- | --- | --- |
| rT | Incubation period | 3-22 | days | 3-22 day incubation period | r | 29, 44-46 |
| rP | Fraction of state Y that moves to state D | 0.06 - 0.8 |  | 6 - 80% infected develop symptoms | r | 47 |
| g | Rate of movement from state D to state X | 1.37e-3 - 2.74e-3 | day-1 | Reciprocal of estimated time for an immune person to become susceptible (1-2 years) | g | 48 |
| s | Rate of movement from state D to state Z | 0.033 - 0.33 | day-1 | Reciprocal of duration of symptoms (3 - 30 days) | s | 29 |
| d | Fraction in state D who die due to disease per day | 0 | day-1 | 0% case-fatality due to disease (immunocompetent individuals) | d | 29 |
| z | Rate of pathogen die-off | 0.14 - 0.2 | day-1 | Reciprocal of estimated persistence of pathogen in ocean water (5-7 days) | z | 49 |
| b0 | Background transmission rate | 0 - 5e-4 | day-1 | Calibration simulations using yearly incidence of disease of 10-30 per 100,000 | b0 | 47 |
| bExp | Disease transmission function parameter | 3.6e-6 - 4.8e-5 |  | Result of fitting transmission function to dose response data | b | 33, 45, 46, 50, 51 |
| lF | Rate of pathogen shedding per infectious swimmer | 10e5 - 10e8 | pathogen/ hour | See Appendix A | l |  |

Table 2.7

 Microorganism-Dependent Parameters for Enteroviruses

| Sampled Parameter | Definition | Range of Sampled Parameter | Units of Sampled Parameter | Basis of Sampled Parameter | Dependent Parameter | References |
| --- | --- | --- | --- | --- | --- | --- |
| rT | Incubation period | 2-3 | days | 2-3 day incubation period | r | 49 |
| rP | Fraction of state Y that moves to state D | 0.001 - 0.01 |  | 0.1 - 1% infected develop symptoms | r | 52 |
| g | Rate of movement from state D to state X | 4.21e-5 - 5.48e-4 | day-1 | Reciprocal of estimated time for an immune person to become susceptible (5-65 years) | g | 29 |
| s | Rate of movement from state D to state Z | 0.1 - 0.5 | day-1 | Reciprocal of duration of symptoms (2-10 days) | s | 29 |
| d | Fraction in state D who die due to disease per day | 0 | day-1 | 0% case-fatality due to disease (immunocompetent individuals) | d | 29 |
| z | Rate of pathogen die-off | 0.0125 - 0.33 | day-1 | Reciprocal of estimated persistence of pathogen in ocean water (3-80 days) | z | 53 - 55 |
| b0 | Background transmission rate | 0 - 0.22 | day-1 | Calibration simulations using yearly incidence of disease of 2,000-4,000 per 100,000 | b0 | See Text |
| bBp | Disease transmission function parameter | 0.3 - 2.3 |  | Result of fitting transmission function to dose response data | b | 33, 56 |
| aBp | Disease transmission function parameter | 0.15 - 0.42 |  | Result of fitting transmission function to dose response data | b | 33, 56 |
| lF | Rate of pathogen shedding per infectious swimmer | 10e2 - 10e5 | pathogen/ hour | See Appendix A | l |  |

# 3.0	RESULTS

A simulation study in which a risk assessment model (used as a comparative analysis tool) was designed to estimate the level of public health risk associated with recreational exposure to microbial agents in Mamala Bay waters. This chapter presents the results of the study.

## 3.1	Giardia and Ala Moana Beach

One microorganism and beach combination (Giardia and Ala Moana beach) was selected for in-depth exploration. For this combination six transmission scenarios were compared to analyze the relative risk of contracting waterborne disease. The first scenario described the background in which recreational use of Mamala Bay waters was not the exposure vehicle.  The results were used to establish a baseline prevalence with which to compare the effects of the next five scenarios. The six scenarios were described in Section 2.3.1, and are summarized below:

Scenario 1:	Background - Exposure to Recreational Water Is Not the Vehicle of Disease Transmission

Scenario 2:	Shedding of Pathogen by Swimmers/Surfers, No Pathogen from Non-Shedding Sources

Scenario 3:	No Shedding of Pathogen by Swimmers/Surfers, Pathogen from Non-Shedding Sources

Scenario 4:	Shedding of Pathogen by Swimmers/Surfers, Pathogen from Non-Shedding Sources

Scenario 5:	Shedding of Pathogen by Swimmers/Surfers, Order of Magnitude Increase in Pathogen from Non-Shedding Sources

Scenario 6:	Shedding of Pathogen by Swimmers/Surfers, Order of Magnitude Decrease in Pathogen from Non-Shedding Sources

1,000 nine-month simulations were performed for each scenario. Table 3.1 presents the results. The mean, variance, minimum and maximum values for average daily prevalence modeled for giardiasis at Ala Moana beach for Scenarios 2 through 5 did not vary significantly from Scenario 1, the modeled background prevalence. (Background prevalence is the expected prevalence in the population when exposure to recreational water is not the vehicle of disease transmission).

To determine which parameters played the most important role in determining the prevalence output, a multiple linear regression analysis was performed for Scenarios 1 and 4.  The dependent variable in these regressions was the natural log of the average daily prevalence.

The linear regression for Scenario 1 (background - exposure to recreational water not the vehicle of disease transmission) with all nine parameters included provided a good fit (R2 = 0.82).  The most important determinants of the level of disease prevalence in order of importance were as follows:

	the fraction of individuals in state D who move to state Z per day

0	the background transmission rate

the remaining parameters were much less important in determining the prevalence.  A linear regression with only  and 0 still produced a good fit (R2 = 0.77).

The linear regression for Scenario 4 (shedding of pathogen by swimmers/surfers, pathogen from non-shedding sources) with all thirty-two parameters included provided a good fit (R2 = 0.82).  The most important determinants of prevalence were found to be the same as those in Scenario 1 ( and 0).  A linear regression with only  and 0 still produced a good fit (R2 = 0.77).

The above results are consistent with the fact that the mean, variance, minimum and maximum values for average daily prevalence modeled for Scenario 4 did not vary significantly from Scenario 1, the modeled background prevalence.

Analysis of FD

The parameter FD, the water flow rate out of the swimming/surfing area, is based on modeling and therefore subject to uncertainty and variability.  This motivated a preliminary analysis of the importance of FD in removing pathogen shed by swimmers and surfers and affecting the model output.

To perform the analysis, the parameter sets for Scenario 4 were reused, except for the time-varying parameter FD, which was multiplied by a factor sampled between 0.01 and 100 for each simulation. Figure 3.1a shows the relationship between average daily prevalence and the factor by which FD is multiplied. Figure 3.1b shows the relationship between average daily pathogen concentration in the recreational water and the factor by which FD is multiplied. Examination of these figures reveals that despite a decrease in pathogen concentration with increasing flow rate out, average daily prevalences did not vary and remained at or below the background level of approximately 10 per 100,000.

It should be noted that this analysis was not a sensitivity analysis; rather, it was a simple preliminary analysis to generally illustrate the effect of increasing water flow rate out of the swimming area on the model output (i.e., disease prevalence).

## 3.2	Other Pathogen and Beach Combinations

The four selected pathogens and two beaches included in the risk assessment result in a total of eight pathogen/beach combinations. In addition to the Giardia and Ala Moana beach simulations described above, simulations were performed for the seven other microorganism and beach combinations. For these simulations three of the previously described six scenarios were performed to analyze the relative risk of contracting waterborne disease. The three scenarios were Scenario 1 (the background scenario in which recreational use of Mamala Bay waters was not the exposure vehicle), Scenario 4 (pathogen from shedding by swimmers and surfers and from non-shedding sources) and Scenario 5 (shedding of pathogen by swimmers/surfers, order of magnitude increase in pathogen from non-shedding sources). Table 3.2 and Figures 3.2 and 3.3 present the results for the eight pathogen and beach combinations.

Examination of Table 3.2 and Figures 3.2 and 3.3 reveals that the mean, variance, minimum and maximum values for average daily prevalences modeled for Scenarios 4 and 5 did not vary significantly from Scenario 1, the modeled background prevalences. Since maximum average daily prevalences never significantly exceeded background maximum prevalences, none of the simulations for Scenarios 4 and 5 would be considered an outbreak.

Figures 3.4 and 3.5 present comparisons of the following:

	The maximum pathogen concentration values of the beach monitoring data provided by Mamala Bay Study group MB-7 that were used to define the rate of shedding of pathogen by swimmers and surfers (Appendix B). Detection limits were used as maximum values for Salmonella at both beaches and for enteroviruses at Waikiki, since these organisms were not detected at these locations.

	The daily concentrations of pathogens at the beach due to shedding and non-shedding sources generated by the risk assessment model.

Examination of these figures reveals that the maximum beach concentrations generated by the risk assessment model always exceeded the maximum for the water quality monitoring data. The pathogen concentrations in the swimming area therefore did not appear to be underestimated by the risk model.

Figures 3.1a and 3.1b

The figure above shows the results for Scenario 1 (the background scenario) and Scenario 4 (shedding of pathogen by swimmers and surfers and pathogen from non-shedding sources) for each microorganism.  1,000 simulations were performed for each scenario.  The average daily prevalence per 100,000 was calculated for each simulation.  The mean of the average daily prevalence is shown with the standard deviation indicated with error bars.

The figure above shows the results for Scenario 1 (the background scenario) and Scenario 4 (shedding of pathogen by swimmers and surfers and pathogen from non-shedding sources) for each microorganism.  1,000 simulations were performed for each scenario.  The average daily prevalence per 100,000 was calculated for each simulation.  The mean of the average daily prevalence is shown with the standard deviation indicated with error bars.

Beach monitoring pathogen concentration is the maximum measured pathogen concentration or detection limit, if pathogen was not detected in any samples.  See Appendix B for pathogen monitoring data provided by Mamala Bay Study group MB-7.
Scenario 4 (shedding of pathogen by swimmers/surfers, pathogen from non-shedding sources) pathogen concentration is the mean of the 1,000 average daily pathogen concentrations output from Scenario 4 simulations.  Minimum and maximum average daily pathogen concentrations for the 1,000 simulations are shown with error bars.

Beach monitoring pathogen concentration is the maximum measured pathogen concentration or detection limit, if pathogen was not detected in any samples.  See Appendix B for pathogen monitoring data provided by Mamala Bay Study group MB-7.
Scenario 4 (shedding of pathogen by swimmers/surfers, pathogen from non-shedding sources) pathogen concentration is the mean of the 1,000 average daily pathogen concentrations output from Scenario 4 simulations.  Minimum and maximum average daily pathogen concentrations for the 1,000 simulations are shown with error bars.

| Table 3.1 Average Daily Prevalence per 100,000 Statistics for Six Scenarios for Giardia and Ala Moana Beach |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario 1 -- Background |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| variance | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| min | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| max | 9.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario 2 -- Shedding of Pathogen by Swimmers / Surfers, No Pathogen from Non-Shedding Sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| variance | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| min | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| max | 9.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario 3 -- No Shedding of Pathogen by Swimmers / Surfers, Pathogen from Non-Shedding Sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| variance | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| min | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| max | 9.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario 4 -- Shedding of Pathogen by Swimmers / Surfers, Pathogen from Non-Shedding Sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| variance | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| min | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| max | 9.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario 5 -- Shedding of Pathogen by Swimmers / Surfers, Order of Magnitude Increase in Pathogen |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| from Non-Shedding Sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| variance | 2.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| min | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| max | 9.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario 6 -- Shedding of Pathogen by Swimmers / Surfers, Order of Magnitude Decrease in Pathogen |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| from Non-Shedding Sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| variance | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| min | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| max | 9.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3.2
Average Daily Prevalence per 100,000 Statistics for Three Scenarios, Four Pathogens and Two Beaches

| Giardia |  |  | Cryptosporidium |  |  |  |  |  |  | Salmonella |  |  |  |  |  | Enteroviruses |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Background -- Scenario 1 -- Exposure to Recreational Water is Not the Vehicle of Disease Transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.5 |  | mean |  | 0.6 |  | mean |  |  | 0.4 |  |  | mean |  |  | 229.3 |  |
| variance | 1.9 |  | variance |  | 0.3 |  | variance |  |  | 0.1 |  |  | variance |  |  | 22303.6 |  |
| min | 0.3 |  | min |  | 0.1 |  | min |  |  | 0.1 |  |  | min |  |  | 13.2 |  |
| max | 9.0 |  | max |  | 3.6 |  | max |  |  | 1.9 |  |  | max |  |  | 1115.5 |  |
| Ala Moana -- Scenario 4 -- Shedding of Pathogen by Swimmers / Surfers, Pathogen from Non-Shedding Sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.6 |  | mean |  | 0.6 |  | mean |  |  | 0.4 |  |  | mean |  |  | 229.4 |  |
| variance | 1.9 |  | variance |  | 0.3 |  | variance |  |  | 0.1 |  |  | variance |  |  | 22305.7 |  |
| min | 0.3 |  | min |  | 0.1 |  | min |  |  | 0.1 |  |  | min |  |  | 13.3 |  |
| max | 9.0 |  | max |  | 3.6 |  | max |  |  | 1.9 |  |  | max |  |  | 1115.6 |  |
| Ala Moana -- Scenario 5 -- Shedding of Pathogen by Swimmers / Surfers, Order of Magnitude Increase in Pathogen from Non-Shedding Sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.6 |  | mean |  | 0.6 |  | mean |  |  | 0.4 |  |  | mean |  |  | 229.4 |  |
| variance | 2.0 |  | variance |  | 0.3 |  | variance |  |  | 0.1 |  |  | variance |  |  | 22296.7 |  |
| min | 0.3 |  | min |  | 0.1 |  | min |  |  | 0.1 |  |  | min |  |  | 13.7 |  |
| max | 9.1 |  | max |  | 3.6 |  | max |  |  | 1.9 |  |  | max |  |  | 1115.6 |  |
| Waikiki -- Scenario 4 -- Shedding of Pathogen by Swimmers / Surfers, Pathogen from Non-Shedding Sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.6 |  | mean |  | 0.6 |  | mean |  |  | 0.4 |  |  | mean |  |  | 229.4 |  |
| variance | 2.0 |  | variance |  | 0.3 |  | variance |  |  | 0.1 |  |  | variance |  |  | 22302.0 |  |
| min | 0.3 |  | min |  | 0.1 |  | min |  |  | 0.1 |  |  | min |  |  | 13.3 |  |
| max | 9.1 |  | max |  | 4.3 |  | max |  |  | 1.9 |  |  | max |  |  | 1115.6 |  |
| Waikiki -- Scenario 5 -- Shedding of Pathogen by Swimmers / Surfers, Order of Magnitude Increase in Pathogen from Non-Shedding Sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mean | 1.7 |  | mean |  | 0.6 |  | mean |  |  | 0.4 |  |  | mean |  |  | 229.5 |  |
| variance | 2.2 |  | variance |  | 0.3 |  | variance |  |  | 0.1 |  |  | variance |  |  | 22287.8 |  |
| min | 0.3 |  | min |  | 0.1 |  | min |  |  | 0.1 |  |  | min |  |  | 14.1 |  |
| max | 9.2 |  | max |  | 4.3 |  | max |  |  | 1.9 |  |  | max |  |  | 1115.6 |  |

# 4.0	CONCLUSIONS

Based on the results of the public health risk assessment modeling we reached the following conclusions:

	Average daily prevalences modeled for disease associated with exposure to selected pathogens (Giardia, Cryptosporidium, Salmonella and enteroviruses) during recreational use of two Mamala Bay beaches did not vary significantly from modeled background prevalences in the population. (Background prevalence is the expected prevalence in the population when exposure to recreational water is not the vehicle of disease transmission).

	Increasing the concentration of pathogen from sources other than direct shedding by swimmers and surfers by an order of magnitude did not significantly increase the prevalence of disease in the population above background. The risk assessment modeling results therefore suggest that water quality management strategies designed to prevent additional pathogens from point and non-point sources from reaching the beach would not appear to affect the disease prevalences associated with the selected pathogens. This conclusion is based on available water quality pathogen monitoring and modeling results and the assumption that the uncertainty in the water quality modeling results used as input to the risk assessment model is less than approximately an order of magnitude.

	Based on a simple preliminary analysis for Giardia and Ala Moana beach, the rate of water flow out of the swimming/surfing area was not important in determining the prevalence of disease. Despite a decrease in pathogen concentration with increasing flow rate out, average daily prevalences did not vary and remained at or below the background prevalence level of approximately 10 per 100,000.

	Based on the results of the uncertainty analysis for Giardia and Ala Moana beach, the most important determinants of prevalence were the same for Scenario 1 (background - exposure to recreational water not the vehicle of disease transmission) and Scenario 4 (shedding of pathogen by swimmers/surfers, pathogen from non-shedding sources). These parameters were , the fraction of individuals in state D who move to state Z per day (based on the duration of disease symptoms) and 0, the background transmission rate. The fact that the uncertainty analysis produced the same results for Scenarios 1 and 4 is consistent with the fact that the mean, variance, minimum and maximum values for average daily prevalence modeled for Scenario 4 did not vary significantly from Scenario 1, the modeled background prevalence.

# Appendix A:  Model Parameterization

This appendix describes the use of the 20 sampling parameters in the model.  The parameters are divided into three groups: 12 biological, 5 community and 3 water quality/flow parameters.  Most of the biological parameters are based on properties of the microorganism under study.  Community parameters are based on properties of the community and the exposure scenario under study. Water quality/flow parameters are derived from fate and transport modeling data from other Mamala Bay study groups.

Biological Parameters

1. and 2.	T and P

The dependent parameter , the fraction of individuals in state Y who move to state D per day, is a ratio of sampling parameters p, the fraction in state Y that move to state D and T, the incubation period:

 = p / T

3.		Rand

The dependent parameter , the fraction of individuals in state Y who move to state D per day, depends on the value of .  For each unit of time, a fraction of the population in state Y will move to state D, at the rate .  The remaining population in state Y either remains in state Y or moves to state Z.  The sampling parameter Rand was sampled from a uniform distribution of 0 to 1 and determined the fraction of the remaining population in state Y that moves to state Z. Therefore  was defined as follows:

 = Rand  (1 - )

4.		

The parameter  is the fraction of individuals in state Z who move to state X per day. This parameter was based on the reciprocal of the estimated time for an immune person to become susceptible.

5.		

The parameter  is the fraction of individuals in state D who move to state Z per day. This parameter was based on the reciprocal of the estimated duration of symptoms.

6.		

The parameter  is the fraction of individuals in state D who die due to modeled disease per day. It was based on case-fatality rate data for the disease.

7.		a

The parameter a, the number of new susceptible individuals who migrate into the population per day, was set equal to the birth rate of the community, which was assumed to be equal to the global birth rate provided by Raven and Johnson.1

8.		

The parameter , the fraction of individuals who die from natural causes per day, was set equal to the death rate of the community, which was assumed to be equal to the global death rate provided by Raven and Johnson.1

9.		F

The dependent parameter  is the number of pathogen shed per liter of water in the swimming/surfing area per day per infectious/asymptomatic individual. This parameter is a function of three community parameters, Ts, the number of hours spent swimming per day, BM, the fraction of the population that visits the beach each day during a given month of the year, and SM, the fraction of beachgoers that swim or surf each day during a given month of the year; one biological parameter, F, the number of pathogen shed per swimmer per hour; and one water quality/flow parameter, VD, the volume of the swimming/surfing area:

 = (F  BM  SM  Ts)/VD

The parameters BM, SM, Ts, and VD will be further discussed later in this appendix.

The parameter F, the number of pathogens shed per infected swimmer per hour, was defined using water quality data provided by Mamala Bay Study monitoring groups (see Appendix B). This data included pathogen monitoring data from Ala Moana and Waikiki beaches. For each of the selected pathogens, 1,000 Monte Carlo simulations were performed for Scenario 2 (shedding of pathogen by swimmers and surfers but no pathogen from non-shedding sources) at Ala Moana beach. The initial sampling range for F was set from 1 to 1011 pathogen/hour.  This very wide range was selected to span the range of uncertainty and variability associated with this parameter.  For each simulation an average daily pathogen concentration was calculated.

Using the pathogen monitoring data at the beaches, an upper bound concentration of 0.1 pathogen per liter in the swimming area due to shedding by swimmers and surfers was defined (it should be noted that it was assumed that surfers would shed pathogen at the same rate as swimmers). A lower bound of pathogen concentration three orders of magnitude lower than the 0.1 upper bound was selected. Thus a range of pathogen concentration in the swimming area of 0.0001 to 0.1 pathogen per liter due to shedding by swimmers and surfers was defined as being approximately consistent with the beach water quality monitoring data. The approximate range of F values which produced pathogen concentrations within this range were then determined.

Figures A.1 through A.4 are scatter plots of the above Monte Carlo simulation results showing the relationship between average daily pathogen concentration and F. This relationship is approximately linear.  From these figures it was determined that 0.0001 to 0.1 pathogen per liter corresponds to the following ranges for F (in pathogen/hour):

Giardia		105 - 108

Cryptosporidium	106 - 109

Salmonella		105 - 108

Enteroviruses		102 - 105

The above ranges for F were used for the risk modeling.

To further support the use of the above ranges, we estimated the fecal coliform shedding rate using data from a report prepared by the Mamala Bay Study group MB-7 that appears to show a relationship between the number of swimmers at Waikiki Beach and the concentration of fecal coliforms.2 A copy of a page from this report with figures comparing the change over time in the number of people and the concentration of fecal coliforms at a 35-meter length of Waikiki Beach is attached at the end of this appendix. To calculate the approximate fecal coliform shedding rate using this data, we made the assumption that for the time period from 0800 to 1600 all of the increase in fecal coliform concentration was due to the increase in people in the water. For this eight hour time period the approximate areas under the number of people curve and the fecal coliform concentration curve were determined to yield people-hours and coliform concentration-hours. Fecal coliform concentration-hours were then converted to total fecal coliform number-hours using an assumed swimming area volume of 1,400 cubic meters (35 meters length x 2 meters depth x 20 meters width). The ratio of fecal coliform number-hours to people-hours was then calculated and divided by an assumed swimming time of 2 hours per person to yield a shedding rate of approximately 2 X 107 fecal coliforms per person per hour. This rate is in the same order of magnitude as the mean rate of shedding of total coliform by swimmers of 2.3 X 108 indicators per hour swimming determined in a study by Hanes and Fossa.3

The rate of indicator shedding can be related to the rate of pathogen shedding by making the following assumption: the ratio of concentration of pathogen to concentration of indicator organisms in an infected person's feces is equal to the ratio of the rate of shedding of pathogen by an infected person during swimming or surfing to the rate of shedding of indicators during swimming. For total coliforms, the concentration of indicators in feces is approximately 107 to 109 per gram of feces4 (we assume that this range is independent of whether a person is infected or not). For Giardia, infected persons may shed 106 cysts per gram of feces.5 Thus it could be assumed that the shedding rate of Giardia by an infected person would be about 1 to 3 orders of magnitude lower than the shedding rate of indicators. This is generally consistent with the rates of shedding of indicators given above and the 105 to 108 pathogen per hour rate used for Giardia for the modeling.

While the above calculations are very approximate and based on numerous assumptions, they do appear to support the rate of shedding ranges used for Giardia in the risk modeling.

10.		0

The parameter 0, the background transmission rate, is based on the expected background incidence of disease in the community.  The sampled range of 0 is established by running a series of calibration simulations.  First, simulations of the model are run with the value of 0 sampled from an arbitrarily large range.  The incidence rates generated from these calibration simulations are compared with the expected background incidence rate range, and if within this range, are classified as passes.  The sampling range of 0 is adjusted to reflect the distribution of 0 for the passed simulations.  Hence, the sampling range of 0 is narrowed by these calibration simulations to provide a high likelihood of matching the expected background incidence of disease in the community.

11.		Exp

For Giardia, Cryptosporidium and Salmonella, the standard single-hit exponential model to describe the probability of infection when an individual is exposed to a certain dose of pathogen was used.  This model assumes that infection is a two-step process: 1) the host is exposed to a certain number of microorganisms, and 2) a fraction of the microorganisms ingested survive and cause infection.  From these assumptions the probability of an infection resulting from the ingestion of d organisms is:6

Pij = 1 - exp ( -Exp  d)

where d is the dose that an individual is exposed to and Exp is the fraction of ingested organisms that survive.

For enteroviruses, an alternative model called the beta poisson model was used. This model results in a more gradual response to increasing dose by describing Exp in the above dose-response model with a beta probability distribution:

Pij = 1 - (1 + d/ Bp - Bp

where the parameters Bp and Bp characterize the dose-response curve and d is the dose to which an individual is exposed.

Both of the above functions have been used to calculate the risk of disease due to exposure to various waterborne pathogens, including viral diseases7 and Giardia lamblia.8 The range of the sampled parameters Exp, Bp and Bp were determined using a maximum likelihood estimator (MLE) approach, derived in a previous study on risk assessment of pathogens in drinking water.8 The likelihood equation for the both of the functions was maximized:

where pj is the number of infected at each dosage, nj is the number of non-infected at each dosage and Pij is the probability of infection, as described above. Tables A.1 through A.4 present the dose-response data used for each of the selected pathogens. It should be noted that rotavirus dose-response data was used to represent the dose-response relationship for enteroviruses.

For this study, the pathogen dose d was described by the following function:

d = W  I  Ts

where W is the output variable representing the concentration of pathogen in water in the swimming area, I is the volume of water ingested per hour swimming, and Ts represents the amount of time in hours spent swimming or surfing per day. The parameters I and Ts will be described later in the community parameters section.

12.		

The parameter  is the fraction of pathogen in the recreational use water that become non-viable per day. It was based on the reciprocal of the estimated survival time of pathogen in ocean water.

Community Parameters

1.		X0

X0 is both the initial susceptible population and the total population, since it is assumed that all members of the population are susceptible at the beginning of each simulation. A range for X0 was selected based on the 1992 de facto population of the City and County of Honolulu.9

2. and 3.	BM and SM

BM is the fraction of the population that visits the beach each day during a given month of the year. Monthly ranges for this time-varying parameter were estimated using daily attendance records in a lifeguard station log database provided to EOA by PRC. This database included daily attendance counts at lifeguard stations for three years: 1990 - 1992. The daily attendance at a given beach was determined by summing the daily attendance records from the lifeguard stations located at that beach (see Table A-5 and Figure 2.2) and averaging for each month of the year for each of the three years. For a given month of each of the three years, the lowest of the three monthly average attendance values was divided by the upper bound of the population range (932,000). This value was used as the lower bound of the sampled range of BM. The upper bound of BM was calculated by dividing the highest of the three monthly average attendance values by the lower bound of the population range (800,000). Figures A.5 and A.6 give the ranges that BM was sampled from for each month of the simulation for each of the two beaches included in the risk assessment. Appendix C contains this data in tabulated form.

The parameter SM is the fraction of beachgoers that swim or surf each day during a given month of the year. Monthly ranges for this time-varying parameter were estimated using daily records of the number of individuals who swim or surf and daily beach attendance records in the lifeguard database provided to EOA by PRC. The daily fraction of swimmers and surfers at a given beach was determined by summing the daily number of swimmers and surfers recorded at lifeguard stations at that beach (see Table A-5 and Figure 2.2) and dividing by the daily attendance at that beach.  The daily fractions were averaged and the standard deviation calculated for each month of the year for all three years. The sampling range for SM used in the model was determined for a given month of the year by taking the lowest value of the monthly average minus a standard deviation and the highest value of the monthly average plus a standard deviation for that month over the three years.

Figures A.7 and A.8 give the ranges that SM was sampled from for each month of the simulation for each of the two beaches included in the risk assessment. Appendix D contains this data in tabulated form.

The parameter , the infection rate due to ingestion of pathogen in the recreational use water, was calculated by multiplying the dose response equation describing the probability of a susceptible swimmer or surfer becoming infected by BM and SM:

 = (1-exp (-Exp  d)BMSM

to give the probability of a susceptible individual in the population becoming infected due to the ingestion of pathogen in the recreational water.  As described earlier, BM and SM are also used to calculate , reflecting that only the fraction of the population in state Y that swims or surfs sheds pathogen directly into the recreational water.

4.		I

The parameter I is the amount of water ingested per swimmer or surfer per hour.  As shown earlier, it was used in the calculation of the pathogen dose.

5.		Ts

The dependent parameter , the infection rate due to ingestion of pathogen in recreational water, is a function of the number of hours spent swimming or surfing per day, Ts.  As described earlier, these parameters are used in the  transmission function, where dose is dependent on the fraction of time spent swimming or surfing, and to calculate , reflecting that the infected population contributes pathogen directly to the swimming water only when swimming or surfing.

Water Quality/Flow Parameters

1.		WNS

The parameter WNS is the average daily concentration of in the water in the swimming/surfing area from sources other than shedding by swimmers and surfers. This time-varying parameter was set by averaging over each day of the simulation hourly pathogen concentrations modeled by HydroQual. Figures A.9 through A.16 give the values of WNS used for each day of the simulation for each of the two beaches and four microorganisms included in the risk assessment. Appendix E contains this data in tabulated form.

2.		FD

The parameter FD is the flow rate out of water from the swimming/surfing area. FD is used to calculate the number of pathogen leaving the swimming/surfing area due to water flow. This time-varying parameter was set by averaging over each day of the simulation the sum of the hourly flows and dispersions out of the beach segment modeled by HydroQual. Dispersion values from HydroQual were given in a volume per unit time basis and were treated as flows for our purposes. Figures A.17 and A.18 give the values of FD used for each day of the simulation for each of the two beaches included in the risk assessment. Appendix E contains this data in tabulated form.

3.		VD

The parameter VD is the volume of the swimming/surfing area. This time-varying parameter was set by averaging over each day of the simulation hourly beach segment volumes modeled by HydroQual. Figures A.19 and A.20 give the values of VD used for each day of the simulation for each of the two beaches included in the risk assessment. Appendix E contains this data in tabulated form.

VD is used to calculate the number of pathogen leaving the swimming/surfing area due to water flow and is used in the differential equation describing the change over time in the state variable WS, the concentration of pathogen in the swimming/surfing area due to shedding by swimmers and surfers:

dWS/dt = -(FD  WS)/VD + Y - WS

where (FD  WS)/VD is the rate that pathogen shed by swimmers and surfers leave the swimming/surfing area per liter of water in this area, Y is the rate of shedding by infected/asymptomatic individuals swimming or surfing per liter of water in the swimming/surfing area and WS is the rate of pathogen die-off per liter of water in the swimming/surfing area. The following two assumptions were made: 1) pathogen shed by swimmers and surfers completely mix in the swimming/surfing area and 2) pathogen shed by swimmers and surfers that leave the swimming/surfing area do not return to this area.

The state variable W, the concentration of pathogen in the swimming/surfing area water, is the sum of the pathogen concentration from non-shedding and shedding sources:

W = WNS + WS

Appendix A - References

1.	Raven, P.H., and G.B. Johnson, Biology (Times Mirror/Mosby College Publishing, Saint Louis, Missouri, 1989), 2nd ed.

2.	Dobbs, C.D., M.R. Landry and L. Campbell, "Microbial Aspects of Point- and Nonpoint Source Pollution in Mamala Bay: Bather and Light Effects on Sewage-Indicator Bacteria, Mamala Bay Study Project MB-7, (May 31, 1995).

3.	Hanes, N.B., and J. Fossa, "A Quantitative Analysis of the Effects of Bathers in Recreational Water Quality," Proceedings of the Fifth International Water Pollution Research Conference, HA-9/1-9 (1970).

4.	Sanitation and Disease: Health Aspects of Excreta and Wastewater Management, Feacham, R.G. et al. (John Wiley and Sons, New York, N.Y., 1983).

5.	Cooper, R.C., A.W. Olivieri, R.E. Danielson, P.G. Badger, R.C. Spear and S. Selvin, Evaluation of Mililtary Field-Water Quality, Volume 5: Infectious Organisms of Military Concern Associated with Consumption: Assessment of Health Risks and Recommendations for Establishing Related Standards (Lawrence Livermore National Laboratory, 1986).

6.	Haas, C.N., "Estimation of Risk Due to Low Doses of Microorganisms:  A Comparison of Alternative Methodologies," American Journal of Epidemiology, 55, 573-82 (1983).

7.	Haas, C.N., "Effect of Effluent Disinfection on Risks of Viral Disease Transmission Via Recreational Water Exposure," Journal of the Water Pollution Control Federation, 55, 1111-15 (1981).

8.	Regli, S., J.B. Rose, C.N. Haas, and C.P. Gerba, "Modeling the Risk from Giardia and Viruses in Drinking Water," Journal of the American Water Works Association, 83(11), 76-84 (1991).

9.	The State of Hawaii Data Book, 1992, A Statistical Abstract, Hawaii Department of Business, Economic Development and Tourism (Honolulu, Hawaii, 1993).

10.	Rendtorff, R.C., and C.J. Holt, "The Experimental Transmission of Human Intestinal Protozoan Parasites, IV. Attempts to Transmit Entamoeba Coli and Giardia lamblia Cysts by Water," American Journal of Hygiene, 60, 327-338 (1954).

11.	Rendtorff, R.C., "The Experimental Transmission of Human Intestinal Protozoan Parasites, II. Giardia lamblia Cysts Given in Capsules," American Journal of Hygiene, 59, 209-220 (1954).

12.	Dupont, Herbert L. et al, "The Infectivity of Cryptosporidium parvum in Healthy Volunteers," New England Journal of Medicine, 332(13), 855-9 (1995).

13.	Blaser, M.J., and L.S. Newman, "A Review of Human Salmonellosis: I. Infective Dose," Review of Infectious Disease, 4, 1096-1106 (1982).

14.	A Collaborative Report, "A Waterborne Epidemic of Salmonellosis in Riverside, California, 1965, Epidemiologic Aspects," American Journal of Epidemiology, 93, 33-48 (1971).

15.	Feldman, R.E., W.B. Baine, J.L. Nitzkin, M.S. Saslaw, and R.A. Pollard, "Epidemiology of Salmonella typhi Infection in a Migrant Labor Camp in Dade County, Florida," Journal of Infectious Disease, 130, 334-342 (1974).

16.	Naylor, G.R.E., "Incubation Period and Other Features of Foodborne and Waterborne Outbreaks of Typhoid Fever in Relation to Pathogenesis and Genetics of Resistance," Lancet, 1:8329, 864-866 (1983).

17.	Ward, R.L., D.I. Bernstein, E.C. Young, J.R. Sherwood, D.R. Knowlton and G.M. Schiff, "Human Rotavirus Studies in Volunteers: Determination of Infectious Dose and Serological Response to Infection," Journal of Infectious Disease, 154, 871-880 (1986).

Table A.1
	Dose response for Giardia lamblia10, 11

| Number Cysts Given | Number of Individuals Exposed | Number of Individuals Infected |
| --- | --- | --- |
| 1 | 5 | 0 |
| 10 | 2 | 2 |
| 25 | 20 | 6 |
| 100 | 2 | 2 |
| 10,000 | 3 | 3 |
| 100,000 | 3 | 3 |
| 300,000 | 3 | 3 |
| 1,000,000 | 2 | 2 |
| Controls | 21 | 0 |

Table A.2
	Dose response for Cryptosporidium12

| Intended Number Oocysts Given | Number of Individuals Exposed | Number of Individuals Infected |
| --- | --- | --- |
| 30 | 5 | 1 |
| 100 | 8 | 3 |
| 300 | 3 | 2 |
| 500 | 6 | 5 |
| 1,000 | 2 | 2 |
| 10,000 | 3 | 3 |
| 100,000 | 1 | 1 |
| 1,000,000 | 1 | 1 |

Table A.3Dose response for Salmonella typhi

| Dose | Response (per 1,000) | Number of subjects | Reference |
| --- | --- | --- | --- |
| 103 | 10.0 | 1,300 | 13 |
| 103 | 45.0 | 11,800 | 13 |
| 103 | 40.0 | 10,675 | 13 |
| 103 | 75.0 | 4,293 | 13 |
| 103 | 90.0 | 378 | 13 |
| 103 | 100.0 | 1.6 x 106 | 14 |
| 105 | 275.0 | 116 | 15 |
| 105 | 270.0 | 104 | 15 |
| 105 | 350.0 | 110 | 16 |
| 107 | 500.0 | 32 | 15 |
| 107 | 530.0 | 30 | 15 |
| 107 | 330.0 | 6 | 15 |
| 107 | 500.0 | 30 | 15 |
| 108 | 890.0 | 9 | 15 |
| 109 | 950.0 | 42 | 15 |
| 109 | 1000.0 | 4 | 15 |

Table A.4
Dose response for Rotavirus17

| Dose (focus forming units) | Number of Individuals Exposed | Number of Individuals Infected |
| --- | --- | --- |
| 0.9 | 7 | 1 |
| 9 | 11 | 8 |
| 90 | 7 | 6 |
| 900 | 8 | 7 |
| 9,000 | 7 | 5 |
| 90,000 | 3 | 3 |

Table A.5
Beaches and Corresponding Lifeguard Stations

| BEACH | LIFEGUARD STATION |
| --- | --- |
| Ala Moana | 1B, 1C, 1D, 1E, 1F, 1G |
| Waikiki | 2A, 2B |

Note: See Figure 2.2 for locations of beaches and lifeguard stations.

missing last page of appendix A

missing Appendix B:

# Appendix C:
BM -- Fraction of Population that Visits
the Beach Each Day During a Given Month of the Year

| BM is the fraction of the population that visits the beach each day during a given month of the year. Monthly ranges for this time-varying parameter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| were estimated using daily attendance records in a lifeguard station log database provided to EOA by PRC. This database included daily attendance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| counts at lifeguard stations for three years: 1990 - 1992. The daily attendance at a given beach was was determined by summing the daily attendance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| records from the lifeguard stations located at that beach and averaging for each month of the year for each of the three years. For a given month of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| each of the three years, the lowest of the three monthly average attendance values was divided by the upper bound of the population range (932,000). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This value was used as the lower bound of the sampled range of BM. The upper bound of BM was calculated by dividing the highest of the three |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| monthly average attendance values by the lower bound of the population range (800,000). The below table gives the ranges that BM was sampled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| from for each month of the simulation for each of the two beaches included in the risk assessment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ala Moana Beach |  |  |  |  |  | Waikiki Beach |  |  |  |  |  |
|  | min | max |  |  |  | min | max |  |  |
| April | 0.003323713 | 0.005376542 |  |  | April | 0.023994028 | 0.030287208 |  |  |
| May | 0.004348938 | 0.00568121 |  |  | May | 0.019994567 | 0.028693145 |  |  |
| June | 0.004962379 | 0.006818917 |  |  | June | 0.021833494 | 0.036149583 |  |  |
| July | 0.005735367 | 0.008206774 |  |  | July | 0.021555731 | 0.044886089 |  |  |
| August | 0.005323705 | 0.007778548 |  |  | August | 0.018637416 | 0.043707661 |  |  |
| September | 0.003936416 | 0.00565725 |  |  | September | 0.015580556 | 0.033631875 |  |  |
| October | 0.003168565 | 0.004372056 |  |  | October | 0.013311427 | 0.027113911 |  |  |
| November | 0.002979974 | 0.004484958 |  |  | November | 0.015497443 | 0.025718333 |  |  |
| December | 0.002131399 | 0.003552419 |  |  | December | 0.016113891 | 0.024562903 |  |  |

# Appendix D:
SM -- Fraction of Beachgoers that Swim or Surf
Each Day During a Given Month of the Year

| The parameter SM is the fraction of beachgoers that swim or surf each day during a given month of the year. Monthly ranges for this time-varying |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| parameter were estimated using daily records of the number of individuals who swim or surf and daily beach attendance records in the lifeguard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| database provided to EOA by PRC. The daily fraction of swimmers and surfers at a given beach was determined by summing the daily number of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| swimmers and surfers recorded at lifeguard stations at that beach and dividing by the daily attendance at that beach. The daily fractions were |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| averaged and the standard deviation calculated for each month of the year for all three years. The sampling range for SM used in the model was |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| determined for a given month of the year by taking the lowest value of the monthly average minus a standard deviation and the highest value of the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| monthly average plus a standard deviation for that month over the three years. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ala Moana Beach |  |  |  |  |  | Waikiki Beach |  |  |  |  |  |  |
|  | min | max |  |  |  |  | min |  |  | max |  |
| April | 0.285777728 | 0.503350332 |  |  | April |  | 0.081535702 |  |  | 0.202061903 |  |
| May | 0.284427765 | 0.553114561 |  |  | May |  | 0.074488535 |  |  | 0.244772665 |  |
| June | 0.361504951 | 0.521853104 |  |  | June |  | 0.086458817 |  |  | 0.207086199 |  |
| July | 0.362100438 | 0.518827459 |  |  | July |  | 0.097419496 |  |  | 0.281808274 |  |
| August | 0.391750536 | 0.546245513 |  |  | August |  | 0.096985769 |  |  | 0.230217203 |  |
| September | 0.362784365 | 0.487971257 |  |  | September |  | 0.073668059 |  |  | 0.28970808 |  |
| October | 0.318429558 | 0.488961922 |  |  | October |  | 0.084170377 |  |  | 0.354443614 |  |
| November | 0.289028161 | 0.562968405 |  |  | November |  | 0.014764274 |  |  | 0.366316231 |  |
| December | 0.264841611 | 0.532771949 |  |  | December |  | 0.049892297 |  |  | 0.341898095 |  |

# Appendix E:
Ala Moana Beach
Time-Varying Parameters From Hydroqual Data

| The parameter VD is the volume of the swimming/surfing area. This time-varying parameter was set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| by averaging over each day of the simulation hourly beach segment volumes modeled by HydroQual. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The parameter FD is the flow rate out of water from the swimming/surfing area. This time-varying parameter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| was set by averaging over each day of the simulation the sum of the hourly flows and dispersions out |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| of the beach segment modeled by HydroQual. Dispersion values from HydroQual were given in a volume per |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| unit time basis and were treated as flows for our purposes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The parameter WNS is the average daily concentration of pathogen in the water in the swimming/surfing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| area from sources other than shedding. This time-varying parameter was set by averaging over each day |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| of the simulation hourly pathogen concentrations modeled by HydroQual. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Average |  |  | Average Daily |  |  | Average Daily |  |  | Average Daily |  |  |  | Average Daily |  |  |  |  |  |  | Average Daily |  |  |  |  |
|  |  | Daily |  |  | Flow |  | Concentration of |  |  |  |  | Concentration of |  |  |  |  | Concentration of |  |  |  | Concentration of |  |  |  |  |  |
| Day |  | Volume (L) |  |  | Rate Out (L/Day) |  | Giardia (Cysts/L) |  |  |  |  | Cryptosporidium (Cysts/L) |  |  |  |  | Salmonella (#/L) |  |  |  | Enteroviruses (#/L) |  |  |  |  |  |
|  |  | VD |  |  | FD |  | WNS |  |  |  |  | WNS |  |  |  |  | WNS |  |  |  | WNS |  |  |  |  |  |
| 1 |  | 259196666.7 |  | 195705000 |  |  | 0.150218333 |  |  |  |  | 0.014368583 |  |  |  |  | 0.393497083 |  |  |  | 0.215544583 |  |  |  |  |  |
| 2 |  | 264782916.7 |  | 168246291.7 |  |  | 0.116738292 |  |  |  | 0.011165208 |  |  |  |  | 0.329121667 |  |  |  | 0.167505417 |  |  |  |  |  |  |
| 3 |  | 266775000 |  | 140740458.3 |  |  | 0.058339167 |  |  |  | 0.00558065 |  |  |  |  | 0.27846625 |  |  |  | 0.083707708 |  |  |  |  |  |  |
| 4 |  | 266296250 |  | 167339291.7 |  |  | 0.016946454 |  |  |  | 0.001624668 |  |  |  |  | 0.253469167 |  |  |  | 0.024308221 |  |  |  |  |  |  |
| 5 |  | 270001666.7 |  | 185020166.7 |  |  | 0.004215563 |  |  |  | 0.000409586 |  |  |  |  | 0.2199175 |  |  |  | 0.006035438 |  |  |  |  |  |  |
| 6 |  | 275658333.3 |  | 248184416.7 |  |  | 0.000971603 |  |  |  | 0.000100084 |  |  |  |  | 0.208122083 |  |  |  | 0.001378273 |  |  |  |  |  |  |
| 7 |  | 276189166.7 |  | 307332125 |  |  | 0.000447307 |  |  |  | 4.66973E-05 |  |  |  |  | 0.20176125 |  |  |  | 0.00063288 |  |  |  |  |  |  |
| 8 |  | 278995833.3 |  | 367364571 |  |  | 0.000428189 |  |  |  | 4.34392E-05 |  |  |  |  | 0.200600417 |  |  |  | 0.000608967 |  |  |  |  |  |  |
| 9 |  | 275633750 |  | 398045083.3 |  |  | 0.001510482 |  |  |  | 0.000145638 |  |  |  |  | 0.197320833 |  |  |  | 0.002164203 |  |  |  |  |  |  |
| 10 |  | 278775000 |  | 412444166.7 |  |  | 0.02644275 |  |  |  | 0.002530058 |  |  |  |  | 0.21153875 |  |  |  | 0.037928958 |  |  |  |  |  |  |
| 11 |  | 283290000 |  | 432642500 |  |  | 0.0323985 |  |  |  | 0.003098754 |  |  |  |  | 0.21404 |  |  |  | 0.046473125 |  |  |  |  |  |  |
| 12 |  | 283385416.7 |  | 403402083.3 |  |  | 0.02369575 |  |  |  | 0.002267367 |  |  |  |  | 0.185990833 |  |  |  | 0.033987792 |  |  |  |  |  |  |
| 13 |  | 281178750 |  | 351358916.7 |  |  | 0.010318675 |  |  |  | 0.000991912 |  |  |  |  | 0.194372917 |  |  |  | 0.014791488 |  |  |  |  |  |  |
| 14 |  | 274851250 |  | 285500916.7 |  |  | 0.004824067 |  |  |  | 0.000468103 |  |  |  |  | 0.1989925 |  |  |  | 0.006906488 |  |  |  |  |  |  |
| 15 |  | 271368333.3 |  | 228404791.7 |  |  | 0.003924925 |  |  |  | 0.00037885 |  |  |  |  | 0.18303875 |  |  |  | 0.005623221 |  |  |  |  |  |  |
| 16 |  | 267792500 |  | 179920833.3 |  |  | 0.005687638 |  |  |  | 0.000545158 |  |  |  |  | 0.160380417 |  |  |  | 0.008156088 |  |  |  |  |  |  |
| 17 |  | 270343333.3 |  | 181899458.3 |  |  | 0.00535555 |  |  |  | 0.000512639 |  |  |  |  | 0.151151667 |  |  |  | 0.007681183 |  |  |  |  |  |  |
| 18 |  | 271982083.3 |  | 190460458.3 |  |  | 0.002979179 |  |  |  | 0.000285967 |  |  |  |  | 0.172211667 |  |  |  | 0.004271163 |  |  |  |  |  |  |
| 19 |  | 272975416.7 |  | 222459625 |  |  | 0.000999237 |  |  |  | 9.82273E-05 |  |  |  |  | 0.189454167 |  |  |  | 0.001427812 |  |  |  |  |  |  |
| 20 |  | 270482500 |  | 261763083.3 |  |  | 0.000324037 |  |  |  | 3.52989E-05 |  |  |  |  | 0.189468333 |  |  |  | 0.000455267 |  |  |  |  |  |  |
| 21 |  | 270492916.7 |  | 289542916.7 |  |  | 0.000222751 |  |  |  | 2.53215E-05 |  |  |  |  | 0.182002917 |  |  |  | 0.000311237 |  |  |  |  |  |  |
| 22 |  | 271325833.3 |  | 313649166.7 |  |  | 0.002207808 |  |  |  | 0.000213267 |  |  |  |  | 0.163512917 |  |  |  | 0.003164695 |  |  |  |  |  |  |
| 23 |  | 278827083.3 |  | 344704625 |  |  | 0.003135383 |  |  |  | 0.000301424 |  |  |  |  | 0.149703333 |  |  |  | 0.004496988 |  |  |  |  |  |  |
| 24 |  | 275732500 |  | 357432916.7 |  |  | 0.002536608 |  |  |  | 0.000243704 |  |  |  |  | 0.152790417 |  |  |  | 0.003638442 |  |  |  |  |  |  |
| 25 |  | 271642916.7 |  | 329766833.3 |  |  | 0.003118467 |  |  |  | 0.000298705 |  |  |  |  | 0.150012083 |  |  |  | 0.004473975 |  |  |  |  |  |  |
| 26 |  | 274922500 |  | 315659583.3 |  |  | 0.016785308 |  |  |  | 0.001605373 |  |  |  |  | 0.153570833 |  |  |  | 0.024079346 |  |  |  |  |  |  |
| 27 |  | 282242916.7 |  | 308018958.3 |  |  | 0.0257575 |  |  |  | 0.002463775 |  |  |  |  | 0.1613775 |  |  |  | 0.036947167 |  |  |  |  |  |  |
| 28 |  | 281273333.3 |  | 300135833.3 |  |  | 0.022081292 |  |  |  | 0.002111396 |  |  |  |  | 0.1649875 |  |  |  | 0.031674583 |  |  |  |  |  |  |
| 29 |  | 278342500 |  | 279947500 |  |  | 0.013563996 |  |  |  | 0.001297949 |  |  |  |  | 0.170485417 |  |  |  | 0.019454708 |  |  |  |  |  |  |
| 30 |  | 274121666.7 |  | 246686666.7 |  |  | 0.00694775 |  |  |  | 0.000666275 |  |  |  |  | 0.175496667 |  |  |  | 0.009963004 |  |  |  |  |  |  |
| 31 |  | 275084166.7 |  | 223476566.7 |  |  | 0.004144371 |  |  |  | 0.000398639 |  |  |  |  | 0.17674625 |  |  |  | 0.005940925 |  |  |  |  |  |  |
| 32 |  | 274167500 |  | 209405500 |  |  | 0.001744874 |  |  |  | 0.000170449 |  |  |  |  | 0.178975 |  |  |  | 0.002496154 |  |  |  |  |  |  |
| 33 |  | 270360416.7 |  | 209639166.7 |  |  | 0.000597808 |  |  |  | 6.12914E-05 |  |  |  |  | 0.1812125 |  |  |  | 0.000849236 |  |  |  |  |  |  |
| 34 |  | 269079583.3 |  | 240382083.3 |  |  | 0.000224883 |  |  |  | 2.48143E-05 |  |  |  |  | 0.18241875 |  |  |  | 0.000315982 |  |  |  |  |  |  |
| 35 |  | 265132083.3 |  | 285092920.8 |  |  | 0.000509305 |  |  |  | 4.98768E-05 |  |  |  |  | 0.1697575 |  |  |  | 0.000728028 |  |  |  |  |  |  |
| 36 |  | 264809166.7 |  | 307667375 |  |  | 0.02935255 |  |  |  | 0.002807966 |  |  |  |  | 0.1827725 |  |  |  | 0.042114546 |  |  |  |  |  |  |
| 37 |  | 265490833.3 |  | 358221791.7 |  |  | 0.144246833 |  |  |  | 0.013798071 |  |  |  |  | 0.356007083 |  |  |  | 0.206969083 |  |  |  |  |  |  |
| 38 |  | 269830833.3 |  | 374676666.7 |  |  | 0.183719583 |  |  |  | 0.017574833 |  |  |  |  | 0.410678333 |  |  |  | 0.263581667 |  |  |  |  |  |  |
| 39 |  | 270250416.7 |  | 405947125 |  |  | 0.135788708 |  |  |  | 0.012989763 |  |  |  |  | 0.3461925 |  |  |  | 0.19480875 |  |  |  |  |  |  |
| 40 |  | 269512083.3 |  | 391993333.3 |  |  | 0.060359667 |  |  |  | 0.005775779 |  |  |  |  | 0.267277083 |  |  |  | 0.086587833 |  |  |  |  |  |  |
| 41 |  | 267107083.3 |  | 378067208.3 |  |  | 0.028702125 |  |  |  | 0.002748721 |  |  |  |  | 0.208993333 |  |  |  | 0.04117 |  |  |  |  |  |  |
| 42 |  | 263875416.7 |  | 352955929.2 |  |  | 0.111291375 |  |  |  | 0.010646617 |  |  |  |  | 0.318335417 |  |  |  | 0.159644167 |  |  |  |  |  |  |
| 43 |  | 266543333.3 |  | 328227083.3 |  |  | 0.12648625 |  |  |  | 0.012099083 |  |  |  |  | 0.3445375 |  |  |  | 0.18143375 |  |  |  |  |  |  |
| 44 |  | 269417083.3 |  | 283788875 |  |  | 0.089274792 |  |  |  | 0.008539046 |  |  |  |  | 0.2866925 |  |  |  | 0.128059 |  |  |  |  |  |  |
| 45 |  | 265990000 |  | 238543916.7 |  |  | 0.036671083 |  |  |  | 0.003508588 |  |  |  |  | 0.239455833 |  |  |  | 0.052599042 |  |  |  |  |  |  |
| 46 |  | 265813333.3 |  | 202391495.8 |  |  | 0.021973417 |  |  |  | 0.00210355 |  |  |  |  | 0.207315833 |  |  |  | 0.031516042 |  |  |  |  |  |  |
| 47 |  | 265106666.7 |  | 183340166.7 |  |  | 0.017093875 |  |  |  | 0.001636475 |  |  |  |  | 0.174595833 |  |  |  | 0.024517167 |  |  |  |  |  |  |
| 48 |  | 264273750 |  | 199648916.7 |  |  | 0.035035708 |  |  |  | 0.003351738 |  |  |  |  | 0.175975417 |  |  |  | 0.050262958 |  |  |  |  |  |  |
| 49 |  | 262138333.3 |  | 228323833.3 |  |  | 0.12004625 |  |  |  | 0.011482433 |  |  |  |  | 0.334914167 |  |  |  | 0.172256625 |  |  |  |  |  |  |
| 50 |  | 262995416.7 |  | 234850788.3 |  |  | 0.154727083 |  |  |  | 0.014798833 |  |  |  |  | 0.410115833 |  |  |  | 0.222005417 |  |  |  |  |  |  |
| 51 |  | 264614166.7 |  | 263456208.3 |  |  | 0.123250833 |  |  |  | 0.011787108 |  |  |  |  | 0.34391125 |  |  |  | 0.176839583 |  |  |  |  |  |  |
| 52 |  | 264573750 |  | 288780166.7 |  |  | 0.097136833 |  |  |  | 0.00929 |  |  |  |  | 0.327728333 |  |  |  | 0.1393725 |  |  |  |  |  |  |
| 53 |  | 262260833.3 |  | 297444500 |  |  | 0.090604583 |  |  |  | 0.008665508 |  |  |  |  | 0.331580833 |  |  |  | 0.129990833 |  |  |  |  |  |  |
| 54 |  | 261685000 |  | 314075000 |  |  | 0.092730208 |  |  |  | 0.008868654 |  |  |  |  | 0.324950833 |  |  |  | 0.13303125 |  |  |  |  |  |  |
| 55 |  | 263886250 |  | 310596250 |  |  | 0.094233917 |  |  |  | 0.009012821 |  |  |  |  | 0.307841667 |  |  |  | 0.135195417 |  |  |  |  |  |  |
| 56 |  | 266744583.3 |  | 315764166.7 |  |  | 0.086013625 |  |  |  | 0.008227279 |  |  |  |  | 0.27579375 |  |  |  | 0.123409583 |  |  |  |  |  |  |
| 57 |  | 264531666.7 |  | 313609833.3 |  |  | 0.066018625 |  |  |  | 0.006314725 |  |  |  |  | 0.244305 |  |  |  | 0.0947285 |  |  |  |  |  |  |
| 58 |  | 267215000 |  | 316729916.7 |  |  | 0.040033667 |  |  |  | 0.003830296 |  |  |  |  | 0.211031667 |  |  |  | 0.057443708 |  |  |  |  |  |  |
| 59 |  | 267510416.7 |  | 292949583.3 |  |  | 0.020504667 |  |  |  | 0.001962867 |  |  |  |  | 0.18860375 |  |  |  | 0.029419833 |  |  |  |  |  |  |
| 60 |  | 269648333.3 |  | 274177500 |  |  | 0.023026292 |  |  |  | 0.002203525 |  |  |  |  | 0.176065833 |  |  |  | 0.03303975 |  |  |  |  |  |  |
| 61 |  | 272025416.7 |  | 267920416.7 |  |  | 0.039141625 |  |  |  | 0.003745188 |  |  |  |  | 0.185015833 |  |  |  | 0.056162167 |  |  |  |  |  |  |
| 62 |  | 274050833.3 |  | 238303791.7 |  |  | 0.037377458 |  |  |  | 0.003576438 |  |  |  |  | 0.188374167 |  |  |  | 0.053629625 |  |  |  |  |  |  |
| 63 |  | 275229583.3 |  | 245000083.3 |  |  | 0.020700875 |  |  |  | 0.001982283 |  |  |  |  | 0.181172917 |  |  |  | 0.029698625 |  |  |  |  |  |  |
| 64 |  | 277603750 |  | 265553416.7 |  |  | 0.007592308 |  |  |  | 0.000729913 |  |  |  |  | 0.187237083 |  |  |  | 0.010886471 |  |  |  |  |  |  |
| 65 |  | 274602083.3 |  | 322366458.3 |  |  | 0.002935046 |  |  |  | 0.000282583 |  |  |  |  | 0.183209167 |  |  |  | 0.004201708 |  |  |  |  |  |  |
| 66 |  | 271991250 |  | 350254583.3 |  |  | 0.024899296 |  |  |  | 0.002382648 |  |  |  |  | 0.18969 |  |  |  | 0.035723671 |  |  |  |  |  |  |
| 67 |  | 272377916.7 |  | 373234166.7 |  |  | 0.051043875 |  |  |  | 0.004882625 |  |  |  |  | 0.20980375 |  |  |  | 0.0732485 |  |  |  |  |  |  |
| 68 |  | 275756666.7 |  | 392251250 |  |  | 0.048504333 |  |  |  | 0.004633121 |  |  |  |  | 0.206754167 |  |  |  | 0.06961825 |  |  |  |  |  |  |
| 69 |  | 278222083.3 |  | 409554208.3 |  |  | 0.031625625 |  |  |  | 0.003015033 |  |  |  |  | 0.20062125 |  |  |  | 0.045403958 |  |  |  |  |  |  |
| 70 |  | 278494583.3 |  | 420223750 |  |  | 0.013146446 |  |  |  | 0.001259546 |  |  |  |  | 0.2014875 |  |  |  | 0.018861625 |  |  |  |  |  |  |
| 71 |  | 282900416.7 |  | 402355000 |  |  | 0.004740113 |  |  |  | 0.000458663 |  |  |  |  | 0.201915 |  |  |  | 0.006790179 |  |  |  |  |  |  |
| 72 |  | 284146666.7 |  | 384498250 |  |  | 0.003197708 |  |  |  | 0.000299132 |  |  |  |  | 0.183354583 |  |  |  | 0.004595033 |  |  |  |  |  |  |
| 73 |  | 284603333.3 |  | 348030791.7 |  |  | 0.003433038 |  |  |  | 0.000321098 |  |  |  |  | 0.180187083 |  |  |  | 0.004942988 |  |  |  |  |  |  |
| 74 |  | 283797083.3 |  | 319315000 |  |  | 0.002710508 |  |  |  | 0.000255345 |  |  |  |  | 0.182919167 |  |  |  | 0.003897446 |  |  |  |  |  |  |
| 75 |  | 280708750 |  | 219268833.3 |  |  | 0.001258303 |  |  |  | 0.000121288 |  |  |  |  | 0.1846675 |  |  |  | 0.001803333 |  |  |  |  |  |  |
| 76 |  | 280986666.7 |  | 237298750 |  |  | 0.000572529 |  |  |  | 5.75155E-05 |  |  |  |  | 0.17850375 |  |  |  | 0.000815577 |  |  |  |  |  |  |
| 77 |  | 283103750 |  | 250462016.7 |  |  | 0.000420072 |  |  |  | 4.15427E-05 |  |  |  |  | 0.176920417 |  |  |  | 0.000599832 |  |  |  |  |  |  |
| 78 |  | 284002916.7 |  | 262050416.7 |  |  | 0.000335664 |  |  |  | 2.95497E-05 |  |  |  |  | 0.174977083 |  |  |  | 0.000509255 |  |  |  |  |  |  |
| 79 |  | 284693333.3 |  | 266704000 |  |  | 0.001685293 |  |  |  | 0.000161421 |  |  |  |  | 0.158514167 |  |  |  | 0.002428633 |  |  |  |  |  |  |
| 80 |  | 289985000 |  | 271527875 |  |  | 0.012163121 |  |  |  | 0.001164434 |  |  |  |  | 0.156399167 |  |  |  | 0.017453875 |  |  |  |  |  |  |
| 81 |  | 290383333.3 |  | 275061958.3 |  |  | 0.012166583 |  |  |  | 0.001164616 |  |  |  |  | 0.155546667 |  |  |  | 0.017458083 |  |  |  |  |  |  |
| 82 |  | 290410833.3 |  | 304800000 |  |  | 0.007411979 |  |  |  | 0.000710238 |  |  |  |  | 0.157900833 |  |  |  | 0.010634258 |  |  |  |  |  |  |
| 83 |  | 290938333.3 |  | 319983333.3 |  |  | 0.004407008 |  |  |  | 0.00042404 |  |  |  |  | 0.1486975 |  |  |  | 0.006320521 |  |  |  |  |  |  |
| 84 |  | 291834583.3 |  | 350068583.3 |  |  | 0.003624338 |  |  |  | 0.000347167 |  |  |  |  | 0.13844375 |  |  |  | 0.005201238 |  |  |  |  |  |  |
| 85 |  | 289127083.3 |  | 372242416.7 |  |  | 0.002794938 |  |  |  | 0.000267979 |  |  |  |  | 0.140555 |  |  |  | 0.004010533 |  |  |  |  |  |  |
| 86 |  | 289900833.3 |  | 379698750 |  |  | 0.004094817 |  |  |  | 0.000392875 |  |  |  |  | 0.156175833 |  |  |  | 0.005875283 |  |  |  |  |  |  |
| 87 |  | 292496250 |  | 383272083.3 |  |  | 0.014737929 |  |  |  | 0.001411378 |  |  |  |  | 0.18284625 |  |  |  | 0.0211437 |  |  |  |  |  |  |
| 88 |  | 295706666.7 |  | 361175416.7 |  |  | 0.022956417 |  |  |  | 0.002196813 |  |  |  |  | 0.2281 |  |  |  | 0.032928 |  |  |  |  |  |  |
| 89 |  | 292262916.7 |  | 314821625 |  |  | 0.031160667 |  |  |  | 0.002981213 |  |  |  |  | 0.2483825 |  |  |  | 0.044693292 |  |  |  |  |  |  |
| 90 |  | 291548750 |  | 289218625 |  |  | 0.029961958 |  |  |  | 0.002866992 |  |  |  |  | 0.24799 |  |  |  | 0.042977583 |  |  |  |  |  |  |
| 91 |  | 292092083.3 |  | 264381620.8 |  |  | 0.022517833 |  |  |  | 0.002154558 |  |  |  |  | 0.229542917 |  |  |  | 0.032301833 |  |  |  |  |  |  |
| 92 |  | 294573750 |  | 262663375 |  |  | 0.011152092 |  |  |  | 0.001073644 |  |  |  |  | 0.21866375 |  |  |  | 0.015985971 |  |  |  |  |  |  |
| 93 |  | 291668750 |  | 281647416.7 |  |  | 0.003603779 |  |  |  | 0.000356863 |  |  |  |  | 0.199320833 |  |  |  | 0.005145979 |  |  |  |  |  |  |
| 94 |  | 290329583.3 |  | 313523000 |  |  | 0.001490146 |  |  |  | 0.000152935 |  |  |  |  | 0.195525 |  |  |  | 0.002114921 |  |  |  |  |  |  |
| 95 |  | 289615833.3 |  | 351410000 |  |  | 0.001066379 |  |  |  | 0.000107478 |  |  |  |  | 0.18973875 |  |  |  | 0.001518408 |  |  |  |  |  |  |
| 96 |  | 286523333.3 |  | 378634583.3 |  |  | 0.000558617 |  |  |  | 5.77395E-05 |  |  |  |  | 0.178249167 |  |  |  | 0.000793456 |  |  |  |  |  |  |
| 97 |  | 284965416.7 |  | 403211500 |  |  | 0.000216809 |  |  |  | 2.48972E-05 |  |  |  |  | 0.1792225 |  |  |  | 0.000303221 |  |  |  |  |  |  |
| 98 |  | 285438333.3 |  | 416227166.7 |  |  | 8.19018E-05 |  |  |  | 1.07209E-05 |  |  |  |  | 0.189429583 |  |  |  | 0.000113342 |  |  |  |  |  |  |
| 99 |  | 284736666.7 |  | 410472416.7 |  |  | 2.89126E-05 |  |  |  | 6.39229E-06 |  |  |  |  | 0.18926 |  |  |  | 4.41808E-05 |  |  |  |  |  |  |
| 100 |  | 283220416.7 |  | 379610000 |  |  | 4.50195E-05 |  |  |  | 8.82672E-06 |  |  |  |  | 0.184265 |  |  |  | 6.0917E-05 |  |  |  |  |  |  |
| 101 |  | 286669166.7 |  | 356102916.7 |  |  | 0.000194207 |  |  |  | 2.39621E-05 |  |  |  |  | 0.1708775 |  |  |  | 0.000278485 |  |  |  |  |  |  |
| 102 |  | 284951250 |  | 321567916.7 |  |  | 0.000433891 |  |  |  | 4.15968E-05 |  |  |  |  | 0.158514583 |  |  |  | 0.00065091 |  |  |  |  |  |  |
| 103 |  | 283987500 |  | 276235250 |  |  | 0.000512523 |  |  |  | 4.70114E-05 |  |  |  |  | 0.1776 |  |  |  | 0.000751617 |  |  |  |  |  |  |
| 104 |  | 282310416.7 |  | 216211250 |  |  | 0.000373142 |  |  |  | 3.56961E-05 |  |  |  |  | 0.175325417 |  |  |  | 0.000539789 |  |  |  |  |  |  |
| 105 |  | 281285416.7 |  | 177620916.7 |  |  | 0.000161222 |  |  |  | 1.73058E-05 |  |  |  |  | 0.168607083 |  |  |  | 0.000227681 |  |  |  |  |  |  |
| 106 |  | 280715833.3 |  | 172435375 |  |  | 7.29552E-05 |  |  |  | 8.98561E-06 |  |  |  |  | 0.1592025 |  |  |  | 9.95927E-05 |  |  |  |  |  |  |
| 107 |  | 282080416.7 |  | 176613250 |  |  | 3.82993E-05 |  |  |  | 5.58917E-06 |  |  |  |  | 0.1552975 |  |  |  | 4.90215E-05 |  |  |  |  |  |  |
| 108 |  | 283602500 |  | 219378333.3 |  |  | 2.58606E-05 |  |  |  | 4.08006E-06 |  |  |  |  | 0.15978125 |  |  |  | 3.25998E-05 |  |  |  |  |  |  |
| 109 |  | 280030416.7 |  | 249387333.3 |  |  | 4.99835E-05 |  |  |  | 5.8638E-06 |  |  |  |  | 0.165440417 |  |  |  | 6.96286E-05 |  |  |  |  |  |  |
| 110 |  | 283982500 |  | 255462916.7 |  |  | 0.000834723 |  |  |  | 8.07699E-05 |  |  |  |  | 0.16326125 |  |  |  | 0.001195955 |  |  |  |  |  |  |
| 111 |  | 286181250 |  | 307449583.3 |  |  | 0.001728842 |  |  |  | 0.000166003 |  |  |  |  | 0.170007083 |  |  |  | 0.002479592 |  |  |  |  |  |  |
| 112 |  | 287011666.7 |  | 330927416.7 |  |  | 0.00142035 |  |  |  | 0.000136614 |  |  |  |  | 0.17377875 |  |  |  | 0.002036621 |  |  |  |  |  |  |
| 113 |  | 291744166.7 |  | 363741458.3 |  |  | 0.000838736 |  |  |  | 8.2431E-05 |  |  |  |  | 0.17846375 |  |  |  | 0.001199048 |  |  |  |  |  |  |
| 114 |  | 290231666.7 |  | 382454250 |  |  | 0.000294075 |  |  |  | 3.06217E-05 |  |  |  |  | 0.1923425 |  |  |  | 0.000416551 |  |  |  |  |  |  |
| 115 |  | 289120416.7 |  | 402973750 |  |  | 0.000287706 |  |  |  | 2.90471E-05 |  |  |  |  | 0.19284 |  |  |  | 0.00040954 |  |  |  |  |  |  |
| 116 |  | 286288750 |  | 392462083.3 |  |  | 0.000593473 |  |  |  | 5.76033E-05 |  |  |  |  | 0.194439583 |  |  |  | 0.000849502 |  |  |  |  |  |  |
| 117 |  | 287165416.7 |  | 367404583.3 |  |  | 0.000862388 |  |  |  | 8.34933E-05 |  |  |  |  | 0.204655 |  |  |  | 0.001235091 |  |  |  |  |  |  |
| 118 |  | 289960416.7 |  | 352181250 |  |  | 0.00076858 |  |  |  | 7.48809E-05 |  |  |  |  | 0.199204583 |  |  |  | 0.001099756 |  |  |  |  |  |  |
| 119 |  | 289125416.7 |  | 310508333.3 |  |  | 0.000462368 |  |  |  | 4.59592E-05 |  |  |  |  | 0.197026667 |  |  |  | 0.00065986 |  |  |  |  |  |  |
| 120 |  | 289269583.3 |  | 272902208.3 |  |  | 0.000339351 |  |  |  | 3.45145E-05 |  |  |  |  | 0.194708333 |  |  |  | 0.000482575 |  |  |  |  |  |  |
| 121 |  | 289338750 |  | 246536458.3 |  |  | 0.000272188 |  |  |  | 2.82915E-05 |  |  |  |  | 0.19583875 |  |  |  | 0.000385809 |  |  |  |  |  |  |
| 122 |  | 288932083.3 |  | 247700500 |  |  | 0.000375858 |  |  |  | 3.71735E-05 |  |  |  |  | 0.190814167 |  |  |  | 0.000536758 |  |  |  |  |  |  |
| 123 |  | 285981666.7 |  | 273120416.7 |  |  | 0.001325625 |  |  |  | 0.00012742 |  |  |  |  | 0.176170833 |  |  |  | 0.001900455 |  |  |  |  |  |  |
| 124 |  | 286669166.7 |  | 295795891.7 |  |  | 0.008389429 |  |  |  | 0.000803215 |  |  |  |  | 0.156061667 |  |  |  | 0.012032467 |  |  |  |  |  |  |
| 125 |  | 286524166.7 |  | 312955125 |  |  | 0.008363788 |  |  |  | 0.000800953 |  |  |  |  | 0.163200417 |  |  |  | 0.011994992 |  |  |  |  |  |  |
| 126 |  | 287902083.3 |  | 349991625 |  |  | 0.004363542 |  |  |  | 0.000419918 |  |  |  |  | 0.177450833 |  |  |  | 0.006253933 |  |  |  |  |  |  |
| 127 |  | 290729166.7 |  | 365304583.3 |  |  | 0.001630217 |  |  |  | 0.000160255 |  |  |  |  | 0.18581 |  |  |  | 0.0023298 |  |  |  |  |  |  |
| 128 |  | 292274166.7 |  | 363319791.7 |  |  | 0.000713302 |  |  |  | 7.22727E-05 |  |  |  |  | 0.17470375 |  |  |  | 0.001015393 |  |  |  |  |  |  |
| 129 |  | 293351250 |  | 358788333.3 |  |  | 0.000292843 |  |  |  | 3.16543E-05 |  |  |  |  | 0.17444125 |  |  |  | 0.000412772 |  |  |  |  |  |  |
| 130 |  | 292952500 |  | 344211250 |  |  | 0.000582775 |  |  |  | 5.74743E-05 |  |  |  |  | 0.182482083 |  |  |  | 0.000832959 |  |  |  |  |  |  |
| 131 |  | 291051666.7 |  | 298239833.3 |  |  | 0.001580465 |  |  |  | 0.000152256 |  |  |  |  | 0.183821667 |  |  |  | 0.002266021 |  |  |  |  |  |  |
| 132 |  | 292639583.3 |  | 230034666.7 |  |  | 0.001681408 |  |  |  | 0.000160947 |  |  |  |  | 0.186753333 |  |  |  | 0.002412579 |  |  |  |  |  |  |
| 133 |  | 293586250 |  | 201553750 |  |  | 0.00194095 |  |  |  | 0.000185603 |  |  |  |  | 0.181482917 |  |  |  | 0.002785271 |  |  |  |  |  |  |
| 134 |  | 290192916.7 |  | 145008583.3 |  |  | 0.056078813 |  |  |  | 0.005364213 |  |  |  |  | 0.237257083 |  |  |  | 0.080462825 |  |  |  |  |  |  |
| 135 |  | 289056250 |  | 145194291.7 |  |  | 0.090101292 |  |  |  | 0.008618871 |  |  |  |  | 0.29267625 |  |  |  | 0.12928 |  |  |  |  |  |  |
| 136 |  | 286531666.7 |  | 138042362.5 |  |  | 0.070980167 |  |  |  | 0.006789563 |  |  |  |  | 0.253655 |  |  |  | 0.101850792 |  |  |  |  |  |  |
| 137 |  | 288396250 |  | 157056291.7 |  |  | 0.063948417 |  |  |  | 0.006116496 |  |  |  |  | 0.229084167 |  |  |  | 0.09176825 |  |  |  |  |  |  |
| 138 |  | 289545000 |  | 191585791.7 |  |  | 0.06208725 |  |  |  | 0.005938183 |  |  |  |  | 0.217962083 |  |  |  | 0.089102 |  |  |  |  |  |  |
| 139 |  | 286463333.3 |  | 226699833.3 |  |  | 0.041630583 |  |  |  | 0.003986321 |  |  |  |  | 0.194190417 |  |  |  | 0.059735458 |  |  |  |  |  |  |
| 140 |  | 280237083.3 |  | 267402083.3 |  |  | 0.019258875 |  |  |  | 0.001851404 |  |  |  |  | 0.173455833 |  |  |  | 0.027620583 |  |  |  |  |  |  |
| 141 |  | 281054166.7 |  | 319504641.7 |  |  | 0.0185425 |  |  |  | 0.001779683 |  |  |  |  | 0.159907083 |  |  |  | 0.02659375 |  |  |  |  |  |  |
| 142 |  | 281609166.7 |  | 369551333.3 |  |  | 0.020263875 |  |  |  | 0.001940404 |  |  |  |  | 0.165452917 |  |  |  | 0.029068167 |  |  |  |  |  |  |
| 143 |  | 284606250 |  | 384628958.3 |  |  | 0.015812833 |  |  |  | 0.001514213 |  |  |  |  | 0.1760875 |  |  |  | 0.022683333 |  |  |  |  |  |  |
| 144 |  | 286790833.3 |  | 393712041.7 |  |  | 0.022042333 |  |  |  | 0.002108404 |  |  |  |  | 0.198536667 |  |  |  | 0.031627417 |  |  |  |  |  |  |
| 145 |  | 290921250 |  | 384941250 |  |  | 0.018272833 |  |  |  | 0.001748629 |  |  |  |  | 0.194407917 |  |  |  | 0.02621775 |  |  |  |  |  |  |
| 146 |  | 290695833.3 |  | 374923333.3 |  |  | 0.008788042 |  |  |  | 0.000844233 |  |  |  |  | 0.18967625 |  |  |  | 0.012602996 |  |  |  |  |  |  |
| 147 |  | 288505833.3 |  | 308798333.3 |  |  | 0.003084396 |  |  |  | 0.000299926 |  |  |  |  | 0.1924875 |  |  |  | 0.004416388 |  |  |  |  |  |  |
| 148 |  | 286048750 |  | 245277708.3 |  |  | 0.001261813 |  |  |  | 0.000124783 |  |  |  |  | 0.182783333 |  |  |  | 0.001802513 |  |  |  |  |  |  |
| 149 |  | 285326250 |  | 207585416.7 |  |  | 0.001605539 |  |  |  | 0.000155583 |  |  |  |  | 0.175807083 |  |  |  | 0.00229995 |  |  |  |  |  |  |
| 150 |  | 284130416.7 |  | 190513833.3 |  |  | 0.005459013 |  |  |  | 0.000523128 |  |  |  |  | 0.165458333 |  |  |  | 0.007830817 |  |  |  |  |  |  |
| 151 |  | 284052916.7 |  | 220801250 |  |  | 0.005920804 |  |  |  | 0.00056564 |  |  |  |  | 0.15832875 |  |  |  | 0.008496908 |  |  |  |  |  |  |
| 152 |  | 281950416.7 |  | 246067708.3 |  |  | 0.004610579 |  |  |  | 0.000440768 |  |  |  |  | 0.154755833 |  |  |  | 0.006616029 |  |  |  |  |  |  |
| 153 |  | 280358750 |  | 262291791.7 |  |  | 0.002613829 |  |  |  | 0.000251667 |  |  |  |  | 0.155355417 |  |  |  | 0.003747204 |  |  |  |  |  |  |
| 154 |  | 278676250 |  | 292274125 |  |  | 0.001871071 |  |  |  | 0.000181282 |  |  |  |  | 0.156537083 |  |  |  | 0.002679875 |  |  |  |  |  |  |
| 155 |  | 281003333.3 |  | 306692083.3 |  |  | 0.022152613 |  |  |  | 0.00212079 |  |  |  |  | 0.167530417 |  |  |  | 0.031790042 |  |  |  |  |  |  |
| 156 |  | 285344583.3 |  | 312801666.7 |  |  | 0.035816083 |  |  |  | 0.003427517 |  |  |  |  | 0.178781667 |  |  |  | 0.05140125 |  |  |  |  |  |  |
| 157 |  | 288983333.3 |  | 320451375 |  |  | 0.029080292 |  |  |  | 0.002783279 |  |  |  |  | 0.180901667 |  |  |  | 0.041733333 |  |  |  |  |  |  |
| 158 |  | 287145000 |  | 305575041.7 |  |  | 0.017985875 |  |  |  | 0.0017229 |  |  |  |  | 0.1757925 |  |  |  | 0.025808667 |  |  |  |  |  |  |
| 159 |  | 288447500 |  | 302690125 |  |  | 0.009210804 |  |  |  | 0.000888374 |  |  |  |  | 0.16866375 |  |  |  | 0.013205833 |  |  |  |  |  |  |
| 160 |  | 287976250 |  | 286782875 |  |  | 0.004464738 |  |  |  | 0.000437504 |  |  |  |  | 0.174295417 |  |  |  | 0.006388579 |  |  |  |  |  |  |
| 161 |  | 286740416.7 |  | 268112916.7 |  |  | 0.00209835 |  |  |  | 0.00021093 |  |  |  |  | 0.17685625 |  |  |  | 0.002991171 |  |  |  |  |  |  |
| 162 |  | 285732916.7 |  | 228712500 |  |  | 0.000784843 |  |  |  | 8.5244E-05 |  |  |  |  | 0.182811667 |  |  |  | 0.00110501 |  |  |  |  |  |  |
| 163 |  | 288687083.3 |  | 188651995.8 |  |  | 0.000322698 |  |  |  | 3.59408E-05 |  |  |  |  | 0.180666667 |  |  |  | 0.000452123 |  |  |  |  |  |  |
| 164 |  | 286701666.7 |  | 165409208.3 |  |  | 0.000128854 |  |  |  | 0.000015101 |  |  |  |  | 0.1894475 |  |  |  | 0.000178538 |  |  |  |  |  |  |
| 165 |  | 283020416.7 |  | 158533783.3 |  |  | 8.51649E-05 |  |  |  | 9.98962E-06 |  |  |  |  | 0.180381667 |  |  |  | 0.00011692 |  |  |  |  |  |  |
| 166 |  | 281755416.7 |  | 161031583.3 |  |  | 7.72394E-05 |  |  |  | 9.19853E-06 |  |  |  |  | 0.173877083 |  |  |  | 0.000106969 |  |  |  |  |  |  |
| 167 |  | 281930000 |  | 187056666.7 |  |  | 5.2973E-05 |  |  |  | 7.1147E-06 |  |  |  |  | 0.1656025 |  |  |  | 7.09755E-05 |  |  |  |  |  |  |
| 168 |  | 283136666.7 |  | 231306071.3 |  |  | 2.38351E-05 |  |  |  | 4.06898E-06 |  |  |  |  | 0.177693333 |  |  |  | 2.91939E-05 |  |  |  |  |  |  |
| 169 |  | 284099583.3 |  | 288370708.3 |  |  | 1.09181E-05 |  |  |  | 2.38983E-06 |  |  |  |  | 0.18079125 |  |  |  | 1.14289E-05 |  |  |  |  |  |  |
| 170 |  | 284501666.7 |  | 321797666.7 |  |  | 5.52781E-06 |  |  |  | 1.5103E-06 |  |  |  |  | 0.1980375 |  |  |  | 4.48517E-06 |  |  |  |  |  |  |
| 171 |  | 284284166.7 |  | 374975458.3 |  |  | 1.57567E-05 |  |  |  | 1.88772E-06 |  |  |  |  | 0.2148625 |  |  |  | 2.14504E-05 |  |  |  |  |  |  |
| 172 |  | 288417500 |  | 408493458.3 |  |  | 3.23697E-05 |  |  |  | 1.03566E-05 |  |  |  |  | 0.208034167 |  |  |  | 3.63035E-05 |  |  |  |  |  |  |
| 173 |  | 294021250 |  | 424774166.7 |  |  | 5.86943E-05 |  |  |  | 1.51004E-05 |  |  |  |  | 0.193235 |  |  |  | 6.84652E-05 |  |  |  |  |  |  |
| 174 |  | 294643750 |  | 420720000 |  |  | 0.000527046 |  |  |  | 5.57272E-05 |  |  |  |  | 0.193830833 |  |  |  | 0.000746196 |  |  |  |  |  |  |
| 175 |  | 295004166.7 |  | 371098333.3 |  |  | 0.00171108 |  |  |  | 0.000166153 |  |  |  |  | 0.182059583 |  |  |  | 0.002450147 |  |  |  |  |  |  |
| 176 |  | 299512083.3 |  | 334042833.3 |  |  | 0.001684371 |  |  |  | 0.000162516 |  |  |  |  | 0.181961667 |  |  |  | 0.002414546 |  |  |  |  |  |  |
| 177 |  | 296300000 |  | 271844916.7 |  |  | 0.001764517 |  |  |  | 0.000169751 |  |  |  |  | 0.17394 |  |  |  | 0.002530867 |  |  |  |  |  |  |
| 178 |  | 291338750 |  | 242462291.7 |  |  | 0.000956413 |  |  |  | 9.38009E-05 |  |  |  |  | 0.17299 |  |  |  | 0.001367908 |  |  |  |  |  |  |
| 179 |  | 286217916.7 |  | 230307037.5 |  |  | 0.000648575 |  |  |  | 6.40637E-05 |  |  |  |  | 0.165629167 |  |  |  | 0.000926497 |  |  |  |  |  |  |
| 180 |  | 285747916.7 |  | 221795000 |  |  | 0.000495583 |  |  |  | 4.90892E-05 |  |  |  |  | 0.16422875 |  |  |  | 0.000706419 |  |  |  |  |  |  |
| 181 |  | 285129583.3 |  | 242193333.3 |  |  | 0.000389803 |  |  |  | 3.97165E-05 |  |  |  |  | 0.157099167 |  |  |  | 0.000552382 |  |  |  |  |  |  |
| 182 |  | 287059166.7 |  | 268913333.3 |  |  | 0.000835295 |  |  |  | 8.13457E-05 |  |  |  |  | 0.152379583 |  |  |  | 0.00119467 |  |  |  |  |  |  |
| 183 |  | 283672916.7 |  | 291242500 |  |  | 0.000800089 |  |  |  | 7.75969E-05 |  |  |  |  | 0.156805 |  |  |  | 0.001145312 |  |  |  |  |  |  |
| 184 |  | 284018750 |  | 284616250 |  |  | 0.00299294 |  |  |  | 0.000287248 |  |  |  |  | 0.137819583 |  |  |  | 0.004290925 |  |  |  |  |  |  |
| 185 |  | 287636666.7 |  | 298261250 |  |  | 0.010324183 |  |  |  | 0.000985244 |  |  |  |  | 0.116508833 |  |  |  | 0.014818721 |  |  |  |  |  |  |
| 186 |  | 287715833.3 |  | 280649875 |  |  | 0.048451875 |  |  |  | 0.004632929 |  |  |  |  | 0.153426667 |  |  |  | 0.069515333 |  |  |  |  |  |  |
| 187 |  | 286150416.7 |  | 305327833.3 |  |  | 0.0579725 |  |  |  | 0.005543625 |  |  |  |  | 0.18092 |  |  |  | 0.083169167 |  |  |  |  |  |  |
| 188 |  | 285677916.7 |  | 292185416.7 |  |  | 0.053415583 |  |  |  | 0.005105083 |  |  |  |  | 0.204524167 |  |  |  | 0.076636208 |  |  |  |  |  |  |
| 189 |  | 287989166.7 |  | 272620000 |  |  | 0.051930875 |  |  |  | 0.004965263 |  |  |  |  | 0.244838333 |  |  |  | 0.074509292 |  |  |  |  |  |  |
| 190 |  | 289757916.7 |  | 240998875 |  |  | 0.028087417 |  |  |  | 0.002686438 |  |  |  |  | 0.241117083 |  |  |  | 0.040300875 |  |  |  |  |  |  |
| 191 |  | 289278333.3 |  | 218371666.7 |  |  | 0.022987708 |  |  |  | 0.002199133 |  |  |  |  | 0.251590417 |  |  |  | 0.032993708 |  |  |  |  |  |  |
| 192 |  | 285199583.3 |  | 188866425 |  |  | 0.041075792 |  |  |  | 0.003929508 |  |  |  |  | 0.29524875 |  |  |  | 0.058954792 |  |  |  |  |  |  |
| 193 |  | 287051250 |  | 164292750 |  |  | 0.053976667 |  |  |  | 0.005166742 |  |  |  |  | 0.297832917 |  |  |  | 0.0774745 |  |  |  |  |  |  |
| 194 |  | 290597500 |  | 148734958.3 |  |  | 0.051159167 |  |  |  | 0.0048947 |  |  |  |  | 0.28118 |  |  |  | 0.073414375 |  |  |  |  |  |  |
| 195 |  | 291347500 |  | 153203041.7 |  |  | 0.074258125 |  |  |  | 0.007104429 |  |  |  |  | 0.293605417 |  |  |  | 0.106549958 |  |  |  |  |  |  |
| 196 |  | 294391666.7 |  | 179539291.7 |  |  | 0.100709792 |  |  |  | 0.009635179 |  |  |  |  | 0.31936625 |  |  |  | 0.144495833 |  |  |  |  |  |  |
| 197 |  | 292937916.7 |  | 227386250 |  |  | 0.087161917 |  |  |  | 0.008340313 |  |  |  |  | 0.279815 |  |  |  | 0.125052917 |  |  |  |  |  |  |
| 198 |  | 290315000 |  | 254631333.3 |  |  | 0.068216292 |  |  |  | 0.006528358 |  |  |  |  | 0.236137917 |  |  |  | 0.0978675 |  |  |  |  |  |  |
| 199 |  | 289901666.7 |  | 312752500 |  |  | 0.129351667 |  |  |  | 0.012375104 |  |  |  |  | 0.33194 |  |  |  | 0.185557083 |  |  |  |  |  |  |
| 200 |  | 291343333.3 |  | 354645583.3 |  |  | 0.151329167 |  |  |  | 0.014477708 |  |  |  |  | 0.360998333 |  |  |  | 0.217085417 |  |  |  |  |  |  |
| 201 |  | 296516250 |  | 374420416.7 |  |  | 0.096059625 |  |  |  | 0.009193654 |  |  |  |  | 0.306110833 |  |  |  | 0.137799208 |  |  |  |  |  |  |
| 202 |  | 296107916.7 |  | 400579583.3 |  |  | 0.03329475 |  |  |  | 0.003190854 |  |  |  |  | 0.244765 |  |  |  | 0.047755417 |  |  |  |  |  |  |
| 203 |  | 294330000 |  | 379671791.7 |  |  | 0.010301092 |  |  |  | 0.000989919 |  |  |  |  | 0.220830833 |  |  |  | 0.014770708 |  |  |  |  |  |  |
| 204 |  | 295481250 |  | 387496708.3 |  |  | 0.003150604 |  |  |  | 0.000305893 |  |  |  |  | 0.2100275 |  |  |  | 0.004511796 |  |  |  |  |  |  |
| 205 |  | 295882500 |  | 346995833.3 |  |  | 0.001327408 |  |  |  | 0.000131171 |  |  |  |  | 0.196363333 |  |  |  | 0.001896438 |  |  |  |  |  |  |
| 206 |  | 292829166.7 |  | 307538333.3 |  |  | 0.00109165 |  |  |  | 0.000107372 |  |  |  |  | 0.176255833 |  |  |  | 0.001560608 |  |  |  |  |  |  |
| 207 |  | 292447500 |  | 222499458.3 |  |  | 0.006111942 |  |  |  | 0.000585601 |  |  |  |  | 0.162158333 |  |  |  | 0.008763904 |  |  |  |  |  |  |
| 208 |  | 291855416.7 |  | 222467083.3 |  |  | 0.011140696 |  |  |  | 0.001066108 |  |  |  |  | 0.159064583 |  |  |  | 0.015976708 |  |  |  |  |  |  |
| 209 |  | 288551250 |  | 205775333.3 |  |  | 0.008755475 |  |  |  | 0.000836627 |  |  |  |  | 0.173355417 |  |  |  | 0.012558771 |  |  |  |  |  |  |
| 210 |  | 284092916.7 |  | 208769708.3 |  |  | 0.003619508 |  |  |  | 0.000347052 |  |  |  |  | 0.185483333 |  |  |  | 0.005189442 |  |  |  |  |  |  |
| 211 |  | 282635000 |  | 227033041.7 |  |  | 0.001663333 |  |  |  | 0.000161655 |  |  |  |  | 0.179396667 |  |  |  | 0.002380708 |  |  |  |  |  |  |
| 212 |  | 282917916.7 |  | 245080000 |  |  | 0.001668163 |  |  |  | 0.000161301 |  |  |  |  | 0.158699583 |  |  |  | 0.002390121 |  |  |  |  |  |  |
| 213 |  | 283182916.7 |  | 262616875 |  |  | 0.002682908 |  |  |  | 0.00025754 |  |  |  |  | 0.146174583 |  |  |  | 0.003847821 |  |  |  |  |  |  |
| 214 |  | 284521250 |  | 277897125 |  |  | 0.003214217 |  |  |  | 0.000306933 |  |  |  |  | 0.140341667 |  |  |  | 0.004612938 |  |  |  |  |  |  |
| 215 |  | 285725000 |  | 270947208.3 |  |  | 0.002549479 |  |  |  | 0.000239439 |  |  |  |  | 0.17196875 |  |  |  | 0.003668 |  |  |  |  |  |  |
| 216 |  | 285684583.3 |  | 286963583.3 |  |  | 0.001403263 |  |  |  | 0.00013282 |  |  |  |  | 0.172999167 |  |  |  | 0.002015942 |  |  |  |  |  |  |
| 217 |  | 285952083.3 |  | 284532958.3 |  |  | 0.00097446 |  |  |  | 9.38608E-05 |  |  |  |  | 0.162376667 |  |  |  | 0.001396925 |  |  |  |  |  |  |
| 218 |  | 287608333.3 |  | 268492391.7 |  |  | 0.000736173 |  |  |  | 7.07608E-05 |  |  |  |  | 0.1928275 |  |  |  | 0.001055008 |  |  |  |  |  |  |
| 219 |  | 287510416.7 |  | 263996041.7 |  |  | 0.000299108 |  |  |  | 2.98323E-05 |  |  |  |  | 0.207976667 |  |  |  | 0.000426413 |  |  |  |  |  |  |
| 220 |  | 283232500 |  | 253967916.7 |  |  | 0.000127914 |  |  |  | 1.38978E-05 |  |  |  |  | 0.1921025 |  |  |  | 0.000178987 |  |  |  |  |  |  |
| 221 |  | 278197916.7 |  | 232522875 |  |  | 0.0001613 |  |  |  | 1.63155E-05 |  |  |  |  | 0.186836667 |  |  |  | 0.000228309 |  |  |  |  |  |  |
| 222 |  | 273740000 |  | 222023375 |  |  | 0.006466259 |  |  |  | 0.000619097 |  |  |  |  | 0.170234167 |  |  |  | 0.00927533 |  |  |  |  |  |  |
| 223 |  | 275145416.7 |  | 195108458.3 |  |  | 0.061408833 |  |  |  | 0.005874167 |  |  |  |  | 0.22604875 |  |  |  | 0.088105375 |  |  |  |  |  |  |
| 224 |  | 275587916.7 |  | 188320833.3 |  |  | 0.070470917 |  |  |  | 0.006740913 |  |  |  |  | 0.239518333 |  |  |  | 0.101109167 |  |  |  |  |  |  |
| 225 |  | 273021250 |  | 183313291.7 |  |  | 0.049089 |  |  |  | 0.004696938 |  |  |  |  | 0.201450833 |  |  |  | 0.070428417 |  |  |  |  |  |  |
| 226 |  | 275775833.3 |  | 190352750 |  |  | 0.021172125 |  |  |  | 0.002030179 |  |  |  |  | 0.196276667 |  |  |  | 0.030367125 |  |  |  |  |  |  |
| 227 |  | 272331250 |  | 228777958.3 |  |  | 0.006592758 |  |  |  | 0.000633554 |  |  |  |  | 0.201392083 |  |  |  | 0.0094532 |  |  |  |  |  |  |
| 228 |  | 265540416.7 |  | 253695366.7 |  |  | 0.001897775 |  |  |  | 0.000184018 |  |  |  |  | 0.196160417 |  |  |  | 0.00271725 |  |  |  |  |  |  |
| 229 |  | 266224166.7 |  | 305951458.3 |  |  | 0.001999075 |  |  |  | 0.000193735 |  |  |  |  | 0.185301667 |  |  |  | 0.002860729 |  |  |  |  |  |  |
| 230 |  | 272631250 |  | 352103666.7 |  |  | 0.003749725 |  |  |  | 0.000359203 |  |  |  |  | 0.187720833 |  |  |  | 0.005374454 |  |  |  |  |  |  |
| 231 |  | 274496250 |  | 375006833.3 |  |  | 0.001136391 |  |  |  | 0.000114914 |  |  |  |  | 0.198248333 |  |  |  | 0.001616388 |  |  |  |  |  |  |
| 232 |  | 276709166.7 |  | 394720416.7 |  |  | 0.000381395 |  |  |  | 4.28778E-05 |  |  |  |  | 0.18736375 |  |  |  | 0.000533472 |  |  |  |  |  |  |
| 233 |  | 276652500 |  | 380659416.7 |  |  | 0.001082876 |  |  |  | 0.000107064 |  |  |  |  | 0.179405417 |  |  |  | 0.001546531 |  |  |  |  |  |  |
| 234 |  | 274780416.7 |  | 362868750 |  |  | 0.061387933 |  |  |  | 0.005874421 |  |  |  |  | 0.247080417 |  |  |  | 0.088071704 |  |  |  |  |  |  |
| 235 |  | 280706250 |  | 322371666.7 |  |  | 0.105671917 |  |  |  | 0.010114175 |  |  |  |  | 0.307637083 |  |  |  | 0.15160125 |  |  |  |  |  |  |
| 236 |  | 279244583.3 |  | 277360666.7 |  |  | 0.063117125 |  |  |  | 0.006044075 |  |  |  |  | 0.25534875 |  |  |  | 0.090545583 |  |  |  |  |  |  |
| 237 |  | 280351666.7 |  | 247374333.3 |  |  | 0.027854833 |  |  |  | 0.002671396 |  |  |  |  | 0.20842 |  |  |  | 0.039951792 |  |  |  |  |  |  |
| 238 |  | 278443750 |  | 224444416.7 |  |  | 0.009232654 |  |  |  | 0.00089013 |  |  |  |  | 0.193175417 |  |  |  | 0.013233133 |  |  |  |  |  |  |
| 239 |  | 280149166.7 |  | 212641166.7 |  |  | 0.002860808 |  |  |  | 0.000282949 |  |  |  |  | 0.201255833 |  |  |  | 0.004085696 |  |  |  |  |  |  |
| 240 |  | 282215000 |  | 198063833.3 |  |  | 0.000915458 |  |  |  | 9.7018E-05 |  |  |  |  | 0.195931667 |  |  |  | 0.001293496 |  |  |  |  |  |  |
| 241 |  | 285613333.3 |  | 212408083.3 |  |  | 0.000320259 |  |  |  | 4.04819E-05 |  |  |  |  | 0.196389583 |  |  |  | 0.000437415 |  |  |  |  |  |  |
| 242 |  | 285932916.7 |  | 248810083.3 |  |  | 0.000107997 |  |  |  | 1.73713E-05 |  |  |  |  | 0.1984875 |  |  |  | 0.000138562 |  |  |  |  |  |  |
| 243 |  | 287794166.7 |  | 276340112.5 |  |  | 3.7336E-05 |  |  |  | 8.0521E-06 |  |  |  |  | 0.198537917 |  |  |  | 4.22752E-05 |  |  |  |  |  |  |
| 244 |  | 285182083.3 |  | 289578666.7 |  |  | 1.68528E-05 |  |  |  | 4.88985E-06 |  |  |  |  | 0.195013333 |  |  |  | 1.49932E-05 |  |  |  |  |  |  |
| 245 |  | 280114583.3 |  | 282736625 |  |  | 1.30345E-05 |  |  |  | 3.01014E-06 |  |  |  |  | 0.18393125 |  |  |  | 1.40448E-05 |  |  |  |  |  |  |
| 246 |  | 282975416.7 |  | 308343375 |  |  | 1.80645E-05 |  |  |  | 2.61355E-06 |  |  |  |  | 0.175464583 |  |  |  | 2.46753E-05 |  |  |  |  |  |  |
| 247 |  | 285500000 |  | 324997083.3 |  |  | 0.000166294 |  |  |  | 1.7141E-05 |  |  |  |  | 0.154417083 |  |  |  | 0.000236159 |  |  |  |  |  |  |
| 248 |  | 286557500 |  | 300531791.7 |  |  | 0.000507244 |  |  |  | 5.01184E-05 |  |  |  |  | 0.141206667 |  |  |  | 0.000724272 |  |  |  |  |  |  |
| 249 |  | 287600833.3 |  | 299169291.7 |  |  | 0.002313491 |  |  |  | 0.000223877 |  |  |  |  | 0.1267975 |  |  |  | 0.003312258 |  |  |  |  |  |  |
| 250 |  | 290933333.3 |  | 302889791.7 |  |  | 0.003718567 |  |  |  | 0.000357382 |  |  |  |  | 0.119565 |  |  |  | 0.005328992 |  |  |  |  |  |  |
| 251 |  | 286883333.3 |  | 254767833.3 |  |  | 0.004453983 |  |  |  | 0.000426903 |  |  |  |  | 0.1258675 |  |  |  | 0.006385688 |  |  |  |  |  |  |
| 252 |  | 286961666.7 |  | 237432375 |  |  | 0.009121404 |  |  |  | 0.000871533 |  |  |  |  | 0.13278125 |  |  |  | 0.013083521 |  |  |  |  |  |  |
| 253 |  | 284008750 |  | 224133333.3 |  |  | 0.010569179 |  |  |  | 0.001007855 |  |  |  |  | 0.1452375 |  |  |  | 0.0151645 |  |  |  |  |  |  |
| 254 |  | 280067083.3 |  | 206319333.3 |  |  | 0.008250908 |  |  |  | 0.000788134 |  |  |  |  | 0.1395875 |  |  |  | 0.011835979 |  |  |  |  |  |  |
| 255 |  | 297576250 |  | 326213125 |  |  | 0.006666269 |  |  |  | 0.000637924 |  |  |  |  | 0.148125625 |  |  |  | 0.00956125 |  |  |  |  |  |  |

# Appendix F:
Waikiki Beach
Time-Varying Parameters From Hydroqual Data

|  |  | The parameter VD is the volume of the swimming/surfing area. This time-varying parameter was set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | by averaging over each day of the simulation hourly beach segment volumes modeled by HydroQual. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The parameter FD is the flow rate out of water from the swimming/surfing area. This time-varying parameter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | was set by averaging over each day of the simulation the sum of the hourly flows and dispersions out |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | of the beach segment modeled by HydroQual. Dispersion values from HydroQual were given in a volume per |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | unit time basis and were treated as flows for our purposes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The parameter WNS is the average daily concentration of pathogen in the water in the swimming/surfing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | area from sources other than shedding. This time-varying parameter was set by averaging over each day |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | of the simulation hourly pathogen concentrations modeled by HydroQual. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Average |  |  |  | Average Daily |  |  |  |  |  | Average Daily |  |  |  | Average Daily |  |  |  |  |  | Average Daily |  |  |  |  |  |  |  | Average Daily |  |  |  |  |  |  |  |  |
|  |  | Daily |  |  |  | Flow |  |  |  |  |  | Concentration of |  |  |  | Concentration of |  |  |  |  |  | Concentration of |  |  |  |  |  |  |  | Concentration of |  |  |  |  |  |  |  |  |
| Day |  | Volume (L) |  |  |  | Rate Out (L/Day) |  |  |  |  |  | Giardia (Cysts/L) |  |  |  | Cryptosporidium (Cysts/L) |  |  |  |  | Salmonella (#/L) |  |  |  |  |  |  | Enteroviruses (#/L) |  |  |  |  |  |  |  |  |  |  |
|  |  | VD |  |  |  | FD |  |  |  |  |  | WNS |  |  |  | WNS |  |  |  |  |  | WNS |  |  |  |  |  |  |  | WNS |  |  |  |  |  |  |  |  |
| 1 |  | 234397500 |  |  |  | 179718333.3 |  |  |  |  |  | 0.13228 |  |  |  | 0.012652725 |  |  |  |  | 0.400789583 |  |  |  |  |  |  |  | 0.189809583 |  |  |  |  |  |  |  |  |  |
| 2 |  | 238754166.7 |  |  |  | 149202083.3 |  |  |  |  |  | 0.105824583 |  |  |  | 0.010122563 |  |  |  |  |  | 0.400875833 |  |  |  |  |  |  |  | 0.151842083 |  |  |  |  |  |  |  |  |
| 3 |  | 242049583.3 |  |  |  | 122872208.3 |  |  |  |  |  | 0.033256975 |  |  |  | 0.003181658 |  |  |  |  |  | 0.282305833 |  |  |  |  |  |  |  | 0.047717208 |  |  |  |  |  |  |  |  |
| 4 |  | 240258750 |  |  |  | 149207083.3 |  |  |  |  |  | 0.002550006 |  |  |  | 0.000244169 |  |  |  |  |  | 0.317002917 |  |  |  |  |  |  |  | 0.003658298 |  |  |  |  |  |  |  |  |
| 5 |  | 243585000 |  |  |  | 168108333.3 |  |  |  |  |  | 8.99851E-05 |  |  |  | 8.52121E-06 |  |  |  |  |  | 0.354244167 |  |  |  |  |  |  |  | 0.000129734 |  |  |  |  |  |  |  |  |
| 6 |  | 249129166.7 |  |  |  | 223733333.3 |  |  |  |  |  | 1.93595E-05 |  |  |  | 3.88841E-06 |  |  |  |  |  | 0.317670417 |  |  |  |  |  |  |  | 2.42781E-05 |  |  |  |  |  |  |  |  |
| 7 |  | 249220416.7 |  |  |  | 276997125 |  |  |  |  |  | 0.000129626 |  |  |  | 1.46709E-05 |  |  |  |  |  | 0.307686667 |  |  |  |  |  |  |  | 0.000181267 |  |  |  |  |  |  |  |  |
| 8 |  | 251655000 |  |  |  | 330900416.7 |  |  |  |  |  | 0.000404375 |  |  |  | 4.02538E-05 |  |  |  |  |  | 0.326636667 |  |  |  |  |  |  |  | 0.00057646 |  |  |  |  |  |  |  |  |
| 9 |  | 249030416.7 |  |  |  | 357463333.3 |  |  |  |  |  | 0.000353004 |  |  |  | 3.35421E-05 |  |  |  |  |  | 0.295282083 |  |  |  |  |  |  |  | 0.00050628 |  |  |  |  |  |  |  |  |
| 10 |  | 251607916.7 |  |  |  | 373705875 |  |  |  |  |  | 0.005873508 |  |  |  | 0.000562073 |  |  |  |  |  | 0.289425 |  |  |  |  |  |  |  | 0.008424921 |  |  |  |  |  |  |  |  |
| 11 |  | 255512916.7 |  |  |  | 384641458.3 |  |  |  |  |  | 0.011742113 |  |  |  | 0.00112331 |  |  |  |  |  | 0.214434583 |  |  |  |  |  |  |  | 0.016843042 |  |  |  |  |  |  |  |  |
| 12 |  | 255832916.7 |  |  |  | 362108833.3 |  |  |  |  |  | 0.01128425 |  |  |  | 0.001079111 |  |  |  |  |  | 0.258831667 |  |  |  |  |  |  |  | 0.0161867 |  |  |  |  |  |  |  |  |
| 13 |  | 253753750 |  |  |  | 317675166.7 |  |  |  |  |  | 0.002996767 |  |  |  | 0.000286364 |  |  |  |  |  | 0.310242083 |  |  |  |  |  |  |  | 0.004298613 |  |  |  |  |  |  |  |  |
| 14 |  | 248445833.3 |  |  |  | 259062500 |  |  |  |  |  | 0.000744725 |  |  |  | 7.16239E-05 |  |  |  |  |  | 0.285316667 |  |  |  |  |  |  |  | 0.001066836 |  |  |  |  |  |  |  |  |
| 15 |  | 245321666.7 |  |  |  | 207593333.3 |  |  |  |  |  | 0.000325788 |  |  |  | 3.167E-05 |  |  |  |  |  | 0.345519167 |  |  |  |  |  |  |  | 0.000466827 |  |  |  |  |  |  |  |  |
| 16 |  | 242085833.3 |  |  |  | 162729583.3 |  |  |  |  |  | 0.001121462 |  |  |  | 0.000107352 |  |  |  |  |  | 0.3253075 |  |  |  |  |  |  |  | 0.001608688 |  |  |  |  |  |  |  |  |
| 17 |  | 243830416.7 |  |  |  | 155341791.7 |  |  |  |  |  | 0.002768883 |  |  |  | 0.000264033 |  |  |  |  |  | 0.258682083 |  |  |  |  |  |  |  | 0.003973617 |  |  |  |  |  |  |  |  |
| 18 |  | 245459166.7 |  |  |  | 169462500 |  |  |  |  |  | 0.001003687 |  |  |  | 9.59853E-05 |  |  |  |  |  | 0.28334 |  |  |  |  |  |  |  | 0.001439764 |  |  |  |  |  |  |  |  |
| 19 |  | 246695416.7 |  |  |  | 198935000 |  |  |  |  |  | 6.18048E-05 |  |  |  | 6.06064E-06 |  |  |  |  |  | 0.333270833 |  |  |  |  |  |  |  | 8.82342E-05 |  |  |  |  |  |  |  |  |
| 20 |  | 244229583.3 |  |  |  | 234470833.3 |  |  |  |  |  | 1.14884E-05 |  |  |  | 1.05155E-06 |  |  |  |  |  | 0.327547917 |  |  |  |  |  |  |  | 1.63976E-05 |  |  |  |  |  |  |  |  |
| 21 |  | 244272916.7 |  |  |  | 259111875 |  |  |  |  |  | 1.1015E-05 |  |  |  | 1.09634E-06 |  |  |  |  |  | 0.32182375 |  |  |  |  |  |  |  | 1.57737E-05 |  |  |  |  |  |  |  |  |
| 22 |  | 245319166.7 |  |  |  | 282256666.7 |  |  |  |  |  | 0.000315941 |  |  |  | 3.0391E-05 |  |  |  |  |  | 0.24958125 |  |  |  |  |  |  |  | 0.000453043 |  |  |  |  |  |  |  |  |
| 23 |  | 251554166.7 |  |  |  | 308075833.3 |  |  |  |  |  | 0.00069471 |  |  |  | 6.65626E-05 |  |  |  |  |  | 0.241215833 |  |  |  |  |  |  |  | 0.000996818 |  |  |  |  |  |  |  |  |
| 24 |  | 248915000 |  |  |  | 321200125 |  |  |  |  |  | 0.001525888 |  |  |  | 0.000147329 |  |  |  |  |  | 0.21632125 |  |  |  |  |  |  |  | 0.00218727 |  |  |  |  |  |  |  |  |
| 25 |  | 245355000 |  |  |  | 297098750 |  |  |  |  |  | 0.001516574 |  |  |  | 0.00014658 |  |  |  |  |  | 0.1996425 |  |  |  |  |  |  |  | 0.002173616 |  |  |  |  |  |  |  |  |
| 26 |  | 248441250 |  |  |  | 286098083.3 |  |  |  |  |  | 0.004335255 |  |  |  | 0.000414662 |  |  |  |  |  | 0.256672083 |  |  |  |  |  |  |  | 0.006219021 |  |  |  |  |  |  |  |  |
| 27 |  | 254554583.3 |  |  |  | 274409583.3 |  |  |  |  |  | 0.018866417 |  |  |  | 0.001803958 |  |  |  |  |  | 0.241872917 |  |  |  |  |  |  |  | 0.027062917 |  |  |  |  |  |  |  |  |
| 28 |  | 253791250 |  |  |  | 268832083.3 |  |  |  |  |  | 0.0118199 |  |  |  | 0.001129879 |  |  |  |  |  | 0.2524225 |  |  |  |  |  |  |  | 0.016955375 |  |  |  |  |  |  |  |  |
| 29 |  | 251235416.7 |  |  |  | 250278750 |  |  |  |  |  | 0.004975479 |  |  |  | 0.000475395 |  |  |  |  |  | 0.294732917 |  |  |  |  |  |  |  | 0.007137604 |  |  |  |  |  |  |  |  |
| 30 |  | 247600833.3 |  |  |  | 220490000 |  |  |  |  |  | 0.001196373 |  |  |  | 0.000114318 |  |  |  |  |  | 0.305015 |  |  |  |  |  |  |  | 0.001716268 |  |  |  |  |  |  |  |  |
| 31 |  | 248296666.7 |  |  |  | 198166666.7 |  |  |  |  |  | 0.000419401 |  |  |  | 3.97406E-05 |  |  |  |  |  | 0.3726725 |  |  |  |  |  |  |  | 0.000602592 |  |  |  |  |  |  |  |  |
| 32 |  | 247433333.3 |  |  |  | 183738333.3 |  |  |  |  |  | 0.000246871 |  |  |  | 2.3682E-05 |  |  |  |  |  | 0.32323375 |  |  |  |  |  |  |  | 0.000353933 |  |  |  |  |  |  |  |  |
| 33 |  | 244154166.7 |  |  |  | 185243208.3 |  |  |  |  |  | 4.77221E-05 |  |  |  | 3.97255E-06 |  |  |  |  |  | 0.30654125 |  |  |  |  |  |  |  | 6.88149E-05 |  |  |  |  |  |  |  |  |
| 34 |  | 243152083.3 |  |  |  | 214937916.7 |  |  |  |  |  | 8.66295E-06 |  |  |  | 7.08479E-07 |  |  |  |  |  | 0.330294167 |  |  |  |  |  |  |  | 1.30548E-05 |  |  |  |  |  |  |  |  |
| 35 |  | 239561250 |  |  |  | 255546250 |  |  |  |  |  | 7.92817E-05 |  |  |  | 7.71589E-06 |  |  |  |  |  | 0.327117083 |  |  |  |  |  |  |  | 0.000113497 |  |  |  |  |  |  |  |  |
| 36 |  | 239165416.7 |  |  |  | 277176666.7 |  |  |  |  |  | 0.009609565 |  |  |  | 0.000919349 |  |  |  |  |  | 0.23048625 |  |  |  |  |  |  |  | 0.013786397 |  |  |  |  |  |  |  |  |
| 37 |  | 239681250 |  |  |  | 323102958.3 |  |  |  |  |  | 0.0932625 |  |  |  | 0.008921058 |  |  |  |  |  | 0.273222917 |  |  |  |  |  |  |  | 0.133816 |  |  |  |  |  |  |  |  |
| 38 |  | 243576250 |  |  |  | 338140416.7 |  |  |  |  |  | 0.181417917 |  |  |  | 0.017353875 |  |  |  |  |  | 0.420480417 |  |  |  |  |  |  |  | 0.26029625 |  |  |  |  |  |  |  |  |
| 39 |  | 244096666.7 |  |  |  | 366234166.7 |  |  |  |  |  | 0.160941042 |  |  |  | 0.015395763 |  |  |  |  |  | 0.4826675 |  |  |  |  |  |  |  | 0.230897083 |  |  |  |  |  |  |  |  |
| 40 |  | 243482500 |  |  |  | 354701250 |  |  |  |  |  | 0.026492713 |  |  |  | 0.002534605 |  |  |  |  |  | 0.282306667 |  |  |  |  |  |  |  | 0.038005525 |  |  |  |  |  |  |  |  |
| 41 |  | 240961666.7 |  |  |  | 343375416.7 |  |  |  |  |  | 0.004969813 |  |  |  | 0.000475507 |  |  |  |  |  | 0.29035625 |  |  |  |  |  |  |  | 0.007129742 |  |  |  |  |  |  |  |  |
| 42 |  | 238490833.3 |  |  |  | 319645833.3 |  |  |  |  |  | 0.1298945 |  |  |  | 0.012426825 |  |  |  |  |  | 0.385307083 |  |  |  |  |  |  |  | 0.186357667 |  |  |  |  |  |  |  |  |
| 43 |  | 240970416.7 |  |  |  | 296792500 |  |  |  |  |  | 0.241305833 |  |  |  | 0.023082792 |  |  |  |  |  | 0.582909583 |  |  |  |  |  |  |  | 0.346149167 |  |  |  |  |  |  |  |  |
| 44 |  | 243057916.7 |  |  |  | 253632833.3 |  |  |  |  |  | 0.0995925 |  |  |  | 0.009525679 |  |  |  |  |  | 0.393135417 |  |  |  |  |  |  |  | 0.142844667 |  |  |  |  |  |  |  |  |
| 45 |  | 240626666.7 |  |  |  | 215873750 |  |  |  |  |  | 0.022315208 |  |  |  | 0.002134721 |  |  |  |  |  | 0.279735833 |  |  |  |  |  |  |  | 0.032005125 |  |  |  |  |  |  |  |  |
| 46 |  | 240037083.3 |  |  |  | 182950833.3 |  |  |  |  |  | 0.004020529 |  |  |  | 0.000383708 |  |  |  |  |  | 0.279722917 |  |  |  |  |  |  |  | 0.005768492 |  |  |  |  |  |  |  |  |
| 47 |  | 239426666.7 |  |  |  | 166382916.7 |  |  |  |  |  | 0.002358629 |  |  |  | 0.000225605 |  |  |  |  |  | 0.30133125 |  |  |  |  |  |  |  | 0.0033833 |  |  |  |  |  |  |  |  |
| 48 |  | 238441250 |  |  |  | 180795208.3 |  |  |  |  |  | 0.020104138 |  |  |  | 0.001923339 |  |  |  |  |  | 0.250425417 |  |  |  |  |  |  |  | 0.028838429 |  |  |  |  |  |  |  |  |
| 49 |  | 236457916.7 |  |  |  | 205463333.3 |  |  |  |  |  | 0.132966958 |  |  |  | 0.012716875 |  |  |  |  |  | 0.356803333 |  |  |  |  |  |  |  | 0.190789417 |  |  |  |  |  |  |  |  |
| 50 |  | 237887500 |  |  |  | 211582083.3 |  |  |  |  |  | 0.28323375 |  |  |  | 0.027086708 |  |  |  |  |  | 0.688217083 |  |  |  |  |  |  |  | 0.406408333 |  |  |  |  |  |  |  |  |
| 51 |  | 238916666.7 |  |  |  | 238850000 |  |  |  |  |  | 0.231515417 |  |  |  | 0.022142125 |  |  |  |  |  | 0.595602083 |  |  |  |  |  |  |  | 0.332134583 |  |  |  |  |  |  |  |  |
| 52 |  | 238705416.7 |  |  |  | 260202083.3 |  |  |  |  |  | 0.097182167 |  |  |  | 0.009292629 |  |  |  |  |  | 0.321479583 |  |  |  |  |  |  |  | 0.1394275 |  |  |  |  |  |  |  |  |
| 53 |  | 237026666.7 |  |  |  | 268855833.3 |  |  |  |  |  | 0.16424625 |  |  |  | 0.015706667 |  |  |  |  |  | 0.521188333 |  |  |  |  |  |  |  | 0.235632917 |  |  |  |  |  |  |  |  |
| 54 |  | 236373333.3 |  |  |  | 283704583.3 |  |  |  |  |  | 0.130963333 |  |  |  | 0.012524375 |  |  |  |  |  | 0.5153025 |  |  |  |  |  |  |  | 0.187840417 |  |  |  |  |  |  |  |  |
| 55 |  | 238505833.3 |  |  |  | 280024166.7 |  |  |  |  |  | 0.1321675 |  |  |  | 0.012640625 |  |  |  |  |  | 0.44888625 |  |  |  |  |  |  |  | 0.189610417 |  |  |  |  |  |  |  |  |
| 56 |  | 240851250 |  |  |  | 282568416.7 |  |  |  |  |  | 0.112127875 |  |  |  | 0.01072325 |  |  |  |  |  | 0.42625125 |  |  |  |  |  |  |  | 0.160893875 |  |  |  |  |  |  |  |  |
| 57 |  | 239139166.7 |  |  |  | 282159583.3 |  |  |  |  |  | 0.015831238 |  |  |  | 0.00151426 |  |  |  |  |  | 0.346657917 |  |  |  |  |  |  |  | 0.022718792 |  |  |  |  |  |  |  |  |
| 58 |  | 241272916.7 |  |  |  | 282856250 |  |  |  |  |  | 0.009514342 |  |  |  | 0.00091029 |  |  |  |  |  | 0.283045 |  |  |  |  |  |  |  | 0.013654229 |  |  |  |  |  |  |  |  |
| 59 |  | 241502083.3 |  |  |  | 263288333.3 |  |  |  |  |  | 0.003386946 |  |  |  | 0.000323874 |  |  |  |  |  | 0.333204167 |  |  |  |  |  |  |  | 0.004860842 |  |  |  |  |  |  |  |  |
| 60 |  | 243687500 |  |  |  | 247105000 |  |  |  |  |  | 0.013328325 |  |  |  | 0.001275425 |  |  |  |  |  | 0.25255875 |  |  |  |  |  |  |  | 0.019127208 |  |  |  |  |  |  |  |  |
| 61 |  | 245552083.3 |  |  |  | 236993412.5 |  |  |  |  |  | 0.0254255 |  |  |  | 0.002432592 |  |  |  |  |  | 0.24874625 |  |  |  |  |  |  |  | 0.036485542 |  |  |  |  |  |  |  |  |
| 62 |  | 247489583.3 |  |  |  | 211056250 |  |  |  |  |  | 0.025998958 |  |  |  | 0.002486825 |  |  |  |  |  | 0.27966125 |  |  |  |  |  |  |  | 0.037306667 |  |  |  |  |  |  |  |  |
| 63 |  | 248482083.3 |  |  |  | 216866666.7 |  |  |  |  |  | 0.007683888 |  |  |  | 0.000735261 |  |  |  |  |  | 0.290275417 |  |  |  |  |  |  |  | 0.011024504 |  |  |  |  |  |  |  |  |
| 64 |  | 250742500 |  |  |  | 238775000 |  |  |  |  |  | 0.001077416 |  |  |  | 0.000103265 |  |  |  |  |  | 0.349913333 |  |  |  |  |  |  |  | 0.001545523 |  |  |  |  |  |  |  |  |
| 65 |  | 247999583.3 |  |  |  | 290811791.7 |  |  |  |  |  | 0.000187333 |  |  |  | 1.77342E-05 |  |  |  |  |  | 0.372477917 |  |  |  |  |  |  |  | 0.000269495 |  |  |  |  |  |  |  |  |
| 66 |  | 245797083.3 |  |  |  | 316487500 |  |  |  |  |  | 0.004925683 |  |  |  | 0.000472992 |  |  |  |  |  | 0.3050125 |  |  |  |  |  |  |  | 0.007064167 |  |  |  |  |  |  |  |  |
| 67 |  | 246258333.3 |  |  |  | 336387916.7 |  |  |  |  |  | 0.023667542 |  |  |  | 0.002264446 |  |  |  |  |  | 0.313590833 |  |  |  |  |  |  |  | 0.033962458 |  |  |  |  |  |  |  |  |
| 68 |  | 248946250 |  |  |  | 351090891.7 |  |  |  |  |  | 0.033223375 |  |  |  | 0.00317735 |  |  |  |  |  | 0.329620417 |  |  |  |  |  |  |  | 0.047679208 |  |  |  |  |  |  |  |  |
| 69 |  | 251051250 |  |  |  | 365518000 |  |  |  |  |  | 0.018041892 |  |  |  | 0.001726612 |  |  |  |  |  | 0.299477083 |  |  |  |  |  |  |  | 0.025888583 |  |  |  |  |  |  |  |  |
| 70 |  | 251491250 |  |  |  | 377054583.3 |  |  |  |  |  | 0.003792583 |  |  |  | 0.000363713 |  |  |  |  |  | 0.333541667 |  |  |  |  |  |  |  | 0.005440342 |  |  |  |  |  |  |  |  |
| 71 |  | 255276250 |  |  |  | 360151958.3 |  |  |  |  |  | 0.00088545 |  |  |  | 8.37565E-05 |  |  |  |  |  | 0.32359875 |  |  |  |  |  |  |  | 0.001272196 |  |  |  |  |  |  |  |  |
| 72 |  | 256603750 |  |  |  | 344289166.7 |  |  |  |  |  | 0.000987265 |  |  |  | 9.39545E-05 |  |  |  |  |  | 0.306387083 |  |  |  |  |  |  |  | 0.001421715 |  |  |  |  |  |  |  |  |
| 73 |  | 256752500 |  |  |  | 308916083.3 |  |  |  |  |  | 0.001804842 |  |  |  | 0.00017242 |  |  |  |  |  | 0.30196625 |  |  |  |  |  |  |  | 0.002594196 |  |  |  |  |  |  |  |  |
| 74 |  | 256160416.7 |  |  |  | 283624045.8 |  |  |  |  |  | 0.00106295 |  |  |  | 0.000100828 |  |  |  |  |  | 0.320217917 |  |  |  |  |  |  |  | 0.001527747 |  |  |  |  |  |  |  |  |
| 75 |  | 253635833.3 |  |  |  | 196279583.3 |  |  |  |  |  | 0.000236581 |  |  |  | 2.24775E-05 |  |  |  |  |  | 0.367731667 |  |  |  |  |  |  |  | 0.000339758 |  |  |  |  |  |  |  |  |
| 76 |  | 253677083.3 |  |  |  | 209942000 |  |  |  |  |  | 3.73528E-05 |  |  |  | 3.26112E-06 |  |  |  |  |  | 0.367208333 |  |  |  |  |  |  |  | 5.45639E-05 |  |  |  |  |  |  |  |  |
| 77 |  | 255667083.3 |  |  |  | 222120416.7 |  |  |  |  |  | 2.95757E-05 |  |  |  | 3.06818E-06 |  |  |  |  |  | 0.34290875 |  |  |  |  |  |  |  | 4.20435E-05 |  |  |  |  |  |  |  |  |
| 78 |  | 256588333.3 |  |  |  | 233149166.7 |  |  |  |  |  | 3.57176E-05 |  |  |  | 3.79159E-06 |  |  |  |  |  | 0.338791667 |  |  |  |  |  |  |  | 5.14903E-05 |  |  |  |  |  |  |  |  |
| 79 |  | 257122083.3 |  |  |  | 238129000 |  |  |  |  |  | 0.000329452 |  |  |  | 3.22496E-05 |  |  |  |  |  | 0.321316667 |  |  |  |  |  |  |  | 0.000471643 |  |  |  |  |  |  |  |  |
| 80 |  | 261428333.3 |  |  |  | 242227333.3 |  |  |  |  |  | 0.006432625 |  |  |  | 0.000615875 |  |  |  |  |  | 0.209792083 |  |  |  |  |  |  |  | 0.009230213 |  |  |  |  |  |  |  |  |
| 81 |  | 262110000 |  |  |  | 245637916.7 |  |  |  |  |  | 0.008208392 |  |  |  | 0.000785104 |  |  |  |  |  | 0.2578625 |  |  |  |  |  |  |  | 0.011779608 |  |  |  |  |  |  |  |  |
| 82 |  | 262349583.3 |  |  |  | 275086666.7 |  |  |  |  |  | 0.001805971 |  |  |  | 0.000172616 |  |  |  |  |  | 0.320120417 |  |  |  |  |  |  |  | 0.002591854 |  |  |  |  |  |  |  |  |
| 83 |  | 262803333.3 |  |  |  | 287674583.3 |  |  |  |  |  | 0.00031029 |  |  |  | 2.96074E-05 |  |  |  |  |  | 0.320347083 |  |  |  |  |  |  |  | 0.0004455 |  |  |  |  |  |  |  |  |
| 84 |  | 263179166.7 |  |  |  | 312123591.7 |  |  |  |  |  | 0.00041854 |  |  |  | 4.01282E-05 |  |  |  |  |  | 0.289360417 |  |  |  |  |  |  |  | 0.000601058 |  |  |  |  |  |  |  |  |
| 85 |  | 260611250 |  |  |  | 332242250 |  |  |  |  |  | 0.000457762 |  |  |  | 4.3992E-05 |  |  |  |  |  | 0.26077 |  |  |  |  |  |  |  | 0.000657129 |  |  |  |  |  |  |  |  |
| 86 |  | 261155000 |  |  |  | 337223333.3 |  |  |  |  |  | 0.001268791 |  |  |  | 0.000121676 |  |  |  |  |  | 0.247492917 |  |  |  |  |  |  |  | 0.001820674 |  |  |  |  |  |  |  |  |
| 87 |  | 263339166.7 |  |  |  | 338858375 |  |  |  |  |  | 0.007967658 |  |  |  | 0.00076288 |  |  |  |  |  | 0.269494583 |  |  |  |  |  |  |  | 0.011432292 |  |  |  |  |  |  |  |  |
| 88 |  | 266558333.3 |  |  |  | 318167625 |  |  |  |  |  | 0.014061667 |  |  |  | 0.001345563 |  |  |  |  |  | 0.2916875 |  |  |  |  |  |  |  | 0.02017375 |  |  |  |  |  |  |  |  |
| 89 |  | 263844583.3 |  |  |  | 282193625 |  |  |  |  |  | 0.012415667 |  |  |  | 0.001187433 |  |  |  |  |  | 0.328729583 |  |  |  |  |  |  |  | 0.01781 |  |  |  |  |  |  |  |  |
| 90 |  | 262957500 |  |  |  | 259159500 |  |  |  |  |  | 0.013248333 |  |  |  | 0.001266423 |  |  |  |  |  | 0.315833333 |  |  |  |  |  |  |  | 0.019004333 |  |  |  |  |  |  |  |  |
| 91 |  | 263721666.7 |  |  |  | 236353958.3 |  |  |  |  |  | 0.008588608 |  |  |  | 0.000821846 |  |  |  |  |  | 0.294658333 |  |  |  |  |  |  |  | 0.012319667 |  |  |  |  |  |  |  |  |
| 92 |  | 265739166.7 |  |  |  | 232623625 |  |  |  |  |  | 0.004064404 |  |  |  | 0.000390656 |  |  |  |  |  | 0.293429167 |  |  |  |  |  |  |  | 0.005828883 |  |  |  |  |  |  |  |  |
| 93 |  | 263487916.7 |  |  |  | 250692375 |  |  |  |  |  | 0.000847888 |  |  |  | 8.41543E-05 |  |  |  |  |  | 0.2934975 |  |  |  |  |  |  |  | 0.001211158 |  |  |  |  |  |  |  |  |
| 94 |  | 262341250 |  |  |  | 278518545.8 |  |  |  |  |  | 0.000152766 |  |  |  | 1.56033E-05 |  |  |  |  |  | 0.303635 |  |  |  |  |  |  |  | 0.000217119 |  |  |  |  |  |  |  |  |
| 95 |  | 261464166.7 |  |  |  | 312217000 |  |  |  |  |  | 2.3296E-05 |  |  |  | 2.47808E-06 |  |  |  |  |  | 0.300104583 |  |  |  |  |  |  |  | 3.31169E-05 |  |  |  |  |  |  |  |  |
| 96 |  | 258640833.3 |  |  |  | 337787302.5 |  |  |  |  |  | 1.78325E-05 |  |  |  | 1.87674E-06 |  |  |  |  |  | 0.323431667 |  |  |  |  |  |  |  | 2.55048E-05 |  |  |  |  |  |  |  |  |
| 97 |  | 257179583.3 |  |  |  | 360681250 |  |  |  |  |  | 9.2172E-06 |  |  |  | 1.10816E-06 |  |  |  |  |  | 0.357659167 |  |  |  |  |  |  |  | 1.30964E-05 |  |  |  |  |  |  |  |  |
| 98 |  | 257798333.3 |  |  |  | 372116375 |  |  |  |  |  | 3.53577E-06 |  |  |  | 5.39288E-07 |  |  |  |  |  | 0.33708875 |  |  |  |  |  |  |  | 5.15505E-06 |  |  |  |  |  |  |  |  |
| 99 |  | 257072083.3 |  |  |  | 368109166.7 |  |  |  |  |  | 2.46206E-06 |  |  |  | 9.86189E-07 |  |  |  |  |  | 0.327687083 |  |  |  |  |  |  |  | 2.2256E-06 |  |  |  |  |  |  |  |  |
| 100 |  | 255891250 |  |  |  | 343643375 |  |  |  |  |  | 5.63651E-06 |  |  |  | 1.7197E-06 |  |  |  |  |  | 0.331382083 |  |  |  |  |  |  |  | 6.08886E-06 |  |  |  |  |  |  |  |  |
| 101 |  | 258963750 |  |  |  | 320610041.7 |  |  |  |  |  | 1.92133E-05 |  |  |  | 2.56403E-06 |  |  |  |  |  | 0.3189 |  |  |  |  |  |  |  | 2.69355E-05 |  |  |  |  |  |  |  |  |
| 102 |  | 257242916.7 |  |  |  | 288282500 |  |  |  |  |  | 4.26725E-05 |  |  |  | 5.08611E-06 |  |  |  |  |  | 0.323903333 |  |  |  |  |  |  |  | 6.03157E-05 |  |  |  |  |  |  |  |  |
| 103 |  | 256560833.3 |  |  |  | 248404583.3 |  |  |  |  |  | 0.000044636 |  |  |  | 4.58065E-06 |  |  |  |  |  | 0.319914167 |  |  |  |  |  |  |  | 6.39113E-05 |  |  |  |  |  |  |  |  |
| 104 |  | 255119583.3 |  |  |  | 196322083.3 |  |  |  |  |  | 2.76928E-05 |  |  |  | 2.6813E-06 |  |  |  |  |  | 0.337875 |  |  |  |  |  |  |  | 4.06084E-05 |  |  |  |  |  |  |  |  |
| 105 |  | 254127916.7 |  |  |  | 161882083.3 |  |  |  |  |  | 1.02682E-05 |  |  |  | 9.53789E-07 |  |  |  |  |  | 0.3485975 |  |  |  |  |  |  |  | 1.53095E-05 |  |  |  |  |  |  |  |  |
| 106 |  | 253625000 |  |  |  | 155425000 |  |  |  |  |  | 1.69265E-06 |  |  |  | 2.31404E-07 |  |  |  |  |  | 0.368047083 |  |  |  |  |  |  |  | 2.43788E-06 |  |  |  |  |  |  |  |  |
| 107 |  | 254882500 |  |  |  | 158491666.7 |  |  |  |  |  | 9.19521E-07 |  |  |  | 1.22054E-07 |  |  |  |  |  | 0.3858725 |  |  |  |  |  |  |  | 1.40301E-06 |  |  |  |  |  |  |  |  |
| 108 |  | 256072083.3 |  |  |  | 194563333.3 |  |  |  |  |  | 5.25347E-07 |  |  |  | 7.84929E-08 |  |  |  |  |  | 0.347301667 |  |  |  |  |  |  |  | 7.89181E-07 |  |  |  |  |  |  |  |  |
| 109 |  | 253179583.3 |  |  |  | 224307916.7 |  |  |  |  |  | 1.16302E-05 |  |  |  | 1.21255E-06 |  |  |  |  |  | 0.345342083 |  |  |  |  |  |  |  | 1.65657E-05 |  |  |  |  |  |  |  |  |
| 110 |  | 256528333.3 |  |  |  | 230659166.7 |  |  |  |  |  | 0.000565816 |  |  |  | 5.41912E-05 |  |  |  |  |  | 0.36063375 |  |  |  |  |  |  |  | 0.000811875 |  |  |  |  |  |  |  |  |
| 111 |  | 258335833.3 |  |  |  | 274349166.7 |  |  |  |  |  | 0.001173314 |  |  |  | 0.000112392 |  |  |  |  |  | 0.347234167 |  |  |  |  |  |  |  | 0.001683477 |  |  |  |  |  |  |  |  |
| 112 |  | 259329583.3 |  |  |  | 298442500 |  |  |  |  |  | 0.000369451 |  |  |  | 3.48618E-05 |  |  |  |  |  | 0.3329675 |  |  |  |  |  |  |  | 0.000531317 |  |  |  |  |  |  |  |  |
| 113 |  | 263053333.3 |  |  |  | 323556666.7 |  |  |  |  |  | 0.000120044 |  |  |  | 1.12331E-05 |  |  |  |  |  | 0.327616667 |  |  |  |  |  |  |  | 0.00017279 |  |  |  |  |  |  |  |  |
| 114 |  | 262268333.3 |  |  |  | 343927916.7 |  |  |  |  |  | 3.20811E-05 |  |  |  | 2.33873E-06 |  |  |  |  |  | 0.314849583 |  |  |  |  |  |  |  | 4.70452E-05 |  |  |  |  |  |  |  |  |
| 115 |  | 260928750 |  |  |  | 359180000 |  |  |  |  |  | 4.85136E-05 |  |  |  | 4.86272E-06 |  |  |  |  |  | 0.349944583 |  |  |  |  |  |  |  | 6.92816E-05 |  |  |  |  |  |  |  |  |
| 116 |  | 258440416.7 |  |  |  | 352150000 |  |  |  |  |  | 0.0001276 |  |  |  | 1.22747E-05 |  |  |  |  |  | 0.308559167 |  |  |  |  |  |  |  | 0.000182965 |  |  |  |  |  |  |  |  |
| 117 |  | 258989583.3 |  |  |  | 325504166.7 |  |  |  |  |  | 0.000395739 |  |  |  | 3.7926E-05 |  |  |  |  |  | 0.304305417 |  |  |  |  |  |  |  | 0.00056773 |  |  |  |  |  |  |  |  |
| 118 |  | 261484166.7 |  |  |  | 311754166.7 |  |  |  |  |  | 0.000316128 |  |  |  | 3.00298E-05 |  |  |  |  |  | 0.333879583 |  |  |  |  |  |  |  | 0.000453732 |  |  |  |  |  |  |  |  |
| 119 |  | 260827500 |  |  |  | 272873333.3 |  |  |  |  |  | 0.000166046 |  |  |  | 1.51218E-05 |  |  |  |  |  | 0.3179875 |  |  |  |  |  |  |  | 0.000239474 |  |  |  |  |  |  |  |  |
| 120 |  | 261122500 |  |  |  | 238554583.3 |  |  |  |  |  | 3.69882E-05 |  |  |  | 3.4407E-06 |  |  |  |  |  | 0.3198625 |  |  |  |  |  |  |  | 5.41452E-05 |  |  |  |  |  |  |  |  |
| 121 |  | 261389583.3 |  |  |  | 218550000 |  |  |  |  |  | 3.03401E-05 |  |  |  | 3.07735E-06 |  |  |  |  |  | 0.379360833 |  |  |  |  |  |  |  | 4.32118E-05 |  |  |  |  |  |  |  |  |
| 122 |  | 260996250 |  |  |  | 218867916.7 |  |  |  |  |  | 9.6636E-05 |  |  |  | 9.207E-06 |  |  |  |  |  | 0.391464167 |  |  |  |  |  |  |  | 0.000138708 |  |  |  |  |  |  |  |  |
| 123 |  | 258577916.7 |  |  |  | 243648750 |  |  |  |  |  | 0.000232527 |  |  |  | 2.23233E-05 |  |  |  |  |  | 0.354410833 |  |  |  |  |  |  |  | 0.000333493 |  |  |  |  |  |  |  |  |
| 124 |  | 258716250 |  |  |  | 264334166.7 |  |  |  |  |  | 0.002797929 |  |  |  | 0.00026762 |  |  |  |  |  | 0.269972083 |  |  |  |  |  |  |  | 0.004013483 |  |  |  |  |  |  |  |  |
| 125 |  | 258798750 |  |  |  | 280771916.7 |  |  |  |  |  | 0.004105458 |  |  |  | 0.000392292 |  |  |  |  |  | 0.333441667 |  |  |  |  |  |  |  | 0.005889717 |  |  |  |  |  |  |  |  |
| 126 |  | 259882083.3 |  |  |  | 313910416.7 |  |  |  |  |  | 0.000990582 |  |  |  | 9.44198E-05 |  |  |  |  |  | 0.354970833 |  |  |  |  |  |  |  | 0.001421404 |  |  |  |  |  |  |  |  |
| 127 |  | 262490000 |  |  |  | 328690416.7 |  |  |  |  |  | 0.000211299 |  |  |  | 1.96418E-05 |  |  |  |  |  | 0.335417917 |  |  |  |  |  |  |  | 0.000304269 |  |  |  |  |  |  |  |  |
| 128 |  | 263714166.7 |  |  |  | 326151666.7 |  |  |  |  |  | 8.09466E-05 |  |  |  | 7.55148E-06 |  |  |  |  |  | 0.318836667 |  |  |  |  |  |  |  | 0.000116821 |  |  |  |  |  |  |  |  |
| 129 |  | 264834166.7 |  |  |  | 322138333.3 |  |  |  |  |  | 2.75744E-05 |  |  |  | 2.84552E-06 |  |  |  |  |  | 0.346360833 |  |  |  |  |  |  |  | 3.998E-05 |  |  |  |  |  |  |  |  |
| 130 |  | 264473333.3 |  |  |  | 307173750 |  |  |  |  |  | 0.00017202 |  |  |  | 1.66905E-05 |  |  |  |  |  | 0.349618333 |  |  |  |  |  |  |  | 0.000246453 |  |  |  |  |  |  |  |  |
| 131 |  | 262712083.3 |  |  |  | 267425416.7 |  |  |  |  |  | 0.000961611 |  |  |  | 9.17348E-05 |  |  |  |  |  | 0.362554583 |  |  |  |  |  |  |  | 0.00138036 |  |  |  |  |  |  |  |  |
| 132 |  | 264444166.7 |  |  |  | 209462500 |  |  |  |  |  | 0.00100733 |  |  |  | 9.56383E-05 |  |  |  |  |  | 0.376039167 |  |  |  |  |  |  |  | 0.001446902 |  |  |  |  |  |  |  |  |
| 133 |  | 265170833.3 |  |  |  | 186945833.3 |  |  |  |  |  | 0.000436544 |  |  |  | 4.15926E-05 |  |  |  |  |  | 0.367585417 |  |  |  |  |  |  |  | 0.000627045 |  |  |  |  |  |  |  |  |
| 134 |  | 261978333.3 |  |  |  | 135436666.7 |  |  |  |  |  | 0.062947654 |  |  |  | 0.006021597 |  |  |  |  |  | 0.356206667 |  |  |  |  |  |  |  | 0.090319554 |  |  |  |  |  |  |  |  |
| 135 |  | 261231666.7 |  |  |  | 131262500 |  |  |  |  |  | 0.160218667 |  |  |  | 0.015324875 |  |  |  |  |  | 0.5642175 |  |  |  |  |  |  |  | 0.229882917 |  |  |  |  |  |  |  |  |
| 136 |  | 258729166.7 |  |  |  | 124271250 |  |  |  |  |  | 0.045032375 |  |  |  | 0.004306971 |  |  |  |  |  | 0.318165417 |  |  |  |  |  |  |  | 0.064613833 |  |  |  |  |  |  |  |  |
| 137 |  | 260598333.3 |  |  |  | 141633333.3 |  |  |  |  |  | 0.029208167 |  |  |  | 0.002794717 |  |  |  |  |  | 0.233725833 |  |  |  |  |  |  |  | 0.041934625 |  |  |  |  |  |  |  |  |
| 138 |  | 261390416.7 |  |  |  | 171300291.7 |  |  |  |  |  | 0.036361167 |  |  |  | 0.003478633 |  |  |  |  |  | 0.310589583 |  |  |  |  |  |  |  | 0.052195333 |  |  |  |  |  |  |  |  |
| 139 |  | 258625416.7 |  |  |  | 203864166.7 |  |  |  |  |  | 0.017901717 |  |  |  | 0.001711979 |  |  |  |  |  | 0.350015417 |  |  |  |  |  |  |  | 0.025695625 |  |  |  |  |  |  |  |  |
| 140 |  | 252941250 |  |  |  | 243155416.7 |  |  |  |  |  | 0.002979379 |  |  |  | 0.000283118 |  |  |  |  |  | 0.31490625 |  |  |  |  |  |  |  | 0.004279808 |  |  |  |  |  |  |  |  |
| 141 |  | 253750833.3 |  |  |  | 289335833.3 |  |  |  |  |  | 0.002174658 |  |  |  | 0.00020809 |  |  |  |  |  | 0.255155 |  |  |  |  |  |  |  | 0.003119825 |  |  |  |  |  |  |  |  |
| 142 |  | 254151250 |  |  |  | 334056250 |  |  |  |  |  | 0.004852275 |  |  |  | 0.000464033 |  |  |  |  |  | 0.191502917 |  |  |  |  |  |  |  | 0.006961317 |  |  |  |  |  |  |  |  |
| 143 |  | 256827916.7 |  |  |  | 346899166.7 |  |  |  |  |  | 0.006534117 |  |  |  | 0.000624633 |  |  |  |  |  | 0.282831667 |  |  |  |  |  |  |  | 0.009375567 |  |  |  |  |  |  |  |  |
| 144 |  | 258902500 |  |  |  | 356759041.7 |  |  |  |  |  | 0.012276925 |  |  |  | 0.001174353 |  |  |  |  |  | 0.29590875 |  |  |  |  |  |  |  | 0.017615375 |  |  |  |  |  |  |  |  |
| 145 |  | 262665833.3 |  |  |  | 348840416.7 |  |  |  |  |  | 0.008249704 |  |  |  | 0.000788291 |  |  |  |  |  | 0.344031667 |  |  |  |  |  |  |  | 0.011839171 |  |  |  |  |  |  |  |  |
| 146 |  | 262287916.7 |  |  |  | 336131666.7 |  |  |  |  |  | 0.00191156 |  |  |  | 0.000182172 |  |  |  |  |  | 0.359271667 |  |  |  |  |  |  |  | 0.002744021 |  |  |  |  |  |  |  |  |
| 147 |  | 260446250 |  |  |  | 279149583.3 |  |  |  |  |  | 0.000391135 |  |  |  | 3.66014E-05 |  |  |  |  |  | 0.353272083 |  |  |  |  |  |  |  | 0.000562755 |  |  |  |  |  |  |  |  |
| 148 |  | 258370000 |  |  |  | 223709166.7 |  |  |  |  |  | 9.97211E-05 |  |  |  | 9.29404E-06 |  |  |  |  |  | 0.358285 |  |  |  |  |  |  |  | 0.000143901 |  |  |  |  |  |  |  |  |
| 149 |  | 257750833.3 |  |  |  | 187963562.5 |  |  |  |  |  | 0.000800374 |  |  |  | 7.66704E-05 |  |  |  |  |  | 0.370411667 |  |  |  |  |  |  |  | 0.001148302 |  |  |  |  |  |  |  |  |
| 150 |  | 256607500 |  |  |  | 170043333.3 |  |  |  |  |  | 0.002590113 |  |  |  | 0.000247911 |  |  |  |  |  | 0.34220625 |  |  |  |  |  |  |  | 0.003716271 |  |  |  |  |  |  |  |  |
| 151 |  | 256460000 |  |  |  | 194905375 |  |  |  |  |  | 0.002733938 |  |  |  | 0.000260771 |  |  |  |  |  | 0.314140833 |  |  |  |  |  |  |  | 0.003924267 |  |  |  |  |  |  |  |  |
| 152 |  | 254641250 |  |  |  | 221025458.3 |  |  |  |  |  | 0.002176067 |  |  |  | 0.000206758 |  |  |  |  |  | 0.295672083 |  |  |  |  |  |  |  | 0.003125321 |  |  |  |  |  |  |  |  |
| 153 |  | 253148750 |  |  |  | 237988333.3 |  |  |  |  |  | 0.000570823 |  |  |  | 5.45504E-05 |  |  |  |  |  | 0.342695417 |  |  |  |  |  |  |  | 0.000819409 |  |  |  |  |  |  |  |  |
| 154 |  | 251760000 |  |  |  | 265512500 |  |  |  |  |  | 0.000512796 |  |  |  | 4.91434E-05 |  |  |  |  |  | 0.33248625 |  |  |  |  |  |  |  | 0.000735767 |  |  |  |  |  |  |  |  |
| 155 |  | 253792916.7 |  |  |  | 280295833.3 |  |  |  |  |  | 0.007494763 |  |  |  | 0.000717333 |  |  |  |  |  | 0.256499167 |  |  |  |  |  |  |  | 0.010755383 |  |  |  |  |  |  |  |  |
| 156 |  | 257331250 |  |  |  | 286122875 |  |  |  |  |  | 0.015559292 |  |  |  | 0.001488929 |  |  |  |  |  | 0.292236667 |  |  |  |  |  |  |  | 0.02232975 |  |  |  |  |  |  |  |  |
| 157 |  | 260695000 |  |  |  | 291225412.5 |  |  |  |  |  | 0.014297375 |  |  |  | 0.001369354 |  |  |  |  |  | 0.258346667 |  |  |  |  |  |  |  | 0.020516333 |  |  |  |  |  |  |  |  |
| 158 |  | 259292916.7 |  |  |  | 279484625 |  |  |  |  |  | 0.012410583 |  |  |  | 0.001188342 |  |  |  |  |  | 0.21816625 |  |  |  |  |  |  |  | 0.017809458 |  |  |  |  |  |  |  |  |
| 159 |  | 260355000 |  |  |  | 274978875 |  |  |  |  |  | 0.005469692 |  |  |  | 0.000524035 |  |  |  |  |  | 0.240902083 |  |  |  |  |  |  |  | 0.007848913 |  |  |  |  |  |  |  |  |
| 160 |  | 259983333.3 |  |  |  | 260092076.7 |  |  |  |  |  | 0.000612015 |  |  |  | 5.67176E-05 |  |  |  |  |  | 0.351868333 |  |  |  |  |  |  |  | 0.00088192 |  |  |  |  |  |  |  |  |
| 161 |  | 258955000 |  |  |  | 242473750 |  |  |  |  |  | 0.000123575 |  |  |  | 1.23286E-05 |  |  |  |  |  | 0.362000833 |  |  |  |  |  |  |  | 0.000176882 |  |  |  |  |  |  |  |  |
| 162 |  | 258156666.7 |  |  |  | 206920416.7 |  |  |  |  |  | 6.3686E-05 |  |  |  | 6.51596E-06 |  |  |  |  |  | 0.329801667 |  |  |  |  |  |  |  | 9.07109E-05 |  |  |  |  |  |  |  |  |
| 163 |  | 260576666.7 |  |  |  | 171243333.3 |  |  |  |  |  | 1.29805E-05 |  |  |  | 1.35483E-06 |  |  |  |  |  | 0.324405417 |  |  |  |  |  |  |  | 1.83195E-05 |  |  |  |  |  |  |  |  |
| 164 |  | 258875000 |  |  |  | 146635958.3 |  |  |  |  |  | 2.71948E-06 |  |  |  | 3.22179E-07 |  |  |  |  |  | 0.32831875 |  |  |  |  |  |  |  | 3.90322E-06 |  |  |  |  |  |  |  |  |
| 165 |  | 255605000 |  |  |  | 138446625 |  |  |  |  |  | 1.13184E-05 |  |  |  | 3.22056E-06 |  |  |  |  |  | 0.336350833 |  |  |  |  |  |  |  | 1.22498E-05 |  |  |  |  |  |  |  |  |
| 166 |  | 254597083.3 |  |  |  | 143122954.2 |  |  |  |  |  | 2.00423E-05 |  |  |  | 4.1069E-06 |  |  |  |  |  | 0.276556667 |  |  |  |  |  |  |  | 2.49958E-05 |  |  |  |  |  |  |  |  |
| 167 |  | 254666666.7 |  |  |  | 167924583.3 |  |  |  |  |  | 2.70684E-05 |  |  |  | 5.26431E-06 |  |  |  |  |  | 0.156124167 |  |  |  |  |  |  |  | 3.43693E-05 |  |  |  |  |  |  |  |  |
| 168 |  | 255682916.7 |  |  |  | 208452375 |  |  |  |  |  | 1.30955E-05 |  |  |  | 2.39893E-06 |  |  |  |  |  | 0.16948375 |  |  |  |  |  |  |  | 1.62963E-05 |  |  |  |  |  |  |  |  |
| 169 |  | 256182916.7 |  |  |  | 258618679.2 |  |  |  |  |  | 5.05068E-06 |  |  |  | 7.87122E-07 |  |  |  |  |  | 0.173604167 |  |  |  |  |  |  |  | 5.86247E-06 |  |  |  |  |  |  |  |  |
| 170 |  | 256680416.7 |  |  |  | 289057083.3 |  |  |  |  |  | 1.78549E-06 |  |  |  | 3.64526E-07 |  |  |  |  |  | 0.206412917 |  |  |  |  |  |  |  | 1.76725E-06 |  |  |  |  |  |  |  |  |
| 171 |  | 256614166.7 |  |  |  | 337456791.7 |  |  |  |  |  | 2.89562E-07 |  |  |  | 1.09286E-07 |  |  |  |  |  | 0.249480417 |  |  |  |  |  |  |  | 2.74246E-07 |  |  |  |  |  |  |  |  |
| 172 |  | 260105416.7 |  |  |  | 363859125 |  |  |  |  |  | 2.48136E-06 |  |  |  | 1.27212E-06 |  |  |  |  |  | 0.224613333 |  |  |  |  |  |  |  | 1.54469E-06 |  |  |  |  |  |  |  |  |
| 173 |  | 265436250 |  |  |  | 379320083.3 |  |  |  |  |  | 1.64944E-05 |  |  |  | 8.93035E-06 |  |  |  |  |  | 0.2801 |  |  |  |  |  |  |  | 9.38941E-06 |  |  |  |  |  |  |  |  |
| 174 |  | 265794166.7 |  |  |  | 373753187.5 |  |  |  |  |  | 0.000185322 |  |  |  | 0.000023218 |  |  |  |  |  | 0.28047125 |  |  |  |  |  |  |  | 0.000256197 |  |  |  |  |  |  |  |  |
| 175 |  | 266313333.3 |  |  |  | 333395916.7 |  |  |  |  |  | 0.000369523 |  |  |  | 3.72805E-05 |  |  |  |  |  | 0.268185417 |  |  |  |  |  |  |  | 0.000526168 |  |  |  |  |  |  |  |  |
| 176 |  | 270308750 |  |  |  | 299744166.7 |  |  |  |  |  | 0.000290959 |  |  |  | 2.93192E-05 |  |  |  |  |  | 0.26198125 |  |  |  |  |  |  |  | 0.000413927 |  |  |  |  |  |  |  |  |
| 177 |  | 267660833.3 |  |  |  | 245081250 |  |  |  |  |  | 0.000123922 |  |  |  | 1.22338E-05 |  |  |  |  |  | 0.30004875 |  |  |  |  |  |  |  | 0.000177151 |  |  |  |  |  |  |  |  |
| 178 |  | 263100000 |  |  |  | 214749583.3 |  |  |  |  |  | 5.54366E-05 |  |  |  | 5.37525E-06 |  |  |  |  |  | 0.360275417 |  |  |  |  |  |  |  | 7.95295E-05 |  |  |  |  |  |  |  |  |
| 179 |  | 258487083.3 |  |  |  | 203625833.3 |  |  |  |  |  | 6.22716E-05 |  |  |  | 6.07805E-06 |  |  |  |  |  | 0.333227917 |  |  |  |  |  |  |  | 0.000089149 |  |  |  |  |  |  |  |  |
| 180 |  | 258115416.7 |  |  |  | 197652083.3 |  |  |  |  |  | 9.0867E-05 |  |  |  | 8.7496E-06 |  |  |  |  |  | 0.295581667 |  |  |  |  |  |  |  | 0.000129761 |  |  |  |  |  |  |  |  |
| 181 |  | 257672083.3 |  |  |  | 217204625 |  |  |  |  |  | 7.94224E-05 |  |  |  | 7.60585E-06 |  |  |  |  |  | 0.266120833 |  |  |  |  |  |  |  | 0.000113987 |  |  |  |  |  |  |  |  |
| 182 |  | 258997500 |  |  |  | 240830833.3 |  |  |  |  |  | 0.000240218 |  |  |  | 2.30793E-05 |  |  |  |  |  | 0.24424625 |  |  |  |  |  |  |  | 0.000344519 |  |  |  |  |  |  |  |  |
| 183 |  | 255969166.7 |  |  |  | 261641333.3 |  |  |  |  |  | 0.000419325 |  |  |  | 4.07245E-05 |  |  |  |  |  | 0.192742917 |  |  |  |  |  |  |  | 0.00060035 |  |  |  |  |  |  |  |  |
| 184 |  | 256581250 |  |  |  | 258918750 |  |  |  |  |  | 0.000669169 |  |  |  | 6.91125E-05 |  |  |  |  |  | 0.163790833 |  |  |  |  |  |  |  | 0.000953844 |  |  |  |  |  |  |  |  |
| 185 |  | 259659583.3 |  |  |  | 270731875 |  |  |  |  |  | 0.002201738 |  |  |  | 0.00021138 |  |  |  |  |  | 0.173687083 |  |  |  |  |  |  |  | 0.003158771 |  |  |  |  |  |  |  |  |
| 186 |  | 259888333.3 |  |  |  | 257670416.7 |  |  |  |  |  | 0.016088121 |  |  |  | 0.001537271 |  |  |  |  |  | 0.244942917 |  |  |  |  |  |  |  | 0.023083817 |  |  |  |  |  |  |  |  |
| 187 |  | 258091666.7 |  |  |  | 279033750 |  |  |  |  |  | 0.051364667 |  |  |  | 0.0049104 |  |  |  |  |  | 0.28896375 |  |  |  |  |  |  |  | 0.073691708 |  |  |  |  |  |  |  |  |
| 188 |  | 257420000 |  |  |  | 260257916.7 |  |  |  |  |  | 0.047822167 |  |  |  | 0.004570313 |  |  |  |  |  | 0.335679583 |  |  |  |  |  |  |  | 0.068604542 |  |  |  |  |  |  |  |  |
| 189 |  | 260102500 |  |  |  | 246376250 |  |  |  |  |  | 0.025815 |  |  |  | 0.002468329 |  |  |  |  |  | 0.331160417 |  |  |  |  |  |  |  | 0.037034125 |  |  |  |  |  |  |  |  |
| 190 |  | 261549583.3 |  |  |  | 215707916.7 |  |  |  |  |  | 0.017244604 |  |  |  | 0.001649388 |  |  |  |  |  | 0.341494583 |  |  |  |  |  |  |  | 0.024742833 |  |  |  |  |  |  |  |  |
| 191 |  | 261573750 |  |  |  | 201433750 |  |  |  |  |  | 0.007334246 |  |  |  | 0.000701796 |  |  |  |  |  | 0.295840417 |  |  |  |  |  |  |  | 0.010525925 |  |  |  |  |  |  |  |  |
| 192 |  | 257321666.7 |  |  |  | 174483750 |  |  |  |  |  | 0.038158417 |  |  |  | 0.003649388 |  |  |  |  |  | 0.365113333 |  |  |  |  |  |  |  | 0.05476 |  |  |  |  |  |  |  |  |
| 193 |  | 259370833.3 |  |  |  | 150990000 |  |  |  |  |  | 0.085249708 |  |  |  | 0.008158054 |  |  |  |  |  | 0.455825 |  |  |  |  |  |  |  | 0.122340375 |  |  |  |  |  |  |  |  |
| 194 |  | 262473333.3 |  |  |  | 135706250 |  |  |  |  |  | 0.074238125 |  |  |  | 0.007107579 |  |  |  |  |  | 0.393040417 |  |  |  |  |  |  |  | 0.106564083 |  |  |  |  |  |  |  |  |
| 195 |  | 263310833.3 |  |  |  | 138378333.3 |  |  |  |  |  | 0.09266575 |  |  |  | 0.008865375 |  |  |  |  |  | 0.404604167 |  |  |  |  |  |  |  | 0.132956792 |  |  |  |  |  |  |  |  |
| 196 |  | 266137916.7 |  |  |  | 161458333.3 |  |  |  |  |  | 0.186867083 |  |  |  | 0.017877667 |  |  |  |  |  | 0.589700417 |  |  |  |  |  |  |  | 0.268125417 |  |  |  |  |  |  |  |  |
| 197 |  | 264386250 |  |  |  | 203787833.3 |  |  |  |  |  | 0.137969583 |  |  |  | 0.013199167 |  |  |  |  |  | 0.49433125 |  |  |  |  |  |  |  | 0.197954167 |  |  |  |  |  |  |  |  |
| 198 |  | 261994166.7 |  |  |  | 230390000 |  |  |  |  |  | 0.083207417 |  |  |  | 0.007960429 |  |  |  |  |  | 0.362712083 |  |  |  |  |  |  |  | 0.119379708 |  |  |  |  |  |  |  |  |
| 199 |  | 261780416.7 |  |  |  | 284818083.3 |  |  |  |  |  | 0.045932667 |  |  |  | 0.004394221 |  |  |  |  |  | 0.237512083 |  |  |  |  |  |  |  | 0.065899083 |  |  |  |  |  |  |  |  |
| 200 |  | 263699166.7 |  |  |  | 317395833.3 |  |  |  |  |  | 0.054457042 |  |  |  | 0.005208967 |  |  |  |  |  | 0.191706667 |  |  |  |  |  |  |  | 0.078119292 |  |  |  |  |  |  |  |  |
| 201 |  | 267436666.7 |  |  |  | 337195000 |  |  |  |  |  | 0.044308625 |  |  |  | 0.004238558 |  |  |  |  |  | 0.2915775 |  |  |  |  |  |  |  | 0.063563083 |  |  |  |  |  |  |  |  |
| 202 |  | 267558750 |  |  |  | 359039166.7 |  |  |  |  |  | 0.007059496 |  |  |  | 0.000675751 |  |  |  |  |  | 0.33561125 |  |  |  |  |  |  |  | 0.010127283 |  |  |  |  |  |  |  |  |
| 203 |  | 265798333.3 |  |  |  | 343382666.7 |  |  |  |  |  | 0.000434572 |  |  |  | 4.12745E-05 |  |  |  |  |  | 0.363829167 |  |  |  |  |  |  |  | 0.000624107 |  |  |  |  |  |  |  |  |
| 204 |  | 266811666.7 |  |  |  | 350755625 |  |  |  |  |  | 0.000110189 |  |  |  | 1.05081E-05 |  |  |  |  |  | 0.36925125 |  |  |  |  |  |  |  | 0.000158341 |  |  |  |  |  |  |  |  |
| 205 |  | 267412083.3 |  |  |  | 314295833.3 |  |  |  |  |  | 0.000123227 |  |  |  | 1.15401E-05 |  |  |  |  |  | 0.374430417 |  |  |  |  |  |  |  | 0.000177678 |  |  |  |  |  |  |  |  |
| 206 |  | 264492916.7 |  |  |  | 280608333.3 |  |  |  |  |  | 0.000261708 |  |  |  | 2.50015E-05 |  |  |  |  |  | 0.355645 |  |  |  |  |  |  |  | 0.000375616 |  |  |  |  |  |  |  |  |
| 207 |  | 263845000 |  |  |  | 202175000 |  |  |  |  |  | 0.004910226 |  |  |  | 0.000469675 |  |  |  |  |  | 0.29057375 |  |  |  |  |  |  |  | 0.007042142 |  |  |  |  |  |  |  |  |
| 208 |  | 263658750 |  |  |  | 199836250 |  |  |  |  |  | 0.008230804 |  |  |  | 0.00078669 |  |  |  |  |  | 0.323795417 |  |  |  |  |  |  |  | 0.011806042 |  |  |  |  |  |  |  |  |
| 209 |  | 260746666.7 |  |  |  | 184644958.3 |  |  |  |  |  | 0.004559617 |  |  |  | 0.000435994 |  |  |  |  |  | 0.342179583 |  |  |  |  |  |  |  | 0.006539817 |  |  |  |  |  |  |  |  |
| 210 |  | 256706666.7 |  |  |  | 187820833.3 |  |  |  |  |  | 0.000628414 |  |  |  | 5.97453E-05 |  |  |  |  |  | 0.361474167 |  |  |  |  |  |  |  | 0.000902051 |  |  |  |  |  |  |  |  |
| 211 |  | 255455833.3 |  |  |  | 203365833.3 |  |  |  |  |  | 0.000145971 |  |  |  | 1.37534E-05 |  |  |  |  |  | 0.321675833 |  |  |  |  |  |  |  | 0.000209911 |  |  |  |  |  |  |  |  |
| 212 |  | 255247916.7 |  |  |  | 220511250 |  |  |  |  |  | 0.000149931 |  |  |  | 1.44065E-05 |  |  |  |  |  | 0.310864167 |  |  |  |  |  |  |  | 0.000215083 |  |  |  |  |  |  |  |  |
| 213 |  | 255752916.7 |  |  |  | 237392083.3 |  |  |  |  |  | 0.00064196 |  |  |  | 6.15105E-05 |  |  |  |  |  | 0.315312917 |  |  |  |  |  |  |  | 0.000921036 |  |  |  |  |  |  |  |  |
| 214 |  | 256635416.7 |  |  |  | 247159541.7 |  |  |  |  |  | 0.001643367 |  |  |  | 0.000156805 |  |  |  |  |  | 0.276164583 |  |  |  |  |  |  |  | 0.002358858 |  |  |  |  |  |  |  |  |
| 215 |  | 258060416.7 |  |  |  | 243812083.3 |  |  |  |  |  | 0.002054638 |  |  |  | 0.000195561 |  |  |  |  |  | 0.286392083 |  |  |  |  |  |  |  | 0.002949479 |  |  |  |  |  |  |  |  |
| 216 |  | 257998333.3 |  |  |  | 259449583.3 |  |  |  |  |  | 0.000704774 |  |  |  | 6.70082E-05 |  |  |  |  |  | 0.3166275 |  |  |  |  |  |  |  | 0.001011056 |  |  |  |  |  |  |  |  |
| 217 |  | 258222500 |  |  |  | 255370833.3 |  |  |  |  |  | 0.000186006 |  |  |  | 1.79502E-05 |  |  |  |  |  | 0.355797083 |  |  |  |  |  |  |  | 0.000266475 |  |  |  |  |  |  |  |  |
| 218 |  | 259853750 |  |  |  | 241612500 |  |  |  |  |  | 0.000166634 |  |  |  | 1.61444E-05 |  |  |  |  |  | 0.348555 |  |  |  |  |  |  |  | 0.00023881 |  |  |  |  |  |  |  |  |
| 219 |  | 259868750 |  |  |  | 238558333.3 |  |  |  |  |  | 7.00911E-05 |  |  |  | 6.73402E-06 |  |  |  |  |  | 0.36009125 |  |  |  |  |  |  |  | 0.000100564 |  |  |  |  |  |  |  |  |
| 220 |  | 256052500 |  |  |  | 229982916.7 |  |  |  |  |  | 1.07161E-05 |  |  |  | 8.6374E-07 |  |  |  |  |  | 0.365559583 |  |  |  |  |  |  |  | 1.5977E-05 |  |  |  |  |  |  |  |  |
| 221 |  | 251371250 |  |  |  | 213219583.3 |  |  |  |  |  | 7.25473E-06 |  |  |  | 7.67434E-07 |  |  |  |  |  | 0.352426667 |  |  |  |  |  |  |  | 1.03571E-05 |  |  |  |  |  |  |  |  |
| 222 |  | 247346250 |  |  |  | 201904166.7 |  |  |  |  |  | 0.002808681 |  |  |  | 0.000268796 |  |  |  |  |  | 0.323665 |  |  |  |  |  |  |  | 0.004029254 |  |  |  |  |  |  |  |  |
| 223 |  | 248380833.3 |  |  |  | 176079166.7 |  |  |  |  |  | 0.045717167 |  |  |  | 0.004372971 |  |  |  |  |  | 0.290885417 |  |  |  |  |  |  |  | 0.065589042 |  |  |  |  |  |  |  |  |
| 224 |  | 249258750 |  |  |  | 169841666.7 |  |  |  |  |  | 0.041835167 |  |  |  | 0.004000692 |  |  |  |  |  | 0.358190833 |  |  |  |  |  |  |  | 0.060022625 |  |  |  |  |  |  |  |  |
| 225 |  | 246849583.3 |  |  |  | 166987500 |  |  |  |  |  | 0.015087954 |  |  |  | 0.001442217 |  |  |  |  |  | 0.300675833 |  |  |  |  |  |  |  | 0.021650788 |  |  |  |  |  |  |  |  |
| 226 |  | 249092500 |  |  |  | 171004166.7 |  |  |  |  |  | 0.002552248 |  |  |  | 0.000243615 |  |  |  |  |  | 0.359789167 |  |  |  |  |  |  |  | 0.003663096 |  |  |  |  |  |  |  |  |
| 227 |  | 246036250 |  |  |  | 207533333.3 |  |  |  |  |  | 0.00065916 |  |  |  | 6.28794E-05 |  |  |  |  |  | 0.334881667 |  |  |  |  |  |  |  | 0.00094613 |  |  |  |  |  |  |  |  |
| 228 |  | 239888333.3 |  |  |  | 231058750 |  |  |  |  |  | 0.000204494 |  |  |  | 1.91543E-05 |  |  |  |  |  | 0.314915833 |  |  |  |  |  |  |  | 0.000294041 |  |  |  |  |  |  |  |  |
| 229 |  | 240757083.3 |  |  |  | 278866666.7 |  |  |  |  |  | 0.00011659 |  |  |  | 1.1393E-05 |  |  |  |  |  | 0.34816 |  |  |  |  |  |  |  | 0.000166719 |  |  |  |  |  |  |  |  |
| 230 |  | 246035416.7 |  |  |  | 315478750 |  |  |  |  |  | 0.000275375 |  |  |  | 2.61319E-05 |  |  |  |  |  | 0.330388333 |  |  |  |  |  |  |  | 0.000395507 |  |  |  |  |  |  |  |  |
| 231 |  | 247966666.7 |  |  |  | 338756250 |  |  |  |  |  | 0.000120571 |  |  |  | 1.04188E-05 |  |  |  |  |  | 0.36822625 |  |  |  |  |  |  |  | 0.000176868 |  |  |  |  |  |  |  |  |
| 232 |  | 250017916.7 |  |  |  | 356266666.7 |  |  |  |  |  | 0.000022916 |  |  |  | 2.4226E-06 |  |  |  |  |  | 0.352198333 |  |  |  |  |  |  |  | 3.24831E-05 |  |  |  |  |  |  |  |  |
| 233 |  | 249748333.3 |  |  |  | 346219166.7 |  |  |  |  |  | 0.000230253 |  |  |  | 2.2312E-05 |  |  |  |  |  | 0.324594167 |  |  |  |  |  |  |  | 0.000329904 |  |  |  |  |  |  |  |  |
| 234 |  | 248062083.3 |  |  |  | 326329500 |  |  |  |  |  | 0.026152954 |  |  |  | 0.0025027 |  |  |  |  |  | 0.25040125 |  |  |  |  |  |  |  | 0.037520838 |  |  |  |  |  |  |  |  |
| 235 |  | 253195000 |  |  |  | 288590750 |  |  |  |  |  | 0.102137083 |  |  |  | 0.009773788 |  |  |  |  |  | 0.387541667 |  |  |  |  |  |  |  | 0.146533167 |  |  |  |  |  |  |  |  |
| 236 |  | 252466666.7 |  |  |  | 249765833.3 |  |  |  |  |  | 0.051345208 |  |  |  | 0.004914188 |  |  |  |  |  | 0.380320833 |  |  |  |  |  |  |  | 0.073663875 |  |  |  |  |  |  |  |  |
| 237 |  | 253230000 |  |  |  | 221045833.3 |  |  |  |  |  | 0.004499633 |  |  |  | 0.000430427 |  |  |  |  |  | 0.355434583 |  |  |  |  |  |  |  | 0.006456025 |  |  |  |  |  |  |  |  |
| 238 |  | 251622083.3 |  |  |  | 201708333.3 |  |  |  |  |  | 0.000273745 |  |  |  | 2.59674E-05 |  |  |  |  |  | 0.411517083 |  |  |  |  |  |  |  | 0.000393304 |  |  |  |  |  |  |  |  |
| 239 |  | 253286666.7 |  |  |  | 189779166.7 |  |  |  |  |  | 3.39277E-05 |  |  |  | 3.24433E-06 |  |  |  |  |  | 0.3929025 |  |  |  |  |  |  |  | 4.88767E-05 |  |  |  |  |  |  |  |  |
| 240 |  | 254877500 |  |  |  | 179854166.7 |  |  |  |  |  | 3.94847E-06 |  |  |  | 4.37018E-07 |  |  |  |  |  | 0.418620833 |  |  |  |  |  |  |  | 5.64536E-06 |  |  |  |  |  |  |  |  |
| 241 |  | 258118750 |  |  |  | 193045833.3 |  |  |  |  |  | 3.97889E-07 |  |  |  | 7.16638E-08 |  |  |  |  |  | 0.388984583 |  |  |  |  |  |  |  | 5.1553E-07 |  |  |  |  |  |  |  |  |
| 242 |  | 258455833.3 |  |  |  | 224541666.7 |  |  |  |  |  | 5.64833E-08 |  |  |  | 1.49343E-08 |  |  |  |  |  | 0.407787083 |  |  |  |  |  |  |  | 6.35373E-08 |  |  |  |  |  |  |  |  |
| 243 |  | 260043750 |  |  |  | 250355833.3 |  |  |  |  |  | 4.41535E-08 |  |  |  | 1.81862E-08 |  |  |  |  |  | 0.4051625 |  |  |  |  |  |  |  | 3.57865E-08 |  |  |  |  |  |  |  |  |
| 244 |  | 257406666.7 |  |  |  | 263612500 |  |  |  |  |  | 6.2909E-08 |  |  |  | 3.57798E-08 |  |  |  |  |  | 0.390139167 |  |  |  |  |  |  |  | 3.00722E-08 |  |  |  |  |  |  |  |  |
| 245 |  | 253206666.7 |  |  |  | 256625833.3 |  |  |  |  |  | 1.17086E-06 |  |  |  | 3.48965E-07 |  |  |  |  |  | 0.3643175 |  |  |  |  |  |  |  | 1.23202E-06 |  |  |  |  |  |  |  |  |
| 246 |  | 255502500 |  |  |  | 277205416.7 |  |  |  |  |  | 3.26564E-06 |  |  |  | 5.42275E-07 |  |  |  |  |  | 0.322585833 |  |  |  |  |  |  |  | 4.23325E-06 |  |  |  |  |  |  |  |  |
| 247 |  | 257837083.3 |  |  |  | 292859125 |  |  |  |  |  | 1.31259E-05 |  |  |  | 1.67447E-06 |  |  |  |  |  | 0.275744583 |  |  |  |  |  |  |  | 1.83083E-05 |  |  |  |  |  |  |  |  |
| 248 |  | 259010000 |  |  |  | 273012833.3 |  |  |  |  |  | 9.60695E-05 |  |  |  | 1.10302E-05 |  |  |  |  |  | 0.235856667 |  |  |  |  |  |  |  | 0.000134966 |  |  |  |  |  |  |  |  |
| 249 |  | 259739583.3 |  |  |  | 272097666.7 |  |  |  |  |  | 0.000438056 |  |  |  | 4.25072E-05 |  |  |  |  |  | 0.254034167 |  |  |  |  |  |  |  | 0.000627458 |  |  |  |  |  |  |  |  |
| 250 |  | 262253333.3 |  |  |  | 273563875 |  |  |  |  |  | 0.002438575 |  |  |  | 0.000233538 |  |  |  |  |  | 0.23577 |  |  |  |  |  |  |  | 0.003496446 |  |  |  |  |  |  |  |  |
| 251 |  | 259247083.3 |  |  |  | 232798125 |  |  |  |  |  | 0.004068983 |  |  |  | 0.000389428 |  |  |  |  |  | 0.207019167 |  |  |  |  |  |  |  | 0.005834771 |  |  |  |  |  |  |  |  |
| 252 |  | 258882083.3 |  |  |  | 213680695.8 |  |  |  |  |  | 0.002330229 |  |  |  | 0.000222354 |  |  |  |  |  | 0.171899167 |  |  |  |  |  |  |  | 0.003342888 |  |  |  |  |  |  |  |  |
| 253 |  | 256586666.7 |  |  |  | 203033083.3 |  |  |  |  |  | 0.003245942 |  |  |  | 0.000313415 |  |  |  |  |  | 0.157702083 |  |  |  |  |  |  |  | 0.004649979 |  |  |  |  |  |  |  |  |
| 254 |  | 252877083.3 |  |  |  | 185896458.3 |  |  |  |  |  | 0.002087567 |  |  |  | 0.000202093 |  |  |  |  |  | 0.134798333 |  |  |  |  |  |  |  | 0.002989563 |  |  |  |  |  |  |  |  |
| 255 |  | 268813750 |  |  |  | 292546875 |  |  |  |  |  | 0.001612525 |  |  |  | 0.000155933 |  |  |  |  |  | 0.186626875 |  |  |  |  |  |  |  | 0.002309813 |  |  |  |  |  |  |  |  |

?Same lower bound used for persistence of Giardia cysts.

?Based on oocysts remaining infective in a moist environment up to 6 months (ref. 29).

?Based on incidence data for Giardia lamblia.

?For typhoid fever.

?Based on duration of protection for typhoid vaccine.

?Based on duration of infection for salmonellosis.

?Based on 2-3 day incubation period for minor illnesses associated with enteroviruses.

?This range was selected based on lifelong type-specific immunity to polio virus.

?For minor illnesses associated with enteroviruses.