Table 12 . 	Organic contaminants measured in tissues of Ostrea sandvicensis sampled in 1986-1988, and 1990-1992 at the National Mussel Watch Stations at Barber’s Pt. and Honolulu Harbor (Keehi Lagoon), Mamala Bay, Oahu, Hawaii. Data from NOAA (1989, 1995).

|  |  | Tissue Concentration (ng/g dry weight) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stressor | Location | 1986 | 1987 | 1988 | 1990 | 1991 | 1992 |
| Total PCBs1 | Barber’s Pt Honolulu Hbr. | 596 126 | 43 276 | 22 135 | 14 126 | 24 168 | 7.2 81 |
| Total DDT2 | Barber’s Pt Honolulu Hbr. | 53 7.5 | 4.7 32 | 1.3 28 | 1.7 39 | 2.6 41 | nd 9.2 |
| Chlordane3 | Barber’s Pt Honolulu Hbr. | 47 4.6 | 7.7 32 | 1.9 41 | 5.3 36 | 0.2 100 | nd 8.4 |
| Dieldrin | Barber’s Pt Honolulu Hbr. | 14 0.13 | 3.7 4.3 | 1.1 13 | nd 17 | nd 72 | nd 3.5 |
| Lindane | Barber’s Pt Honolulu Hbr. | 1.1 nd6 | nd 0.33 | 0.88 0.35 | nd 2.7 | 0.02 nd | nd nd |
| Hexachloro- benzene | Barber’s Pt Honolulu Hbr. | 29 nd | 0.33 nd | nd 0.14 | nd 0.37 | nd nd | nd nd |
| LMWPAHs4 | Barber’s Pt Honolulu Hbr. | 1,523 nd | 256 1,277 | 23 269 | 61 1,011 | 472 2,350 | 96 889 |
| HMWPAHs5 | Barber’s Pt Honolulu Hbr. | 6,050 nm7 | nd 2,477 | 11 3,343 | 133 5,774 | 56 15,722 | 79 5,877 |

1. 	PCBs were the total of eight “chlorination classes” in 1986 and 1987. Beginning in 1988 between 18 and 20 congeners of varying chlorine substitution were summed for total PCB concentration.

2.	Total DDTs comprise o,p’-DDT, p,p’-DDT and the p,p’ and o,p’-DDD and DDE metabolites.

3.	“Total chlordane” is the sum of -chlordane, trans-nonachlor, heptachlor, and heptachlor epoxide.

4.	Low Molecular weight polycyclic aromatic hydrocarbons (2-, and 3-rings).

5.	High Molecular weight polycyclic aromatic hydrocarbons (>4 rings).

6.	nd indicates compound below the detection limit for the method used.

7.	nm idicates the compound was not included in analyses from the site in question.