Table 15.  	Comparison of estimated effluent concentrations of pollutants after initial dilution (factor of 400) with Hawaii Water Quality Standards. Stressors which, after dilution, are within a factor of 100 of the existing WQS would be considered candidates for evaluation in Phase II. BOD, Oil and Grease and TSS concentrations are in mg/L. All other concentrations are in ug/L.

| Pollutant | Hawaii Acute Saltwater Standard | Hawaii Chronic Saltwater Standard | Sand Island Effluent | Honouliuli Effluent | Fort Kam. Effluent |
| --- | --- | --- | --- | --- | --- |
| Ammonia | 3.5 | ns1 | - | - | 0.0047 |
| Benzene | 1,700 | ns | 0.025 | - | - |
| BOD (mg/L) | ns | ns | - | - | - |
| Cadmium | 43 | 9.3 | 0.0065 | - | - |
| Chloride ion | 13 | 7.5 | na2 | na2 | 0.0065 |
| Chromium | 1,100 | 50 | 0.05 | - | - |
| Copper | ns | ns | 0.045 | 0.082 | 0.045 |
| Diethylphthalate | ns | ns | - | 0.053 | - |
| Lead | 140 | ns | 0.100 | - | 0.06 |
| Methylchloride | ns | ns | 0.017 | - | - |
| Oil & Grease (mg/L) | ns | ns |  |  |  |
| Phenol | 170 | ns | 0.04 | - | - |
| Tetrachloroethylene | 3,400 | 145 | 0.012 | - | - |
| Toluene | 2,100 | ns | 0.015 | - | - |
| TSS (mg/L) | ns | ns |  |  |  |
| Zinc | 95 | 86 | 0.15 | 0.198 | 0.262 |

1.	“ns” indicates no standard has been established.

2.	“na” indicates not applicable; chlorination was performed only at Fort Kamehameha WWTP.