PROJECT MB-10

ENVIRONMENTAL IMPACTS ON RECEPTORS AND RESOURCES

Part III

EFFECTS OF EFFLUENT FROM THE BARBERS POINT AND SAND ISLAND

OUTFALLS ON THE MAMALA BAY ECOSYSTEM

William J. Kimmerer

Joseph M. O'Connor

Executive Summary

The purpose of this report is to present results of analyses of existing data on chemical and biological conditions in Mamala Bay. The objective of these analyses was to determine whether ecological effects of sewage discharge from the Sand Island and Barbers Point outfalls could be discerned in the data, and the severity of any effects. Sediment chemical composition in the two areas showed considerable spatial and interannual variability. However, almost none of that variability suggested sewage input. The single exception was for TKN in 1990, which was somewhat elevated at and near the ZID station. This general lack of clear gradients in sediment chemistry with distance from the outfalls could arise for several reasons:

1.	Rates of input of sewage-derived material from the outfall are too low to produce a gradient in deposition rates with distance from the outfall

2.	A gradient in deposition rate of organic matter exists, but benthic oxidation of organic matter is so rapid that a gradient in organic matter concentration cannot be detected

3.	The spatial scale of sampling is too small: either the ZID station is outside the area of influence of the outfall, or the far-field stations are substantially affected by effluent

4.	Sampling or analytical techniques are not sensitive enough to detect a sewage effect.

Some evidence shows an effect of sewage inputs on sediments near the outfall, but the flux of sewage-derived sediment to the bottom is small and the benthic oxidation of these sediments seems to be rapid.

High values of some of the variables occurred at all stations at a site in one year, but never in both areas in the same year. This suggests that much of the variability is either seasonal or irregular in time. Without further information on the spatial and intra-annual variability in these data, the cause of the variability cannot be determined.

The apparent effects of sewage discharge on sediment and benthic fauna in the vicinity of the two outfalls are small and confined to a few taxonomic groups. There was no significant trend in sediments that could be explained as a sewage effect. In spite of the lack of sewage effect on sediments, there was an apparent effect on some of the taxonomic groups in the region. Molluscs, copepods, polychaetes, and oligochaetes were elevated near the Sand Island outfall, but molluscs were less abundant near the Barbers Point outfall than far from it. There were also several effects of depth, sediment grain size, and distance along an east-west transect that make it difficult to determine whether sewage was having an effect.

The Mamala Bay outfalls apparently do not provide sufficient loading of organic matter to overwhelm the processes of dissipation and oxidation in Mamala Bay. The effects on benthic fauna are minor and localized, and probably confined to the soft-bottom communities within ca. 500 meters of the outfalls.

The potential impact of sediment loading on the benthos is a major concern for alteration of the function or structure of the benthic communities in the vicinity of the outfalls. Although nutrient and oxygen fluxes differ between outfall and control stations, there is no evidence of buildup of organic sediments on the bottom, and no evidence that anaerobic metabolism in sediments is more common near than far from the outfalls. In other words, the soft-bottom benthic community appears able to absorb the excess organic matter without undergoing qualitative changes in metabolism, as are seen in eutrophication.