Mamala Bay Study

Plume Dynamics and Dispersion in Mamala Bay, Hawaii

Project MB-SP-2

Principal Investigators:

Burton H. Jones

Tommy D. Dickey

Ocean Physics Group

Dept. of Earth Sciences

University of Southern California

Los Angeles, CA 90089-0740

Graduate Assistant:

Anne Petrenko

Phone:

(213) 740-5765 (BJ)

(213) 740-6734 (TD)

(213)740-1971 (AP)

E-Mail:

bjones@usc.edu

dickey@usc.edu

petrenko@usc.edu

Submitted:

August 15, 1995

# 1. Executive Summary

In situ studies of the Sand Island Treatment Plant (SITP) treated wastewater effluent plume were performed during the period of September 25 - October 1, 1994 for the purposes of understanding the characteristics of the plume including its dilution and dispersion, and for comparison with the plume modeling of Roberts (MB-4) and larger scale bay modeling by Hydroqual (MB-5). During the period of our study both Trade Wind conditions and Kona Wind conditions were observed with different physical structure and dynamics occurring during the two periods.

The plume was detected and mapped using natural tracers that included physical (temperature and salinity) and bio-optical variables (beam attenuation, chlorophyll fluorescence, spectral beam attenuation and spectral absorption).   During the period of our study, stratification was sufficient to prevent the plume from rising above 45 m depth. Along with the deeper plume, initial dilution was in the range of 200:1, lower than is normally expected for the SITP diffuser.  Semi-diurnal tides were a significant source of  variability in both current speed and direction and stratification during our study.  As one result of this relatively high frequency variability, the dynamics affecting the plume changed rapidly.

The plume was distributed widely within the sampling region, as indicated by the presence of appropriate anomalies in the natural tracers.  While the main body of the effluent plume appeared to advect with the predominant currents in the lower half of the water column, evidence of effluent was found in other parts of the bay.  This suggests that remnants of the plume were broken off from the newly forming plume as current speed and direction and stratification changed in response to various modes of forcing including tidal fluctuations, internal waves and synoptic scale weather systems.  The change from Trade Wind to Kona Wind conditions resulted in reduced stratification within the bay and higher elevation of the plume in the water column.  During Trade Wind conditions, the upper layer currents were eastward, and during Kona Winds  current direction in the upper layer was predominantly westward.  Both cases are opposite in direction to what had been expected, which may suggests that local wind forcing of the currents may not be a dominant controlling factor.  The lower layer currents, although variable in direction, were more generally westward.

## Table of Contents

1. EXECUTIVE SUMMARY

Table of Contents

List of Figures

List of Tables

2. INTRODUCTION

2.1 Scope of Work

2.2 Objectives

2.3 Project Organization

3. METHODS

3.1 Task Summary

3.2 Task Methodology

3.2.1 Experimental description

3.2.2 Instrument characteristics

3.2.2.1 Beam transmissometer

3.2.2.2 Chlorophyll fluorometer

3.2.2.3 Absorption/attenuation meter (AC-9)

3.2.2.4 Spectral absorption and fluorescence instrument

3.2.2.5 Particle size analyzer

3.2.3 Data sampling frequency

3.2.4 Biological water sample analysis

4. RESULTS

4.1 Mamala Bay Background

4.1.1 Meteorological conditions

4.1.2 Temperature time series

4.1.3 Current time series

4.1.4 Time series statistics

4.1.5 Wave data

4.1.6 Towyo measurements

4.1.7 Particle field characterization

4.2 Sand Island Outfall Wastewater Dilution

4.2.1 Sewage outfall design

4.2.2 Initial dilution.

4.2.3 Plume dispersion.

4.2.3.1 Plume dispersion on 09/25/94

Westward plume

Westward Plume extent

4.2.3.2 Plume dispersion on 10/01/94

5. CONCLUSIONS

6. RECOMMENDATIONS

6.1 Recommended Actions

6.2 Future Studies

7. REFERENCES

8. APPENDICES

APPENDIX A - Moored observations (based on SAIC data sets)

APPENDIX B - Towyo surveys

09/25/94

09/26/94

09/27/94

09/28/94

10/01/94

APPENDIX C - Methods of Estimating In Situ Dilution

Method 1 - Based on Rapid Mixing with Ambient Water at Diffuser Depth:

Method 2 - Based on the Background Salinity/Depth Profile:

Method 3 - Estimating Sa from the Ambient T-S Relationships:

APPENDIX D - Spatial Maps of Dilution from Towyo Sections

## List of Figures

Figure 2.1	Time series at D2 a) alongshore velocity at 63m, b) cross-shelf velocity at 63m, and c) temperature at 13 and 63m (based on SAIC moored data).

Figure 3.1	Diagram of the instrumented towyo platform used for spatial mapping of physical and bio-optical variables

Figure 3.2 	Diagram of the towyo operations with the towyo platform towed in a sawtooth pattern behind the boat

Figure 3.3	An example towyo section (Towyo 4) of beam c showing the contours of beam  c and the path of the towyo across the shelf

Figure 4.1	Towyo transect location and direction of sampling on 09/25/94 and 09/26/94

Figure 4.2	Wind (airport) a) Stick plot, b) speed (knots)

Figure 4.3	Temperature, alongshore velocity (positive eastward) and cross-shelf velocity (positive northward) at D2, time versus depth, a) for 09/24/94-10/01/94, b) to i) daily during this period

Figure 4.4	Number of temperature variations of more than a degree per 1/2 hour at D2 vs. depth during 09/24/94-10/01/94

Figure 4.5	Tide at Honolulu from 09/24/94 through 10/01/94

Figure 4.6	Daily averaged wave energy at Makapuu Point, for each day between 09/25/94 and 10/01/94, wave height (per bands) vs. energy

Figure 4.7	Towyo 16, upcast 5, a) beam c vs. salinity, and b) chlorophyll fluorescence vs. beam c.

Figure 4.8	a) Minimum dilution, and b) depth of the plume with 'o' for station calculation, '*' for towyo calculation, - for model results

Figures A.1	Time series for temperature and velocities at D2, a) during 09/24/94 to 10/01/94, and daily from b) 09/24/94 to i) 10/01/94

Figures A.2	Time series for velocities (stick-plots) and temperature (contours) at D2 daily from a) 09/24/94 to h) 10/01/94

Figures A.3	Power spectra for temperature, alongshore velocity (positive eastward) and cross-shelf velocity (positive northward) at D2, a) to k) 13 to 63m

Figures A.4	Temperature 3-D plots vs. time and depth daily from a) 09/24/94 to h) 10/01/94

Figures B.1	Towyo transect location and direction of sampling a) 09/25-26/95, b) 09/27-28/94, and c) 09/29-30/94 and 10/01/94

Figures B.2 	Temperature, salinity, density, beam c and chlorophyll fluorescence contours for towyo 2 (a) to towyo 33 (af). Towyo 20 is divided in two sections. Towyo directions are indicated by arrows in Figures B.1 a-c

Figure B.3	Station 19, a) beam c vs. salinity, and b) chlorophyll fluorescence vs. beam c.

Figures C.1	Salinity profiles and corresponding dilutions for station 9 on 09/29/94

Figure C.2	T-s diagram with lines of constant density anomaly for station 9 on 09/29/94

Figures D.1	Dilution calculation on 09/25/94, 2m-bin vertically averaged and mapped horizontally between a) 50-52m and j) 68-70m

Figures D.2	Dilution calculation on 10/01/94, 2m-bin vertically averaged and mapped horizontally between a) 42-44 m and j) 60-62 m

## List of Tables

Table 4.1 - 	Number of CT (changes of temperature > 1 degree per half hour) at each depth during 09/24/94 (0000) to 10/01/94 (2400)

Table 4.2 - 	Number of CT (changes of temperature > 1 degree per half hour) per day during 09/24/94 (0000) to 10/01/94 (2400)

Table 4.3 - 		Ranking of magnitude of the energy peaks

Table 4.4 -	 	Measured (Z-measured) and modeled (Z-model) depth of the minimum

dilution and measured (D-measured) and modeled (D-model) minimum

dilution

# 2. Introduction

The purpose of this study is to obtain in situ measurements of the characteristics of the effluent plume from the outfall of the Sand Island Treatment Plant (SITP), including the processes of initial dilution and dispersion of the effluent field under varying tidal and meteorological conditions.

Until the Mamala Bay Study program, there had been few oceanographic studies focusing specifically on the atmospheric forcing and current system in the Mamala Bay region. In order to understand the outfall plume dynamics, it is important to examine the plume in the context of good meteorological and physical oceanographic measurements. This program provided an excellent opportunity to examine plume initial dilution and dispersion concurrently with vertically and temporally resolved current and temperature measurements, and to compare these observations with state-of-the-art plume and coastal circulation models.

The hydrographic sampling program addressed two fundamental scales of variability. The initial dilution studies were performed in the immediate vicinity of the diffusers (the near-field as described by Roberts in an earlier section). Vertical profiles were obtained directly over the outfall (depth of the outfall diffusers is ~70 m) enabling us to estimate height of rise and initial dilution of the plume. The second scale was of the order of 5 km alongshore centered on the outfall, and 1-3 km cross-shelf, depending on the currents and the distributions observed. The purpose of these scales of observations was to determine the plume dispersion and distribution. The sampling included cross-shelf transects separated by 0.5-1 km alongshore (e.g. Figure 4.1) to provide a measure of the cross-sectional area of the effluent plume when flow is generally along-shelf. Along-shelf transects approximately following the 70 m isobath were used to examine the alongshore continuity of features observed in the cross-shelf transects. Past experience indicates that effluent plumes tend to be advected in the alongshore direction with little cross-shelf dispersion. However, tidally varying currents can result in complex plume fields that have a cross-shelf as well as an along-shelf component (Wu et al., 1993).

The study of plume dilution and dispersion described within this report complements several other components of the Mamala Bay Study. The results of this study provide field validation for the near-field plume model of Phil Roberts (MB-4), and also for the far-field modeling of Alan Blumberg and his associates at Hydroqual (MB-5). In addition, our results provide useful background information to the University of Hawaii investigators (MB-9) who are examining the effects of point and non-point sources on the primary productivity and contaminant dispersion along the beach.  Our study also complements the microbiological study of Roger Fujioka at University of Hawaii  (MB-7). He provided microbiological sampling and analysis during our study, and our results provided the microbiologists with an understanding of the context and spatial implications of their observations.

## 2.1 Scope of Work

Mapping of the plume was carried out during the period of September 25 through October 1, 1994. The plume mapping was intended to utilize both natural tracers such as salinity, optically determined particle characteristics including beam attenuation, chlorophyll fluorescence, and spectral attenuation and absorption (Jones, et al., 1993; Washburn, et al., 1992; Wu, et al., 1994). We also planned to use artificial tracers, i.e., man-made tracers introduced into the effluent which have unique characteristics compared with both the effluent and background ambient ocean characteristics. Both the natural and artificial tracers were to be used to map the plume field and to determine its characteristics including height of rise, initial dilution, and dispersion.

The mapping was carried out over a week long period in order to provide a variety of tidal and meteorological forcing conditions. This period happened to be unusual compared to the rest of the annual time series study at the Sand Island diffuser.  Indeed the stratification was much stronger during the end of September and beginning of October than the rest of the period from June to October 1994 (Figure 2.1).  Fortuitously, the wind conditions changed during the study from Trade Wind conditions during the first half of the study to Kona Wind conditions during the second half of the study (Figures 4.2), accompanied by a weakening of the stratification.

In addition to the core measurements of the study, we were able to include additional new measurement systems to study their effectiveness for detecting and mapping the effluent plume and its components.

## 2.2 Objectives

The primary goals of this project are:

1. to provide field observations of the outfall effluent plume in three dimensions,

2. to determine the initial dilution of the plume

3. to estimate the dispersion of the plume away from the outfall

The results of this study are intended to provide field validation for the near-field plume modeling effort of Phil Roberts, and for the far-field coastal circulation model of Blumberg et al. at Hydroqual (MB-5).

In addition, we intended to determine how these processes vary under differing oceanographic conditions, dependent upon the natural variability during the sampling period. Two major meteorological forcing conditions characterize the region around the Hawaiian Islands. Trade wind conditions are the prevailing meteorological forcing on the coastal ocean. Kona wind conditions are more likely to occur in late winter and early spring (Armstrong, 1983). Under Kona wind conditions, the winds may blow from the south setting up onshore waves and currents that will affect the dispersion of contaminants from both point and non-point sources. The region is affected by tidal forcing which may be a major contributor to the current variability in the coastal region (e.g. Noda, 1992; SAIC report, 1995). We have also found this to be true in the southern California region and have observed that plume direction is highly dependent on the tidal currents under most conditions.

## 2.3 Project Organization

This project was directed and performed by the Ocean Physics Group at the University of Southern California. The principal investigators were:

Burton H. Jones and Tommy D. Dickey, Ocean Physics Group, Department of Earth Sciences, University of Southern California, Los Angeles, CA 90089-0740:

Drs. Jones and Dickey are oceanographers with a broad experience in both coastal and open ocean oceanography including physical, chemical and biological processes. They, along with Dr. Libe Washburn and their students, have developed the in situ mapping technology that was used to map the effluent plume and have applied this technique to other outfall systems.

Other USC Ocean Physics Group personnel involved in the project include:

Derek Manov, Research Engineer, Ocean Physics Group, USC:

He is responsible for instrumentation development and maintenance for the Ocean Physics Group. His participation in the cruise was directed toward ensuring proper performance of the instrumentation during the field operations

Anne Petrenko, Graduate Student, Ocean Physics Group, USC:

Ms. Petrenko has a major responsibility in this effort. She helped to design the experiment and execute the field sampling program and has a major responsibility in the analysis of the data set. She is incorporating the results from this project into her Ph.D. dissertation.

David Foley, Graduate Student, Ocean Physics Group, USC

Mr. Foley assisted with the sampling and field studies of the sewage outfall plume in Mamala Bay.

Mr. Jerome Wiggert, Graduate Student, Ocean Physics Group, USC:

Mr. Wiggert assisted with the sampling and field studies of the sewage outfall plume in Mamala Bay.

Subcontracted Participants:

Jonathan Marsh, Environmental Tracing Systems, Ltd., Cornwall, England:

Mr. Marsh was subcontracted to provide the tracer and tracer detection systems for use with the SITP effluent.

Other Personnel Participating in the Field Studies, but not funded by this project:

Mr. Casey Moore, President, WetLabs, Inc., Philomath, Oregon:

WetLabs has developed several new spectral bio-optical instruments for in situ oceanographic sampling. He participated in a portion of our field work and made available a spectral fluorometer and a second AC-9 to attach to our measurement system. These measurements are expected to provide for the detection of additional components of the effluent plume that are not detected with our standard instrumentation set.

Mr. Michel Lehaitre, IFREMER, Brest, France:

Mr. Lehaitre provided an in situ particle size analyzer for deployment in the vicinity of the Sand Island Outfall diffuser. The intent of this was to examine the particle size distributions that were present inside and outside of the outfall plume. It was expected that the effluent would produce a particle field with unique size distributions, as well as unique bio-optical characteristics compared with the ambient particle field (phytoplankton, bacteria, and inorganic particles).

# 3. Methods

## 3.1 Task Summary

The tasks that were completed during our study of the SITP effluent plume include:

1. 	Plume mapping and estimation of initial dilution, height of rise, and dispersion using natural tracers. The effluent plume was mapped using a towyo system equipped to measure natural characteristics of the effluent plume that are unique to the plume. These characteristics include salinity, chlorophyll fluorescence, and beam attenuation coefficient (beam c).

2.	Complementary studies of bacterial concentrations (abundances) to corroborate our observations. Bottle sampling near the outfall by R. Fujioka and his colleagues (U. Hawaii) was used to verify that regions where natural and artificial tracers indicate the presence of the effluent plume also contain bacterial abundances characteristic of an effluent plume.

3.	New in situ technologies were also employed to evaluate their usefulness in the detection and mapping of the effluent field. The new technology included a WetLabs AC-9 spectral absorption and attenuation meter, a WetLabs SAFIRE spectral fluorometer, and a CILAS in situ particle analyzer.

Plume mapping and estimation of initial dilution, height of rise, and dispersion were originally planned by using tracers introduced at the treatment plant. Using fluorescent particles introduced at the sewage treatment plant, the plume was to be mapped using an in situ, laser excited, fiber optic detection system that would allow simultaneous fluorescent measurements at multiple depths. Because the subcontractor was unable to deliver the in situ detection system for these particles, this aspect of the study was not carried out.

## 3.2 Task Methodology

The basic towyo methodology that was used to map the distribution of the effluent plume has been developed by the Ocean Physics Group at USC and has proven successful in the study of sewage effluent plumes offshore of Southern California (Jones et al, 1990; Jones et al, 1991; Washburn et al, 1992; Wu et al, 1994).

Measurements were obtained from an instrumented towyo platform (Figure 3.1 )

which carried conductivity, temperature and depth sensors (CTD, Sea-Bird Electronics model SBE 9/11+), a beam transmissometer (660 nm wavelength, 0.25 m path length, Sea Tech, Inc.), a chlorophyll fluorometer (Sea Tech, Inc.), a nine wavelength absorption/attenuation meter (AC-9, WETLabs, Inc.), and a photosynthetically available radiation (PAR) sensor (Biospherical Inst.). In addition to the above instruments, either an AC-9 or a Spectral Absorption and Fluorescence Instrument (SAFIRE, WETLabs, Inc.) was mounted on the towyo package during 09/29-10/01/94. A particle size analyzer (CILAS, France) was added to the platform for two stations on 09/29/94 but cable difficulties prevented further use of that configuration and the particle size analyzer was then used in the ship laboratory.

### 3.2.1 Experimental description

Two primary modes of sampling were used:

1) Towyo platform towed in a sawtooth pattern behind the boat (Figure 3.2 ). The word towyo implies that the platform is yo-yoed up and down while the package is towed by the research vessel.

2) Vertical profiles (on station) with instruments of the towyo platform.

Towyos were done on a grid pattern around the diffuser.  Cross-shelf towyos generally were spaced at either 500 m or 1 km intervals along the coast and alongshore towyos generally followed the 70 m isobath. An example of a cross-shelf towyo is shown for beam attenuation coefficient (beam c) on 09/25/94 (Figure 3.3). Alongshore towyos were done twice on the western side of the diffuser, once on the eastern and southern side of it, and once from west to east of the diffuser (going south along the diffuser).

During towyo operations, the R/V Kila, University of Hawaii, steamed at a speed of 1-2 knots and the winch speed was about 25 m·min-1. Thus,  good spatial coverage of the area was obtained with horizontal resolution of 250 m near mid-depth and vertical resolution of at least 0.5 m. Most cross-shelf transects were obtained from inshore to offshore to minimize of the likelihood of the instrumented platform hitting bottom. Generally, the bottom of the towyo profiles was  within 10 meters of the sea floor.

Stations were done repetitively at the western end of the diffuser, and also offshore and inshore of the diffuser.

### 3.2.2 Instrument characteristics

#### 3.2.2.1 Beam transmissometer

The beam transmissometer measures the attenuation of a beam of  light at 660 nm wavelength along a path of known distance, 0.25 m.  A decrease in beam transmission at 660 nm (T)  corresponds to an increase in the beam attenuation coefficient or beam c (c) which is derived from the former by the following relation:

[m-1]

where T = percent transmission of light

and l is the path length, l = 0.25 m.

In this report we will use the beam attenuation coefficient, c, rather than the percent transmission to report the results from this measurement.  Beam attenuation coefficient is generally linearly correlated with the abundance of suspended particulate material in the water column (e.g., Spinrad, 1986).  The regression coefficient and intercept will vary from area to area depending on the characteristics of the particles. These characteristics include size, light absorption and scattering, spectral characteristics, particle type such as phytoplankton, bacteria, detritus, sand, etc.

#### 3.2.2.2 Chlorophyll fluorometer

The Sea Tech chlorophyll-sensitive fluorometer measures the fluorescence generated by exciting the sample with light centered at 425 nm wavelength (200 nm Full Width Half Maximum (FWHM)) and measuring the fluorescence emission centered at 685 nm (30 nm FWHM), the peak emission wavelength for chlorophyll fluorescence (Bartz et al., 1988). The fluorescence is highly correlated with chlorophyll concentration under most circumstances.

#### 3.2.2.3 Absorption/attenuation meter (AC-9)

The AC-9 is configured with two 25 cm paths, and filters for the following nine wavelengths: 412, 440, 488, 510, 560, 630, 650, 676, and 715 nm. The beam attenuation (c) path has blackened sidewalls to absorb the light scattered from the beam. In contrast, the flow tube of the absorption (a) path is reflective in order to transmit forward scattered light towards the large area detector (Zaneveld et al., 1990). Individual bandwidths are 10 nm FWHM.  In addition to providing basic optical characteristics, the AC-9 also provides an alternative method for measuring chlorophyll concentration that is not subject to photoinhibition (as the fluorometer is) and also measures beam attenuation not only at 660 nm, but at 8 other wavelengths as well.

#### 3.2.2.4 Spectral absorption and fluorescence instrument

The SAFIRE is a spectral fluorometer which has 6 wavelengths of excitation, with 3 wavelengths in the UV and  3 wavelengths in the visible portion of the spectrum (228 - 435 nm), and 16 wavelengths of emission spanning from UV through the visible to the beginning of the infrared (228 - 900 nm).  This instrument  yields 59 combinations of excitation and emission (combinations where the emission wavelength is shorter than the excitation wavelength are not considered). Thus, information content is great, but analysis and interpretation of these measurements can be quite complex.

#### 3.2.2.5 Particle size analyzer

The range limits of the CILAS particle size analyzer are 1 to 400 mm.  The instrument is composed of a laser collimated beam (l = 830 nm) which, after crossing the particle suspension, provides a diffraction pattern.  This diffraction pattern is analyzed by using silicon photodetectors to provide the size distribution.

### 3.2.3 Data sampling frequency

Sampling rates varied for the different sensors. Pressure, temperature, conductivity, beam transmission and chlorophyll fluorescence were sampled at 24 Hz. However, the time constant for the fluorometer is about 1 second. A portion of the data was analyzed with the full 24 Hz resolution in order to resolve possible interleaving processes and features on very small spatial scales. AC-9 data were collected at 5.2 Hz and SAFIRE data at 5 Hz. All the data from these sensors were acquired simultaneously and time-stamped with the Modular Ocean Data And Power System (MODAPS, WETLabs, Inc.). The data were then converted into engineering units and averaged into 0.5-s or 1-s bins for further analysis.

### 3.2.4 Biological water sample analysis

Following vertical profiling with the towyo instrument package, discrete water samples were obtained with 5-liter Niskin bottles at target depths based on features apparent from the instrument profiles. Biological analyses of these samples included extracted chlorophyll concentration, particulate and filtrate absorption spectra, nutrient concentration, particle size using CILAS particle size analyzer, flow cytometry (Campbell, pers. comm.), HPLC pigment analysis, and direct counts of bacteriological and viral abundances using acridine orange epifluorescence and transmission electron microscopy.

# 4. Results

## 4.1 Mamala Bay Background

The MB-SP2 cruise took place from September 25 to October 1, 1994 in a rectangular region of approximately 6 km alongshore by 3 km cross-shelf centered on the Sand Island outfall diffuser (Figure 4.1) which is at depth of about 70m. The coastline is nearly east-west in this region, as are the outfall diffuser and the isobaths. The cruise period covered two different meteorological conditions which were reflected in the currents and stratification.

### 4.1.1 Meteorological conditions

Wind speed and direction were measured at the Honolulu International Airport about 2-3 km northwest of the study site. During the cruise period, wind speed varied diurnally with reduced speed in the morning and increased velocity in the afternoon. During the first part of the MB-SP2 cruise (09/24-09/27), Trade Winds prevailed (Figures 4.2 a and b); the average amplitude of the wind vectors was 9.6 knots (4.9 m·s-1) from the northeast (55.5 degrees clockwise from true North). During the second part of the cruise, Kona conditions occurred with an average wind speed of 4.4 knots (2.3 m·s-1) from the southeast (139 degrees clockwise from true North). During the Kona Wind conditions, a land breeze - sea breeze oscillation was observed in the wind data. Offshore flow occurred during the late night - early morning and onshore flow in the late evening. As shown in Figure 4.2a, there was a brief period of northwesterly winds early on 09/30.

### 4.1.2 Temperature time series

The SAIC group obtained temperature and/or current data from 11 locations in the region (see SAIC report, 1995).  Results pertinent to our study periods are summarized below.  The SAIC mooring at site D2 (near the outfall diffuser) obtained temperature measurements at 5 m depth intervals from 13 m to 63 m and acoustic current

measurements at 2 m intervals from 7 m  to 67 m beneath the surface. Both temperature and currents were sampled at 30-minute intervals. Two distinct periods were evident in the temperature time series during our field study (Figure 4.3). During the first half of the study,  09/24 through 09/27, the water column was strongly stratified with average 13 m temperatures of about 27.5 °C and 63 m temperatures of  ~24.5 °C. On 09/29 the stratification decreased as the shallow temperature at 13 m cooled by nearly 0.5 °C to 27.0 °C and  the temperature at 60 m warmed to 25.1 °C. Because the wind was weak on 09/29 (4 to 11 knots), wind-induced mixing is not likely to explain the decrease in stratification that was observed. This change in stratification coincided with the onset of  Kona conditions and therefore the warming of the lower layer may have been due to advection of warmer water from the center of the bay into the study area (Figures 4.2a and 4.3). This resulted in a well mixed upper column where temperature differences between 13 m and 63 m were sometimes less than one degree (Figures A.4 g-h).

Temperature data exhibited an apparent semi-diurnal signal at all depths (Figure 4.3), confirmed by the spectral analysis described below in section 4.1.4. Alongshore currents showed the same semi-diurnal periodicity during the beginning of the cruise, i.e. during the Trade Wind conditions. During the 09/24-09/25 period, westward velocity was associated with cooling in the lower part of the water column and eastward currents with warming. After 09/25, the current/temperature relationship was more ambiguous.

Because of the tidal influence on temperature distribution in the lower half of the water column, stratification could change rapidly. Hence, the dynamics of the effluent plume could change correspondingly.  Changes of more than 1 degree in one half hour are noted for the time period from 09/24, 0000 to 10/01, 1200 and hereafter will be referred to as CT (Change of Temperature of more than 1 degree in half an hour). These CTs occurred most often in the lower half of the water column (Figure 4.4); the number of CTs was 27 at 63 m, compared with none at 13 m. The number of CTs at each thermistor depth are shown in Table 1. The average of these temperature changes is 1.29 degree. A semi-diurnal tidal signature can be discerned in these results especially during the first part of the cruise (see Table 2) and below 40 m (see Table 1). This is explained by the fact that, during the beginning of the cruise, the water column was more stratified, so internal waves, and tidal fluctuations resulted in larger temperature excursions than if the water column had been isothermal or weakly stratified. Tidal fluctuations are shown in figure 4.5.  Here the process of formation of these internal tides has been proposed to be due to the merging of two components of the tidal wave in Mamala Bay, with the formation of a  nodal point between transect B and D, instead of a generation via coastal topography (SAIC, 1995). These rapid changes of temperature at depths above the sewage diffusers are likely to play an important role in the rise of the plume and its dilution level. Since the near-field model uses these temperature data and stratification, model results should show a strong variability in the height of rise and dilution levels on small temporal scales (Roberts, pers. comm.).

Table 4.1 - Number of CTs (changes of temperature > 1 degree per half hour) at each depth during 09/24/94 (0000) to 10/01/94 (2400)

| Depth | 13 | 18 | 23 | 28 | 33 | 38 | 43 | 48 | 53 | 58 | 63 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of CT | 0 | 1 | 0 | 0 | 3 | 5 | 7 | 11 | 15 | 19 | 27 |

Table 4.2 - Number of CTs (changes of temperature > 1 degree per half hour) per day at all depths  during 09/24/94 (0000) to 10/01/94 (2400)

| Date | 09/24 | 09/25 | 09/26 | 09/27 | 09/28 | 09/29 | 09/30 | 10/01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of CT | 15 | 14 | 12 | 22 | 10 | 2 | 6 | 7 |

### 4.1.3 Current time series

In the upper water column (0 to 30 m), the alongshore current direction at D2 was opposite in direction to that expected.  During the Trade Wind period, the upper layer alongshore current direction was eastward, but during the Kona conditions the along-shore current direction was westward (Figure 4.3). This suggests the presence of a non wind-driven upper layer circulation, or an island effect in Mamala Bay. The circulation model of Hydroqual (MB-7)  indicates that there is a nodal point located in the middle of the bay.

At depths of 40 to 70 m, the alongshore velocity was dominantly westward (Figure 4.3). This confirms the field observations in which the sewage plume was found most often west of the diffuser.

### 4.1.4 Time series statistics

Power spectral densities have been calculated with half-overlapping temporal windows of 128 hours for a total period of 136 days for velocity (June 5 - Oct. 18, 1994) and  188 days for temperature (April 14 - Oct. 18, 1994). The largest peak of the energy spectra for the two velocity components and for temperature is at the semi-diurnal frequency (Figures A.3 a to k), except for temperature at 13 m where the diurnal signal is dominant apparently due to the daily solar cycle.  The secondary peaks are either the diurnal signal or at a period of about 6 hours (see Table 4.3).

Table 4.3 - Ranking of magnitude of the energy peaks: f: inertial, D: diurnal, SD: semi-diurnal, n: 6 hours

|  | U |  |  |  | V |  |  |  | T |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Depth | f | D | SD | n | f | D | SD | n | f | D | SD | n |
| 13 | - | 2 | 1 | 3 | - | 2 | 1 | - | - | 1 | 2 | - |
| 18 | - | 2 | 1 | 3 | - | 2 | 1 | - | 2 | 1 | 1 | - |
| 23 | - | 2 | 1 | 3 | - | 2 | 1 | - | 2 | 2 | 1 | - |
| 28 | - | 2 | 1 | 3 | - | - | 1 | - | 3 | 2 | 1 | - |
| 33 | - | 2 | 1 | 2 | - | - | 1 | 2 | - | 2 | 1 | 3 |
| 38 | - | 2 | 1 | 2 | - | - | 1 | 2 | - | 2 | 1 | 3 |
| 43 | - | 2 | 1 | 2 | - | - | 1 | - | - | 2 | 1 | 3 |
| 48 | - | 2 | 1 | 2 | - | - | 1 | - | - | 2 | 1 | 3 |
| 53 | - | 2 | 1 | 2 | - | - | 1 | 2 | - | 2 | 1 | 3 |
| 58 | - | 2 | 1 | 2 | - | - | 1 | 2 | - | 2 | 1 | 3 |
| 63 | - | 2 | 1 | 2 | - | - | 1 | 2 | - | 2 | 1 | 3 |

### 4.1.5 Wave data

Four sources of ocean waves are found in deep water around the Hawaiian islands, varying seasonally in direction, amplitude (Armstrong, 1983):

1. Trade wind waves have wave heights of 4-12 ft and periods of 5-8 seconds. They impinge on the islands from the northeast most of the year, but are strongest in the summer.

2. Kona storm waves have wave heights of 10-15 ft and periods 8-10 seconds. These waves are generated by local fronts and occur at any time of the year, but are most common in late winter or early spring.

3. Southern Swell have wave heights of 1-4 ft and periods of 14-22 seconds. They originate from distant storms occurring in the Southern hemisphere during the austral winter (summer and early autumn in Hawaii).

4. Northern Swell have wave heights of 8-14 ft and periods of 10-15 seconds. They originate from distant storms of the North Pacific occurring in winter and early spring.

During the period of our sampling, the wave energy spectra at Makapuu Point was dominated by northeast Trade Wind waves during the first part of the cruise (09/25-27), and by  Kona Storm waves during the latter part of the cruise (09/28-10/01) (Figure 4.6, data from SIO wave database).

### 4.1.6 Towyo measurements

During the plume study, the water column was stratified in both temperature and salinity. In general, The plume was trapped below 55 m during the Trade wind conditions and below 45 m during the Kona period.  Plume thickness was generally of the order of 10 meters, and sometimes divided into multiple thinner layers with the shallower ones overlapping with chlorophyll fluorescence layers. Detailed day by day analyses of all the towyos are presented in Appendix B with the corresponding figures.

During the Kona Wind condition, towyo sections were obtained only on 10/01/94. Nonetheless, it is clear from these data that the plume characteristics were different than during the Trade Wind period. Beam c levels were much lower than previously observed and the rise height of the plume was higher in the water column, both suggesting that initial dilution was greater than previously observed. Because the two layer system of currents and water column stratification observed earlier was weakening during the Kona period, the possibility of the plume rising to the surface where it would become subject to wind forcing was increasing.

### 4.1.7 Particle field characterization

In general, the determination of particle fields is facilitated by small variability in plume characteristics.  During our previous studies near Palos Verdes, California, (Washburn, et al, 1992; Wu et al., 1994; Jones, et al., 1993) comparison of  beam c,   salinity, and chlorophyll fluorescence allowed the distinction of three particle fields:

1) the effluent field where beam c was negatively correlated with salinity and varied independently from chlorophyll fluorescence;

2) the resuspended sediment field where beam c was elevated above ambient levels within 10-15 m of the bottom and varied independently from both salinity and chlorophyll fluorescence;

3) phytoplankton where beam c and chlorophyll fluorescence tend to be positively correlated.

For these studies, the threshold salinity S varied depending either on current speed, being 33.6 psu for low mean current 0.01 m·s-1 and 33.65 psu for stronger currents of average 0.07 m·s-1 (Washburn et al, 1992),  or on temporal changes, being 33.52 psu on March 7, 1989 and 33.7 psu on March 9, 1989 (Wu et al, 1994).

Since our study covered a time period of more than two (Washburn et al, 1992) and three (Wu et al, 1994) days, the situation was expected to be more complex, especially since it has been shown that currents and water stratification were highly variable at the site during the cruise period.  Nonetheless it was possible to determine two primary particle fields in the water column, the sewage effluent and the phytoplankton population.  The instrument package was carefully kept away from the bottom during towyoing so that no resuspended sediments were observed, which makes the particle fields characterization simpler than in the Palos Verdes study described above.

It was possible to differentiate between plume and ambient water as described above, but sometimes varying threshold values of beam c were obtained for the same profile.  For example, on 09/27/94 for towyo 16, the transition between the upper layer of the water column and the plume occurred when beam c values rose above 0.5 m-1 and the transition between the plume and the lower layer of the water column occurred when beam c values fell below 0.58  m-1 (Figures 4.7).  The plot exhibits the expected relation between beam c and salinity for the plume with beam c increasing with decreasing salinity.  In figures 4.7A and B, the circles (crosses) represent the data above (below) the depth of the plume salinity minimum.  It can be noted that, in the plume, at equal beam c, the salinity values in the upper part of the plume were much lower than the salinity values in the corresponding deeper part of the plume.  This hysteresis could explain the fact that the depth of minimum dilution does not always correspond to the depth of the beam c maximum.  Within the plume chlorophyll fluorescence was nearly  uncorrelated with beam c.  Beam c increased to more than 1.1 m-1 while chlorophyll fluorescence values remained nearly constant around 0.6 mg·m-3 and rose to a maximum of ~0.7 mg·m-3 at the core of the plume.

At the end of the cruise, the beam c values observed in the plume were much lower than the ones of the beginning of the cruise.  On 10/01/94 for the downcast of station 19, a unique values of beam c equal to 0.45 m-1 indicated the transition between ambient water and the plume (Figure B.3).  The hysteresis between upper and deeper part of the plume occurred not only for the beam c vs. salinity plot but also for the chlorophyll fluorescence vs. beam c plot.  In the latter case, at equal beam c, the chlorophyll fluorescence values in the upper part of the plume were much lower than the chlorophyll fluorescence values in the corresponding deeper part of the plume.

## 4.2 Sand Island Outfall Wastewater Dilution

### 4.2.1 Sewage outfall design

The ideal goal of an outfall design is for the effluent plume to always be submerged. In Mamala Bay the winter stratification is usually weak and the surface mixed layer deep (Fischer et al., 1979). Hence to meet the legal submergence standard, the sewage diffuser needs to be located at depths deeper than 90 m. But this raises other problems, such as:

- the summer stratification can be very strong which then leads to a very deep submergence of the plume. This is associated with low dilution and increased risk of viral survival which could pose a problem should upwelling bring the contaminated seawater to the surface.

- below 90 m, the bottom slope becomes steeper and more rocky. Pipe attachment becomes difficult under these circumstances.

A compromise between dilution, feasible engineering design, and construction costs on the one hand and submergence on the other hand was reached by placing the sewage diffuser built at its present location (Fischer et al, 1979). The ports of the sewage diffusers are located over the last 1030 m of the outfall at depths 69 to 71m, at the bottom of a margin slope (2.8o slope over 1000 m).

### 4.2.2 Initial dilution.

From the velocity direction and amplitude and the temperature stratification, the Roberts, Snyder, and Baumgartner (RSB) plume model (Baumgartner et al, 1994; Roberts et al, 1989 a, b, and c) predicts the initial mixing length, the rise height of the plume, and the minimum dilution of the plume. The plume is assumed to be advected in the direction

of the currents and the initial mixing length is defined from the middle of the diffuser line when the currents are orthogonal to the diffuser, and from the end of the diffuser when the currents are parallel to it. In order to compare the results of the near-field model (Roberts, pers. comm.) with in situ data, dilution is derived from in situ measurements of minimum salinity in the plume near the diffuser (Washburn, 1992; detailed methods of estimating dilution from the in situ are described in Appendix C).  Comparisons of the in situ dilutions (estimated using the second method in  Appendix C) and rise heights (shown as equilibrium depth of the plume) with the model predictions are shown in Figure 4.8 and table 4.4.

Table 4.4 -  Measured (Z-measured) and modeled (Z-model) depth of the minimum dilution and measured (D-measured) and modeled (D-model) minimum dilution

| Date Time | Z-measured | Z-model | D-measured | D-model |
| --- | --- | --- | --- | --- |
| Trade Winds: |  |  |  |  |
| 9/25/94 17:31 | -58.0 | -53.1 | 145.0 | 274.7 |
| 9/27/94 13:55 | -65.5 | -56.7 | 114.0 | 150.1 |
| 9/28/94 13:26 | -62.0 | -58.3 | 65.0 | 283.6 |
| 9/28/94 17:31 | -44.5 | -56.5 | 260.0 | 332.2 |
| Average: | -57.5 | -56.2 | 146.0 | 260.2 |
| Kona Winds: |  |  |  |  |
| 9/29/94 17:02 | -57.0 | -50.0 | 405.0 | 269.2 |
| 9/29/94 17:02 | -54.0 | -50.0 | 285.0 | 269.2 |
| 9/29/94 18:28 | -59.0 | -50.8 | 340.0 | 341.6 |
| 9/30/94 10:04 | -68.0 | -41.7 | 210.0 | 963.6 |
| 10/1/94 10:33 | -54.0 | -55.3 | 200.0 | 342.3 |
| 10/1/94 12:57 | -58.0 | -50.4 | 113.0 | 417.7 |
| 10/1/94 13:55 | -59.0 | -53.9 | 220.0 | 596.7 |
| Average: | -58.4 | -50.3 | 253.3 | 457.2 |
| Trade + Kona |  |  |  |  |
| Overall Average | -58.1 | -52.4 | 214.3 | 385.5 |

The RSB model predicts initial dilutions which are 45% higher than dilutions estimated from the in situ data, and heights of rise which are higher in the water column

(by 35%) than the in situ measured height of rise. The measured and modeled results are of the same order of magnitude.  The observed structure of the in situ plume was considerably more complex than the compact shape of the modeled plume. This is probably due to the high variability both in velocity and in density stratification in the vicinity of the diffuser. Calculating the near-field structure with a high frequency (0.5 hour) is necessary, but with these time steps, the in situ plume does not develop the size and shape characteristics that result from the RSB model assumptions. The model relationships have been empirically determined from laboratory experiments where the environmental conditions are relatively stable (Roberts et al, 1989 a, b, and c). However, as we have already shown, the stratification and current conditions in Mamala Bay can be highly variable and result in complex plume characteristics and distributions.

### 4.2.3 Plume dispersion.

#### 4.2.3.1 Plume dispersion on 09/25/94

Detailed analysis of plume dilution has been performed utilizing the observations from September 25, 1994 (1430-2100), when Trade Wind conditions prevailed and the water column was well stratified. Relatively long cross-shelf towyo sections were obtained allowing an examination of the cross-shelf as well as alongshore spreading of the plume. In the morning, prior to sampling, the currents had been as high as 40 cm·s-1 westward, then had reversed to about 2 cm·s-1 eastward during the afternoon.  By evening the currents were again westward at about 5 cm·s-1.

Dilution has been estimated using the third method described in Appendix C and spatial maps have been plotted to show the distribution of dilution around the diffuser. The dilution calculations have been vertically averaged into 2 m bins and mapped horizontally for each 2 m level between 50 m and 70 m (Appendix D - Figures D.1a-j).  The dilution scale has been fixed to a range of dilution from 150:1 to 2500:1.  The location of the towyo measurements are indicated by the white circles enumerated as towyos 2 to 6, from west to east, in the chronological order they were done. The diffuser is located at the northern end of towyo 4. Areas where no data were collected are shaded black.

The effluent field was evident between 50 and 70 m near the diffuser at the northern end of towyo 4. A pronounced westward plume occurred between 58 and 70 m (Figures D.1 e-j) in towyos 2, 3 and 4, near the northern or coastal edge of these transects (21.28° - 21.285° latitude). This is consistent with the westward flow of 5 cm·s-1 during the evening.

At shallower depths, between 50 and 58 m, low dilutions were found both south and east from the diffuser (Figures D.1 a-d). These low dilutions may have several explanations. First, they could result from the eastward transport during the afternoon of 09/25/94 (Figures A.2 a) when the current velocity was ~2 cm·s-1 eastward. Second, they could also indicate the remnants of a plume from sewage wastewater released prior to 9/25/94 when  stratification was not as strong as on 09/25/94.

#### Westward plume

Four stages of the effluent wastewater behavior in the water column can be described using figures D.1 a to j:

1) Initially, the effluent rises as a positively buoyant plume. It is detected in all transects between 50 and 70 m around the diffuser (Figures D.1 a-j);

2) The plume reaches its density equilibrium depth between 62 and 68 m. Dilution is the lowest between 64 and 66 m (Figure D.1 h);

3) The plume overshoots its buoyancy equilibrium level. The low dilution (< 500:1) at the northern end of towyo 4 can be found up to 50 m, more than 10 meters above the buoyancy equilibrium level (Figures D.1 a-j);

4) The plume is transported and dispersed by local currents and widens with distance away from the diffuser. The cross-shelf extent of the plume at the northern end of towyo 2 is always wider than on towyo 3 between 56 and 68 m (Figures D.1 d-i).

These general processes have been described in more detail in the literature (e.g., Fischer et al, 1979; Roberts et al., 1989a; Wu et al, 1994).

#### Westward Plume extent

The following estimations of the plume extent are made between depths of 64 and 66 m. For dilution ranging from 150 to 1000, the plume width is about 500 m for towyo 2, about 350 m for towyo 3, and about 800 m for towyo 4. In the case of towyo 4, the inshore extent of the plume cannot be estimated since no towyo was done close to the bottom near shore. The plume may extend cross-shelf for more than 800 m.

#### 4.2.3.2 Plume dispersion on 10/01/94

Similar analysis has been performed at the end of the cruise when the water column was less stratified.  The dilution has been estimated with the first method explained in Appendix C (the method is shown not to differ from the third method results) and is plotted in the area around the diffuser where the towyo data were collected (Figure B.1c).  The studied area has been plotted on the same geographical scale as the study on 09/25/94 in order to allow for comparison.  Areas where no data were collected were left in black.

The dilution calculations are 2m-bin vertically averaged and mapped horizontally between 42m and 62m (Appendix D - Figures D2 a-j).  The dilution scale has been fixed to a range of dilution of 150:1 to 500:1 between 42m and 54m, and of 150:1 to 1500:1 between 54m and 62m.

The location of the towyo measurements are indicated by the white circles enumerated as towyos 28 to 33, in the chronological order they were done.   The towyos were done in the morning from 0830 to 1215.  The diffuser is located north of the alongshore towyo 29.

The effluent wastewater is visible between depths of  42 and 62m, west of the diffuser during towyo 30 and during the northern end of towyo 31 and 33.  A pronounced westward plume is observed between 46 and 56m (Figures D.2 c to g) in towyos 30 to 33, near the northern or coastal edge of these transects (latitude 21.275 - 21.285 degree).  This is consistent with the currents which are mostly westward all day long (Figure A.2h) over the whole water column.  Some offshore onshore currents were present between 40 and 50m around 0600-0700, and below 55m at 1100.  They may explain some of the onshore extent of the plume.

The main plume has approximately the same thickness of 10m as the plume on 09/25/94, but is located at more shallow depths in the water column, between 46 and 56m instead of 58 to 68m on the first day of the cruise.  This is due to the weaker stratification on 10/1/94.  The plume also extends west of the diffuser as on 09/25/94.  Since the area studied on 10/01/94 is reduced compared to the area covered on 09/25/94, the full offshore and alongshore extent of the plume influence can not be estimated.  The range of the dilution scale for 10/01/94  is only 150 to 500.  If these data were plotted with the dilution scale used for 9/25/94 (range of 150:1 to 2500:1 as in figures D.1 a to h) the entire sampled area shown in Figures D2a-h would be entirely red to pale green, indicating relatively low dilution throughout.  These low dilutions are not due to a reduced dilution of the plume, but rather to the smaller geographical coverage of the towyos.  In fact, dilution is greater during this low stratification period.

# 5. Conclusions

1. During the period of our study (September 25 - October 1, 1995), the plume was detectable and traceable at all times using natural tracers which included salinity, beam attenuation coefficient at 660 nm wavelength, and spectral absorption and beam attenuation coefficient.  The effluent plume was characterized by decreased salinity, increased beam attenuation at 660 nm and increased beam attenuation and absorption of light at all 9 wavelengths of the AC-9.

2. Under the stratification and current conditions observed during our study, the effluent plume from the SITP remained deep (>45 m) and initial dilution was low (<200:1).

3. Because of the tidal variability in current direction, remnants of the plume were found in both along-shore directions and offshore from the SITP outfall diffuser.

4. On the basis of our results obtained during a period of relatively strong stratification, the plume did not have a significant impact on the surface layer of the coastal ocean.

5. Evidence from this study suggests that the plume may contribute to growth of subsurface phytoplankton layers.  For one towyo (9/28/95 - towyo 22) elevated chlorophyll fluorescence was apparently associated with the plume downstream from the SITP outfall diffuser.

6. Tidal fluctuations in thermocline depth may result in significant changes in plume height of rise and initial dilution.  During our study, there was greater variability in thermocline depth during Trade Wind conditions than during Kona Wind conditions. However, local wind forcing may not be the most important physical oceanographic effect

7. Observations and near-field model results are in general agreement with the model overestimating dilution and rise of height of the plume.

# 6. Recommendations

## 6.1 Recommended Actions

1.	On the basis of our results which were obtained during a period of relatively strong stratification,  the plume did not have a significant impact on the surface layer of the coastal ocean.  However, freshwater surface outflows such as the Ala Wai canal seem more likely to have an immediate and significant effect on local beaches and recreational areas, and therefore may require more direct attention.

2.	A long-term operational scheme for the Sand Island and Honouliuli Treatment Plants should include a telemetered monitoring system coupled with real-time data assimilation modeling (e.g. Dickey et al., 1993).   Such a system could be used to tune the treatment plant operations and perhaps result in significant cost savings. While current and temperature sensors are of fundamental importance, conductivity and bio-optical sensors should also be included for the purpose of detecting the effluent plume in the water column.

## 6.2 Future Studies

1.	Additional studies of the Sand Island Treatment Plant outfall plume should be made during periods of weak stratification in order to evaluate the performance of the model during those periods.  These studies should include a detailed diurnal sampling of the plume processes over a 24 hour period because of the significant semi-diurnal tidal and higher frequency variations in Mamala Bay.  Our in-situ measurement package should be complemented with underway ADCP current profiling measurements to better resolve spatial inhomogeneities in the flow field.

2.	Further field validation of the plume model is needed for periods of weak stratification.

3.	If a long term monitoring program is to be carried out in the Bay, bio-optical and conductivity sensors should be included on the mooring as well as temperature sensors to enable the detection of the effluent plume and differentiation of particle fields in the water column.  Ideally, these sensors should be placed at several depths in the water column to allow for measurements of high variability in the vertical structure of currents and stratification and for determination of the plume height of rise depending on ambient stratification and currents.

4.	Future studies of bacteria and viruses related to the outfall plume should include physical/bio-optical profiling and tow-yoing in order to target sampling and to ensure that the samples are taken from water of known characteristics.

# 7. References

Armstrong, R.W. (ed.), 1983, Atlas of Hawaii, second edition, University of Hawaii Press, Honolulu, Hawaii, 238pp.

Bartz, R., R.W. Spinrad, and J.C. Kitchen, 1988, A low power, high resolution, in situ fluorometer for profiling and moored applications in water, SPIE 9:157-170.

Baumgartner, D.J., Frick, W.E., and Roberts, P.J.W., 1994, Dilution Models for Effluent Discharges, 3rd Edition, U.S. Environmental Protection Agency, Office of Research and Development, Washington, DC, EPA/600/R-93/139.

Dickey, T., 1991, The emergence of concurrent high resolution physical and bio-optical measurements in the upper ocean and their applications, Reviews of Geophysics, 29, 383-413.

Dickey, T., R.H. Douglass, D. Manov, D. Bogucki, P.C. Walker, and P. Petrelis, 1993, An experiment in duplex communication with a multi-variable moored system in coastal waters, Journal of Atmospheric and Oceanic Technology, 10:637-644.

Fischer, H.B., E.J. List, R.C.Y. Koh, J. Imberger, and N.H. Brooks, 1979, Mixing in Inland and Coastal Waters, Academic Press, New York, 483pp. JKK Look Laboratory, University of Hawaii, 1990, Progress report for the Honouliuli WWTP Barbers Point Ocean Outfall.

JKK Look Laboratory, University of Hawaii, 1990, Progress report for the Honouliuli WWTP Barbers Point Ocean Outfall.

Jones, B.H., A. Bratkovich, T. Dickey, G. Kleppel, A. Steele, R. Iturriaga, and I. Haydock, 1990, Variability of physical, chemical, and biological parameters in the vicinity of an ocean outfall plume, in Stratified Flows: Proceedings of the Third International Symposium on Stratified Flows, E.J. List and H.H. Jirka (eds.), American Society of Civil Engineers, New York, 877-890.

Jones, B.H., L. Washburn, and Y. Wu, 1991, The dispersion of ocean outfall plumes: physical and biological dynamics, Coastal Zone '91, Proc. of 7th Symp. on Coastal & Ocean Management, American Society of Civil Engineers, Long Beach, CA, July 8-12, 1991, 74-85.

Jones, B.H., L. Washburn, R.C. Smith, and A. Petrenko, 1992, Physical and Bio-optical variability in the Sub-polar North Atlantic during MLML. Abstract in EOS 73(43):263; presented at fall American Geophysical Union Meeting, San Francisco, December 6-11, 1992.

Jones, B.H., T.D. Dickey, L. Washburn, and D. Manov. 1993. Physical and biological dynamics of sewage outfall plumes in the coastal region: an integrated observational approach.  In: Water Pollution II: Modeling, Measuring and Prediction, L.C. Wrobel and C.A. Brebbia (eds.), Computational Mechanics Publications, Southampton, pp. 527-534.

Noda, E.K., 1992, Near-field and far-field plume analysis Sand Island outfall system, progress report for the Department of the Corporation Counsel City and County of Honolulu, Honolulu, Hawaii.

Parnell, P.E., 1992, Impacts to phytoplankton and water quality of Hawaiian coastal waters by primary and secondary sewage outfalls. M.S. Thesis, University of Hawaii, 47 p.

Roberts, P.J.W., Snyder, W.H., and Baumgartner, D.J., 1989a, Ocean Outfalls I: Submerged Wastefield Formation, Journal of Hydraulic Engineering, ASCE, 115(1),1-25.

Roberts, P.J.W., Snyder, W.H., and Baumgartner, D.J., 1989b, Ocean Outfalls II: Spatial Evolution of Submerged Wastefield, Journal of Hydraulic Engineering, ASCE, 115(1),26-48.

Roberts, P.J.W., Snyder, W.H., and Baumgartner, D.J., 1989c, Ocean Outfalls III: Effect of Diffuser Design on Submerged Wastefield, Journal of Hydraulic Engineering, ASCE, 115(1), 49-70.

Roberts, P.J.W. and D. Wilson, 1990, Field and model studies of ocean outfalls, Proc. 1990 National Conference on Hydraulic Engineering, San Diego, July 30-Aug. 3, Ed. H. Chang, American Society of Civil Engineers, New York.

Roberts, P.J.W. and D. Wilson, 1990, Field and model studies of ocean outfalls, Proc. 1990 National Conference on Hydraulic Engineering, San Diego, July 30-Aug. 3, Ed. H. Chang, American Society of Civil Engineers, New York.

Spinrad, R.W. 1986. A calibration diagram of specific beam attenuation, Journal of Geophysical Research 91(C6):7761-7764.

Washburn, L, B. Jones, A. Bratkovich, T.D. Dickey, and M.-S. Chen, 1992, Mixing, dispersion, and resuspension in vicinity of an ocean wastewater plume, Journal of Hydraulic Engineering, 118, 38-58.

Wu, Y., L. Washburn, and B. H. Jones. 1994. Buoyant plume dispersion in a coastal environment: Observations of plume structure and dynamics. Continental Shelf Research, 14(9):1001-1023.

Zaneveld, J.R., R. Bartz, and J.C. Kitchen, 1990, Reflective-tube absorption meter, In: Ocean Optics X, R.W. Spinrad (ed.), Proc. SPIE, 1302:124-136.

# 8. Appendices

## APPENDIX A - Moored observations (based on SAIC data sets)

## APPENDIX B - Towyo surveys

### 09/25/94

On 09/25/94, the sewage effluent was detected between 50 and 70 m at the northern end of towyo 4 (Figure B.2c), a cross-shelf towyo at the center of the diffuser, and between 58 and 70 m at the northern end of towyos 2 and 3 (Figures B.2 a & b), cross-shelf transects respectively 2 km and 1 km west of the center of the diffuser (Figure B1.a). For these towyos, the cross-shelf extent of the plume is estimated to be about 1 km. Freshly released wastewater was not detected to the east of the diffuser.

The mixed tide had an intermediate low tide at 1517, height 0.7 ft, followed by an intermediate high tide at 1912, height 0.9 ft and a low tide at 1255, height 0.4 ft. All towyos were done in the afternoon and evening (between 1430 and 2150), hence in a period of minimal tidal range (Figure 4.5).

Below 50 m, the currents were relatively weak during most of the day except for a strong onshore northwest current between 0930 and 1100 (Figure A.2b). This was immediately followed by offshore flow for about 1 hour. While the alongshore flow remained weakly westward, cross-shelf currents reversed several times between 1900 and 2200. The high temporal variability of the cross-shelf currents may explain the limited cross-shelf extent of the plume

### 09/26/94

Six towyos were done on 09/26/94 in the area west of the diffuser; towyos 8 to 12 were cross-shelf transects and towyo 13 was an alongshore transect toward the west end of the diffuser (Figure B.1a).

At the west end of the diffuser (towyo 8), the plume was detectable at the northern end below 48 m (Figure B.2f). Further offshore, the high beam c associated with the effluent was located between 40 and 50 m, within a layer with high chlorophyll fluorescence (30-55 m).

For towyo 9, 1 km west of the center of the diffuser, the highest peaks in beam c (beam c = 0.9 m-1) were located below 45 m in the northern part of the transect and were associated with decreases in chlorophyll concentration (Figure B.2g). Offshore, at the third downcast of towyo 9 (0.45-0.52 km), the plume separated into two layers between 48 and 58 m. About 150 m further offshore at ~0.65 km, the stronger beam c signal was found in the lower plume (53-56 m) and the upper plume (45-48 m) had a lower beam c signal, but a higher chlorophyll fluorescence signal.

The plume was found at the beginning of towyo 10, 1.5 km west of the center of the diffuser from 43 to 62 m (Figure B.2h), and at the beginning of towyo 11, 2 km west of the diffuser from 53 to 73 m (Figure B.2i). Further south, the beam c signal was also separated into basically two vertically separated horizontal layers. The lowest one was dominated by the effluent plume waters, and the upper one consisted of a combination of effluent and phytoplankton detected by chlorophyll fluorescence.

In towyo 12, 3 km west of the center of the diffuser, the plume was located deeper than during the previous towyos. The final plume level after overshooting was located deeper than the sewage diffuser depth (Figure B.2j). Moreover, the weak signal of the plume deeper offshore suggests gravitational spreading. Plume wastewater was present between 72 and 80 m on the 95 m isobath, and between 78 and 90 m 500 m further south.

The alongshore towyo 13, obtained at the end of the day, clearly shows that the plume was located below a chlorophyll layer, about 2.5 km west of the diffuser (Figure B.2k distance 0-1 km). Closer to the diffuser (i.e., distance superior to 1 km), the plume signals were weaker than the values detected earlier during the cross-shelf towyos. The plume may have been displaced shoreward by the strong onshore currents present at depths below 40 m after 2 p.m. (Figure A.2c).

The cross-shelf extent of the plume was about 0.5 to 1 km narrower than on 09/25/94. The plume extended deeper, west of the diffuser, and also traveled south along towyo 12, 2.5 km west of the center of the diffuser. The tidal cycle was similar to the cycle of the previous day, but the towyos were done later in the day, between 1815 and 2210 (Figure 4.5). The cross-shelf component of the currents was strong (peaks higher than 20 cm·s-1) and exhibited high temporal variability.

### 09/27/94

In the early afternoon (1330 to 1445), the plume was detected at the west end of the diffuser (towyo 16), and 1 km west of the center of the diffuser (towyo 17; Figure B.1b). Remnants of the plume were detected by their low salinity signature as far away as 3.5 km west from the end of the diffuser (towyo 19). The plume depth on this transect was between 55 and 58 m (Figure B.2q). The plume was not detected east of the diffuser.

Across the western end of the diffuser, the sewage plume was located between 60 and 70 m (towyo 16; Figure B.2n), and 550 m to the west below 55 m (towyo 17; Figure B.2o). The plume was about 10 m thick. The strongest signals in beam c measured during the entire sampling period (about 1.18 m-1) were found within towyos 16 and 17. These high beam c values correspond to the deeper and less diluted parts of the effluent plume, between 60 and 70 m. In the case of towyo 16, these values were correlated with a sharp decrease in chlorophyll concentration (from 1 mg·m-3 of chlorophyll below the plume to 0.4 mg·m-3 in the plume). Beam c also reached 1.18 m-1 in towyo 17, but the chlorophyll concentration remained high in the plume (0.7 mg·m-3).

A long west-east transect - in fact composed of two alongshore towyos, 20 and 21, - was done in order to determine whether there was offshore dispersion of the plume (Figures B.2 r-t). The end of towyo 20 showed the plume extending over a distance of 2 km west to south-west of the diffuser (Figure B.2r distance 3-4 km and Figure B.2s distance 4-5 km).

### 09/28/94

Seven towyos were done in the afternoon, six cross-shelf and one alongshore (Figure B.1b). A strong signal of the plume was found at the western end of the diffuser between 48 to 70 m and was overlapped by a layer of high chlorophyll fluorescence (towyo 22; Figure B.2u). Chlorophyll fluorescence and beam c values indicative of phytoplankton population and diluted wastewater were tracked as far west as 2.5 km from the center of the diffuser (Figure B.2y). Towyo 27, alongshore towyo, confirmed the spatial distribution of the effluent plume and clearly showed the overshooting stage of the plume before it reached a 50 m equilibrium depth (Figure B.2z).

All the cross-shelf transects were done between 1320 and 1630. High tide, 1.9 ft, was at 1106  and low tide, 0.5 ft, at 1840. Hence the towyo measurements were collected during the accelerating phase of the ebb flow when the currents were expected to be westward. But the currents measured at D2 were quite weak and did not exhibit any specific trend during this day. The alongshore towyo, towyo 27, was done around 1700 during the decreasing ebb flow, which did not change the western position of the plume in contrast with what was observed on 09/26/94.

### 10/01/94

The transects were done first on the east side of the diffuser since eastward currents were expected to accompany Kona conditions, and then on the west side of the diffuser (Figure B.1c).

Weak but distinct signals in beam c indicated a trace of plume at about 80 m south-east of the diffuser, during towyo 28 (Figure B.2aa, distance 0.6 km). Other weak, but also very clear, indications of the plume were found at shallower depths west of the diffuser, in towyos 30 to 33.

During towyo 30, the plume's presence between 45 and 55 m did not affect the monotonic increase of chlorophyll concentration with depth (Figure B.2ac). During towyos 31 and 32, chlorophyll fluorescence did not increase, but exhibited a constant level across the sewage plume (Figures B.2ad & ae). Since towyo 30 was done west of towyos 31 and 32 (Figure B.1c), this shows that the further away from the diffuser where the wastewater is more diluted, the distribution of chlorophyll fluorescence is less affected by the plume presence.

During towyo 33, the plume was tracked southward (Figure B.2af, distance 1 km). Its maximum height of rise was 45 m, with a thickness of about 15 m. Offshore, its minimum detected thickness was 2 m, at depth 52 m, and its characteristics were still undoubtedly related to the effluent plume.

Beam c levels on 10/01/95 were among the lowest values observed during the week-long cruise, but were nonetheless due to wastewater presence, as supported by salinity and AC-9 optical measurements. The low levels of beam c indicated that the wastewater plume was more diluted than the plume at the beginning of the cruise. This can explain the undisturbed, if not increased, presence of phytoplankton detected by high levels of chlorophyll fluorescence. Kona conditions began on 09/29/94 and apparently led to the warming of the top 60 m of the water column, weakening the stratification and resulting in greater height of rise and dilution of the wastewater plume. The increased thickness of the plume was likely related to the weaker stratification and the higher dilution level. It seems that this environment was highly favorable for inter-leaving conditions since thin plume layers (2 m) were easily detected.

## APPENDIX C - Methods of Estimating In Situ Dilution

Three approaches for estimating dilution from salinity and temperature are described below. Dilution is calculated from the effluent wastewater with discharge volume Ve and discharge salinity Se, and ambient water, of volume Va and salinity Sa.

The dilution D is defined as :

D = (Ve + Va)/ Ve			          		(1)

S is usually used in the literature to identify the dilution instead of D (e.g. Roberts and Wilson, 1990), but for clarity in the following equations with salinity terms, D is used to define dilution and S is used to define salinity. The dilution of the conservative tracer, here salinity, is derived from the equation of conservation of flux of the tracer:

Ve * Se + Va * Sa = (Ve + Va) * Sm					(2)

Sm is the salinity of the mixed (effluent and ambient) waters of volume (Ve + Va); here Sm corresponds to the measured salinity and, in principle, Se < Sm < Sa. Equation (2) divided by Ve is:

Se + Va/Ve * Sa  = D * Sm					(3)

By solving for Va in equation (1) and substituting it into (2), we can write:

Se - Sa = D * (Sm - Sa)				 	(4)

The dilution can then be written as:

D = (Sa - Se) / (Sa - Sm)				      (5)

In the present analysis, the salinity of the effluent is assumed to be zero, Se = 0; hence equation 5  becomes:

D = Sa / (Sa - Sm) 					 (6)

### Method 1 - Based on Rapid Mixing with Ambient Water at Diffuser Depth:

The first approach uses a constant salinity for Sa, the ambient salinity. Sa is chosen to be the salinity at a depth of 70 m, the depth of the diffuser. Hence this dilution can be interpreted as the initial dilution assuming that effluent wastewater was mixed with entrained water at 70 m and rose without additional mixing to the equilibrium depth where dilution is calculated. Although it is unrealistic to expect that no mixing occurs as the plume rises, the calculated dilutions seem to be very reasonable in the plume core and agree with the results of the third method described below for depths greater than 50 m.

Figure C.1 shows the salinity measured at station 9 on 09/29/94.  The ambient salinity is assumed to be Sa = 34.93 PSU, the salinity observed at 70m.  Above 70 m, this method results in a conservative estimate of dilution, i.e. an underestimate of the in situ dilution.  But at  70 m or deeper, where the measured salinity was ³34.93 PSU, the dilution can reach infinite and/or negative values. This calculation is not applicable in the upper layer either where low salinity is likely to result from runoff and far away from the diffuser where the source of the low salinity water may not be certain. More fundamentally, although large initial dilution occurs in the diffuser jets, it is likely that mixing continues in the rising plume. At the Palos Verdes, CA (near Los Angeles) sewage outfall, active mixing has been found up to the equilibrium depth (Wu et al., 1994). Hence, alternative approaches to estimating dilution are presented below.

### Method 2 - Based on the Background Salinity/Depth Profile:

An alternative method to Method 1 utilizes a depth-varying  Sa, Sa(z). From the background vertical profiles of salinity, a characteristic profile is chosen which is more or less representative of the water column which is unaffected by the effluent.  Ideally, this profile comes from along the diffuser isobath where the water column is unaffected by the plume. Because parcels of effluent may be distributed throughout the region, it may be difficult to find a profile that is uncontaminated by effluent to create a characteristic Sa(z). In addition, considerable horizontal variability in the vertical structure over the shelf due to the effects of topography, coastal inputs of freshwater, and the effects of internal waves and tides, could limit the effectiveness of this method. In an effort to mitigate these problems, and to only consider the question of initial dilution, we have chosen to use vertical profiles from directly over the diffuser and to interpolate an ambient salinity structure through the salinity profile in the region that is obviously affected by the effluent plume.  Figure C.1 shows a sample profile from station 9 where both the observed salinity profile and the interpolated salinity (Sa2) in the region of the plume are plotted. Dilution is then calculated from Equation 6 using the interpolated salinity field as Sa(z) and the measured salinity field as Sm(z).

This method is somewhat subjective since it depends on the choices of end points which are used for the interpolation. Dilutions calculated by this method should be considered as an upper limit of dilution. These results were used for the comparison with the modeled initial dilution  in section 4.2.3.

### Method 3 - Estimating Sa from the Ambient T-S Relationships:

Another method for calculating dilution using a varying Sa is to use the background temperature/salinity (T-S) relation (taken as a constant each day) and to estimate Sa using the measured temperature (Figure C.2). This approach assumes that the effluent has a negligible effect on the temperature of the water into which it mixes. Thus, a value for Sa could be calculated for each data point as a function of the measured temperature. This value is then used in Equation 6 to calculate the dilutions discussed in section 4.2.4.  Advantages of this method include 1) the calculation of  Sa is independent of depth and 2) Sa is allowed to vary as it would in the water column.

The range of dilutions computed with this approach is large. Within the plume core and below 50 m, the dilutions calculated with this method are similar in value to the dilutions estimated using constant salinity of Method 1 (Figure C.1).  But above 50 m, dilutions calculated with the third method show a wide range of variations, as would be expected. Because the first method may result in negative dilutions below 70 m, and the second method is not as easily implemented because of the spatial heterogeneity of isohaline depths, the third method was chosen for the analysis of plume dispersion (section 4.2.4).

It should be noted that a fundamental problem with all of these methods is that a parcel of effluent passes through water of varying salinity and temperature characteristics both as it rises and as it advects laterally.  Without tracking that particular parcel continuously to measure both its characteristics and the characteristics of the ambient water, it is impossible to estimate dilution using any of these approaches with complete accuracy.

## APPENDIX D - Spatial Maps of Dilution from Towyo Sections