THE NODC STANDARD SUBSET OF SHIPBOARD ADCP DATA

The NODC provides shipboard ADCP data in two formats:

1) a high-density set in CODAS binary format consisting of

currents and ancillary parameters at the sampling interval

with which the data were recorded and processed and 2) an

ASCII standard subset of absolute currents at hourly and

10 m intervals.  If absolute currents are not available

due to lack of navigation, then relative currents are provided.

The standard subset includes the hourly means and standard

deviations of ship velocity components and the transducer

temperature.  A description of the methodology and format of

the standard subset is given below.

METHODOLOGY

The standard subset will consist of absolute currents

if the ship navigation was satisfactory.  Otherwise the

currents will be given relative to the mean over a reference

layer, usually 50 to 150 m.

In the vertical, each velocity profile is linearly interpolated

to a regular grid at integral multiples of 10 m.  The shallowest

level is the first multiple of 10 m that is greater than or equal

to the depth of the first good bin of the recorded data, typically

a function of the instrument configuration and the transducer depth.

The deepest level is that at which at least 50% of the ensembles

over the course of the cruise have a Percent Good greater than 30%.

For each depth bin, the Percent Good is the percentage of individual

pings in an ensemble with valid data.

In time, averages are calculated for one-hour intervals centered

on the hour.  The time column in the ASCII subset gives the mean of

the times of the ensembles that fall into the interval.  Hence the

mean may be slightly off the integral hour, even when ensembles

are evenly available over the entire hour.  The mean time may be

even less centered in the case where data are only partially available

over the interval because instrument recording was turned off (e.g.,

at port calls).  In the case where data are totally unavailable over

the entire interval so no mean time can be calculated, a "placeholder"

entry is made and assigned a time value exactly at the middle of the

interval, i.e., on the hour, followed by bad value flags for the

longitude, latitude and velocity fields.  It is left to the user to

manipulate the time column if a truly even time grid is desired.

At each level, a bad value flag is given if less than 50% of the

ensembles in the hour have valid data.  Bad value flags (99999)

for velocity components also occur at depth layers where less than 50%

of the ensembles in the interval contribute to the average because of

low percent good and other editing criteria, or in the case of

absolute currents, occur over the entire depth range because ship

velocity was not available as a reference.

The transducer temperature is provided as a mean and standard

deviation over the hour.  The ship velocity is provided as means

and standard deviations of the components over the hour and is

derived from the final absolute ship velocities of each ensemble

as stored in the CODAS high-density database.  The ship velocity

information allows one to see at a glance if the ship was on-station,

underway, or both over the hour.

FORMAT DESCRIPTION

====> HEADER RECORD

sac_id=xxxxx yr_base=yyyy start_lev=ddm num_lev=ddd type

e.g.

sac_id=00001 yr_base=1993 start_lev= 20m num_lev= 45 absolute

where sac_id     is a unique identifier for the cruise (NODC bookkeeping)

yr_base    is the year inwhich the cruise begins

start_lev  is starting level (meters)

num_lev    is the number of levels

type       means either "absolute" or "relative" currents

A complete documentation file which describes the cruise, instrumentation,

processing, and other pertinent comments for the given cruise will be

available separately.

====> DATA RECORD

The format is designed for ease of application; therefore,

the date-time and positions are provided in decimal form as a

single field.  Each data record will be fixed length although

the length may vary from cruise to cruise due to differences in

the number of depth bins.

The order of each record is as follows:

<DAY-TIME> <POSITION> <TEMPERATURE> <SHIP VELOCITY> <U,V pairs repeated>

With a template such as:

ddd.fffff dddd.dddd ddd.dddd tt.t tt.tt suu.u su.uu svv.v sv.vv uuuuu vvvvv ..

e.g.

350.00002  157.9365   6.9120 28.9  0.01  -4.6  0.11  -3.4  0.09   419   177 ..

field         template               comment

<DATE-TIME>     ddd.fffff:              decimal day; e.g. 0.50000 is

1200 UTC on January 1 of the year

given by yr_base; always present

<POSITION>      dddd.dddd ddd.dddd:     longitude and latitude decimal

degrees; lon positive east,

lat positive north;

missing flag= 1E38;

e.g. -145.1667 is 145 deg 10' W

<TEMPERATURE>   tt.t tt.tt              transducer temperature mean and

standard deviation (deg C);

missing = 1E38

<SHIP VELOCITY> suu.u su.uu svv.v sv.vv absolute ship velocity components

means and standard deviations in

m/sec; missing = 1E38

<U V PAIRS>     uuuuu vvvvv             current components in mm/sec;

bad or missing = 99999;

e.g.   121 is 12.1 cm/s