Protocol for determining amorphous Fe by ascorbic acid extraction

Standard preparation

Ferrozine-acetate reagent: 50mL of 0.01 M ferrozine and 50mL of 5M ammonium acetate

Hydroxylamine: 100mL of 0.2M hydroxylamine in 0.1M HCl

Fe2+ stock solution: for 1mM, add 0.039214g of ferrous ammonium sulfate to 100mL degassed de-ionized H2O. add HCl to prevent Fe2+ oxidation (to pH=2)

Make standards: blank, 1,2,5,10,15,20M Fe2+

(2.5mL degassed de-ionized water, 2.5mL ferrozine reagent, appropriate aliquot of Fe2+ solution, 100 L hydroxylamine) (aliquots will be 5, 10, 25, 50, 75, 100 L of 1mM stock)

Color development takes 5 minutes

Run on UV-VIS at =562nm

Plot abs vs concentration (in molar).  Slope should be close to 27000.

# Samples

Mass approximately 0.4g of wet mud.

Prepare ascorbic acid solution

10g sodium citrate and 10g sodium bicarbonate in 200mL degassed de-ionized

onto this solution, slowly add 4g of ascorbic acid.

Check that pH is near to 8. If not, adjust with acid or base.

Add mud to 10mL of ascorbic acid solution in plastic vial

Shake vials in water bath at 200 rpm at 25oC for 24 hours.

Remove vials and filter with 0.2m

To determine amorphous iron, add 100L of filtered sample, 100L hydroxylamine, 2.5mL ferrozine, 2.5mL de-ionized H2O.

Make a blank with 100L hydroxylamine, 2.5 mL H20, 2.5mL ferrozine, and 100L of original ascorbate soln.

Run on UV-VIS at 562nm.

# Sample Data

| Sample | Abs. | Corrected Abs. | Dilute Conc | Corrected Conc. (M) | Sediment mass (g) | Conc. (mol/g) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | .263 | .211 | 8.69233436 | 452.001439 | 0.4317 | 10.470267 |

Abs. is actual sample absorbance

Corrected Abs. is actual, minus absorbance of the blank, in this example blank abs. was 0.052 (not shown).

Dilute Conc. is the sample absorbance divided by the slope of the standard curve. Here that slope was 24274 (calculation based on standard not shown).

Corrected conc. is the actual concentration accounting for the dilution of .1mL of sample in a 5.2mL solution (100L sample, 100L hydroxylamine, 2.5mL ferrozine, 2.5mL H2O=5.2mL total, but only 0.1mL was sample, therefore here we multiply dilute conc by 52)

Sediment mass was the original wet mass of the sediment sample.

Final moles in sediment calculated by multiplying the corrected conc. (M) by 0.01L to account for the original 10mL of ascorbate solution used as extract. That value is then divided by the original sediment mass.