# Field & Laboratory Protocols

Field & Laboratory Protocols	1

CTD PROTOCOLS AND SALINITY1	9

DISSOLVED OXYGEN1	13

DISSOLVED INORGANIC CARBON1	24

NITRATE, NITRITE and DISSOLVED ORGANIC NITROGEN1	29

ORTHOPHOSPHATE AND DISSOLVED ORGANIC PHOSPHORUS1	36

SOLUBLE REACTIVE SILICA1	41

PARTICULATE CARBON AND NITROGEN1	45

PARTICULATE PHOSPHORUS1	48

FLUOROMETRIC ANALYSIS OF CHLOROPHYLL A. AND PHAEOPIGMENTS1	52

HPLC ANALYSIS OF ALGAL PIGMENTS1	55

PRIMARY PRODUCTIVITY1	58

BACTERIA AND CYANOBACTERIA1	62

MICROBIAL ATP1	65

DISSOLVED AND PARTICULATE LIPOPOLYSACCHARIDE1	69

SEDIMENT TRAP PROTOCOLS1	72

MEASUREMENT OF SOLAR RADIATION1	76

SHIPBOARD METEOROLOGICAL OBSERVATIONS AND NDBC BUOY1	78

LOW LEVEL NITRATE AND NITRATE BY CHEMILUMINESCENCE1	80

TITRATION ALKALINITY1	85

MEASUREMENT OF pH1	89

ALKALINITY1	92

Particulate S82	97

Salinity, pH, & electrochemistry2	98

Iron: Fe(II), total dissolved Fed, total FeT 2	99

H2S by UV-vis 2	100

FeS & FeS22	101

Bimane (thiols, thiosulfate, polysulfides)2	102

Sulfate, nitrate, nitrite, ammonium2	103

Chloride & phosphate2	104

Metals by ICP (Mg, Mn, Fe, Co, Ni, Zn)2	105

Field & Laboratory Protocols Sources	107

QUALITY CONTROL/QUALITY ASSURANCE PROGRAM

_________________________________________________________________

SUMMARY:  The primary objective of the HOT program

is to collect and interpret biological, chemical and

hydrographic time-series data.  In order to provide

accurate and reliable data, to the oceanographic

community, the JGOFS component of HOT has established

a quality control/quality assurance (QC/QA) program

that is designed to assess and maintain data quality.

These QC/QA procedures encompass all aspects of the

program from sample collection through data reporting.

_________________________________________________________________

1.  Introduction

Our QC/QA program is designed to ensure that data of the

highest quality are obtained from the HOT program.  A fundamental

component of this QC/QA program is the documentation of the detailed

analytical procedures that are presented in the following chapters.

These procedures are consistently applied in our laboratory analyses.

This chapter describes the HOT program QC/QA procedures that are

independent of the specific analytical protocols presented in

subsequent chapters.  These QC/QA procedures include field sampling,

analytical facilities and instrument maintenance, interlaboratory

comparisons and data reporting.

2.  Precision and Accuracy

The precision and accuracy of each analytical procedure is

discussed in the appropriate chapter.  Accuracy is a measure of

how close an analyzed value is to the true value.  In  general,

the accuracy of an analytical method is determined by the use

of calibrated, traceable reference standards.  However, it is

important to bear in mind that the assessment of accuracy based

upon primary standards can be misleading if the standards are not

prepared in seawater because many of our chemical determinations

exhibit matrix (i.e., salt) effects.  In addition, it must be

recognized that most of the HOT program core measurements (e.g.,

dissolved oxygen, pH, pCO2, primary production, etc.), do not

have readily available reference materials.

Precision is a measure of the variability of individual

measurements (i.e., the analytical reproducibility) and in the

HOT program two categories of replicates are measured; field and

analytical replicates.  Analytical replication is the repeated

analysis of a single sample and is a measure of the greatest

precision possible for a particular analysis.  Field replication

is the analysis of two or more samples taken from a single

sampling bottle and has an added component of variance due to

subsampling, storage, and natural within sample variability.

The variance of field and analytical replicates should be equal

when sampling and storage have no effect on the analysis (assuming

the analyte is homogenously distributed within the sampling bottle).

Therefore, the difference between field and analytical replicates

provides a first order evaluation of the field sampling procedure.

Higher level variance due to sample bottle replication (multiple

bottles on same cast or multiple casts) is not well-resolved in the

current HOT sampling protocols.

It is apparent from these definitions that precision and accuracy

are not necessarily coupled.  An analysis may be precise yet inaccurate,

whereas the mean of a variable result may be quite accurate.  Therefore,

precision and accuracy must be evaluated independently.

3.  Overview of the Quality Control/Assurance Program

The basic framework of the HOT-JGOFS QC/QA program addresses field

sampling, laboratory facilities, laboratory analysis and data reporting.

Quality control in the field is primarily attained by utilizing modern

sampling equipment that is properly maintained.  The quality of field

and laboratory instruments is preserved with appropriate instrument

maintenance, periodic calibration and careful documentation procedures.

Laboratory analysis QC/QA is evaluated on the basis of periodic review

of methodology, variance evaluations (control charts), reference

materials (where available) and inter- and intralaboratory comparisons.

Quality control procedures associated with data reporting include sample

documentation, tracking and evaluation of analytical results, relative

to sample documentation, and comparison of results to historical values.

4.  Field Sampling and Strategy

Specific aspects of the time-series field sampling strategy have

been presented (see Chapter 2), and will not be repeated here except

to emphasize key aspects of our QC/QA program.  Station KAHE (see map

in Chapter 1) serves as a representative coastal site for the collection

and interpretation of long-term environmental data and as an equipment

test station.  At Station KAHE, an initial cast is performed using

only a weight to test the winch and to inspect the condition of the

hydrowire.  This test cast is followed by a CTD-rosette cast to 1000 m

with a full complement of 24 12-liter water bottles.  The latter serves

to test the CTD, pylon and deck box, to collect water column samples

and to provide a hands-on opportunity for novice members of the

scientific party to participate in the deployment and retrieval of

the rosette and the collection of water samples.  Ideally, samples

are collected and processed exclusively by experienced personnel.

However, the HOT program encourages graduate and undergraduate

participation and endeavors to combine marine research with marine

science education.  Consequently, we conscientiously schedule an

at-sea training session to ensure that the procedures followed are

identical from month to month.

During each HOT cruise, at least 20% of the samples are routinely

collected in duplicate or triplicate to evaluate field precision.

In addition, salinity samples are drawn and on-deck sample temperatures

(for those casts where oxygen samples are drawn) recorded  from each

water bottle sampled.  Both procedures are useful for the identification

of sample mistrips (i.e., the collection of water from a depth other

than intended).

The collection of representative samples is paramount to a successful

time-series program and is contingent upon the use of appropriate

sampling equipment which is well-maintained and operationally sound.

A field sampling equipment maintenance program is administered by our

Marine Technician.  The program consists of a documented inspection

of field equipment at regular intervals.  A record of repairs,

modifications and any other pertinent information is also maintained.

In addition, diagrams outlining all sampling equipment and assembly

procedures for sediment trap and in situ primary productivity arrays,

radio direction finder tracking equipment, Argos satellite transmitter

and on-deck incubation system have been drawn and are updated as necessary.

Sample collection quality control measures are based upon the

concept of applying time-tested oceanographic sampling techniques in

a standardized and coordinated manner supervised or conducted by

experienced personnel (details of each sampling procedure are outlined

in the following chapters).  Specific sampling data are recorded on

log sheets at the time of collection, identifying the type of samples

collected, cruise, station, time, cast number, sample number and any

other pertinent metadata.  These "metadata," along with copies of the

CTD console log and property vs. depth plots are retained in the

appropriate HOT cruise notebook.  Records are maintained to identify

sample tracking from collection through analysis and data reporting.

Any problems associated with a particular sample are noted on the

appropriate log sheet or data file and are evaluated relative to

routine quality control proceures (Fig. 2).

5.  Analytical Facilities

All analyses are conducted at the University of Hawaii at Manoa

in modern, well-equipped research laboratories.  Specialized analytical

equipment used in the JGOFS project include:  Packard model #4640

liquid scintillation counter, UIC model #5011 coulometer, Biospherical

microprocessor-controlled ATP photometer, Perkin-Elmer model #2400

carbon/nitrogen analyzer, Technicon autoanalyzer and accessories,

automatic Winkler titration system consisting of Brinkmann Dosimat 665,

Orion EA 940 ion analyzer and IBM compatible computer, Guildline Autosal

model 8400A salinometer, Antek model #720 nitrogen oxides analyzer,

Zeiss epifluorescence,  phase contrast and inverted microscopes, Coulter

Epics dural-laser flow cytometer, Ionics model #555 carbon analyzer,

Beckman DU-640 ultraviolet-visible light spectrophotometer, Spectra-

Physics model SP8800 HPLC equipped with Waters model 440 absorbance

and model 470 fluorescence detectors, Waters model 990 photodiode array

spectrometer,  Turner model AU-10 fluorometer, Perkin-Elmer model #LS-5

fluorescence spectrophotometer with data station, Cahn C-31 microbalance

and LAL model #5000 optical analyzer.

In addition to the above, the JGOFS laboratories are well equipped

with standard laboratory equipment, including:  fume hoods, analytical

and toploading  balances, centrifuges, freezers, refrigerators,

volumetric glassware, pipettes, muffle furnaces, pH meters, computers

and other general laboratory equipment and glassware.  The facilities

are maintained to provide optimum conditions for a wide scope of

analytical procedures.  Quality control measures include service

contracts (balances and selected equipment), verification of performance

through the use of calibration curves, standards bracketing samples,

wavelength verification and calibration, measurement of secondary

standards, utilization of NIST Class S weights, NIST traceable

thermometers and analysis of appropriate known and unknown reference

samples.  Instrument operating, service, and calibration manuals

are retained and the calibration, repair and service history of

JGOFS-utilized equipment documented and retained for use by laboratory

personnel.

6.  Chemicals and Reagents

All chemicals and reagents used in our routine sample analyses are

ACS quality, or better.  Incoming chemicals are marked with date

received and recorded on the chemical inventory sheet.  Distilled

deionized water (DDW) is used in the preparation of solutions and

the chemical resistivity of the DDW is continuously monitored to

confirm purity.  New chemicals or reagents are compared to previous

reagent performance and are discarded when:  (1) the expiration date

has elapsed or (2) when the analytical performance is deemed inadequate.

7.  Laboratory Analysis

The specific analytical methodologies outlined in the subsequent

chapters have resulted from extensive methods evaluation (Table 1).

These procedures are conducted by experienced personnel familar with

oceanographic laboratory protocols and instrumentation.  Where applicable,

analytical runs include a series of standards bracketing samples,

replicates (>20%), analysis of reference (control) samples, as well

as procedural, reagent, refractive index, salt, dilution and time-zero

blank corrections.  All analytical results are documented and original

hard copies are archived in the appropriate HOT notebooks.

Sample analysis quality assurance relies heavily on replicate

analysis and use of certified reference standards as determinants of

precision and accuracy, respectively.  Replicates are the primary

determinants of variance and, as discussed previously, can be divided

into two categories; field replicates, providing a measure of sampling

and natural variability, and analytical replicates, providing a measure

of the analytical precision.  As previously stated, at least 20% of the

samples are collected and processed as field replicates.  An additional

number of analytical replicates are analyzed to evaluate analytical

variance.  Where appropriate, internal standards are analyzed on

selected samples.  When necessary additional quality control measures

may include matrix matching, standard additions and comparison of

results with independent methodologies.  When available, traceable

certified reference standards are used to assess the accuracy of each

set of determinations.

Analogous to the field preventative maintenance program is an

instrumental service and calibration program.  This program identifies

and documents service intervals for balances and other specialized

equipment.  Because the analytical equipment used in the JGOFS program

experiences regular use the performance is routinely evaluated.  If a

problem develops, sample analysis is terminated until normal operation

is restored.  In addition, the dependency of many analytical procedures

on the proper and accurate operation of the analytical balance is

recognized and evaluated by weighting secondary standards during periods

of use and periodic comparison to NIST traceable class S weights.  All

calibration, repair, modifications and service histories are maintained

in written logs.

Where relatively small temporal changes are expected in the ambient

concentrations of dissolved and particulate analytes of interest to

the HOT program scientists, it is of paramount concern to quantify

the temporal performance of resulting analytical results in terms of

the accuracy of primary standards.  This can be achieved by routinely

analyzing the same reference standard over a relative long period of

time.  These need not be certified reference standards, however, the

analyte must be temporally stable (ideally greater than 1 year) in

the sample matrix.  We have found that frozen  (-20  C) unfiltered

inorganic nutrient samples (for inorganic nutrient analysis),  dried

pulverized net plankton (particulate carbon, nitrogen and phosphorus),

and a (-20  C) stored pure chlorophyll a standard (fluorometric analysis)

are adequate for assessing temporal variability.

A great asset to any analytical quality control and assurance

program is  participation in inter-laboratory programs.  Interlaboratory

programs allow an independent evaluation of analytical quality and

performance relative to participating laboratories.   The HOT program

has had the opportunity to participate in the following intercomparison

studies:

* NSF-JGOFS intercalibration of plant pigments coordinated by R. Bidigare

and M. C. Kennicutt of Texas A&M

* ICES Marine Chemistry Working Group-sponsored intercomparison of

inorganic nutrient analyses coordinated by D. Kirkwood, United Kingdom

* monthly total CO2 intercomparison with C. D. Keeling, Scripps

Institution of Oceanography

* periodic total CO2 intercomparison with P. Quay, University of Washington

* periodic dissolved oxygen intercomparison with S. Emerson, University

of Washington

* periodic dissolved oxygen intercomparison with Omar Calvario-Martinez,

Instituto de Ciencias del mar y Limnologia, Estacion Mazatlan

* periodic salinity intercomparisons with C. Collins, Naval Postgraduate

School

* methods intercomparison with Bermuda da Atlantic Time-Series, Bermuda

Biological Station for Research

* DON intercomparison coordinated by C.S. Hopkinson, Jr. Marine

Biological Laboratory, Woods Hole, Massachusetts

* DOC intercomparisons coordinated by J. Hedges University of Washington

and J. Sharp, College of Marine Studies, University of Delaware

8.  Data Evaluation and Reporting

Data evaluation and reporting are the final steps in the quality

control process and comprise an essential part of the quality assurance

program.  Here the data are reviewed in the context of the entire

sample collection, storage and analytical process. Discrepancies or

anomalous results are noted at various stages of the analytical process

(Fig. 2) and the final data evaluated for correctness of analysis by

plotting the analyte profile vs. depth and density and investigating

those points outside the historic data envelope.  Data outside the

historic data envelope are not automatically flagged as "bad," but

rather investigated for the source of the problem through the sample

documentation.  If the problem remains unidentificable the data are

flagged "questionable" if  the values are outside the 95% confidence

interval (greater than 2 standard deviations from the historical mean),

and "good" if within this error envelope.  If a source for the discrepancy

is discovered the data are flagged "bad."  At this point all data inside

the historic envelope are flagged "good" and together with the

"questionable" data added to the historic data set.  Finally, all

the data are summarized and reported in our annual data report along

with the appropriate quality flag.

9.  References

American Public Health Association.  1989.  Standard Methods for

the Examination of Water and Wastewater, 17th edition.  American

Public Health Association, Washington, DC.

Dux, J.P.  1990.   Handbook of Quality Assurance for the Analytical

Chemistry Laboratory, New York:  Van Nostrand Reinhold,  203 pages.

Dickson, A.G.  1991.   Measuring Oceanic CO2:  Progress on Quality

Control.  U.S. JGOFS Newsletter vol. 3, number 2, pp. 4-5.

### CTD PROTOCOLS AND SALINITY1

_________________________________________________________________

SUMMARY:  A commercially-available instrument package is

lowered into the ocean on a conducting cable to obtain real-

time, high-resolution profiles of temperature, conductivity

and pressure from which salinity and depth are calculated.

Additional sensors also detect dissolved oxygen concentration

and phytoplankton fluorescence.  Water samples are collected

in bottles attached to the rosette sampler which can be

activated by a surface control unit.

_________________________________________________________________

1.  Principle

High vertical resolution environmental data are collected with

a SeaBird CTD having external temperature (T), conductivity (C) and

dissolved oxygen (DO) sensors and an internal pressure sensor.  There

is also a provision for adding fluorescence (F) and other sensors.

A General Oceanics 24-place pylon and an aluminum rosette containing

24 12-liter Niskin bottles is used to obtain water samples from

predetermined depths.  The CTD and rosette are deployed on a 3-conductor

cable allowing for the real-time display of data and acquisition, and

for tripping the bottles in areas of interest in the water column.

The CTD system takes 24 samples per second of pressure (P), T, C, DO

and F.  The raw data are stored both on the computer and, for redundancy,

on VHS-format video tapes.

2. CTD Data Collection

2.1. CTD data are taken with a SeaBird SBE-09 CTD having internal

pressure sensor, and external temperature, conductivity and

dissolved oxygen sensors.

2.2. The CTD is mounted in an aluminum Scripps-type rosette with 24

places for Niskin bottles.  Water samples are taken on the upcast

for calibration of the conductivity and oxygen sensors.

2.3. CTD data are recorded during both down and up casts.  When the

Niskin bottles are tripped, an event mark is made in the data

files to record the time and CTD data.

2.4. One deep cast to approximately 4700 m (total water depth = 4750 m)

is made on each cruise.

2.5. Ten to twelve consecutive casts to 1000 m are made over 36 hours

to span the local inertial period (~31 hour) and three semi-diurnal

tidal cycles.  This sampling is performed so that tidal or near-

inertial variability can be estimated and removed, thereby

preventing an aliasing of the data records by these components.

During this 36 hour intensive sampling period, most of the water

samples for the GOFS program objectives are collected.

2.6. In 1989, SeaBird introduced a ducting arrangement which is designed

to minimize salinity spiking.  This "T-C" duct has been used in

cruises since HOT-11.

3.  CTD Sensors and Calibrations

3.1. Pressure:  CTD pressure is measured with a 6,000 dbar

Paroscientific Digiquartz pressure transducer with an internal

temperature sensor.  Pressure calibrations are conducted twice

yearly against a reference Paroscientific pressure standard

using a dead weight pressure tester to impose test pressures.

The pressure standard is recalibrated at the Northwest Regional

Calibration Center (NWRCC) every two years.  Pressure is

corrected for thermal shock effects using a linear response

method (Chiswell, 1990):

P = aPm + h * T + b

where: *= temporal convolution operator

h= impulse response function

T= water temperature

Pm= measured pressure

The a and b calibration coefficients are determined from a

calibration at constant temperature and pressure.  The impulse

response function, h, is computed by measuring pressure

perturbations when the CTD is plunged into a cold water bath.

3.2. Temperature sensor:  CTD temperature sensors can be removed and

calibrated independently.  The SeaBird model SBE-3-02/F temperature

sensors are calibrated annually at the NWRCC.  A check on the

temperature sensors is maintained by intercomparing our pool of

sensors between each cruise.  Two sensors (#741 and #866) were

used during 1989.  These sensors were calibrated at Northwest

Regional Calibration Center prior to HOT-1, and again in November

1989.  Between each cruise, they were compared against each other

and against two additional sensors acquired in August 1989.  These

intercomparisons allowed us to map sensor drift that occurred

between the NWRCC calibrations.  Each HOT program data report

will provide a Table of temperature sensor corrections for the

period of investigation.

3.3. Conductivity sensor:  The SeaBird model SBE-4 conductivity cell

is calibrated by comparing CTD-recorded conductivities with

conductivities computed from the discrete water samples.  At

least three samples are used per cast; salinity minimum,

salinity maximum and mixed layer.  On three casts, together

comprising the "WOCE deep-cast," much higher vertical resolution

sampling of the salinity is done to get the best possible

salinity profile.

3.4. Dissolved oxygen (DO) sensor:  A SeaBird model SBE-13 dissolved

oxygen sensor is used employing a polargraphic sensor,

manufactured by SensorMedics.  The DO sensor consists of a

teflon membrane covering a layer of KCl  gel.  A constant

voltage applied across two electrodes results in a current

nearly proportional to the activity of oxygen diffusing across

the membrane.  This current and the temperature of the cell

are measured, and DO is calculated using an algorithm based on

Owens and Millard (1985):

DO = (a1 OC + a2) OSAT (P,T,S) exp (a3 T + a4 OT + a5 p + a6 dOC/dt)

where: OC= sensor current

OT= sensor temperature

OSAT= saturation concentration of oxygen

P = pressure

T = temperature

S = salinity

The coefficients a1,....,a6 are determined from a nonlinear

least-squares fit against check samples taken from bottles

during the deep cast.  Because the DO sensor shows considerable

hysteresis, the calibration is made using the down cast values

of OC, OT, P, T, S at the same density levels of the bottles.

3.5. Fluorescence sensor:  Stimulated fluorescence is measured using

a Sea-Tech flash fluorometer with an excitation wavelength of

425 nm peak emission and a peak response at 685 nm (30 nm FWHM).

4.  Salinity Determinations

Salinity samples are collected directly from the Niskin bottles

into 250 ml polyethylene bottles and stored at room temperature,

in the dark, for subsequent analysis at our shore-based laboratory.

The time between sample collection and analysis is generally less

than 1 week.  Prior to analysis, the samples are equilibrated to

laboratory temperature and the salinity measured using an AGE

model 2100 Minisal salinometer which is calibrated against IAPSO

standard (Wormley) seawater.  Typical precision of replicate

analyses from the same water sample is 0.0003 o/oo; for triplicate

sample bottles the precision is less than 0.001 o/oo.  The effects

of sample storage in polyethylene bottles was systematically

evaluated during year 1 of the HOT program.  The data, summarized

in the 1988-89 HOT Data Report, indicate that S o/oo changes were

negligible.

5.  CTD Post-processing

Once the CTD data are collected and laboratory-determined pressure

and temperature calibrations are applied, they are subjected to

screening and quality control; they are checked for spikes or missing

data caused by electrical interference in the hydrowire.  Spikes are

removed with a 9-point median filter, and missing data are replaced

with interpolated values.  After this initial screening, the data

are averaged to 1/2-second values.  Pressure and conductivity are

then corrected for thermal hysterisis effects.  This processing in

the time-domain is required to allow for correction of lags between

the C- and T- sensor responses.  After the data have been reduced to

1/2-second values, corrections derived from the calibration methods

described above are applied to the conductivity and dissolved oxygen.

Finally, the data are pressure-sorted to remove effects of shiproll

(i.e., only data taken when the CTD is moving downwards are kept),

and averaged into 2 decibar values.

6.  References

Chiswell, S. M.  Dynamic reaponse of SeaBird CTD pressure sensors

to temperature.  In prep.

Owens, W. B. and R. C. Millard, Jr.  1985.  A new algorithm for

CTD oxygen calibration.  Journal of Physical Oceanography, 15,

621-631.

### DISSOLVED OXYGEN1

_________________________________________________________________

SUMMARY:  Seawater is collected from known depths using

CTD-rosette sampling protocols.  Subsamples are drawn

into precalibrated iodine flasks and dissolved oxygen is

chemically bound by the formation of a manganese (III)

hydroxide floc.  The floc is subsequently dissolved under

acidic conditions which stochiometrically converts the

original dissolved oxygen (DO) oxidizing equivalents to

triiodide.  The latter is quantitatively titrated with

sodium thiosulfate to a potentiometric end-point using

a high-precision computer-controlled titration system.

_________________________________________________________________

1.  Principle

The oxygen content of seawater is a fundamental measurement in

oceanography providing information which can elucidate water mass

movements, net primary productivity, atmosphere-ocean interactions

and carbon remineralization processes.  The oxygen content of a

seawater sample is largely determined by a balance between:  (a)

the exchange of atmospheric oxygen with the upper mixed layer, (b)

net increases due to photosynthetic processes and (c) net decreases

due to respiratory demands and heterotrophic processes.  From an

oceanographic perspective, the measurement of dissolved oxygen is a

parameter of fundamental importance.

In this procedure, a divalent manganese solution and a strong

alkali are added to the water sample which results in the formation

of a floc (1).  In the presence of oxygen the manganese II is

oxidized to manganese III (2).  On subsequent acidification (1 < pH < 2.5),

the manganese hydroxide floc dissolves, and I2 is produced in

stoichiometric proportion to the original O2 concentration (3).

The iodine, in the form of triiodide (4), is titrated with a

standardized thiosulfate solution (5), and the oxygen content is

calculated from the quantity of thiosulfate consumed.

(1)	Mn2+ + 2OH ---------- Mn(OH)2

(2)	2Mn(OH)2 + ½O2 + H2O ---------- 2Mn(OH)3

(3)	2Mn(OH)3 + 2I- + 6H+ ---------- 2Mn2+ + I2 + 6H2O

(4)	I2 + I- ---------- I3-

(5)	I3- + 2S2O32- ---------- 3I- + S4O62-

2.  Precautions

It is extremely important to prevent contamination of the

sample with atmospheric oxygen during sampling, fixation and storage.

Drawing tubes and sample flasks must be free of bubbles (3.1 and

3.2.1), as must the autopipette system which dispenses the reagents

(3.2.2).  If the water samples cool significantly or the seal dries

during storage, air can infiltrate the flask.  The rims of the flasks

are filled with seawater and the samples are stored in a location

where temperature fluctuations are minimized (3.4).

The solubility of oxygen in seawater is temperature-dependent and

deep samples may warm up as they are brought to the surface (i.e.

through the thermocline).  It is therefore necessary to measure the

temperature in the Niskin bottle at the time the sample is drawn in

order to calculate the oxygen concentration in situ (3.3).  Oxygen

samples are always the first samples drawn from the Niskin bottles.

3.  Field Sampling

3.1. Sample collection

3.1.1. Subsamples are drawn as soon as the rosette arrives on deck.

DO samples are the first samples drawn from the water

sampling bottle.

3.1.2. The drawing tube is flushed with sample and any bubbles are

displaced.  The tube is then inserted (with water flowing)

almost to the bottom of the sample flask.  Care is taken to

avoid creating turbulence in the flask in order to minimize

the intrusion of atmospheric oxygen.

3.1.3. The flask is overflowed with two to three volumes of sample.

With the sample flowing, the tube is slowly withdrawn from

the flask so that the bottle remains brimful when the tube

is completely withdrawn.

3.2. Sample fixation

3.2.1. Before fixing the sample, the iodine flask is carefully

examined for any bubbles that may have adhered to the walls.

If any bubbles are present, the sample is discarded and drawn

again.

3.2.2. Immediately after examining the flask, the two fixing

reagents (manganous chloride and alkaline iodide) are added

simultaneously using two 1 ml autopipettes mounted side-

by-side and activated by a single injection lever arm.

Care is taken so that bubbles are not added in this process.

The pipette tips are positioned at least 3 cm below the

surface of the sample, the lever arm is pressed slowly and

gently and released while the pipette tips are still

submerged, and the pipettes and tubing are checked frequently

for bubbles.

3.2.3. The glass stopper is carefully inserted and the flask is

shaken vigorously for at least 20 seconds with a rapid wrist

action (it is extremely important that the floc be dispersed

throughout the flask).

3.3. Sample temperature

3.3.1. The temperature of the water in the Niskin bottle is measured

and recorded immediately following sample collection and

fixation.  This is accomplished by a second person to

reduce the time interval between sampling and temperature

determination.

3.3.2. An insulated holding container is rinsed thoroughly and

filled with the sample.  The temperature is then measured

with a NBS traceable quick-response digital thermometer.

3.3.3. In situ temperature is measured at the time of bottle trip

using a calibrated thermistor (see Chapter 4).

3.4. Storage

3.4.1. Approximately 20 minutes after the samples are collected,

the sample bottles are shaken again to resuspend the floc.

The flared mouth of the flask is then filled with seawater

and bottles are stored in a cool location where temperature

fluctuations are minimized.

4.  Analysis

4.1. Computer controlled potentiometric titration

The computer program (DO05HOT.BAS) is written in GW Basic and

located in the DO subdirectory on the AT 286 hard drive.  This

program interfaces and operates the Dosimat (model 665) and pH

meter (Orion model 720A or EA940).  The program parameters are

chosen to optimize the equivalence point determination based on

the computer controlled addition of µl quantities of titrant.  The

titration time is approximately 10 minutes per sample, which is a

function of the response time of the electrode and the preset

electrode stability criteria.  The titration time is minimized by

the manual addition of titrant by the operator until the color of

the sample is a very light yellow (almost clear).  At this point

the computer-controlled functions are enabled.  The computer

terminates the titration when the end point criteria are met.

The following steps outline the Incremental mv Titration program

operation:

1.  After the computer has booted and all peripheral instruments

are connected to the computer and turned on, change

directories to C:\DO.  The titration program and GWBasic

reside here.  To initialize the program, type "GWBASIC DO05HOT"

and return.

2.  If the system is ready to go (i.e., all connections are "OK"

and all peripherals are working) the program will list a

description of the task keys available.  When you are done

with this screen, press return.

3.  You will be presented with the option to load preset parameters

into the program.  For work at Sta. ALOHA you should answer "Y"

or "y".

4.  Next you will be asked to input the name of the file containing

the parameters.  The default answers are ALOHA05 and C:\DO.

These default answers should be accepted for routine work at

Sta. ALOHA.

5.  The program will list the default operational parameters which

have been found to optimize the sensitivity and precision of

the equivalence point determination at Sta. ALOHA and will

prompt you to either accept the default values or change them.

In most instances the statement is self-explanatory.

Statement                   Comments

Sodium thiosulfate normality (N) = 0.10      This is the standardized

normality of the Na2SO3

titrant.  This together

with the change in mv and

volume added determines

the slope.

Buret size (ml) = 5                          Capacity of the Dosimat

buret on the exchange

unit.

Titration increment size (ml) = 0.02         This is the initial

volume of titrant added

before the first slope

criteria is enabled.

Preadd volume (ml) = 0                       This is set at 0 since

we add the preadd manually.

Total vol. of titrant (ml) = 4               This is one of the three

titration end point

criteria.  If 4 ml of

titrant is reached, the

titration is terminated.

Stop potential (mv) = 100                    This is the second of the

three end point criteria.

If the potential reaches

100 mv, the titration is

terminated.

Electrode drift time (sec/.2mv) = 5          Criteria for electrode

stability.

First slope increment change (mv/meq) =      This is a calculated

value which controls the

reduction in titrant

increment size (300 mv/

titrant normality).

Second slope increment change (mv/meq) =     calculated (750 mv/titrant

normality)

Third slope increment change (mv/meq) =      calculated (3000 mv/titrant

normality)

First increment volume change (ml) = .01    Titrant volume increment

when first slope criteria

is enabled.

Second increment volume change (ml) = .002  Titrant volume increment

when second slope criteria

is enabled.

Third increment volume change (ml) = .0005  Titrant volume increment

when third slope criteria

is enabled.

6.  If you wish to accept these values (which you should for routine

work at Sta. ALOHA), then enter a "Y" or "y" and return.  If you

do not want to accept the default parameters enter a "N" or "n".

You will have to input all values except those calculated.

7.  Next you will be asked to input:

(a)  the date of analysis

(b)  the pH meter model used

(c)  if you have a printer connected to the computer

(d)  if you want a detailed or summarized printout of the

titrations (use summarized for routine work at Sta. ALOHA).

(e)  if you want an alarm to be activated at the end of each run

(f)  the path of data storage (C:\DO\HOT##)

(g)  the sample name.  This is an 8 character max file name and

for standardization purposes the following convention is

used.  For blanks and standards an alphanumeric designation

is used which represents the date and sample type.  For

samples, an alphanumeric system is also used which

represents the iodine flask number and the HOT cruise

number (see examples below, assume the date of the analysis

is 3/25/92 for these examples):

Reagent Blank                3252bl1 (this creates a prn

file with the name 3252bl1.prn)

Primary KIO3 Standard        3252std1 (if you run more than

1 std then number sequentially)

CSK Certified KIO3 Standard  3252csk1

DO Sample                    1H36 (iodine flask number

followed by HOT cruise number).

DO Sample Backtitrated       1BH36

Different DO Sample          1AH36

analyzed in a previously

used flask

(h)  your name or ID

8.  At this point a statement will appear on the screen instructing

you that the Dosimat is now in the manual titration mode and

to titrate your sample to a pale yellow color.  To reduce the

titration time, manually titrate to a very pale yellow (almost

clear), then press "x".

9.  From this point on the computer controls the titration,

adjusting the volume of titrant added in response to the

slope criteria.  The titration is terminated when one of

three events occurs.  Either the volume of titrant or mv

criteria is reached or the calculated slope increases four

consecutive times.

10.  At this point, the results are printed out and you are

prompted to make any comments which are specific to that

sample.  Record any problems, observations or concerns.

This is a great help in interpreting the data.  Entering

return will close that file and you will be prompted if you

would like to run another sample.  By entering "Y" or "y"

the process begins again.  If you do not want to run any

more samples press "N" or "n" and you will see "ok".  Type

"system" to return to the C: prompt.  Type "run" to start

the program again.  Normally the program is not terminated

until the end of the cruise.  If all of the samples have

been run but more are to be collected, do not terminate

the program (i.e., do not enter "n"; leave it as is).

Other concerns:

(a)  Backtitrating

The program adds incremental volumes of titrant to the sample

based on selected slope criteria.  If the operator gets too close

to the equivalence point or passes it, during the manual titration

step, then the program will not reach its end point criteria and

will continue to run.  To avoid this, check the mv reading.  For

our samples this reading should be approximately 320-360 mv at the

end of the manual addition.  If you find that you have titrated

just past this point, go ahead and enable computer controlled

titration and observe 3 or 4 cycles checking the slope.  It should

decrease to a minimum value (~-500,000 to -900,000), and then

increase.  If it initially increases and the mv readings are below

300 mv then you have gone too far.  In this case, press F5 and add

volumetrically a known amount of primary standard.  Ideally, you

want to add an amount which is equivalent to the normal manual

addition end point.  I have found this to usually be around 100-

200 µl, depending on how far you went over or how long the program

ran.  The program will have continued to add 20 µl if you passed

the end point during the manual addition.  If you walk away and

come back 1/2 hr later, it will still be adding titrant at 20 µl

every 2-3 minutes.  In this case the 200-300 µl of titrant have

been added.  Therefore, you need to add enough primary standard

to equal this amount plus an amount equal to the manual addition

end point.  Our primary standard is about 0.025 N and the titrant

about 0.05 N so you would need to add 400-600 µl of primary

standard to equal the added titrant, plus approximately 100 µl

to reach the manual addition end point.  From this point, the

program should function properly.

(b)  Interrupting an analysis in progress

If for some reason you need to interrupt an analysis, you can do

so by pressing F1.

(c)  Restarting an interrupted analysis

After the buret refills, the program will prompt you whether you

want to run another sample.  Continue in the normal manner.

(d)  Volumetric delivery of standards

To obtain the maximum precision and accuracy it is critical to

deliver the primary standard in a manner which is accurate and

reproducible.  At sea and in the laboratory we use class A

volumetric pipets which have been gravimetrically calibrated.

At the 5000 µl range, these glass pipets are accurate to within

+4.0 µl (0.08%) with a precision of +2.5 µl (0.05%).  This is

equal to the precision of the Dosimat within the delivery range

of 500-3000 µl.  To obtain this precision it is necessary to be

very reproducible in the delivery operation.  A technique which

produces good results is outlined below:

1)  Fill the pipet past the calibration mark and slowly deliver

the contents into the container until the meniscus is level

with the mark.

2)  Invert the pipet and wipe off any excess solution from the

tip and barrel (the level of the solution within the pipet

will be below the tip at this point).

3)  Invert again and touch the pipet tip to the sample vessel

wall.  Release finger pressure and allow pipet to drain.

After the pipet has drained keep the tip against the vessel

wall for an additional 15-20 seconds.

4)  Remove pipet and rinse any solution that may be on the vessel

wall into the sample with DDW.

To maintain accuracy and precision it is necessary to have an

absolutely clean pipet where none of the delivered solution clings

to the pipet walls.  It is also good practice to rinse the pipet

with the filling solution prior to filling (unless this is the

first use of the pipet).

4.2. Reagent blank determination

The reagent blank is determined by adding the fixing reagents in

reverse order and titrating a known volume of primary standard

three separate times in the same flask.  The volume of the first

titration includes the equivalents of primary standard added plus

any oxidizing or reducing substances in the reagents (V1); the

second titration should theoretically be equal to the equivalents

of primary standard only (V2).  However, it has been our experience

that we rarely observe an obvious difference between V1 and V2

which could be attributed to the reagents and not a variable end

point estimation or pipetting.  To help quantify this relationship

we have added a third aliquot titration (V3), which again should

be equivalent to only the primary standard.  A statistical

comparison between the estimates of |V1-V2| and |V2-V3| is

performed to determine whether a significant blank exists.

Fill an iodine flask with distilled water and add in the following

order, mixing after each addition, 1 ml of sulfuric acid reagent

(10 N), 1 ml alkaline iodide reagent (4 M sodium iodide in 8 N

sodium hydroxide), and 1 ml manganese chloride reagent (3 M).

[Care must be taken to dispense the reagents below the blank

solution surface or in such a manner as to ensure that no

precipitate forms in the neck of the flask.  If this occurs, a

high blank will be encountered if the precipitate subsequently

dissolves in the acidified blank solution during the titration

step.  This is especially prevalent when using the redox

electrode which displaces a larger volume of sample into the neck

of the flask.]  Add 1.0 ml of 0.01 N potassium iodate from a

precalibrated glass class A volumetric pipette.  Titrate

manually, with adequate stirring, to a very pale yellow color,

enable computer operation of the titration and run to completion.

Record the volume at the equivalence point as V1.  Add a second

1.0 ml aliquot of potassium iodate to the flask and titrate as

above.

Record as V2 the sum of the volume at the equivalence point and

the residual titrant from the first titration (this is equal to

the total amount of titrant added minus the volume of titrant

at the equivalence point).  Add a third 1.0 ml aliquot of

potassium iodate, titrate and record volume as V3 in the same

manner as V2.  Repeat this procedure a minimum of 3 times or

until a clear pattern is established.

4.3. Standardization of the thiosulfate titrant

Prepare a blank solution as outlined section 4.1.  Add a known

volume (at least 5 ml) of potassium iodate primary standard (usually

0.02 N). It is necessary to know the volume added and the normality

of the potassium iodate to the highest degree of accuracy that is

reasonably possible.  Therefore, class A volumetric pipets which

have been gravimetrically calibrated should be used and the potassium

iodate should be thoroughly dried, weighed in large aliquots relative

to the sensitivity of the balance to reduce weighing errors, and the

final primary standard volume determined gravimetrically.  Titrate as

outlined above.  Since precision is usually increased with increasing

number of titrations, it is our general practice to titrate 3-7

replicates at the beginning and end of each analytical run.  In

addition to these, we typically run 1-4 standards per day since our

samples are analyzed at sea over a 3-4 day period.

4.4. Calculate the concentration of thiosulfate working solution

using the following relationship:

VT * NT = VI * NI

where: VT= the blank corrected volume of thiosulfate used to

reach the end-point

NT= the normality of thiosulfate solution

VI= the volume of potassium iodate added

NI= the normality of potassium iodate solution

4.5. Sample titration

Remove the ovelying seawater seal and remove the stopper.  Add

1 ml of sulfuric acid reagent (10 N), and carefully slip a clean

teflon-coated magnetic stir bar into the flask.  Place the flask

on a magnetic stirrer, insert the clean redox electrode and titrant

delivery tube.  Titrate as outlined above.  Record the presence of

bubbles or any unusual aspects of the titration or sample condition

in the comment section.  Rinse electrode and titrant delivery tube

with DDW and repeat above procedure for the next sample.

5.  Calculations

Calculate the DO concentration using the following formula:

(Vt-Vb) * Nt * E

µmol O2 l-1 = ________________ - RDO

(Vf - Vr)

where: Vt= volume of titrant (µl)

Vb= volume of blank (µl)

Nt= normality of titrant (µeq µl-1)

E= 0.2500 (µmol O2 µeq-1)

Vf= volume of flask (l)

RDO= dissolved oxygen content of the reagents = (0.804 * 0.14)/Vf

Vr= volume of fixing reagents (l)

This value is derived from Carpenter 1965 (i.e., DO content

of fixing reagents when 1 ml of each reagent is used in 140 ml of

sample equals 0.018 ml O2 l-1).

6.  Precision and Accuracy

The method as outlined above is capable of a precision of 0.1%

or less (as defined by the coefficient of variation for triplicate

samples).  The accuracy of the Winkler titration procedure, when

the Carpenter modifications are employed, has been determined to be

0.1% (Carpenter, 1965a,b).

Currently, we are using a high quality commercially prepared

potassium iodate certified reference standard to independently

assess the accuracy of our potassium iodate primary standard

normality.  A batch of primary standard is prepared and the

theoretical normality normalized to the certified primary

standard value.  Our primary standard is periodically checked

against the certified standard until the batch or lot is exhausted.

7.  Equipment

5 ml Dosimat titrator (Brinkmann)

calibrated iodine titration flasks

teflon-coated magnetic stir bars and retriever

squirt bottle with deionized distilled water (DDW)

reagent dispensers

magnetic stir plate

calibrated volumetric pipettes and flasks

8.  Reagents

Manganese chloride reagent (3 M)

Dissolve 600 g of manganese chloride tetrahydrate (MnCl2.4H2O)

in 800 ml distilled water and make up to 1 liter in a

volumetric flask.

Alkaline iodide reagent

Dissolve 320 g sodium hydroxide (NaOH) in 500 ml distilled

water and, separately, dissolve 600 g sodium iodide in 500

ml distilled water.  Mix the he two solutions 1:1, by volume.

Sulfuric acid reagent (10 N)

Mix 280 ml of concentrated sulfuric acid into distilled water

using a 1 liter volumetric flask

Sodium thiosulfate reagent (0.05 N)

Dissolve 24.82 g of reagent grade sodium thiosulfate (Na2S2O3.

5H2O and make up to one liter with distilled water.  Determine

exact normality as described under 4.3 above.

Potassium iodate reagent (0.025 N)

Dissolve 0.8918 g of dry (100oC, 2 hours) KIO3 into 800 ml

distilled water and bring up to 1 liter in a volumetric flask.

CSK 0.0100 N KIO3 1o standard

Wako Chemicals, Inc., 1600 Bellwood Rd., Richmond, VA  23237.

9.  References

American Public Health Association.  1981.  Standard Methods for

the Examination of Water and Wastewater, 15th edition.

Carpenter, J. H.  1965a.  The accuracy of the Winkler method

for dissolved oxygen analysis.  Limnology and Oceanography, 10,

135-141.

Carpenter, J. H.  1965b.  The Chesapeake Bay Institute technique

for the Winkler dissolved oxygen method. Limnology and Oceanography,

10, 141-143.

CSK Certified Seawater Reference Standards.  Sagami Chemical

Research Center, Sagimahara, Japan.

### DISSOLVED INORGANIC CARBON1

_________________________________________________________________

SUMMARY: Seawater samples are collected at discrete depths

using CTD-rosette sampling protocols.  Subsamples for

dissolved inorganic carbon are collected, immediately

preserved with HgCl2 and stored for subsequent analysis

in the laboratory using a commercial CO2 coulometer.

_________________________________________________________________

1.  Principle

The accurate and precise determination of dissolved inorganic

carbon (DIC) concentrations over annual and interannual time scales

is central to the achievement of GOFS objectives.  In the central

ocean basins, DIC concentration in surface seawater is believed to

be controlled by air-sea exchange reactions.  However, physical

processes and biological activity also influence the concentration

of DIC in surface waters.  Beneath the mixed layer the concentration

of DIC increases as a result of the decomposition of organic material.

DIC concentrations are determined using a commercial CO2 coulometer.

The coulometric determination of carbon dioxide has the unique distinction

of performing with a high degree of both precision and accuracy while

maintaining relatively high sample throughput.  The coulometric

determination of carbon dioxide involves the stripping of an acidified

seawater sample with a carbon dioxide-free air stream and subsequent

absorption of the carbon dioxide by a solution of ethanolamine.

The weak acid generated by carbon dioxide absorbed in ethanolamine is

titrated by a strong base produced electrolytically.  The equivalence

point is detected photometrically with thymolphthalein as the indicator.

The number of coulombs required to reach the end-point is proportional

to the quantity of carbon dioxide evolved from the sample.

2.  Precautions

DIC samples should be the first samples taken from the water bottle

unless dissolved oxygen (DO) or pH is also sampled from the same

hydrocast.  In this case, DIC samples are collected immediately after

the DO and/or pH samples.

Careful subsampling is important for all dissolved gases.  Samples

should be taken as soon as possible and in the same manner as DO samples

(i.e., no bubbles, low turbulence with adequate flushing; see Chapter 5).

3.  Water Sampling

3.1. Drawing the sample

3.1.1. Samples are drawn into clean 300 ml glass reagent bottles as

soon as the rosette arrives on deck.

3.1.2. The drawing tube is completely filled with sample by raising

the end of the drawing tube.  Bubbles are simultaneously

dislodged by manipulation of the tube.  The drawing tube is

flushed and inserted to the bottom of the sample bottle.

3.1.3. The sample bottle is overflowed with at least two volumes of

sample.

3.1.4. The tube is slowly withdrawn from the bottle while the sample

is flowing so that the bottle remains brimful when the tube

is completely withdrawn.

4.  Preserving the Sample

4.1. Some of the sample is removed from the reagent bottle using a

plastic pipette equipped with a rubber bulb.  Enough water is

removed so that approximately 1 ml of air is contained in the

bottle when the glass stopper is inserted.

4.2. 100 µl of saturated HgCl2 is added to each sample.  The tapered

ground glass bottle neck is dried with a Kimwipe wrapped on an

applicator stick.  The bottle is sealed with a ground glass

stopper coated with a light covering of Apiezon grease.  The

stopper is pressed firmly into the bottle to make a good seal.

The stopper is secured with polyethylene tape or a large rubber

band.

4.3. The samples are stored in a cool location, in the dark.

5.  Coulometric Determination of DIC

5.1. Maintenance of extraction and analysis system and temperature control

5.1.1. The glassware used in the extraction system is combusted (450oC,

3 hours) on a regular basis in order to prevent the buildup of

organic films within the extraction system.

5.1.2. The titration cell and rubber stopper are dried overnight at 55oC

before use.

5.1.3. The air stream leaving the extractor is passed through a condensor

and then through a Balston air filter.

5.1.4. The temperature of the titration cell is maintained at 25oC with

circulating water.

5.2.Analysis

5.2.1. A 50 ml plastic syringe is rinsed with sample and then filled and

weighed on a microbalance.

5.2.2. Five ml of 6% phosphoric acid is added to the extractor and the

acid is purged for 2-5 minutes with carbon dioxide-free carrier gas.

5.2.3. The contents of the syringe are injected into the extractor through

a septum.

5.2.4. The syringe is weighed again and the mass of the extracted sample

determined.

5.2.5. The acidified sample is purged with carrier gas.  Successive

coulometer readings are integrated at 1 minute intervals until

they differ by less than 0.01%

6.  Determination of the Coulometer Blank

The coulometer blank is determined at intervals throughout each

day by allowing the coulometer to titrate a CO2-free air stream.  The

blank is taken as the µg C per minute value detected by the coulometer

when a steady-state reading is achieved.

7.  Calibration

Although the digital coulometer output is fairly accurate, the

coulometer response per unit carbon may vary with time.  In order to

achieve maximum accuracy, it is necessary to calibrate the coulometer

with samples containing known quantities of inorganic carbon.  We are

presently using anhydrous sodium carbonate standards.  The dried (270oC

for 3 hours) reagent is carefully weighed on a microbalance to the

nearest 0.1 µg and introduced into a degassed acidified sample solution

in a combusted aluminum boat through a port in the side of the extractor.

Recoveries are generally slightly less than 100%.

8.  Data Reduction and Calculations

In order to compute the absolute concentration of DIC in a water

sample, the integrated reading given at the titration endpoint must

be corrected for both the coulometer blank and the recovery of NaCO3

standards.  These corrections are made by multiplying the blank µg C

min-1 by the time taken to reach the endpoint and subtracting this

value from the integrated reading.  This value is then corrected for

the recovery of standards by dividing by the average percentage

recovery of known standards run on the day of analysis.

9.  Precision and Accuracy

Three replicate samples from a single Niskin bottle generally yield

a coefficient of variation of less than 0.1% (+2 µmol/kg).  The

precision of our replicate determinations of liquid standards averages

approximately 0.5 µmol/kg over periods of months.  The accuracy of our

determinations is within 1 µmol/kg as determined by the routine analysis

of liquid standards provided by Dr. Andrew Dickson of Scripps Institution

of Oceanography under the U.S. Department of Energy Global Change

Research Program.

10.  Quality Control

As a safeguard on the quality of our results, we maintain a set

of secondary seawater standards.  These are made from a large surface

seawater sample, which is preserved with saturated HgCl2 and subdivided

into 300 ml bottles.  These are sealed as described above and stored

in the dark.  These secondary standards are analyzed relative to a

primary seawater standard provided by Dr. Andrew Dickson.

11.  Equipment/Supplies

5011 Coulometer (UIC Inc.) and modified glassware

300 ml ground glass stoppered reagent bottles

kimwipes and applicator sticks

large plastic pipette

Apiezon grease

Cahn microbalance

volumetric flasks

analytical balance / pan balance

muffle furnace

carbon dioxide-free carrier gas

data acquisition system

water bath

12.  Reagents

distilled deionized water

high purity sodium carbonate

potassium iodide

coulometer cathode solution (UIC Inc.)

coulometer anode solution (UIC Inc.)

orthophosphoric acid

13.  References

Johnson, K. M., A. E. King and J. McN. Sieburth.  1985.

Coulometric TCO2 analysis for marine studies; An introduction.

Marine Chemistry, 16, 61-82.

Johnson, K. M., J. McN. Sieburth, P. J. LeB. Williams and L.

Brandstrom.  1987.  Coulometric total carbon dioxide analysis

for marine studies:  Automation and calibration.  Marine

Chemistry, 21, 117-133.

Lindberg, A. O.  1978.  Automatic coulometric titration with

photometric end-point detection. Part II. Coulometric

determination of nanomole amounts of carbon dioxide by non-

aqueous titration.  Analytica Chimica Acta, 96, 327-333.

Robinson, C. and P. J. LeB. Williams.  1991.  Development

and assessment of an analytical system for the accurate and

continual measurement of total inorganic carbon. Marine

Chemistry, 34, 157-175.

### NITRATE, NITRITE and DISSOLVED ORGANIC NITROGEN1

_________________________________________________________________

SUMMARY:  Seawater is collected from known depths using

CTD-rosette sampling protocols.  Subsamples are carefully

drawn and stored in acid-washed polyethylene bottles.

Nitrate/nitrite is measured with an azo dye either before

(nitrite) or after (nitrite plus nitrate) subsamples are

passed through a cadmium reduction column.  Dissolved

organic nitrogen is determined after quantitative

conversion to inorganic N by exposure to UV radiation.

_________________________________________________________________

1.  Principle

In seawater the forms of dissolved nitrogen of greatest interest

are, in order of decreasing oxidation state:  nitrate, nitrite, ammonium

and organic nitrogen.  All these forms of nitrogen, as well as nitrogen

gas (N2), are biochemically interconvertible and are components of the

biological nitrogen cycle.

In this method nitrate is quantitatively reduced to nitrite in a

copperized cadmium reduction column.  The nitrite thus produced, along

with any nitrite present in the original sample, is coupled with an

aromatic amine, which in turn is reacted with a second aromatic amine

to produce an azo dye.  The extinction due to the dye is then read

spectrophotometrically. A second subsample is analyzed without prior

reduction in order to determine the nitrite level.  Nitrate is

calculated by difference between the [nitrate+nitrite] and nitrite

concentrations, using standard solutions.  For surface water samples

(<100 m) where the [nitrate+nitrite] concentration is generally <0.05

µM, we have employed a low-level assay procedure which is based on the

production and detection of nitrous oxides.

Total dissolved nitrogen (TDN) is determined by UV oxidation of the

sample and subsequent analysis for dissolved inorganic nitrogen (DIN =

nitrite + nitrate + ammonia).  Dissolved organic nitrogen (DON) is

computed from the relationship DON = TDN - DIN, where TDN is total

dissolved nitrogen after UV oxidation and DIN is the sum of the

dissolved inorganic nitrogen species before UV oxidation.  As an

alternative to the UV oxidation method, Walsh (1989) has described

a high-temperature (1100oC) combustion method which has been applied

to open ocean samples collected in the North Pacific Ocean.  No

significant differences were observed between these two procedures

(Walsh, 1989).

2.  Precautions

Contamination is the primary concern with these samples.  This

is particularly true with samples collected from the euphotic zone,

where inorganic nutrient concentrations are extremely low (<0.2 µM).

In order to avoid contamination, sample bottles must be meticulously

cleaned with dilute HCl and rinsed with deionized distilled water

(DDW) before use.  Samples are stored frozen until analysis, generally

within 1-2 weeks of sample collection.

3.  Sampling Collection and Storage

NOTE:  The currently held "dogma" in the oceanographic literature is

that seawater samples must be processed fresh and on board ship for

high-precision, low-level inorganic nutrient analyses (Morse et al.,

1982; Venrick and Hayward, 1985).  However, extensive results from

automated analyses of nutrients in tropical seawaters (Ryle et al.,

1981) and the VERTEX program (D. Karl and S. Moore, unpubl. results)

which included direct comparisons of [NO3+NO2], PO4 and SiO3

determinations in fresh vs. frozen samples would suggest otherwise.

Provided that caution is taken to collect and store the samples in an

environment free of potential contamination, we found no significant

treatment effect.  A similar conclusion was presented by Walsh et al.

(manuscript) following the analysis of a wide range of seawater

samples that were either analyzed fresh or frozen and stored for

varying periods of time.  They conclude that, "Despite published and

voiced opinions to the contrary, there appears to be no adequate

basis either from the literature or from our experiments for

across-the-board dismissal of high-precision nutrient analyses

undertaken on properly stored seawater samples."  As we are not

able to take our autoanalyzer to sea on the HOT program cruises,

we have focussed our attention on maintaining a contamination-free

environment during collection and storage of nutrient samples.

3.1. Sample collection

3.1.1. Rinse the nutrient sample bottle (acid-washed, 125 ml

polyethylene bottle) 3 times before filling.  Fill to

approximately 2/3 full, tighten cap and freeze.

3.1.2. Record cruise, cast and Niskin bottle number on the bottle

and data sheet.

4.  Sample Analysis

4.1. Standard procedure

Currently, GOFS and WOCE nutrients collected during the Hawaii

Ocean Time Series cruises are analyzed by the Hawaii Institute

of Marine Biology Analytical Facility.  Mr. Ted Walsh has

provided us with the following procedure for the analysis of

dissolved N.

4.1.1. Nitrate (NO3-) plus Nitrite (NO2-)

[NO3-+NO2-] analyses are performed on a four-channel

Technicon Autoanalyzer IIR continuous flow system.  The

automated wet chemistries generally follow the standard

methods of seawater analysis as given by Technicon (1977),

which involve:  (1) reduction of nitrate to nitrite using a

copperized cadmium reduction column, (2) reaction of nitrite

with sulfanilamide for diazotization and (3) coupling with

N-1-napthylethylenediamine dihydrochloride (NED) forming a

purple azo dye (Armstrong et al., 1967).  The dye absorbance

is read through a 15 mm pathlength flowcell at 550 nm.  The

reduction column is looped in line using a Hamilton 4-way

valve and can be by-passed for nitrite analysis only.  Both

nitrate and nitrite standards are run to check column

efficiency.  If speciation is desired, nitrite is determined

separately by omitting the reduction step.  Nitrate is

calculated by difference.

4.1.2. Dissolved Organic Nitrogen (DON)

The method for DON involves initial UV digestion of a seawater

sample followed by autoanalysis of the digestion products for

[nitrate+nitrite], as above, and ammonium using the Berthelot

(indophenol) method.  The modified photooxidation technique

(Armstrong et al., 1966) utilizes a 24 hour irradiation.

Details are given in Walsh (1989).  Periodic calibration

checks of the UV lamp efficiency are made using a dissolved

organic nitrogen (2,2-bypyridyl) standard.   As a general rule,

the UV lamp is replaced after approximately 700-800 hr of use.

DON is calculated by difference between the sum of [nitrate+

nitrite+ammonium] before and after UV treatment.

4.2. Low-level procedures for nitrate (NO3-) and nitrite (NO2-) by

chemiluminescence

Nanomolar quantitites of nitrate and nitrite are routinely analyzed

using the chemiluminescence techniques of Cox (1980) and Garside

(1982).  This technique relies on the wet chemical reduction of

nitrate and nitrite to nitric oxide in a highly acidic solution

of sulfuric acid, ferrous ammonium sulfate, and ammonium molybdate.

The reduced nitric oxide is carried by an inert carrier gas (argon),

scrubbed of acid and water vapors in a cold finger filled with 6 M

sodium hydroxide solution followed by an anhydrous sodium carbonate

filled drying tube.  The gas stream is then routed through a

membrane dryer and the nitric oxide is combined with ozone and

simultaneously exposed to a photomultiplier tube.  The nitric

oxide is further oxidized to excited nitrogen dioxide emitting a

photon as it returns to ground state, and the emitted light is

detected by the photomultiplier.

4.2.1. [NO3- + NO2-] determinations

[NO3- + NO2-] analyses are performed on an Antek model 720

nitrogen oxide analyzer.  Ten ml of concentrated sulfuric

acid (36 N) plus 2 ml each of ferrous ammonium sulfate (4%,

w/v) followed by ammonium molybdate (2%, w/v) are dispensed

into the reaction tube.  The reaction tube is inserted into

the carrier gas line and the reagents degassed.  A 10 ml

water sample or standard solution is introduced into the

reaction tube with a syringe through a septum fitted to the

side arm of the reaction tube.  Total reaction light emission

is recorded using an automated integrator.

4.2.2. NO3- determinations

Sample analysis is conducted, as above (Chapter 7, section

4.2.1), except that the sample (10 ml) is pretreated with

0.2 ml of sulfanilamide prior to injection into the reaction

tube.  Since sulfanilamide quantitatively binds nitrite, the

integral is the result of nitrate only.  Therefore, nitrite

can be obtained from the difference of the two analyses.

5.  Calibration, Data Reduction and Calculations

5.1. Calibration stocks and regression standards

The calibration of dissolved inorganic nutrients in the autoanalysis

of seawater samples is performed using standard solutions containing

N, P and Si.  A nutrient stock solution is prepared by dissolving

dried (65oC, 72 hours) analytical grade reagent chemicals with

distilled-deionized water in 1 liter glass volumetric flasks

containing 1 ml of chloroform.  Once dissolved, this stock solution

is immediately transferred into 1 liter amber polypropylene bottles

and stored at 4oC.  The reagent chemicals and concentrations are:

phosphate (KH2PO4, 1 mM), nitrate (KNO3, 5 mM) and silica (Na2SiF6,

4 mM).

Working standards are prepared daily by volumetric dilutions of the

stock using glass pipettes and a plastic (polymethylpentene; PMP)

volumetric flask.  All pipettes and PMP flasks are acid-washed

(1 M HCl) and gravimetrically calibrated prior to use.  The daily

regression standards are prepared by diluting the working standard

with low nutrient natural seawater diluent (SWDIL).  The SWDIL is

filtered open ocean surface seawater that is stored in a carboy at

room temperature.  By using this technique all standards are

matrix-matched with the seawater samples and any cross-nutrient

interference effect should be accounted for.

Cross-nutrient interference and reagent contamination was evaluated

by preparing separate solutions, as above, but with one of the three

standards omitted.  Only phosphate showed a slightly measurable

increase (+0.014 µM) in the presence of 40 µM-NO3 and 160 µM-Si.

The linear regressions of the standards were applied to all seawater

sample peaks for calculating each batch of cruise samples.  Typical

correlations produced r2 values that were between 0.9999 and 0.99999.

5.2. Blank corrections

All seawater standard absorbance peaks were corrected for the

absorbance of the seawater diluent (SWDIL).  All seawater sample

peaks were corrected for the refractive index absorbance for each

unique nutrient detection system.  The refractive index corrections

(in apparent uM units) ranged from approximately 0.13 (for P), 0.23

(for N) to 2.41 (for Si), and represent the increase in absorbance

that is due strictly to the presence of dissolved salts in seawater

when compared to the distilled-deionized water baseline.  These

corrections are made running seawater (35 o/oo salinity) through

the autoanalyzer with DDW only in reagent lines.  The Levor

surfactant used routinely in the phosphate channel was omitted

from the DDW lines during the refractive index measurement because

Levor reacts erratically with seawater in the absence of the acidic

color reagent.

6.  Accuracy and Precision

The detection limit for nitrate plus nitrite is approximately 0.03

uM with a coefficient of variation for field-collected replicates

of 0.3%.  The detection limit for DON is 0.05 with a coefficient of

variation of 4%.

7.  Equipment/Supplies

Niskin bottles and rosette/CTD unit

acid-washed, 125 ml polyethylene bottles

Autoanalyzer (Technicon Corp.) and accessories

UV oxidizer unit

nitrogen oxide analyzer (Antek model #720, operated in vacuum mode)

reaction tube, cold finger and drying tube glassware array

8.  Reagents

glass distilled deionized water (DDW)

1 M HCl for cleaning

concentrated H2SO4 (36 N)

Ammonium Chloride Reagent:  Dissolve 10 g of ammonium chloride in DDW,

adjust pH to 8.5 with concentrated ammonium hydroxide and dilute to

1 liter.  Add 0.5 ml Brij-35 (Technicon No. T21-0110).

Color Reagent:  To approximately 1500 ml of distilled water, add 200

ml of concentrated phosphoric acid and 20 g of sulfanilamide.

Dissolve completely (heat if necessary).  Add 1 g of N-1-naphthyl-

ethylenediamine dihydrochloride and dissolve.  Dilute to 2 liters.

Add 1.0 ml of Brij-35. Store in a cold, dark place.  Stability: one

month.

Cadmium Powder (Technicon No. T11-5063):  Use coarse cadmium powder

(99% pure).  Rinse the powder once or twice with a small quantity

of clean diethyl ether and 1 M HCl  to remove grease and dirt.

Follow with a DDW rinse.  Allow the metal to air-dry and store in

a well-stoppered bottle.

Ferrous Ammonium Sulfate (4% w/v):  Dissolve 4 g of reagent grade

ferrous ammonium sulfate in 100 ml DDW.  Prepare fresh daily.

Ammonium molybdate (2% w/v):  Dissolve 2 g reagent grade ammonium

molybdate in 100 ml DDW.  Prepare fresh daily.

Sulfanilamide (1% in 10% HCl).  Dissolve 1 g reagent grade

sulfanilamide in 100 ml of 10% HCl.

Sodium hydroxide (6 M):  Dissolve 240 g of reagent grade sodium

hydroxide and make up to 1 liter with DDW.

Preparation of Reduction Column:  See Technicon Industrial System,

1977.

Stock Standard (1000 µM):  Dissolve 0.101 g of potassium nitrate

in DDW and dilute to 1 liter.  Store in a dark bottle.  Add 1 ml

of chloroform as a preservative.

Working Standard (50 µM):  Dilute 5 ml of stock standard in a

volumetric flask with DDW or seawater diluent.

9.  References

Armstrong, F. A. J., C. R. Sterns and J. D. H. Strickland.  1967.

The measurement of upwelling and subsequent biological processes

by means of the Technicon Autoanalyzer and associated equipment.

Deep-Sea Research, 14, 381-389.

Armstrong, F. A. J., P. M. Williams and J. D. H. Strickland.

1966.  Photo-oxidation of organic matter in seawater by ultraviolet

radiation, analytical and other applications. Nature, 211 481-483.

Cox, R. D.  1980.  Determination of nitrate at the parts per

billion level by chemiluminescence. Analytical Chemistry, 52,

332-335.

Garside, C.  1982.  A chemiluminescent technique for the

determination of nanomolar concentrations of nitrate and nitrite

in seawater. Marine Chemistry, 11, 159-167.

Grasshoff, K., M. Ehrhardt, and K. Kremling.  1983.  Methods

of Seawater Analysis.  Verlag Chemie.

Morse, J. W., M. Hunt, J. Zullig, A. Mucci, and T. Mendez.  1982.

A comparison of techniques for preserving dissolved nutrients in

open ocean seawater samples. Ocean Science and Engineering, 7,

75-106.

Ryle, V. D., H. R. Mueller, and P. Gentien.  1981.  Automated

analysis of nutrients in tropical sea waters. Australian Institute

of Marine Science Technical Bulletin, Oceanography Series No. 3.

Standard Methods for the Examination of Water and Wastewater,

15th Edition.

Strickland, J. D. H. and T. R. Parsons.  1972.  A Practical

Handbook of Seawater Analysis.  Fisheries Research Board of

Canada, 167.

Technicon Industrial Systems.  1977.  Nitrate & Nitrite in

Water and Seawater.  Autoanalyzer II R Industrial Method No.

155-71W.  W. Tarrytown, New York 10591.

Venrick, E. L. and T. L. Hayward.  1985.  Evaluation of some

techniques for preserving nutrients in stored seawater samples.

CalCOFI Report, 26, 160-168.

Walsh, T. W.  1989.  Total dissolved nitrogen in seawater:

A new high temperature combustion method and a comparison to

photo-oxidation. Marine Chemistry, 26, 295-311.

POSTSCRIPT:  During year 1 of the HOT program, we routinely measured

NH4+ concentrations using the standard Berthelot (indophenol) method.

Concentrations of NH4+ were consistently at, or below, our detection

limits (<0.05 µM) throughout the water column.  Although a new method

has been described for low-level determinations of NH4+ in seawater

(Brzezinski, 1988, Limnology and Oceanography, 33, 1176-1182), we have

not yet successfully adapted this procedure for our routine determinations.

Until that method, or a suitable alternative, is available we have decided

to delete NH4+ measurements from our list of "standard procedures."

### ORTHOPHOSPHATE AND DISSOLVED ORGANIC PHOSPHORUS1

_________________________________________________________________

SUMMARY:  Seawater is collected from known depths using

CTD- rosette sampling protocols.  Subsamples are drawn and

stored in acid-washed polyethylene bottles.  Soluble

reactive phosphorus (SRP) is measured spectrophotometrically

following the formation of phosphomolybdic acid.  Total

dissolved phosphorus (TDP) is measured in a separate

sample after exposure to ultraviolet (UV) light and dissolved

organic phosphorus (DOP) is estimated by difference.

_________________________________________________________________

1.  Principle

Phosphorus (P) is one of several macronutrients required for the

growth of marine organisms.  In open ocean marine ecosystems, P is

often present in low and, perhaps, limiting concentrations for

microalgal and bacterial populations.  Therefore, P is a central

element in oceanic biogeochemical cycles.

In seawater, inorganic phosphorus (also referred to as

orthophosphate or soluble reactive phosphorus, SRP) occurs chiefly

as ions of HPO42-, with a small percentage present as PO43-.

Dissolved organic phosphorus (DOP) exists in a variety of forms

(primarily P-esters) which result from excretion, decomposition,

death and autolysis.

In this analytical procedure, we make direct measurements of

SRP and total dissolved phosphorus (TDP).  The latter includes all

organic and inorganic phosphorus compounds.  Bound phophorus is

released from organic matter by ultraviolet light (UV) oxidation

and the liberated orthophosphate is reacted with an acidified

molydate reagent and potassium antimonyl tartrate.  The resulting

compound, a heteropoly acid (phosphomolybdic acid), is reduced to

the intensely colored molybdenum blue by ascorbic acid and measured

spectrophotometrically.  SRP samples are prepared in the same manner

as TDP samples, but without the prior UV oxidation step.  DOP is

calculated by difference (i.e., TDP-SRP).

2.  Precautions

Contamination is the primary concern with P determinations.

This is particularly true with samples collected from the euphotic

zone, where SRP concentrations are extremely low (<0.2 µM).  In

order to avoid contamination, sample bottles must be meticulously

cleaned with dilute HCl and rinsed with deionized distilled water

(DDW) before use.

3.  Sample Collection and Storage

3.1. Sample collection (also see "NOTE" in Chapter 7, section 3)

3.1.1. Rinse the nutrient sample bottle (acid-washed, 125 ml

polyethylene bottle) 3 times before filling.  Fill to

approximately 2/3 full, tighten cap and freeze.

3.1.2. Record cruise, cast and Niskin bottle number on the

bottle and data sheet.

4.  Sample Analysis

Currently, GOFS and WOCE nutrient samples collected during

the Hawaii Ocean Time Series cruises are analyzed by the Hawaii

Institute of Marine Biology Analytical Facility.  Mr. Ted Walsh has

provided us with the following procedure for the analysis of phosphate.

Phosphorus analyses are performed on a four-channel Technicon

Autoanalyzer II continuous flow system.  The automated wet chemistries

generally follow the standard methods of seawater analysis as given

by Technicon (1973).  Slight modifications have been incorporated to

achieve the optimum range and sensitivity for each nutrient at

concentration levels specific for the Station ALOHA seawaters.

4.1. Soluble Reactive Phosphorus (SRP)

This method employs a single color reagent consisting of an

acidified solution of ammonium molybdate, ascorbic acid and

antimony-tartrate.  The blue phosphomolybdenum complex formed

is read colorimetrically through a 50 mm pathlength flowcell

at 880 nm (Murphy and Riley, 1962).  The specific automated

method used is described in Technicon (1973) with the following

modifications:

sample pump tube size is increased to allow for a flow rate

of 0.8 ml min-1, and the color reagent

solution concentration is diluted by a factor of 2.

4.2. Total Dissolved Phosphorus (TDP)

The method for TDP involves initial UV digestion of a seawater

sample followed by autoanalysis of the digestion product (SRP;

see Chapter 8, section 4.1).  The modified photoxidation technique

(Armstrong et al., 1966) utilizes a 2 hour UV irradiation period.

Exact details of the photoxidation unit are described in Walsh

(1989).  Periodic calibration checks of the UV lamp efficiency

are made using dissolved organic phosphorus (β-glycerol-

phosphate) standards.  As a general rule, the UV lamp is replaced

after approximately 700-800 hr of use.

4.3. Low-level procedures

We have recently developed a method for the determination of P

dissolved in seawater at low concentrations (<0.1 µM; Karl and

Tien, in preparation).  The method is based upon the quantitative

removal of SRP by co-precipitation with magnesium hydroxide which

is formed by the addition of high purity sodium hydroxide to

selected seawater samples.  The precipitate is collected by

centrifugation, washed with a solution of weak sodium hydroxide

(0.01 M) and then dissolved in a known volume of high purity

hydrochloric acid.  At this point, the concentrated samples are

treated as above (see Chapter 8, section 4.1) for the determination

of SRP.  The detection limit is determined by the volume of

seawater initially used (50-250 ml for our routine assays), but

P concentrations as low as 5 nM can be easily and reproducibly

detected.  All low level P determinations are corrected for

arsenic interference by thiosulfate reduction of dissolved

arsenate (Johnson, 1971).

5.  Calibration, Data Reduction and Calculations

5.1. Calibration stocks and regression standards

The calibration of dissolved inorganic nutrients in the

autoanalysis of seawater samples is performed using standard

solutions containing N, P and Si.  A nutrient stock solution

is prepared by dissolving dried (65oC, 72 hours) analytical

grade reagent chemicals with distilled-deionized water in 1

liter glass volumetric flasks containing 1 ml of chloroform.

Once dissolved, this stock solution is immediately transferred

into 1 liter amber  polypropylene bottles and stored at 4oC.

The reagent chemicals and concentrations are:  phosphate (KH2PO4,

1 mM), nitrate (KNO3, 5 mM) and silica (Na2SiF6, 4 mM).

Working standards are prepared daily by volumetric dilutions

of the stock using glass pipettes and a plastic (polymethyl-

pentene; PMP) volumetric flask.  All pipettes and PMP flasks

are acid-washed (1M HCl) and gravimetrically calibrated prior

to use.  The daily regression standards are prepared by diluting

the working standard with low nutrient natural seawater diluent

(SWDIL).  The SWDIL is filtered open ocean surface seawater that

is stored in a carboy at room temperature.  By using this technique

all standards are matrix-matched with the seawater samples and any

cross-nutrient interference effect should be accounted for.

Cross-nutrient interference and reagent contamination was evaluated

by preparing separate solutions, as above, but with one of the three

standards omitted.  Only phosphate showed a slightly measurable

increase (+0.014 µM) in the presence of 40 µM-NO3 and 160 µM-Si.

The linear regressions of the standards were applied to all seawater

sample peaks for calculating each batch of cruise samples.  Typical

correlations produced r2 values that were between 0.9999 and 0.99999.

5.2. Blank corrections

All seawater standard absorbance peaks were corrected for the

absorbance of the seawater diluent (SWDIL).  All seawater sample

peaks were corrected for the refractive index absorbance for each

unique nutrient detection system.  The refractive index corrections

(in apparent µM units) ranged from approximtely 0.13 (for P), 0.23

(for N) to 2.41 (for Si), and represent the increase in absorbance

that is due strictly to the presence of dissolved salts in seawater

when compared to the distilled-deionized water baseline.  These

corrections are made by running sampling seawater (35 o/oo salinity)

through the autoanalyzer with DDW only in reagent lines and also with

all reagents except the color producing reagent.  The Levor surfactant

used routinely in the phosphate channel was omitted from the DDW lines

during the refractive index measurement.

6.  Accuracy and Precision

The detection limit for phosphorus is approximately 0.02 µM with a

coefficient of variation for field-collected replicates of 0.3%  For DOP

the detection limit is 0.02 µM with a coefficient of variation of 1%.

7.  Equipment/Supplies

Niskin bottles and rosette/CTD unit

acid-washed, 125 ml polyethylene sample bottles

Autoanalyzer (Technicon Corp.) and accessories

8.  Reagents

glass distilled deionized water (DDW)

HCl (1M) for cleaning

concentrated H2SO4

ultrapure NaOH and HCl for low-level P determinations

Ammonium molybdate solution: Dissolve 40 g of ACS grade ammonium

paramolybdate [(NH4)6M O7O24 . 4H2O into 800 ml DDW and dilute

to 1 liter in a volumetric flask.  Store  in plastic bottle in

the dark.  Solution is stable indefinitely.

Ascorbic acid solution (Prepare fresh):  Dissolve 1.8 g of ACS

ascorbic acid into 100 ml DDW (1.8% wt/vol).

Antimony potassium tartrate solution:  Dissolve 3.0 g of ACS

antimony potassium tartrate (C8H4K2O12Sb2 . 3H2O) into 800 ml

DDW and dilute to 1 liter in a volumetric flask.  Solution is

stable for several months.

Mixed reagent (Prepare fresh):  Mix together in the following

order 15 ml ammonium molybdate, 50 ml 5 N sulfuric acid, 30 ml

1.8% ascorbic acid and 5 ml potassium antimony tartrate.

Stock phosphate standard solution (1 mM):  Dissolve 0.1361 g of

dry (65oC for 72 hours) potassium phosphate monobasic (KH2PO4)

into 800 ml of DDW and dilute to 1 liter in a volumetric flask.

Store in a dark bottle with 1 ml of chloroform.

Working phosphate standard (40 µM):  Dilute 4.0 ml of the stock

standard to 100 ml (use volumetric flask).  Then dilute the

working standard to prepare a series of standards to cover

a range of P concentrations.

9.  References

Armstrong, F. A. J., P. M. Williams and J. D. H. Strickland.  1966.

Photo-oxidation of organic matter in seawater by ultraviolet radiation,

analytical and other applications. Nature, 211, 481-483.

Grasshoff, K., M. Ehrhardt and K. Kremling.  1983.  Methods of Seawater

Analysis.  Verlag Chemie.

Johnson, D. L.  1971.  Simultaneous determination of arsenate and

phosphate in natural waters.  Environmental Science and Technology,

5, 411-414.

Murphy, J. and J. P. Riley.  1962.  A modified single solution

method for the determination of phosphate in natural waters.

Analytica Chimica Acta, 27, 30.

Strickland, J. D. H. and T. R. Parsons.  1972.  A Practical

Handbook of Seawater Analysis.  Fisheries Research Board of Canada,

167.

Technicon Industrial Systems.  1973.  Orthophosphate in water and

seawater.  Autoanalyzer IIR Industrial Method No. 155-71W.  W.

Tarrytown, New York 10591.

Walsh, T. W.  1989.  Total dissolved nitrogen in seawater: A new

high temperature combustion method and a comparison to photo-

oxidation.  Marine Chemistry, 26, 295-311.

Walsh, T. W., R. E. Young and S. V. Smith.  Seawater nutrient

storage and analysis.  Manuscript.

### SOLUBLE REACTIVE SILICA1

_________________________________________________________________

SUMMARY:  Seawater is collected from known depths using

CTD- rosette sampling protocols.  Subsamples are drawn and

stored in acid-washed polyethylene bottles.  Soluble

reactive silica is measured spectrophotometrically following

the formation of silico- molybdic acid from the reaction of

ammonium molybdate and silica at acidic pH.

_________________________________________________________________

1.  Principle

Silicon is the second most abundant element in the Earth's crust.

Subaerial weathering processes produce orthosilicic acid Si(OH)4 which

eventually is deposited in the oceans.  In seawater, various groups

of organisms (diatoms, radiolarians, silicoflagellates, sponges and

some fungi) utilize silica primarily as a structural component.

The analysis of soluble reactive silica is based upon the formation

of yellow silicomolybdic acid from the reaction of ammonium molybdate

and silica at low pH.  Phosphate also reacts to produce a positive

interference due to the formation of molybdophosphoric acid.  The

addition of oxalic acid eliminates the phosphate interference.  The

sensitivity of the analysis is increased by a further reduction of

the yellow silicomolybdic acid using ascorbic acid, in order to

produce "molybdenum blue".

2.  Precautions

Contamination is the primary concern with these samples.  This

is particularly true with samples collected from the euphotic zone,

where inorganic nutrient concentrations are extremely low.  In order

to avoid contamination, all sample bottles must be meticulously

cleaned with dilute HCl and rinsed with deionized distilled water

(DDW) before use.  It is important to realize that silica is leached

from glass at seawater pH.  Therefore, plastic should be used for

sample handling and storage.  Finally, special care must be taken

when performing dissolved Si analyses on frozen seawater samples

(Macdonald et al., 1986).

3.  Sample Collection and Storage (also see "NOTE" in Chapter 7,

section 3)

3.1. Rinse the nutrient sample bottle (acid-washed, 125 ml

polyethylene bottle) 3 times before filling.  Fill to

approximately 2/3 full, tighten cap and freeze.

3.2. Record cruise, cast and Niskin bottle number on the bottle

and data sheet.

4.  Sample Analysis

Currently, GOFS and WOCE nutrient samples collected during

the Hawaii Ocean Time Series cruises are analyzed by the Hawaii

Institute of Marine Biology Analytical Facility.  Mr. Ted Walsh has

provided us with the following procedure for the analysis of

reactive Si.

Si analyses are performed on a four-channel Technicon Autoanalyzer

II continuous flow system.  The automated wet chemistries generally

follow the standard methods of seawater analysisas given by Technicon

(1977).  This method involves a reaction of the sample with oxalic acid,

molybdate, and ascorbic acid.  The absorbance is read at 660 nm using

a 15 mm pathlength flowcell.

5.  Calibration, Data Reduction and Calculations

5.1. Calibration stocks and regression standards

The calibration of dissolved inorganic nutrients in the

autoanalysis of seawater samples is performed using standard

solutions containing N, P and Si.  Nutrient stock solution "A"

is prepared by dissolving dried (65oC, 72 hours) analytical

grade reagent chemicals with distilled-deionized water in 1

liter glass volumetric flasks containing 1 ml of chloroform.

Once dissolved, this stock solution is immediately transferred

into 1 liter amber polypropylene bottles and stored at 4oC.

The reagent chemicals and concentrations are:  phosphate

(KH2PO4, 1 mM), nitrate (KNO3, 5 mM) and silica (Na2SiF6, 4 mM).

Working standards are prepared daily by volumetric dilutions of

the stock using glass pipettes and a plastic (polymethylpentene;

PMP) volumetric flask.  All pipettes and PMP flasks are acid-washed

(1 M HCl) and gravimetrically calibrated prior to use.  The daily

regression standards are prepared by diluting the working standard

with low nutrient natural seawater diluent (SWDIL).  The SWDIL is

filtered open ocean surface seawater that is stored in a carboy

at room temperature.  By using this technique all standards are

matrix-matched with the seawater samples and any cross-nutrient

interference effect should be accounted for.

Cross-nutrient interference and reagent contamination was evaluated

by preparing separate solutions, as above, but with one of the

three standards omitted.  Only phosphate showed a slightly

measurable increase (+0.014 µM) in the presence of 40 µM-NO3

and 160 µM-Si.  The linear regressions of the standards were

applied to all seawater sample peaks for calculating each batch

of cruise samples.  Typical correlations produced r2 values that

were between 0.9999 and 0.99999.

5.2. Blank corrections

All seawater standard absorbance peaks were corrected for the

absorbance of the seawater diluent (SWDIL).  All seawater sample

peaks were corrected for the refractive index absorbance for each

unique nutrient detection system.  The refractive index corrections

(in apparent µM units) ranged from approximately 0.13 (for P), 0.23

(for N) to 2.41 (for Si), and represent the increase in absorbance

that is due strictly to the presence of dissolved salts in seawater

when compared to the distilled-deionized water baseline.  These

corrections were measured by sampling seawater (35 o/oo salinity)

with DDW only in reagent lines and also with all reagents except

the color producing reagent.  The Levor surfactant used routinely

in the phosphate channel was omitted from the DDW lines during the

refractive index measurement.

6.  Accuracy and Precision

The detection limit for dissolved Si is approximately 0.3 µM.  The

coefficient of variation of field-collected replicates is 6%.

7.  Equipment/Supplies

Niskin bottles and rosette/CTD unit

acid-washed, 125 ml polyethylene bottles

Autoanalyzer (Technicon Corp.) and accessories

8.  Reagents

Because Si is the principle component of glass, all solutions

should be made up and contained in plastic.  Glass distilled deionized

water will have minimal silica leaching due to the low pH of distilled

water.

Glass distilled deionized water (DDW)

Ammonium molybdate solution:  Dissolve 10 g of ammonium molybdate

into 1 liter of sulfuric acid (0.1 N).  Filter and store in an

amber plastic container.

Oxalic acid (0.56 M):  Dissolve 50 g of oxalic acid into 900 ml

of DDW and dilute to 1 liter.

Ascorbic acid solution (1.76% wt/vol):  Dissolve 17.6 g of ACS

quality ascorbic acid in 500 ml of DDW containing 50 ml of

acetone.  Mix and dilute to 1 liter with DDW.  Add 0.5 ml

of Levor V per liter of reagent.

9.  References

Grasshoff, K., M. Ehrhardt, and K. Kremling.  1983.  Methods

of Seawater Analysis.  Verlag Chemie.

Macdonald, R. W., F. A. McLaughlin and C. S. Wong.  1986.

The storage of reactive silicate samples by freezing. Limnology

and Oceanography, 31, 1139-1142.

Standard Methods for the Examination of Water and Wastewater,

15th Edition.

Strickland, J. D. H. and T. R. Parsons.  1972.  A Practical

Handbook of Seawater Analysis. Fisheries Research Board of

Canada, 167 p.

Technicon Industrial Systems.  1977.  Silicates in Water and

Seawater.  Autoanalyzer II R Industrial Method  No. 155-71W.

W. Tarrytown, New York 10591.

### PARTICULATE CARBON AND NITROGEN1

_________________________________________________________________

SUMMARY: Seawater samples are collected at discrete depths

in 12-liter Niskin bottles.  The water samples are prefiltered

(202 µm) and transferred to specially designed, precalibrated

filtration bottles, pressure filtered through combusted GF/F

filters and stored frozen for subsequent analysis.  In the

laboratory, the filters are dried and analyzed for C and N

using a Perkin-Elmer model 2400 analyzer.

_________________________________________________________________

1.  Principle

Particulate carbon (PC) and particulate nitrogen (PN), including

both inorganic and organic forms, are present in seawater primarily

as by-products of biological activity.  Living organisms also contribute

a variable amount (<1% in deep waters to >50% in the euphotic zone)

to the total particulate carbon and nitrogen content of seawater.

Both PC and PN can be measured using commercially-available

instruments which detect the gaseous by-products of high-temperature

combustion.  The Perkin-Elmer model 2400 CHN analyzer used in the

HOT-GOFS project, combines the classical Pregal and Dumas methods

for the determination of carbon and nitrogen, respectively.  The

samples are combusted in a pure oxygen environment, the gases mixed

and separated by frontal chromatography.  The separated gases are

determined stepwise by a thermal conductivity detector.

2.  Precautions

Care must be taken to ensure that contamination with PC and PN

is avoided.  Samples should, therefore, be kept away from paper,

wood, food or other carbon-containing compounds.  The filtration

system that is set up to collect these samples is constructed of

plastic.  Plastic contains carbon and, therefore, is a potential

source of contamination.  The carboys and tubing used in this system

must be kept clean and free of abrading surfaces.

3.  Sampling, Filtration and Storage

3.1. Seawater samples are collected in 12-liter Niskin bottles and

transferred directly to acid-cleaned filtration bottles.  The

samples are transferred via Tygon tubing which incorporates an

in-line 202 µm Nitex screen prefilter to remove zooplankton or

any other rare particles which might otherwise affect the

precision of the estimate.  The filtration bottles are 4- and

12-liter polyethylene aspirator bottles fitted with a valve

assembly and tubing connection.

Once the rosette sampler is on deck, the vent valve of each

Niskin bottle is opened and one end of the drawing tube is

attached to the outflow spigot of the sampling bottle and the

other end to the tubing connector on the cap of the filtration

bottle.  Particular attention is paid to the orientation of

the in-line screened drawing tube (the shorter, larger bore

section is attached to the Niskin bottle).  The filtration

bottle valve is opened and 100-200 ml is run through the

transfer tube and valve assembly to rinse the sampling bottle.

The bottle and cap are rinsed 3 times in this fashion.  During

the rinsing and filling operation, the filtration bottle cap

valve is used to control and direct the sample flow.  After

rinsing,the cap is placed on the bottle mouth (without

tightening), the valve is opened and the polyethylene bottled

is filled to the calibration mark.

3.2. After filling, the filtration bottles are inverted and placed

into the filtration rack.  The contents are then pressure

filtered (4-7 psi nitrogen gas) through combusted in-line 25

mm GF/F filters.

3.3. Following filtration, clean forceps are used to transfer each

filter to a clean plastic petri dish containing a 3 cm square

piece of combusted foil.  The sample is labeled and stored

frozen (-20oC).  Usually only a few ml or less of water remains

in the carboy after filtration; this residual water is ignored

in assessing the volume filtered unless it appears to be

significant compared to the volume of the carboy.  All appropriate

data are entered on the data sheet.

4.  Analysis

Standard procedures for instrument warm-up are followed.  Primary

PC/PN standards are prepared using acetanilide (C8H9NO; mol. wt. =

135.16).  Standards are made to encompass the range 20-500 ug/sample.

Blanks are prepared by analyzing combusted GF/F "field filters" and

the mean value subtracted from the sample value.  Typical ranges in

analytical blanks, expressed as percentages of the total signal

produced by a sample in the course of a given analysis, are 5-17%

for both PC and PN.

5.  Data Reduction and Calculations

External standard data are used to prepare a standard curve of C

(or N) versus corrected signal counts, and linear regression statistics

are calculated.  An EXCEL spreadsheet is used to calculate PC and PN

(µg l-1) for each sample based on standard curve, corrected signal

counts and volume of seawater filtered.

6.  Precision and Accuracy

Replicate samples are routinely analyzed to estimate the precision

of our PC and PN analyses.  The average coefficients of variation for

such replicate analyses are 12% and 10%, respectively.  Accuracy is

estimated from determinations of the C and N contents of reference

standards analyzed along with samples during each analytical run.

This accuracy, expressed as the mean percent difference between the

analytical determination and the known C and N contents of the

standards, is found to be 6% for PC and 13% for PN.  Note that the

estimates of accuracy apply only to standards containing >10 µg N

and >70 µg C.  Below these levels, we have found the performance of

the PE-2400 to be suspect.  Therefore sample volumes large enough

to keep above this lower limit are used.

7.  Equipment/Supplies

PE-2400 (or equivalent) Carbon/Nitrogen analyzer with integrator

Cahn (or equivalent) electronic balance

combusted 2.5 cm GF/F filters (450oC, 4 hours)

ethanol-cleaned forceps for handling filters

combusted foil (450oC, 4 hours)

aluminum foil for covering work area

clean petri dishes

low pressure filtration apparatus (4-7 psi)

valved polyethylene aspirator bottles

8.  Reagents

acetanilide standard

9.  References

Sharp, J. H.  1974.  Improved analysis for particulate organic

carbon and nitrogen from seawater. Limnology and Oceanography, 19,

984-989.

### PARTICULATE PHOSPHORUS1

_________________________________________________________________

SUMMARY: Seawater samples are collected at discrete

depths in 12-liter Niskin bottles.  The water samples are

prefiltered (202 µm) and transferred to specially designed,

precalibratedfiltration bottles, pressure filtered through

combusted acid-rinsed GF/F filters and stored frozen for

subsequent analysis.  In the laboratory, the filters are

combusted at 450-500oC and the concentration of the resulting

inorganic phosphorus is determined by colorimetric analysis.

_________________________________________________________________

1.  Principle

The procedure presented here is a modification of one used by the

Hawaii Institute of Marine Biology Analytical Services laboratory at

the University of Hawaii.  It is a method pioneered and used by soil

scientists and marine chemists for particulates which can be homogenized

into a fine powder.

The method relies on the release of organically-bound phosphorus

compounds as orthophosphate, by high temperature combustion at 450-500oC.

The orthophosphate is then extracted with 0.5 N HCl at 90oC.  The

liberated orthophosphate is reacted with a mixed reagent of molybdic acid,

ascorbic acid and trivalent antimony to form phosphomolybdic acid.  This

heteropoly acid is then reduced to the colored molybdenum blue complex

by ascorbic acid and the solution is measured spectrophotometrically.

This procedure measures all forms of phosphorus which can be released

by combustion and acid hydrolysis.

2.  Precautions

Contamination is the primary problem to be avoided with these samples.

Combusted acid rinsed filters are used.  All sampling bottles, forceps,

tubing and filtration bottles are also acid rinsed.

3.  Sampling, Filtration and Storage

3.1. Seawater samples are collected in 12-liter Niskin bottles and

transferred directly to acid-cleaned filtration bottles.  The

samples are transferred via Tygon tubing which incorporates an

in-line 202 µm Nitex screen prefilter to remove zooplankton or

any other rare particles which might otherwise affect the

precision of the estimate.  The filtration bottles are 4- and

12-liter polyethylene aspirator bottles fitted with a valve

assembly and tubing connection.

Once the rosette/CTD unit is on deck, the vent valve from each

Niskin bottle is opened and one end of the drawing tube is

attached to the outflow spigot of the sampling bottle and the

other end to the tubing connector on the cap of the filtration

bottle.  Particular attention is paid to the orientation of the

in-line screened drawing tube (the shorter, larger bore section

is attached to the Niskin bottle).  The filtration bottle valve

is opened and 100-200 ml is run through the transfer tube and

valve assembly to rinse the sampling bottle. The bottle and cap

are rinsed 3 times in this fashion.  During the rinsing and

filling operation, the filtration bottle cap valve is used to

control and direct the sample flow.  After rinsing, the cap is

placed on the bottle mouth (without tightening), the valve is

opened and the polyethylene bottle is filled to the calibration mark.

3.2. After filling, the filtration bottles are inverted and placed

into the filtration rack.  The contents are then pressure

filtered (4-7 psi nitrogen gas) through combusted in-line

acid-rinsed 25 mm GF/F filters.

3.3. Following filtration, clean forceps are used to transfer each

filter to a combusted 16 x 100 mm glass test tube which is

then covered with a 3.5 cm square piece of combusted foil.

Each sample is labeled and stored frozen (-20oC).  Any water

remaining in the carboy is measured to calculate the volume

filtered.  This information and any other appropriate data

are entered on the data sheet.

4.  Blank Determination

Standards are corrected for reagent blanks while samples are

corrected for field filter blanks.

4.1. Standards:  At least 2 reagent blanks are prepared and individual

standard absorbances are corrected by the mean blank value.

4.2. Samples:  The mean absorbance from three field filter blanks,

stored and processed in the same manner as samples, are used

to correct individual sample absorbances for filter, reagent

and systematic procedural contamination.

5.  Analysis

5.1. Samples are combusted in 16 x 100 mm test tubes at 450oC for

4.5 hours in a muffle furnace.  The samples are then allowed

to cool and are immersed in 10 ml of 0.5 M HCl. The test tube

is then heated for 60 minutes at 90oC in a heating block.

5.2. The samples are allowed to cool, and centrifuged for 30 min at

2800 g.  5 ml of the supernatant is volumetrically subsampled

into another combusted acid washed 16 x 100 mm test tube.

5.3. One half ml of mixed reagent is added to samples and standards

and mixed thoroughly.  Color is developed for 60 minutes and

absorbance is read at 880 nm against a DDW reference.  Standards

are corrected for absorbance of reagent blanks and samples are

corrected for absorbance of filter or procedural blanks.

6.  Data Reduction and Calculations

6.1. Calculations

6.1.1. Calculate µmol l-1 of phosphate from standard curve using:

µmol l-1 = (x - b)/m

where:x = blank-corrected absorbance of sample

b = y intercept of regression line

m = slope of regression line

6.1.2. Calculate ug P-PO4 l-1 using:

µmol l-1 smpl x 30.97376 µg µmol-1 = µg P l-1 in sample extract

(µg P l-1 in extract) /(1000 ml l-1) = µg P ml-1

(µg P ml-1 x 20.0 ml of extract) / vol in liters filtered =

µg P-P04 l-1

7.  Equipment/Supplies

Niskin bottles and rosette/CTD unit

low pressure filtration apparatus (4-7 psi)

muffle furnace

heating block

combusted, acid-rinsed GF/F filters

acid-rinsed vacuum filtering assembly

spectrophotometer (Perkin-Elmer Lambda 3B) and 1-cm cuvette

combusted (450oC, 3 hours), acid-washed 16 x 100 mm glass tubes

8.  Reagents

Glass distilled deionized water (DDW)

0.5 M HCl

HCl for cleaning (1 M)

Ammonium molybdate solution: Dissolve 15 g of ACS grade ammonium

paramolybdate [(NH4)6 MO7O24 . 4H2O], in 500 ml DDW. Store in

plastic bottle in the dark.  Solution is stable indefinitely.

Sulfuric acid solution (5 N)

Ascorbic acid solution:  Dissolve 0.54 g of ACS ascorbic acid in

10 ml DDW (5.4% wt/vol).  Prepare fresh.

Potassium antimonyl-tartrate solution:  Dissolve 0.34 g of ACS

potassium antimonyl-tartrate (tartaremetic), in 250 ml DDW.

Solution is stable for many months.

Mixed reagent:  Mix together 10 ml ammonium molybdate, 25 ml 5 N

sulfuric acid, 10 ml 5.4% ascorbic acid and 5 ml potassium

antimony tartrate.  Prepare fresh.

Stock phosphate standard solution (1000 µM):  Dissolve 0.1361 g

of dry KH2PO4, in 1000 ml of DDW. Store in a dark bottle with

1 ml of chloroform.

Working phosphate standard (100 µM):  Dilute 10 ml of the stock

standard to 100 ml, using a volumetric flask.

Dilute the working standard to prepare a series of standards to

cover the range from 0.05 - 10 µM.

9.  References

Strickland, J. D. H. & T. R. Parsons.  1972.  A Practical Handbook

of Seawater Analysis.  Fisheries Research Board of Canada, 167 p.

### FLUOROMETRIC ANALYSIS OF CHLOROPHYLL A. AND PHAEOPIGMENTS1

_________________________________________________________________

SUMMARY: Concentrations of chlorophyll a and

phaeopigments, considered an index of the phytoplankton

standing stock, are measured by fluorometry after

particulate sample concentration by vacuum filtration

and extraction in 100% acetone.

_________________________________________________________________

1.  Principle

Historically chlorophyll a (chl a) has been used as a measure

of phytoplankton standing stock.  Also, when related to photosynthetic

carbon production, the concentration of chl a and distribution in the

water column allows one to estimate an index of the efficiency of

phytoplankton in harvesting light.

Fluorescence is produced by atoms or molecules that, under exited

state, return to their state of minimum energy (or ground state) by

losing energy in the form of light.  The absorption of blue light

by chlorophylls and phaeopigments produces emission of red light.

In a sample, the fraction of light transformed is proportional to

the amount of pigment.  Hence, by knowing the amount of incident light

and by measuring the light emitted at a second wavelength, it is

possible to estimate the concentration of these pigments in the sample.

2.  Sampling, Filtration, Extraction and Storage

2.1. Seawater samples are collected from 5, 25, 45, 60, 75, 85, 95, 105,

115, 125, 150, 175 and 200 m using 12-liter Niskin bottles attached

to the rosette sampler and from the primary productivity cast (see

Chapter 14) using Go-Flo bottles attached to a Kevlar line.

2.2. A 500 ml sample from each bottle is collected in clean 4-liter

polyethylene bottles and stored in the dark.  The polyethylene

bottles are rinsed three times with 100-200 ml of sample before

the collection of the final sample.

2.3. Filtration and storage

2.3.1. As soon as possible, triplicate 100 ml subsamples from the

12-liter Niskin bottles from 30, 95 and 125 m are filtered

through 25 mm GF/F filters.  For the remaining depths of the

same cast a single 100 ml sample is filtered.

2.3.2. Triplicate 100 ml samples from each Go-Flo bottle are filtered

as in Chapter 12, section 2.3.1.

2.3.3. Immediately following filtration, the filter is transferred

to a glass screw cap tube containing 5 ml of cold (-20oC)

100% acetone.  Then the tubes are wrapped in aluminum foil

and stored at -20oC to prevent photodegradation of pigments.

2.4. Sample Analysis

2.4.1. Samples are brought to room temperature 2 hours before the

analysis and the fluorometer is allowed to stabilize for

30 minutes before the samples are analyzed.

2.4.2. The external standard, prepared with reagent grade chl a

(see Chapter 12, section 3), is read before other samples.

2.4.3. Samples are analyzed from deepest to shallowest depth.  Each

time the door of the fluorometer is changed, the dial is

brought to the "zero" setting using 100% acetone as a blank.

2.4.4. Each sample is acidified by adding 2 drops of HCl (1 M) to

the sample in the cuvette.  The new reading must be stable

before recording the value and is made using the same door

used for the reading before the acidification.

2.4.5. The fluorometer door used, the readings before and after

acidification and the dilution factor are recorded in the

chlorophyll log book.

3.  Standard Preparation and Analysis

3.1. A chl a standard stock is prepared every four months using

commercially-available chl a (SIGMA) and 100% acetone.  The

stock is wrapped in aluminum foil and stored at -20oC.

3.2. To determine the concentration of the stock, a 50 ml sample

is brought to room temperature avoiding exposure to the light.

An absorbance spectrum from 350-750 nm is obtained using a

scanning spectrophotometer and the concentration is calculated

considering an extinction coefficient of 88.15 (l g-1 cm-1)

for the absorbance at 662 nm.

3.3. Using the same stock sample, three dilutions (1/10 each) are

prepared gravimetrically using 100% acetone and read in the

fluorometer.  During this step, chl a must be kept under dim

light.

3.4. A calibration of the fluorometer is performed every 6 months

following the procedure described by Strickland and Parsons

(1972).

4.  Data Reduction and Calculations

4.1. Concentrations of chl a and phaeopigments are calculated using

the following equations:

chl a = (T/(T-1))*(Rb-Ra)*Fd*volex/volfilt

phaeo = (T/(T-1))*((T*Ra)-Rb)*Fd*volex/volfilt

where:  chl a = concentration of chl a (mg m-3)

phaeo = concentration of phaeopigments (mg m-3)

T = acidification coefficient (Rb/Ra average obtained

during the calibration of the fluorometer)

Rb = reading before acidification

Ra = reading after acidification

Fd = door factor (µg/(ml* reading units))

volex = volume of extraction (ml)

volfilt = volume filtered (l)

5.  Equipment/Supplies

Niskin or Go-Flo bottles and rosette/CTD unit

filtration system

glassware

freezer

fluorometer (Turner model 111)

spectrophotometer (Varian model DMS-100S)

6.  Reagents

Filtered seawater

100% acetone

HCl (1 M)

7.  References

Strickland, J. D. H. and T. R. Parsons.  1972.  A Practical

Handbook of Seawater Analysis, Fisheries Research Board of Canada,

167 pp.

### HPLC ANALYSIS OF ALGAL PIGMENTS1

_________________________________________________________________

SUMMARY:  Chlorophylls and carotenes are analyzed by high-

performance liquid chromatography (HPLC).  The diversity as

well as their distributions and concentrations in the water column

are used to describe the structure of the phytoplankton

community.

_________________________________________________________________

1.  Principle

Because different pigments are specific for individual

phytoplankton taxa, the study of plant pigment diversity, concentration

and distribution in the water column has become a useful tool when

trying to describe the phytoplankton community.  The method presented

here is based on the protocol described by Mantoura and Lewellyn (1983)

and modified according to Bidigare et al. (1989).  The difference in

polarity and molecular size between photosynthetic pigments is used

to separate these molecules by high-performance liquid chromatography.

The integration of the area under a particular peak in absorbance or

fluorescence at 436 nm or 640 respectivelly is a measure of the amount

of pigment injected in the column.  Pigments routinely measured are:

chlorophyll a, b and c and their degradation products, fucoxanthin,

diadinoxanthin, β-carotene, zexanthin, lutein, alloxanthin, prasino-

xanthin 19'-hexanoyloxyfucoxanthin and 19'-butanoyloxyfucoxanthin.

2.  Precautions

Light causes deterioration in pigments.  Therefore, the samples

should always be protected from exposure to light.  Pigments can also

decompose spontaneously, particularly when concentrated onto filters.

Therefore, filters are stored in liquid nitrogen to minimize pigment

degradation.

3.  Sampling, Filtration, Extraction and Storage

3.1. Seawater samples (4-10 liters) are collected in 12 liter Niskin

bottles and transferred, via Tygon tubing, to polyethylene

filtration bottles.  The filtration bottles and caps are rinsed

three times with the sample before filling.  Sampling depths

correspond to those used for primary productivity.  Four to six

additional depths are distributed uniformly within the region

of the chl maximum, as determined by the continuous profile of

fluorescence (see Chapter 4).

3.2. After filling, the filtration bottles are placed upside down in

the filtration rack and the contents pressure filtered (4-7 psi

N2) through in-line 25 mm GF/F filters.  All filtration procedures

are done under subdued light conditions.

3.3. After the sample is filtered, each filter is folded and transferred

to a cryotube.  The tube is labeled, flushed with N2 gas and stored

in liquid nitrogen.

3.4. Extraction and concentration of pigments

3.4.1. Filters are extracted for 48 hours in 3 ml 100% acetone at

-20oC.  One  hundred microliters of a known concentration of

canthaxanthin are added as external standard to the extract

to correct for changes in extraction volume resulting from

the water content in the filter and the acetone evaporation.

The sample is vortexed to ensure an homogeneous suspension

and supernatant is decanted by centrifugation (1,500 rpm for

5 min.).

3.4.2. One milliliter of each extract is combined with 300 µl of an

ion pairing solution (IPS) (50 mmol tetrabutyl ammonium

acetate [95% purity] and 1 mol of ammonium acetate [analytical

reagent grade] in 100 ml DDW) in an autosampler vial and store

at 4oC until analysis by HPLC.

4.  Sample Analysis by HPLC

4.1. Two eluant solutions (termed solvent A and solvent B) are prepared

for the chromatographic separation. One liter of solvent A is made

by mixing 50 ml IPS and 150 ml DDW in 800 ml methanol.  The solvent

B is 100% methanol.

4.2. Samples are loaded in a temperature controlled autosampler equipped

with a 500 µl injection loop.  The temperature is kept at 4oC.

The separation of pigments is performed on a Radial-Pak C18 column

(0.8 x 10 cm, 5µ particle size). by linear gradient elution from

100% eluant A to 100% eluant B in 12 min, followed by 20 min of

100% eluant isocratic hold. Flow rate is 6 ml min-1 and upper limit

pressure for the column is 3000 psi.

4.3. Absorbance is read at 436 nm.  The excitation wavelength of the

fluorometer is 424 nm and the emission is read at 640 nm.

4.4. Peak areas are converted to concentration by the external standard

calibration method using standards provided by Dr. Bidigare.

5.  Data Reduction and Calculations

Pigment concentration in the concentrated sample is determined using

the following equation:

(RF) * (Area of the peak)

[P] = -------------------------

(Sample Loop Area)

where  [P] = pigment concentration

(RF) = response factor (obtain from the calibration)

(RF) = [(calibration[P]) * (sample loop volume)]/(calibration

[peak area])

The extraction volume is calculated as follows:

(AESP) * (volume ext. std. added to sample)

vol. extract) = -------------------------------------------

(Area of ext. std. peak in sample)

where (AESP) = Area of the external standard when 1 ml external

standard is mixed with 300 µl of IPS and 500 µl

are injected in the HPLC.

6.  Equipment/Supplies

Niskin bottles and rosette/CTD unit

low pressure filtration apparatus

liquid nitrogen dewer

Spectra-Physics HPLC, Waters model 440 absorbance detector,

Waters model 470 fluorescence detector, Radial-Pak C18 column

(0.8 x 10 cm, 5 µm particle size), guard column

vortex

centrifuge

general laboratory glassware and supplies

N2 gas

freezer

7.  Reagents

Deionized distilled water (DDW)

Acetone (HPLC grade)

Methanol (HPLC grade)

Tetrabutyl-ammonium acetate (95% purity)

Ammonium acetate (reagent grade)

Ion-pairing reagent (made of 1.5 g tetrabutyl ammonium acetate

[95% purity] and 7.7 g of ammonium acetate [analytical reagent

grade] in 100 ml DDW)

Solvent "A" (mix 50 ml of ion-pairing reagent, 150 ml DDW and 800

ml methanol)

Solution "B" (100% methanol)

8.  References

Bidigare, R. R., O. Schofield and B. B. Prezelin.  1989.  Influence

of zeaxanthin on quantum yield of photosynthesis of Synechococcus

clone WH7803 (DC2).  Marine Ecology Progress Series, 56, 177-188.

Mantoura, R. F. and C. A. Lewellyn.  1983.  The rapid determination

of algal chlorophyll and carotenoid pigments and their breakdown

products in natural waters by reverse-phase liquid chromatography.

Analytica Chimica Acta, 151, 297-314.

### PRIMARY PRODUCTIVITY1

_________________________________________________________________

SUMMARY:  The 14C-radiotracer method is used to measure

the assimilation of dissolved inorganic carbon (DIC) by

phytoplankton as an estimate of the rate of photosynthetic

production of organic matter in the euphotic zone.

_________________________________________________________________

1.  Principle

The 14C method, originally proposed by Steeman-Nielsen (1952),

is used to estimate the uptake of dissolved inorganic carbon (DIC)

by planktonic algae in the water column.  The method is based on the

fact that the biological uptake of 14C-labeled DIC is proportional

to the biological uptake of 12C-DIC.  If one knows the initial

concentration of DIC in a water sample, the amount of 14C-DIC added,

the 14C retained in particulate organic matter (14C-POC) at the end

of the incubation and the metabolic discrimination between the two

isotopes of carbon (i.e., 5% discrimination against the heavier 14C

isotope), then it is possible to estimate the total uptake of carbon

from the following relationship:

DIC * 14C-POC * 1.05

C uptake  =   --------------------

14C-DIC added

Due to the potentially toxic effects of trace metals on phytoplankton

metabolism in oligotrophic waters, the following procedure is used to

minimize the contact between water samples and possible sources of

contamination.

2.  Cleaning

2.1. HCl (Baker Instra-Analyzed) solution (1M) is prepared with high

purity hydrochloric acid and freshly-prepared glass distilled

deionized water (DDW).

2.2. 500 ml polycarbonate bottles are rinsed twice with 1M HCl

(Baker Instra-Analyzed) and left overnight filled with the same

acid solution.  The acid is removed by rinsing the bottles three

times with DDW before air drying.

2.3. Go-Flo bottles, fitted with teflon-coated springs, are rinsed

three times with 1M HCl and DDW before use.

2.4. Pipette tips used in the preparation of the isotope stock and

in the inoculation of samples are rinsed three times with

concentrated HCl (Baker Instra-Analyzed), three times with DDW

and once with the sodium carbonate solution (Chapter 14, section

3.2) and stored in a clean polyethylene glove until used.

3.  Isotope Stock

3.1. The preparation of the isotope stock is performed wearing

polyethylene gloves.  A 25 ml acid-washed teflon bottle and a

50 ml acid-washed polypropylene centifuge tube are rinsed three

times with DDW.

3.2. 0.032 g of anhydrous Na2CO3 (ALDRICH 20,442-0, 99.999% purity)

are dissolved in 50 ml DDW in the centrifuge tube to provide

a solution of 6 mmol Na2CO3 per liter.

3.3. 3.5 ml of NaH-14C03 (53 mCi mmol-1; Research Products Inc.)

are mixed with 16.5 ml of the above prepared Na2CO3 solution

in the teflon bottle.

3.4. The new stock activity is checked by counting triplicate 10 µl

samples with 1 ml β-phenethylamine in 10 ml Aquasol-II.

3.5. Triplicate 10 µl stock samples are also acidified with 1 ml of

2 M HCl, mixed intermittently for 1-2 hours and counted in 10

ml Aquasol-II to confirm that there is no 14C-organic carbon

contamination.  The acidification is done under the hood.

The acidified dpm should be <0.001% of the total dpm of the 14C

preparation.

4.  Incubation Systems

Typically we measure primary production using in situ incubation

techniques.

4.1. A free-floating array equipped with VHF radio and strobe light

is used for the in situ incubations.  Incubation bottles are

attached to a horizontal polycarbonate spreader bar which is

then attached to the 200 m, 1/2" polypropylene in situ line at

the depths corresponding to the sample collections.

4.2. Generally eight incubation depths are selected (5-175 m,

approximately).

5.  Sampling

5.1. Approximately 3 hours before local sunrise, seawater samples

are collected with acid- washed, 12-liter Go-Flo bottles using

Kevlar line, metal-free sheave, teflon messengers and a

stainless steel bottom weight.  A dedicated hydrowinch is used

for the primary productivity sampling procedures in a further

effort to reduce/eliminate all sources of trace metal contamination.

5.2. Under low light conditions, water samples are transferred to the

incubation bottles (500 ml polycarbonate bottles) and stored in

the dark.  Polyethylene gloves are worn during sample collection

and inoculation procedures.  No drawing tubes are used.

6.  Isotope Addition and Sample Incubation

6.1. Three light bottles, three dark bottles and 1 time-zero control

(see Chapter 14, section 8) are collected at each depth for in

situ incubation.  In situ dark bottles are deployed in specially-

designed, double-layered cloth bags with VelcroR closures.

6.2. After all water samples have been drawn from the appropriate Go-Flo

bottles, 250 µl of the 14C-sodium carbonate stock solution is added

to each sample using a specially-cleaned pipette tip. The samples

are deployed before dawn on a free-floating, drifter buoy array.

6.3. At local sunset, the free-floating array is recovered and all in

situ bottles are immediately placed in the dark and processed as

soon as possible. The time of recovery is recorded.

7.  Filtration

7.1. Filtration of the samples is done under low light conditions and

begins as soon as the incubation bottles are recovered from the

in situ array.

7.2. 200 µl are removed and placed into a second LSC vial containing

0.5 ml of β-phenethylamine.  This sample is used for the

determination of total radioactivity in each sample.

7.3. The remainder is filtered through a 25 mm diameter GF/F filters.

The filters are placed into prelabelled, clean glass liquid

scintillation counting vials (LSC vials) and stored at -20oC.

8.  14C Sample Processing

8.1. One ml of 2 M HCl is added to each sample vial (under the hood).

Vials are covered with their respective caps and shaken in a

vortex mixer for at least 1 hour with venting at 20 minute

intervals.  To vent, the vials are removed from the shaker,

and the cap opened (under the hood).  After shaking is completed,

the vials are left open to vent under the hood for an additional

24 hours.

8.2. Ten ml of Aquasol-II are added per vial (including vials for total

14C radioactivity) and the samples are counted in a liquid

scintillation counter.  Samples are counted again after 2 and

4 weeks, before discarding.  Counts have shown a consistent

increase during the first two weeks and become stable between

the second and the fourth week.  This is probably the result of

sample hydrolysis or diffusion of radioactivity from the GF/F

filter matrix, thereby reducing the extent of self-absorption.

Only the 4-week count is used for 14C calculations.  Counts per

min (CPM) are converted to disintegration per min (DPM) using

the channels ratio program supplied by the the manufacturer

(Packard Instrument Co.).

9.  Calculations

From the data derived above we can estimate several properties of

the phytoplankton populations at Station ALOHA.  Total daylight organic

carbon production is calculated from the 12-hour uptake data (after

corrections for 12-hour dark activities).  Net daily organic carbon

production is calculated from the 24-hour light/dark samples (corrected

for the time-zero blank activities).  Phytoplankton population respiration

is taken as the difference between the 12-hour light and the 24-hour

light/dark incubations.  Net primary production is used as the estimate

of phytoplankton carbon production for the purposes of comparison to

other ecosystem-level processes (e.g., standing stock assessments,

vertical C-flux, etc.).

10.  Equipment/Supplies

Go-Flo bottles

kevlar hydroline

teflon messengers

stainless steel weight

temperature- and light-controlled deck incubation system

(NORDA/USM incubation system)

free-drifting productivity array (including polypro line,

spreader bars, surface floats, buoy, radio

transmitter and strobe le light)

500 ml wide-mouth polycarbonate bottles

vacuum filtration system

liquid scintillation counting (LSC) vials

pipettes

glassware

vortex mixer

liquid scintillation counter (Packard model 4640; United

Technologies Inc.)

11.  Reagents

distilled deionized water (DDW)

HCl for trace metal analysis (Baker Instra-Analyzed)

Na2CO3 (99.999%)

NaH-14CO3 solution (cat #CMM-50, Research Products Inc.)

β-phenethylamine

Aquasol-II (Dupont)

2 M HCl

12.  References

Steeman-Nielsen, E.  1952.  The use of radioactive carbon (14C)

for measuring organic production in the sea.  Journal du Conseil,

18, 117-140.

### BACTERIA AND CYANOBACTERIA1

_________________________________________________________________

SUMMARY:  Picoplankton populations are subsampled from

seawater collected in Niskin bottles.  The cells are

fixed in the field by the addition of paraformaldehyde

and, in the laboratory, stained with Hoechst 33342 (a DNA

specific dye) and enumerated by dual-beam flow cytometry.

_________________________________________________________________

1.  Principle

Flow cytometric enumeration is based on the method of Monger and

Landry (1993).  This method increases substantially the precision of

bacterial counts, relative to epifluorescence microscopy.  Hoechst

33342 is used because binding to DNA substantially alters its

fluorescence spectrum, which facilitates separation of cell

fluorescence signals from the background fluorescence of the unbound

dye.  A 225 mW UV laser is aligned colinearly with a 1 W visible

(488 nm) laser to permit enumeration of both heterotrophic and

autotrophic picoplankton.  Methods for enumeration of autotrophs

are given by Campbell and Vaulot (1993) and are not described in

detail here.

2.  Precautions

Because this is a procedure for enumerating preserved cells,

sterilization is not a guarantee against contamination.  However new,

sterile plastic containers tend to be among the cleanest containers

available. All reagents (preservative and stain) must be prefiltered

through 0.2 µm filters and the sample should not come in contact with

fingertips or other potentially contaminating surfaces.  No drawing

tubes are used.

Because of the small sample volume (1 ml), frozen samples thaw

quickly.  Therefore they should be kept in liquid nitrogen during

transport from the ship to the shore-based laboratory.

3.  Sampling

3.1. A small volume of sample (10-12 ml) is drawn from the Niskin

bottle into a sterile 15 ml plastic centrifuge tube.  Drawing

tubes are not used.

3.2. 1 ml subsamples are drawn from the sample and placed in 2 ml

cryovials containing 0.02 ml of 0.2 µm prefiltered 10%

paraformaldehyde (final concentration 0.2%).

3.3. Cryovials are let to sit for 10 minutes and then quick frozen

in liquid nitrogen.  They are then stored frozen at -20oC or

colder until analyzed.

4.  Analysis

4.1. Samples are stained with freshly prepared, prefiltered Hoechst

33342 (Molecular Probes Inc.) to a final concentration of 0.5

µg/ml.  Samples stain for 2 h at room temperature in the dark.

215 µL sample

25 µL Hoechst 33342 (5 µg/ml)

10 µL fluorescent beads (internal standard)

4.2. Samples are analyzed by colinear dual-beam flow-cytometry using

a Coulter EPICS flow instrument (Monger and Landry, 1993).

Prior to analyzing samples, fluorescent beads are run to check

the volume calibration and the alignment of the instrument.

100 µL subsamples are analyzed for forward angle light scatter

(FALS), right angle light scatter (RALS), blue (DNA) fluorescence

(BF) and red (chlorophyll) fluorescence (RF).  The scatter parameters

are a function of cell size and are used to separate bacteria from

beads and other blue fluorescence signals.  Marine bacteria form

a distinct cluster on a scatter plot of BF vs. RALS (Monger and

Landry, 1993).  BF and RF signals are used to distinguish

heterotrophic (unpigmented) from autotrophic (chlorophyll-containing)

cells.

A known concentration of beads (0.98 µm, Polysciences) are used as

an external standard for volume calibration.  Beads (0.46 µm,

Polysciences) are also added to each sample as an internal standard

for per cell blue fluorescence.  A record of cruise-to-cruise

variation in fluorescence per cell relative to beads is maintained.

4.3. Data are stored in list (ASCII) files.  Statistical analysis is

carried out on an 80486 microcomputer using the CYTOPC program

developed by Dr. Daniel Vaulot (Station Biologique, Roscoff-sur-mer,

France).  This is not presently available commercially; see Vaulot

(1989) for a description of the program.  Further calculations

(correction for dilution by preservative and stain) are done on

a Microsoft Excel spreadsheet.

5.  Reagents

Hoechst 33342 (5 µg/ml; prepared fresh)

paraformaldehyde (10%)

Add 10 g paraformaldehyde (be careful not to breathe dust) to 90 ml

boiling distilled water and stir (use magnetic stir bar).  Add 1 M

NaOH, dropwise, with constant mixing, until the solution clears.

Cool in ice to room temp, add 10 ml filtered sea water (can be cooling

in ice while on stirrer).  Adjust pH to 7.5 (careful, pH changes

very rapidly below about 8.8; i.e., only one or two more drops

needed).  Use a Sterifil apparatus with 47 mm GF/F filters to filter

this solution.  Filter in two equal aliquots,changing the filter

between aliquots to avoid clogging.  The final preservative strength

is about 10%.  This solution should be prepared fresh for each cruise

and pH checked before use.

6.  Equipment and Supplies

15 ml sterile polypropylene centrifuge tubes

2 ml sterile cryovials

sterile plastic syringes and Acrodisc filters

1 ml autopipet and tips

liquid nitrogen and container

Coulter EPICS flow cytometer with UV and visible lasers

IBM-compatible 80486 microcomputer and software

0.5 µm fluorescent beads

7.  References

Campbell, L. and D. Vaulot.  1993.  Photosynthetic picoplankton

community structure in the subtropical North Pacific Ocean near

Hawaii (Station ALOHA). Deep-Sea Research, in press.

Monger, B. C. and M. R. Landry.  1993.  Flow cytometric analysis

of marine bacteria with Hoechst 33342. Applied and Environmental

Microbiology, 59, 905-911.

Vaulot, D.  1989.  CYTOPC: Processing software for flow cytometric

data. Signal and Noise, 2, 8.

### MICROBIAL ATP1

_________________________________________________________________

SUMMARY:  ATP, an obligate constituent of all living

organisms, is extracted from viable microorganisms in

boiling TRIS buffer following sample concentration by

vacuum filtration.  The extracted ATP is analyzed in a

photometer by the firefly bioluminescence reaction, and

the ATP content is related to total living (biomass)

microbial carbon by the application of a laboratory-

derived extrapolation factor.

_________________________________________________________________

1.  Principle

In field studies it is often desirable to determine the total

amount of living cellular material (biomass).  Conventional methods

(i.e., fresh or dry weight determinations, rate of increase of cell

numbers, etc.) usually cannot be used owing to (a) lack of sensitivity

in the analytical procedures, (b) the presence of a heterogeneous

assemblage of organisms, (c) the presence of dead cells and (d) the

presence of detrital (non-living) organic material which is not

associated with the living cells.  Estimation of cellular biomass

by measurement of adenosine triphosphate (ATP) is not limited by any

of these considerations.

The rationale for using ATP to estimate biomass is the ubiquitous

distribution of ATP in all living cells, the rapid loss of ATP from

dead cells and the fairly uniform concentration of ATP in the protoplasm

of all microbial cells.  Data on ATP concentrations can thus be

extrapolated to biomass parameters, such as cellular organic carbon

or dry or fresh weight (Holm-Hansen, 1973).  ATP is extracted from

cells using boiling TRIS buffer and is stored frozen (-20oC) prior

to analysis by firefly bioluminescence.

2.  Precautions

ATP samples must be processed as rapidly as possible, because the

ATP content of microorganisms can change rapidly when cells are stressed.

Furthermore, a phenomenon referred to as the "filtration effect" causes

a loss of ATP when cells are exposed to dessicating conditions

immediately after the water is drawn through the filter pad (Karl

and Holm-Hansen, 1978).  For this reason it is very important that

the samples are filtered immediately upon sampling and extracted

immediately upon filtration; any delay will cause a decline in ATP

content.  Because the firefly bioluminescence assay is inhibited by

metals, it is also important to use clean stainless steel forceps when

handling the filters.  It is also essential that the extraction buffer

(TRIS) is boiling (100oC), as inefficient extraction results at

temperatures (<95oC).  TRIS buffer boiling must be confirmed before

starting the filtration process.

3.  Sampling, Filtration, Extraction and Storage

3.1. Samples for ATP determinations are collected in clean Niskin

bottles attached to the rosette/CTD unit.

3.2. As soon as the samples arrive on deck, water is prefiltered

through a drawing tube containing an in-line 202 µm Nitex mesh

prefilter to remove large zooplankton and particles which might

otherwise affect the precision and accuracy of microbial biomass

determinations.  Samples are drawn into 4-liter polyethylene

bottles that are rinsed 3 times with approximately 100-200 ml

seawater from the appropriate depth.  Filtration is begun

immediately (be sure heating block is on and has achieved a

temperature >110oC; time required ~1 hour).

3.3. Filter triplicate samples at each depth through 47 mm GF/F filters.

Total volume required per sample will depend on depth:  between

0-150 m use 1 liter per sample, below 150 m use 2 liters per sample.

3.4. As soon as the last few drops of water have passed through the

filter, remove the filter tower from the base, fold the filter

in half then in half again and plunge the folded filter into 5

ml of boiling TRIS buffer (pH 7.4; 0.02 M) which is kept partially

covered to eliminate evaporative volume loss.  Try to avoid

"bumping" of the TRIS buffer (caused by superheating of the

buffer).

3.5. After a 5 minute extraction period, remove the tubes from the heating

block, allow to cool to approximately room temperature, secure the

rubber stoppers and freeze (-20oC) in upright position.

3.6. In order to minimize sample cross-contamination it is best to start

with deepest sample, which, in most cases, contains the lowest

concentration of ATP.

4.  Analysis

4.1. Prepare enough firefly lantern extract (Sigma Chemical Co., FLE-50)

to process all samples and at least ten external ATP reference

standards.  Lyophilized FLE-50 should be reconstituted in 5 ml

distilled water and allowed to "age" at room temperature for at

least 6 hours (but no longer than 24 hours) in order to reduce

the background luminescence.  Approximately 1 hour before

starting the assay, dilute each 50 mg vial of reconstituted

FLE-50 with 15 ml of sodium arsenate buffer (0.1 M, pH 7.4)

and 15 ml of MgSO4  (0.04 M).  Immediately before use, filter

the FLE-50 mixture through a GF/F filter.

4.2. Turn on ATP photometer at least 30 minutes before use.

4.3. Prepare set of ATP reference standards ranging from 0.1-100 ng

ATP ml-1 (see Chapter 16, section 5).

4.4. Using the automatic injector and computer-assisted photometer,

analyze the peak height of light emission (0-15 seconds) for

each sample and standard.

5.  Preparation of ATP Standards

A primary (reference) ATP standard is prepared by dissolving

exactly 10 mg of high purity (99.9%) ATP (sodium salt) into exactly

10 ml of sterile distilled water.  Be sure to weigh out an amount

equivalent to 10 mg of free acid form of ATP (not 10 mg of hydrated

Na-ATP) taking into account the cation contribution and average

hydration state of the molecule.  This latter information is supplied

by the manufacturer.  Immediately prepare 1/10 and 1/100 dilutions

of the primary stock, place a portion of each solution into a 1 cm

quartz spectrophotometercuvette and measure the absorbance at 259 nm.

Calculate the exact concentration as follows:

A = Elc

where:  A = absorption at 259 nm

E = ATP molar extinction coefficient (15.4 x 103)

l = path length (cm)

c = concentration of ATP in moles per liter

After calculating the precise concentration of ATP in the primary

standard, dilute the stock (gravimetrically) into sterile-filtered

(0.2 µm) 0.02 M TRIS buffer pH 7.4 to yield an ATP standard solution

of exactly 1 µg ml-1.  Compare with "old" standard and if different

by >|1%|, repeat the dilution step.  Store frozen in 1 ml aliquots.

Working standards, ranging from 0.1-100 ng ATP ml-1, are prepared

by diluting a vial of the primary standard with freshly-prepared TRIS

buffer (0.02 M) just prior to use.  These working standards are

discarded at the end of the day.

6.  Data Reduction and Calculations

6.1. Plot ATP standard data and calculate linear regression statistics

for the standard curve.  Use LOTUS spreadsheet to calculate ATP

concentration (ng l-1) from data on peak height (relative to

standards), volume of extract and volume of sample filtered.

6.2. ATP concentrations can be related to total biomass by applying

the extrapolation factor; 250 x ATP = C (Karl, 1980).  This

relationship is based upon direct laboratory and field analyses

performed over the past two decades.  While there is a range

in the C/ATP ratio for microorganisms grown under a variety

of environmental conditions, the relationship of C/ATP = 250

appears to be reasonable for samples collected from the

oligotrophic North Pacific Ocean (Laws et al., 1987).

7.  Equipment/Supplies

Niskin bottles and rosette

sample bottles and tygon tubing

heating block and test tubes

ATP photometer (Biospherical Instrument Co.)

stainless forceps

glass fiber filters (Whatman; 47 mm GF/F)

8.  Reagents

TRIS buffer (0.02 M, pH 7.4)

magnesium sulfate (0.04 M)

sodium arsenate buffer (0.1 M; pH 7.4)

firefly lantern extract (FLE-50; Sigma Chemical Co.)

reagent ATP (Sigma Chemical Co.)

9.  References

Holm-Hansen, O.  1973.  Determination of total microbial biomass

by measurement of adenosine triphosphate.  In: Estuarine Microbial

Ecology, L. H. Stevenson and R. R. Colwell, editors, University

of South Carolina Press, Columbia, pp. 73-89.

Karl, D. M.  1980.  Cellular nucleotide measurements and

applications in microbial ecology.  Microbiological Reviews, 44,

739-796.

Karl, D. M. and O. Holm-Hansen.  1978.  Methodology and measurement

of adenylate energy charge ratios in environmental samples.

Marine Biology, 48, 185-197.

Laws, E. A., D. G. Redalje, L. W. Haas, P. K. Bienfang, R. W.

Eppley, W. G. Harrison, D. M. Karl and J. Marra.  1984.  High

phytoplankton growth and production rates in oligotrophic

Hawaiian coastal waters. Limnology and Oceanography, 29, 1161-1169.

### DISSOLVED AND PARTICULATE LIPOPOLYSACCHARIDE1

_________________________________________________________________

SUMMARY:  LPS, an obligate constituent of the cell walls

of bacteria and cyanobacteria, varies in proportion to

total bacterial biomass.  Particulate (bound) and dissolved

LPS samples are collected from standard hydrocasts and are

measured using the Limulus amoebocyte lysate (LAL) reaction.

Bound-LPS can be extrapolated to bacterial biomass using

laboratory-derived LPS/C relationships.

_________________________________________________________________

1.  Principle

The outer membrane of all gram-negative bacteria, including

photosynthetic prokaryotes (e.g., sulfur and nonsulfur purple

bacteria, green sulfur bacteria, cyanobacteria) contains

lipopolysaccharide (LPS).  Total LPS (T-LPS) exists in seawater

in both a particulate or bound form (P-LPS) and a dissolved form

(D-LPS).  The measurement of P-LPS has been suggested as an

indicator of bacterial biomass in the marine environment (Watson

et al., 1977; Watson and Hobbie, 1979).  Watson has adapted the

Limulus amoebocyte lysate (LAL) test for the in vitro detection

of bacterial LPS as endotoxin (or pyrogen) and a commercially-

available automatic system now exists for the quantitative

determination of LPS.  During the assay, LPS reacts with LAL

which causes an increase in the optical density of the seawater

sample.  The rate of optical density increase is proportional to

the pyrogen concentration.  In this analytical procedure, the

rate of optical density increase is converted to LPS concentration

using lipopolysaccharide standards made up in pyrogen-free seawater.

Further extrapolation of LPS to total prokaryote biomass relies

upon empirically-determined LPS-to-carbon relationships.

2.  Precautions

Contamination is the primary concern with the LPS assay, and

great care should be used to ensure that the samples are not

contaminated.  All glass and plastic sampling gear must be pyrogen

free.  Niskin bottles should be acid-washed before use.  The primary

source of contamination is from fingertips, therefore plastic gloves

should be worn when collecting samples at sea and when analyzing

the samples in the laboratory.

3.  Sampling and Storage

3.1. Total LPS (T-LPS)

3.1.1. A 20 to 40 ml sample is collected directly from each Niskin

bottle into a previously unused, 50 ml plastic centrifuge

tube.  Do not use a drawing tube as this too is a potential

site for LPS contamination.  It is important that only

pyrogen-free, disposable tubes are used.  It is not necessary

to rinse the centrifuge tubes.

3.1.2. As soon as the samples have been collected, pipette a 1 ml

aliquot from each 50 ml centrifuge tube into a 1.5 ml

microcentrifuge tube.  Use a new, clean pipette tip for

each sample and wear gloves during this operation.  After

the 1 ml subsample has been removed, tighten the cap securely

on the 50 ml sample and store at -20oC for subsequent

laboratory analysis.  The 1 ml subsample contained in the

centrifuge tube is then immediately processed for D-LPS

(see Chapter 17, section 3.2).

3.2. Dissolved LPS (D-LPS)

3.2.1. The 1.0 ml subsample (Chapter 17, section 3.1.2) is

immediately centrifuged for 10 minutes in a Beckman

microfuge.  The supernatant is then decanted into a

second pyrogen-free microcentrifuge tube and stored

frozen (-20oC) until assayed.

4.  Lab Procedures

4.1. Turn on computer-assisted LAL-5000 and follow menu to set up

the instrument.  The LAL-5000 must reach the appropriate

operating temperature before the assay can begin, this can

take up to 2 hours.  The instrument should, therefore, be

turned on and set up to accept samples before samples are

prepared for analysis.

4.2. Dilute the commercially-available pyrotell-GT reagent with 5 ml

of pyrogen-free seawater, as described on the label.

4.3. Set up an endotoxin standard curve in combusted 10x75 mm glass

culture tubes.  The standard curve should contain two pyrogen-

free blanks and one tube each of the following concentrations:

0.0001, 0.0005, 0.001, 0.005, 0.01, 0.05, 0.1 ng endotoxin ml-1.

Each tube should contain a total of 0.4 ml of the appropriate

solution.

4.4. Dilute three replicate aliquots of each sample into a combusted

10x75 mm glass culture tube.  Dilute each sample 1:20 with

pyrogen-free seawater to make a total of 0.4 ml of diluted

sample per tube (i.e., 380 µl of pyrogen free seawater and

20 µl of sample).  Extreme caution must be taken to avoid

contamination.  Use new pipette tips for each sample and wear

gloves throughout the sample preparation procedure.

4.5. Immediately before positioning the samples and standards

in the LAL-5000 for analysis, add 100 µl of diluted pyrotell-

GT to each tube and vortex for approximately 10 seconds.

Process each tube sequentially and immediately place the tube

in the instrument after vortexing.  Try to do this part of

the assay as reproducibly as possible.  Variability between

samples in this procedure will directly contribute to

variability in measured LPS concentrations between replicates.

5.  Data Reduction and Calculations

The quantity of LPS in each sample is determined by the computer

software program.  LPS concentrations are corrected for dilution in

these calculations.  P-LPS is calculated by difference between "total"

and "dissolved" determinations.

6.  Equipment/Supplies

new, pyrogen-free 50 ml and 1.5 ml centrifuge tubes

LAL-5000 (Associates of Cape Cod) and MS-DOS computer

automatic pipettes and tips

combusted 10x75 mm glass culture tubes and appropriate rack

parafilm

7.  Reagents

pyrogen-free seawater (prepared by UV irradiation of surface seawater)

pyrotell-GT (Associates of Cape Cod)

endotoxin standard (Associates of Cape Cod)

8.  References

Watson, S. W. and J. E. Hobbie.  1979.  Measurement of bacterial biomass

as lipopolysaccharide.  In:  Native Aquatic Bacteria: Enumeration,

Activity and Ecology, J. W. Costerton and R. R. Colwell, editors,

American Society for Testing and Materials, Baltimore, pp. 82-88.

Watson, S. W., T. J. Novitsky, H. L. Quinby and F. W. Valois.  1977.

Determination of bacterial number and biomass in the marine

environment. Applied and Environmental Microbiology, 33, 940-946.

### SEDIMENT TRAP PROTOCOLS1

_________________________________________________________________

SUMMARY:  Passively sinking particulate matter is

collected using a free-floating sediment array and,

after prescreening (335 µm) to remove zooplankton and

micronekton carcasses, the sample materials are analyzed

for C, N, P and mass flux (mg m-2 d-1).

_________________________________________________________________

1.  Principle

Although most of the particulate matter both on the seafloor

and in suspension in seawater is very fine, recent evidence suggests

that most of the material deposited on the benthos arrives via

relatively rare, rapidly sinking large particles (McCave, 1975).

Therefore, in order to describe adequately the ambient particle

field and to understand the rates and mechanisms of biogeochemical

cycling in the marine environment, it is imperative to employ

sampling methods that enable the investigator to distinguish between

the suspended and sinking pools of particulate matter.  This universal

requirement for a careful and comprehensive analysis of sedimenting

particles has resulted in the development, evaluation and calibration

of a variety of in situ particle collectors or sediment traps.

The results, after nearly a decade of intensive field experiments,

have contributed significantly to our general understanding of:

(1) the relationship between the rate of primary production and

downward flux of particulate organic matter, (2) mesopelagic zone

oxygen consumption and nutrient regeneration, (3) biological control

of the removal of abiogenic particles from the surface ocean and

(4) seasonal and interannual variations in particle flux to the

deep-sea.  Future sediment trap studies will, most likely, continue

to provide novel and useful data on the rates and mechanisms of

important biogeochemical processes.

At Station ALOHA, we presently deploy a free-drifting sediment

trap array with 12 individual collectors positioned at 150, 300 and

500 m.  The deployment period is generally 72 hours.  The passively

sinking particles are subsequently analyzed for a variety of chemical

properties, including:  total mass, C, N and P.

2.  Precautions

Because particle fluxes in oligotrophic habitats are expected

to be low, special attention must be paid to the preparation of

individual sediment trap collector tubes so that they are clean and

free of dust and other potentially contaminating particles.  Traps

should be capped immediately after filling and immediately after

retrieval.  Pay particular attention to airborne and/or shipboard

particulate contamination sources.  In addition, the time interval

between trap retrieval and subsample filtration should be minimized

in order to limit the inclusion of extraneous abiotic particles and

the post-collection solubilization of particles.

3.  Field Operations

3.1. Hardware

Our free-floating sediment trap array is patterned after the

MULTITRAP system pioneered by Knauer et al. (1979) and used

extensively in the decade-long VERTEX program.  Twelve individual

sediment trap collectors (0.0039 m2) are typically deployed at

three depths (150, 300 and 500 m).  The traps are affixed to a

PVC cross attached to  1/2" polypropylene line.  The traps are

tracked using VHF radio and Argos satellite transmitters and

strobelights.  Typically we deploy our traps for a period of 72

hours each cruise.

3.2. Trap solutions

Prior to deployment, each trap is cleaned with 1 M HCl,

rinsed thoroughly with deionized water then filled with a high

density solution to prevent advective-diffusive loss of

extractants and preservatives during the deployment period and

to eliminate flushing of the traps during recovery (Knauer et al.,

1979).  The trap solution is prepared by adding 50 g of NaCl to

each liter of surface seawater.  This brine solution is pressure

filtered sequentially through a 1.0 and 0.5 µm filter cartridge

prior to the addition of 10 ml 100% formalin l-1.  Individual

traps are filled and at least 10 l of the trap solution is saved

for analysis of solution blanks (see sections 4.1 and 5.1).

3.3. Post-recovery processing

3.3.1. Upon recovery, individual traps are capped and transported

to the shipboard portable laboratory for analysis.  Care

is taken not to mix the higher density trap solutions with

the overlying seawater.  Trap samples are processed from

deep to shallow in order to minimize potential contamination.

3.3.2. The depth of the interface between the high density solution

and overlying seawater is marked on each trap.  The overlying

seawater is then aspirated with a plastic tube attached to

a vacuum system in order to avoid disturbing the high density

solution.  Because some sinking particulate material collects

near the interface between the high density solution and the

overlying seawater, the overlying seawater is removed only

to a depth that is 5 cm above the previously identified interface.

3.3.3. After the overlying seawater has been removed from all the

traps at a given depth, the contents of each trap is passed

through an acid rinsed 335 µm NitexR screen to remove

contaminating zooplankton and micronekton which entered

the traps in a living state and are not truly part of the

passive flux.  Immediately before this sieving process, the

contents of each trap are mixed to disrupt large amorphous

particles.  The traps are rinsed with a portion of the <335

µm sample in order to recover all particulate matter, and

the 335 µm NitexR screen is examined to determine whether

residual material, in addition to the so-called "swimmers",

is present.  If so, the screens are rinsed again with a

portion of the 335 µm filtrate.  After all traps from a

given depth have been processed, the 335 µm screen is

removed and placed into a vial containing 20 ml of formalin-

seawater solution, and stored at 4oC for subsequent

microscopic examination and organism identification and

enumeration.

4.  Determination of Mass Flux

4.1. Three of the 12 traps deployed at each water depth are used for

the determination of mass flux.  At our shore-based laboratory,

triplicate 250 ml subsamples of the time-zero high density trap

solution (blank) and equivalent volumes individual traps (start

with the deepest depth and work up), are vacuum filtered through

tared 25 mm 0.2 µm Nuclepore membrane filters (see Chapter 18,

sections 4.1.4 to 4.1.3).  The tared filters are prepared as

follows:

4.1.1. Rinse filters three times with distilled water.  Place rinsed

filter on a 2.5 cm2 foil square (to reduce static electricity)

in a plastic 47 mm petri dish.

4.1.2. Fold the foil in half over the filter and place the petri dish

in a drying oven with the lid ajar for 2 hours at 55oC.  Remove

and cool in dessicator for 30 minutes.

4.1.3. Weigh filter to constant weight (i.e., repeat oven drying,

cooling and weighing until a relative standard deviation of

<0.005% is achieved), on a microbalance capable of 0.1 µg

resolution.  Record weights (to the nearest 0.1 µg) on label

tape placed on top of the petri dish.

4.2. After the last of the sample has passed through the filter, the

walls of the filter funnel are washed with three consecutive 5 ml

rinses of an isotonic (1 M) ammonium formate solution to remove

seawater salts.  During each rinse, allow the ammonium formate

solution to completely cover the filter.

4.3. Return the processed filter to its petri dish, record sample number

(on the dish and data sheet), and place in a drying oven at 55oC

for 8 hours.  Alternately, store in a dessicator, if an oven is

not immediately available.  Dry to constant weight (as in Chapter

18, section 4.1.3).

4.4. Mass flux is calculated as follows:

[(Wa-Wb)-Wbl] * Vt

mg (dry wt.) m-2 d-1 =  ----------------------

Vf * 0.0039 * 1000 * t

where: Wa = filter weight after filtration (µg)

Wb = filter weight before filtration (µg)

Wbl = net weight of blank solution (µg)

Vt = volume of trap (l)

Vf = volume filtered (l)

0.0039 = cross-sectional area of trap (m2)

1000 = conversion factor (µg mg-1)

t = deployment period (d)

5.  Determination of C, N and P Flux

5.1. The quantities of particulate C, N and P in the prescreened

trap solutions are determined using methods described in Chapters

10 and 11.  Six replicate traps are used for C/N determinations

and three additional traps for P.  Typically, 1.5-2 liters are

used for a single C/N or P measurement.  An equivalent volume

of the time-zero sediment trap solution, filtered through the

appropriate filters is used as a C, N or P blank.

5.2. C, N and P flux is calculated as follows:

[(Cs-Cb)] * Vt

mg C (or N, P) m-2 d-1 =  ---------------

Vf * 0.0039 * t

where:  Cs = carbon (mg) in sample

Cb = carbon (mg) in blank

Vt = volume of trap (liters)

Vf = volume filtered (liters)

0.0039 = cross-sectional area of trap (m2)

t = deployment period (d)

6.  Equipment/Supplies/Reagents

Nuclepore 25 mm 0.2 µm membrane filters

petri dishes and pre-cut foil squares

vacuum filtration apparatus and glassware

Cahn electronic microbalance or equivalent

graduate cylinders

sediment trap array (spar buoy, radiotransmitter, strobe light,

floats, trap supports, collector tubes)

forceps

335 µm NitexR screen

ammonium formate solution (1 M)

7.  References

Knauer, G. A., J. H. Martin and K. Bruland.  1979.  Fluxes of

particulate carbon, nitrogen and phosphorus in the upper water

column of the Northeast Pacific Ocean. Deep-Sea Research, 26,

97-108.

McCave, I. N.  1975.  Vertical flux of particles in the ocean.

Deep-Sea Research, 22, 491- 502.

### MEASUREMENT OF SOLAR RADIATION1

_________________________________________________________________

SUMMARY:  Knowledge of the flux of radiant energy is

essential to interpret habitat variability in the water

column.  Solar radiation is measured with two automated,

independent instruments.  Solar radiation at the sea

surface (incident radiation) is measured with both a

cosine collector and with a scalar irradiance sensor.

Data are logged over the daylight period.  In addition,

continuous profiles of underwater radiation are obtained

from the surface to approximately the 0.1% light level.

_________________________________________________________________

1.  Introduction

Measurements of solar radiative flux are essential to interpret

variability in primary production.  In addition, the depth distribution

of underwater light is an important physical determinant of phytoplankton

standing stock and species composition.  As part of our regular sampling

program at station ALOHA, we therefore collect data on both solar

radiative flux at the sea surface and the vertical profiles of underwater

quantum scalar irradiance.  Our measurements of radiation are restricted to

the 400 to 700 nm waveband.  This waveband is defined as Photosynthetically

Active Radiation (PAR) and is the waveband limits for photosynthetically

useful light (Booth, 1976).

2.  Shipboard Measurements

A LicorR (model LI-1000) data logger and a LicorR (model LI-192S)

quantum sensor are used to collect continuous measurements of radiative

flux at the sea surface on monthly HOT cruises.  The quantum sensor

is a cosine collector and measures PAR.  The sensor is positioned

approximately 4 meters above the deck in order to avoid shadows from

the ship's superstructure.  Radiance is averaged over 10 minute

intervals and logged throughout the day.

3.  Underwater Measurements

Underwater irradiance is measured using a Biospherical Instruments

Profiling Natural Fluorometer (model PNF-300).  This submersible device

transmits measurements of temperature, pressure, downwelling scalar

irradiance (PAR) and upwelling irradiance at 683 nm to an on-deck

computer which allows one to obtain continuous profiles of the above-

mentioned parameters to a depth of approximately 175 m.  In addition,

the instrument package is equipped with an on-deck 2 π deck cell which

continuously logs surface light for comparison with the Licor system

described above.  The data provided by the Biospherical PNF instrument

are used to determine the depth of the 1.0 and 0.1 % light level, and

to determine the underwater extinction coefficient for PAR.  In addition,

the measurement of upwelled red light (683 nm) is used to estimate the

depth distribution of chlorophyll a and primary production for comparison

with these parameters measured at discrete depths (Kiefer et al., 1989;

Chamberlain et al., 1990).

4.  References

Booth, C. R.  1976.  The design and evaluation of a measurement

system for photosynthetically active quantum scalar irradiance.

Limnology and Oceanography, 21, 326-336.

Chamberlain, W. S., C. R. Booth, D. A. Kiefer, J. H. Morrow

and R. C. Murphy.  1990.  Evidence for a simple relationship

between natural fluorescence, photosynthesis and chlorophyll

in the sea. Deep-Sea Research, 37, 951-973.

Kiefer, D. A., W. S. Chamberlain and C. R. Booth.  1989.

Natural fluorescence of chlorophyll a:  Relationship to

photosynthesis and chlorophyll concentration in the western

South Pacific gyre. Limnology and Oceanography, 34, 868-881.

### SHIPBOARD METEOROLOGICAL OBSERVATIONS AND NDBC BUOY1

_________________________________________________________________

SUMMARY:  Basic meteorological and sea surface

observations are recorded by ship's crew and HOT program

personnel.  These data are used in support of the CTD

measurements and discrete water sample analyses.  In

addition, continuous environmental data are recorded at a

NOAA-NDBC buoy located to the northwest of Station ALOHA.

_________________________________________________________________

1.  Introduction

The surface ocean is inexorably tied to atmospheric forcing,

including such processes as surface winds, solar radiation and heat

flux.  These processes, in turn, affect mixed layer depth and the

distributions of nutrients and microorganisms.  The stochastic

variability of the north Pacific Ocean gyre demands a continuous

observation program to fully assess upper water column variability.

These important environmental data can only be collected with remote

meteorological buoys, equipped with appropriate sensors.

2.  Shipboard Meteorological Observations

Each month while the research vessel is supporting the water

sampling and measurement program at Station ALOHA, basic meteorological

observations (including sea surface temperature, relative humidity,

cloud cover, etc.) are made on 4-hour intervals by the WOCE program

personnel.  In addition to these measurements, the bridge weather logs,

sail-loop data and other meteorological observations made the ship's

officers and crew are also incorporated into the HOT program data base.

Finally, continuous measurements of photosynthetically available

radiance (PAR) are made by the GOFS program personnel for the duration

of each cruise.

3.  NDBC Buoy Measurements

We have recently obtained a nearly continuous 9 year record of

meteorological  and upper water column data from the National

Oceanographic Data Center in Washington, D.C. for environmental

conditions near Station ALOHA.  Since Feb 1981, data have been

collected from NDBC buoy #51001 located to the northwest of Station

ALOHA (23o 23'59"N, 162o 8'00"W).  Data include:  air temperature (in

oC, to nearest 0.1), barometric pressure reduced to sea-level (in mB,

to nearest 0.1), wind speed as 8.5 minutes average (in m sec-1, to

nearest 0.01), wind direction as 8.5 minutes average (in degrees from

true N to nearest 0.1), significant wave height corrected for low

frequency noise (in m, to nearest 0.1), average wave period (in sec,

to nearest 0.1), sea surface temperature (in oC, to nearest 0.01),

wind gust (in m sec-1 to nearest 0.01) and wave spectrum data.

The file structure is the standard NODC 9x120 character records

logged on hourly intervals.  The data, recorded on magnetic tape,

are obtained from NODC, approximately three months after collection.

At the University of Hawaii, the data are translated into ASCII file

format whereupon they are further analyzed by HOT program personnel.

It is also possible to receive these data in real time from the NOAA

weather service as this information is also used for local weather and

surf forecasting.

### LOW LEVEL NITRATE AND NITRATE BY CHEMILUMINESCENCE1

_________________________________________________________________

SUMMARY: Seawater is collected from known depths using

CTD-rosette sampling protocols.  Subsamples are carefully

drawn and stored in acid-washed polyethylene bottles.

Nitrate/nitrite is converted to nitric oxide by wet chemical

reduction in a highly acidic solution.  The nitric oxide

produced is measured by a chemiluminescent detector.

_________________________________________________________________

1.  Principle

Surface water samples (<100m) in oligotrophic waters usually

have nitrate concentrations below the 0.03 µM detection limit of

the Technicon Autoanalyzer.  Except for a narrow band of elevated

nitrite around 100 m (the primary nitrite maximum), nitrite

concentrations are also below standard detection limits.  To achieve

high-precision high-accuracy measurements at these low levels, we

employ the chemiluminescent method of Cox (1980) and Garside (1982).

In this method nitrite and/or nitrate are chemically reduced to

gaseous nitric oxide by an acidic solution of glacial acetic acid

and sodium iodide (nitrite only) or concentrated sulfuric acid,

ferrous ammonium sulfate and ammonium molybdate (nitrate plus

nitrite).  The reduced nitric oxide is carried by an inert carrier

gas (argon) through a cold finger filled with 6M sodium hydroxide,

a drying tube filled with anhydrous sodium carbonate, and a membrane

drier in order to remove acid and water vapors.  The gas stream is

then routed into the chemiluminescent analyzer, where the nitric

oxide is combined with ozone to produce a metastable nitrogen dioxide.

The nitrogen dioxide subsequently emits a photon as it returns to

ground state, and the emitted light is detected by a photomultiplier

tube.  The integrated electrical signal produced by the photomultiplier

is proportional to the content of nitrite (or nitrate plus nitrite)

in the sample.

2.  Precautions

As with the autoanalysis of these nutrients, contamination is the

primary concern.  Because nanomolar concentrations are to be measured,

all sample bottles and laboratory glassware must be meticulously cleaned

with dilute HCl (10%) and distilled deionized water (DDW) before use,

and both the argon carrier gas and the oxygen supplied to the ozone-

generator must be of the highest purity available.  An additional

concern with the chemiluminescent technique is the quenching effect

that water vapor has on the signal.  All drying and scrubbing tubes

must be well maintained to ensure that no water enters the detector.

3.  Sample Collection and Storage

Concerns about the potential "freezing effect" on these samples

are heightened due to the low levels of the nutrients and the high

precision of the measurements.  However, results from experiments

conducted by Constantinou (unpubl.) indicate no evidence for any

discrepancy between replicate samples analyzed immediately on ship

and those frozen and analyzed later in the laboratory.  Samples are

therefore collected and stored exactly as described for autoanalysis

of these nutrients.

4.  Analysis

Nitrate and nitrite analyses are performed on an Antek model 720

nitrogen oxide analyzer.  The reaction apparatus is a slight variation

of that described by Garside (1982).

4.1. [NO2-] determinations

One ml sodium iodide solution (3% wt/vol) and 3 ml glacial acetic

acid are dispensed into the reaction tube.  The reaction tube is

inserted into the carrier gas line and the reagents degassed.

This degassing step removes any contaminating nitrite from the

reagents.  The extent of the degassing is monitored by the

detector signal.  When this step is complete, the detector is

reset and 10 ml of sample or standard are introduced into the

reaction tube with a syringe through a septum fitted to the

side arm of the tube.  The signal produced is integrated and

peak area is recorded.  The waste is then discarded, the

detector reset and the procedure repeated.  Generally each

sample or standard analysis takes about five minutes.

4.2. [NO3- + NO2-] determinations

Ten ml concentrated sulfuric acid (36 N) followed by 2 ml

ferrous ammonium sulfate (4% wt/vol) and 2 ml ammonium

molybdate (2% wt/vol) are dispensed into the reaction tube

(the order of addition is important).  Unlike in the nitrite-

only analysis, the heat generated by the dilution of the acid

is important for the rapid stripping of the nitric oxide

produced from the solution, therefore an insulating sleeve

is used around the reaction tube.  The reagents are degassed

as above, and the sample added immediately after degassing

is complete, in order to maintain an elevated reaction

temperature.  If the reaction temperature gets too low,

the sample may not degas rapidly enough for quantitative

results.  A strip chart recorder is useful for troubleshooting:

if asymmetrical peak tailing is observed, the reaction

temperature is probably not being maintained at a high enough

level (Garside, 1982).  The signal is integrated and recorded

as for the nitrite-only analysis.

4.3. [NO3-] determinations

Our preferred method for low level nitrate determination is to

analyze the samples for both [NO3- + NO2-] and [NO2-] and

calculate [NO3-] by difference.  An alternative method is to

first bind the nitrite in the sample with sulfanilamide to

form a diazonium salt, then analyze the sample for nitrate

plus nitrite.  0.2 ml of a sulfanilamide solution (1% wt/vol

in dilute HCl) is added to the sample and allowed to react

with the nitrite for at least two minutes.  Then the treated

sample is analyzed as for [NO3- + NO2-] above.  The

disadvantages of using the latter method are that:  (1) a

small blank correction for contaminating nitrate in the

sulfanilamide solution must be determined and applied to the

results and (2) some precision is sacrificed in using the [NO3-

+ NO2-] assay twice instead of using the [NO3- + NO2-] assay

once and the more precise [NO2-] assay once.  The advantage is

that two separate sets of analyses involving the time-consuming

generation of two standard curves are not required.

5.  Calibration, Data Reduction and Calculations

5.1. Calibration stocks and regression standards

The calibration of the low level nitrate/nitrite analysis is

performed using standard solutions of NO3- or NO2- in DDW.

Dried (60oC, 72 hours) analytical grade reagent  chemicals are

dissolved in DDW in 500 ml acid-washed glass volumetric flasks.

A few drops of chloroform are added to inhibit microbial

activity.  The stock solutions are stored at 4oC  and discarded

about every three months.  The reagent chemicals and concentrations

are KNO3 (10 mM) and NaNO2 (10 mM).

Working standards are prepared fresh by volumetric dilutions of

the stock using acid-washed glass pipettes and flasks.  Since

there is no salt effect in the chemiluminescent analysis, working

standards are all prepared using DDW.  These working standards

are used to generate a standard curve, and are analyzed at

intervals throughout the sample run in order to detect any drift

in the detector response.  Typical correlations produce r2 values

of 0.9999 or better.

In order to maintain the accuracy of the analysis, a few certified

reference standards (CSK or equivalent), diluted volumetrically

with DDW until within the concentration range of interest, are

included in every sample run in case stock solutions become

contaminated or microbially altered during storage.

5.2. Blank corrections

For the [NO3- + NO2-] analysis, a small blank is observed due

to traces of nitrate in the DDW diluent.  This blank is corrected

for in the calculation of standard concentrations, but is not

subtracted from sample results because they contain no diluent.

For samples, therefore, only the slope of the regression line

is used.  For the [NO2-] analysis, no such diluent blank is

observed.  If the sulfanilamide method is used for [NO3-]

measurements, the blank due to the sulfanilamide solution should

be determined and subtracted from results.

6.  Precision and Accuracy

The detection limit for nitrate plus nitrite is approximately

1-2 nM.  The nitrite-only analysis produces less noisy signals;

standards with as little as 0.4 nM nitrite and samples with as

little as 0.1 nM nitrite have been detected.  Precision and

accuracy of the nitrate plus nitrite analysis are approximately

2-3 nM while for nitrite-only they are generally <2 nM.

7.  Equipment/Supplies

Niskin bottles and rosette/CTD unit

acid-washed 125 ml polyethylene bottles

nitrogen oxide analyzer (Antek model #720, operated in vacuum mode)

reaction apparatus including cold finger and drying tube

ultra-high purity O2 and Ar, with drierite/ascarite scrubbers

volumetric flasks and pipettes, syringe, septa, and glass beakers

8.  Reagents

glass distilled deionized water (DDW)

dilute HCl (10%): add 500 ml concentrated reagent grade HCl to 4500

ml DDW.

glacial acetic acid

sodium iodide (3% wt/vol): dissolve 3 g reagent grade NaI in 100 ml DDW.

Prepare fresh daily.

concentrated H2SO4 (36 N)

ferrous ammonium sulfate (4% wt/vol): dissolve 4 g reagent grade

ferrous ammonium sulfate in 100 ml DDW.  Prepare fresh daily.

ammonium molybdate (2% wt/vol): dissolve 2 g reagent grade ammonium

molybdate in 100 ml DDW.  Prepare fresh daily.

sulfanilamide (1% wt/vol in 10% HCl): dissolve 1 g reagent grade

sulfanilamide in 100 ml 10% HCl.

sodium hydroxide (6 M): dissolve 240 g reagent grade sodium hydroxide

and make up to 1 l with DDW.

nitrite stock solution (10 mM): dissolve 0.345 g dried NaNO2 in 500 ml

DDW.  Add 0.5 ml chloroform and store in the dark at 4oC.

nitrate stock solution (10 mM): dissolve 0.506 g dried KNO3 in 500 ml

DDW.  Add 0.5 ml chloroform and store in the dark at 4oC.

CSK (or equivalent) certified reference standards for seawater:  NO2-

and NO3-.

9.  References

Cox, R. D.  1980.  Determination of nitrate at the parts per billion

level by chemiluminescence.  Analytical Chemistry, 52, 332-335.

CSK Certified Seawater Reference Standards.  Sagami Chemical Research

Center, Sagimahara, Japan.

Garside, C.  1982.  A chemiluminescent technique for the determination

of nanomolar concentrations of nitrate and nitrite in seawater.  Marine

Chemistry, 11, 159-167.

### TITRATION ALKALINITY1

_________________________________________________________________

SUMMARY: Seawater is collected from discrete depths

using PVC sampling bottles attached to the CTD/rosette

system.  Subsamples of approximately 50 ml are measured

gravimetrically and titrated with 0.1 N HCl using an

automated potentiometric titration system.  Total

alkalinity is determined using a Gran plot corrected

for interference from bisulfate and hydrogen fluoride.

_________________________________________________________________

1.  Principle

Titration alkalinity is one of four parameters needed to describe

the carbonate system in seawater.  Titration alkalinity is therefore

useful for assessing the magnitude and direction of ocean-atmospheric

carbon dioxide flux and the saturation state of calcium carbonate

within the interior of the ocean.  Titration or total alkalinity is

usually defined as the quantity of hydrogen ions in millimoles (mmol)

required to neutralize the weak bases in 1 kilogram of seawater

(Grasshoff, 1983).  A more exact definition (Dickson, 1981) of total

alkalinity is the quantity of hydrogen ion required to neutralize

bases formed from weak acids with pKs of a > 4.5.  Our determination

of total alkalinity employs the potentiometric titration of a seawater

sample with hydrochloric acid.  Our methods are slight modifications

of those developed for seawater analysis (Grasshoff, 1983 and references

therein).  Our titration procedure employs an open cell and a computer-

controlled titration system .  The computer-controlled system

incorporates an automated high-precision burette and pH meter.

2.  Precautions

Alkalinity samples should be drawn directly from the Niskin bottle

using clean drawing tubes.  Care should be taken to avoid contamination

from ship's equipment or the atmosphere.

3.  Water Sampling

Alkalinity samples are collected using the same techniques employed

when collecting DIC samples (see Chapter 6).

3.1. Drawing the sample

3.1.1. Samples are drawn into a clean 300 ml glass reagent bottle.

3.1.2. The bottle should be flushed with at least one volume of water.

3.1.3. The tube is withdrawn so that the bottle remains brimful when

the tube is completely withdrawn.

4.  Preserving the Sample

4.1. Some of the sample is removed from the reagent bottle using a 10

ml pipet.  Enough water is removed so that about 1 cc of air is

contained in the bottle when the glass stopper is inserted.

4.2. 100 µl of a saturated HgCl2 solution is added to the sample.

The tapered ground glass bottle neck is dried with a kimwipe

and the bottle is sealed with a ground glass stopper covered

with light coating of ApiezonR grease.  The stopper is pressed

firmly into the bottle to make a good seal and is secured with

polyethylene tape of a large rubber band.

5.  Potentiometric Titration

5.1. The sample is brought to 25oC and approximately 50 ml is placed

in a tared 100 cc beaker.  The weight of the sample is recorded

to the nearest milligram.

5.2. A clean stirbar is added to the beaker and the beaker is placed

on a stirplate.  The antidiffusion burette tip and the

potentiometric electrode is placed in the beaker.

5.3. The temperature of the stirred sample is measured to the nearest

0.1oC.  The temperature of the sample is recorded and the

automated titration is initiated.

5.4. The automated titration procedure adds a 1 to 1.5 ml aliquot of

approximately 0.1 N HCl depending upon the anticipated sample

alkalinity.  After reaching a stable mv reading, the mv value is

recorded and a 15 microliter HCl aliquot is added.  After the

electrode output stabilizes a second 15 µl aliquot is added.

This process is continued until the mv value reaches a

predetermined value beyond the V2 inflection point.  Typically

30 to 35 aliquots are added to reach the predetermined mv value.

6.  Computation of Total Alkalinity

Total alkalinity is determined from the volume of acid required

to reach the second endpoint and the acid normality.  The inflection

point (V2) is determined using a Gran plot (Gran, 1952) which corrects

for the interference from both bisulfate and hydrogen fluoride.  Sulfate

and fluoride concentrations are calculated from salinity and the

equilibrium constants for bisulfate and hydrogen fluoride (Khoo et al.,

1979; Dickson and Riley, 1979).

7.  Preparation of HCl

The accuracy of the total alkalinity measurement is determined, in

large part, by the uncertainty in the normality of the HCl.  The

approximately 0.1 N HCl solution is made by dilution of concentrated

HCl into a 0.7 M solution of high-purity NaCl.  The normality of the

HCl solution is ascertained by titration of solutions made from dried

high-purity sodium carbonate and borax.

8.  Precision and Accuracy

The precision of our total alkalinity determinations is approximately

+4 µequiv/kg.  This gives a coefficient of variation of approximately

0.2% for typical seawater samples.  Because we lack a liquid seawater

standard of known alkalinity, the accuracy of our total alkalinity

determinations is not well known.  In order to help ensure the accuracy

of our alkalinity measurements we regularly collect intercalibration

samples for Dr. Keeling's laboratory at Scripps Institution of Oceanography.

To date, alkalinity values for replicate samples analyzed in both

laboratories are typically within 10 µequiv/kg.

9.  Equipment and Supplies

kimwipes and applicator sticks

300 ml ground glass stoppered reagent bottles

10 ml automatic pipet

Apiezon grease

analytical balance

volumetric flasks, 100 ml beakers, 50 ml glass pipets and mercury

thermometer

Automated titration system

PC and software

Brinkmann model 655 Dosimat and 5 ml top

Orion model 940 pH meter

Orion combination electrode

stir plate

10.  Reagents

distilled deionized water (DDW)

high purity sodium carbonate

high purity sodium chloride

reagent grade hydrochloric acid

11.  References

Almgren, T., D. Dyrssen and S. Fonselius.  1983.  Determination

of alkalinity and total carbonate.  In: Methods of Seawater

Analysis, M. E. K. Grasshoff and K. Kremling, editors, Verlag-

Chemie, pp. 99-123.

Dickson, A. G.  1981.  An exact definition of total alkalinity

and a procedure for the estimation of alkalinity and total

inorganic carbon from titration data.  Deep-Sea Research, 28A,

609-623.

Dickson, A. G. and J. P. Riley.  1979.  The estimation of acid

dissociation constants in seawater media from potentiometric

titrations with strong base. TheThe ionic product of water (Kw).

Marine Chemistry, 7, 89-99.

Gran, G.  1952.  Determination of the equivalence point in

potentiometric titration. Part II.  Analyst, 77, 661-671.

Khoo, K. H., R. W. Ramette, C. H. Culberson and R. G. Bates.

1977.  Determination of hydrogen ion concentrations in seawater

from 5 to 40oC:  standard potentials at salinities from 20 to

45 o/oo.  Analytical Chemistry, 49, 29-34.

### MEASUREMENT OF pH1

_________________________________________________________________

SUMMARY: Seawater samples are collected at discrete

depths with PVC sampling bottles attached to a CTD/

rosette system.  Seawater samples are brought to

constant temperature and pH is determined from the

millivolt output of an Orion reference electrode

calibrated with buffer solutions.  The buffer

solutions are prepared in a special matrix which

mimics the ionic characteristics of seawater.

_________________________________________________________________

1.  Principle

The pH (-log of the hydrogen ion concentration) is a master

variable for describing the acid-base equilibria in seawater.  The

pH of seawater is dependent upon the in situ chemical, physical and

biological conditions, and pH is also one of four variabilities

which can be used to describe the carbonate system in seawater.

This parameter is therefore useful for examining the exchange of

carbon dioxide across the air-sea interface and for the calculation

of the saturation state of calcium carbonate in the interior of the

ocean.  pH is a unique physiochemical quantity because it involves

the activity of a single ion which cannot be measured directly.  As

a consequence, pH is defined operationally in terms of the method

by which it is measured.  In order for pH measurements to be

interpreted correctly the reference scale must be precisely defined.

In this procedure, pH is measured electrochemically using a combination

electrode.  Measurements are made relative to seawater buffers prepared

as described.

2.  Precautions

Because pH is dependent upon temperature, the temperature of the

pH sample must be carefully controlled, or at least accurately measured

before pH is determined.  In addition, the pH of a seawater sample

changes if CO2 is lost or gained on contact with the atmosphere.  This

is a concern particularly with deep water samples where CO2 concentrations

are high.  It is therefore important that samples be drawn as soon as

possible after arrival of the rosette on deck.  Furthermore, contact

with the atmosphere should be avoided during sampling and sample bottles

should remain tightly sealed until pH measurements are made.

3.  Water Sampling

3.1. pH samples are collected from the PVC sampling bottles into a

clean 200 ml polyethylene bottle using a Tygon drawing tube to

minimize sample contact with the atmosphere.

3.2. The sample bottle is overflowed with at least one volume before

the sample bottle is tightly capped.

3.3. The air space with the sample bottle is kept to a minimum in

order to minimize CO2 exchange with the head space.

4.  Sample Analysis

4.1. pH sample bottles are immersed in a constant temperature water

and brought to 25oC.

4.2. After reaching constant temperature, the sample bottle is opened

and a clean stir bar is placed in the bottle.

4.3. The bottle is placed on a stir plate and the sample temperature

is measured to 0.1oC.

4.4. The potentiometric electrode is placed in the bottle and the mv

output is recorded after the reading stabilizes.

5.  Electrode calibration and calculation of pH

The potentiometric electrode is calibrated with the seawater

buffers 2-amino-2-hydroxymethyl-1,3-propanedion (tris) and 2-amino-2-

methyl-1,3-propanediol (bis).  These buffers are made as described by

Dickson (1991) and pH is computed on the "total" hydrogen scale.  The

electrode slope and an isoelectric point is computed from the mv outputs

when the electrode is immersed in the seawater buffers as described in

Grasshoff (1983).  The pH of seawater samples is calculated using the

electrode slope and isoelectric point and sample temperature.

6.  Precision and Accuracy

Because of a variety of problems inherent in electrometric pH

measurements, including electrode drift, electromagnetic interference

and problems with the reference electrode, the precision of these pH

measurements is relatively poor.  On average, we obtain a precision of

+0.02 pH units on replicate samples.  The accuracy of our pH measurements

are difficult to evaluate directly because we have no seawater standard

for pH measurements.  The accuracy is therefore dependent primarily on

the accuracy of the seawater buffers that are used for electrode

calibration.  In order to improve the precision of our time-series pH

measurement data, we are currently evaluating the spectrophotometric

methods for pH measurements described by Byrne et al. (198_).  Although

these measurements are currently being made on a regular basis, the

methodological details are not finalized and are not described here.

7.  Equipment and Supplies

Orion combination electrode

Orion model 940 pH meter

250 ml polyethylene bottles

stir plate and stir bars

water bath at 25oC

chemicals for seawater buffers

NaCl

KCl

MgCl2

CaCl2

Na2SO4

HCl

2-amino-2-hydroxymethyl-1,3-propanediol (tris)

2-amino-2-methyl-1,3-propanediol (bis)

8.  References

Dickson, A. G. 199_.  pH buffers for seawater media based on the

total hydrogen ion concentration scale.  Deep-Sea Research.

Grasshoff, K.  1983.  Determination of pH.  In: Methods of Seawater

Analysis, K. Grasshoff and K. Kremling, editors, Verlag-Chemie, pp.

85-97.

### ALKALINITY1

_________________________________________________________________

SUMMARY: Seawater samples are collected at discrete

depths using CTD-rosette sampling protocols.  Subsamples

for alkalinity are collected and immediately preserved

with HgCl2 for subsequent analysis in the laboratory.

The alkalinity is determined by a potentiometric titration,

and the second end point (V2) is determined with a

modified Gran Plot.

_________________________________________________________________

1.  Principle

The total alkalinity is defined as the number of milliequivalents

of H+ required to titrate one kilogram of seawater to the bicarbonate

equivalence point (~pH 4.5).  The classical chemical formulation

(Harvey, 1966) of this definition is:

Alk(t) = [HCO31-] +2[CO32-] +[B(OH)41-] +[OH1-] -[H+]

In high precision work corrections for other proton acceptors

(e.g. [H3PO3-] and [SiO(OH)3-), should be taken into account (Edmond,

1970; Dickson, 1981; Bradshaw and Brewer, 1988).  From the knowledge

of the total alkalinity, dissolved inorganic carbon, salinity, and

nutrient content of seawater the entire carbonate "balance" can be

calculated from the dissociation constants of carbonic acid, boric

acid and other interacting species.

2.  Precautions

Samples should be drawn as soon as possible and preserved to halt

biological activity.  Samples bottles should be completely sealed and

stored in the dark cool place.

3.  Water Sampling

3.1. Drawing the sample

3.1.1. Samples are drawn into combusted 300 ml glass reagent bottles

in the same manner as dissolved oxygen.

3.1.2. The drawing tube is completely filled with sample while

manipulating the tube to dislodge bubbles.  The drawing tube

is flushed and inserted to the bottom of the sample bottle.

3.1.3. The sample bottle is overflowed with at least two volumes

of sample.

3.1.4. The tube is slowly withdrawn with the sample flowing.

3.2. Preserving the sample

3.2.1. Use a pipet to remove water from the neck of the sample

bottle of sample.  Enough water is removed so that

approximately 1 ml of air is contained in the bottle when

the glass stopper is inserted.

3.2.2. 100 µl of saturated HgCl2 is added to each sample.  The

tapered ground glass bottle neck is dried with a Kimwipe

wrapped on an applicator stick.  The bottle is sealed with

a ground glass stopper coated with a light covering of

Apiezon grease.  The stopper is pressed firmly into the

bottle to make a good seal, and is secured with polyethylene

tape or a large rubber band.

4.  Calibration of Electrode (NBS and SWS)

4.1. NBS

4.1.1. Soak electrode in storage solution overnight.

4.1.2. Place electrode in buffer 7 and soak for 15 minutes.

4.1.3. Pour out old buffer and replace with fresh buffer.

4.1.4. Record relative millivolt reading when electrode stabilizes.

4.1.5. Take temperature of buffer.

4.1.6. Repeat steps 2 through 5 with buffer 4.

4.2. Seawater buffer preparation

4.2.1. 0.4 M Tris and 0.4 M Bis seawater buffers are prepared

according to the method of Dickson (1992).

4.2.2. NaCl, KCl, and NaHSO4 are dried and added gravimetrically.

4.2.3. MgCl2 and CaCl2 are added volumetrically from stock

solutions that have  been standardized with AgNO3.

4.2.4. HCl is added from a 0.1 M stock solution carefully prepared

from a Dilut-it.

4.2.5. Buffers are stored in tightly capped 250 ml polyethylene

Nalgene bottles.

4.3. Calibration

4.3.1. Place electrode in Tris buffer overnight.

4.3.2. Pour out old buffer and replace with fresh buffer.

4.3.3. Record relative millivolt reading when electrode stabilizes.

4.3.4. Take temperature of buffer.

4.3.5. Place electrode in Bis buffer for approximately 15 minutes.

4.3.6. Repeat steps 2 through 6.

5.  Preparation and Standardization of Acid Titrant

5.1. Volumetrically prepare a solution of 0.1000 N HCl in 0.7 M

sodium chloride.

5.2. Standardize the above solution against recrystalyzed sodium

tetraborate (borax) which is stored in a dessicator over a

saturated solution of sodium chloride and sodium citrate

(Vogel, 1978).

5.3. Prepare at least two concentrations of borax primary standard.

5.3.1. Make a 0.0035 to 0.004 M solution of sodium tetraborate.

5.3.2. Weigh out approximately 25 g of the borax solution and

titrate in a similar manner to a seawater titration.

5.3.3. Calculate the endpoint using a Gran Plot.

5.4. Calculate normality of the acid titrant using the known quantity

and concentration of borax primary standard.

5.5. As a final check on the accuracy of our acid standardization,

a subsample is sent to Dr. Andrew Dickson's laboratory at

Scripps Institution of Oceanography for high precision

coulometric determination of the acid normality.

6.  Titration of Samples

6.1. Turn on computer, Dosimat, printer, and pH meter.  Load the

alkalinity program.  Dispense approximately 20 ml of acid

through the Dosimat to purge any air bubbles and to pump fresh

acid into the lines.

6.2. Throughly rinse and dry a 50 ml beaker.

6.3. Place beaker on balance and tare.  Weigh out approximately

50 grams of sample.

6.4. Rinse electrode with seawater and gently dry by touching the

bottom of the electrode with a Kimwipe.  Do not wipe the

electrode dry.

6.5. Place sample on magnetic stir plate.  Put stir bar into sample

taking care not to splash any sample out of the beaker. Adjust

to moderate stirring.  Arrange electrode and buret tip as to

not interfere with the stir bar but to make sure electrode

junction and tip are submerged in sample.

6.6. Start titration program.  The program will request a preadd

volume.  This volume should be sufficient to adjust the pH of

the sample to about 4.  The program will then add HCl in 0.015

ml increments and record the millivolt reading after each

increment is added.  The titration will automatically terminate

when a specified millivolt reading (equivalent to pH 3.0) is

reached.

6.7. After the titration has terminated, check and record the sample

temperature.

6.8. Remove beaker from stir plate and pour out sample.  Rinse with

tap water for approximately 3 minutes and then rinse with

deionized distilled water.  Dry beaker for next sample.

7.  Calculations

Data is fit by a modified Gran Function (f2=(Vo+V)([H+]+[HS04-]+[HF]))

using Matlab.  A best fit Eo and slope are obtained by minimizing the sum

of the residuals for those points in the range pH 3-3.5.  The accuracy

of the calculated slope is determined by comparing to the measured slope

for the buffers described above.

8.  Quality Control

As a safeguard on the quality of our results, we maintain a set of

secondary standards which are run with each analysis.  These are made

from a large surface seawater sample, which is preserved with HgCl2

and subdivided into 300 ml reagent bottles.  These are sealed as

described above and stored in a cool dark place.

9.  Precision and Accuracy

The precision of our titration procedure is approximately 3 µeq/kg.

An absolute alkalinity standard is not yet available, and the accuracy

of the procedure is determined primarily by the standardization for

the normality of the titrant.  We  intercalibrate the determination

of the acid normality with Dr. Andrew Dickson's laboratory at Scripps

Institution of Oceanography, where high-precision coulometric methods

are used to determine the acid normality.

10.  Equipment and Supplies

Niskin bottles and rosette

300 ml combusted reagent bottles

Apiezon grease

Computer interfaced 665 Dosimat and Orion EA 940 Ion Analyzer

Calibrated volumetric glassware

Analytical and topload balance

Magnetic Stirrer

Thermometer

Glass Electrodes

11.  Reagents

Hydrochloric acid (0.1000 N)

Sodium tetraborate (borax)

Distilled and deionized water

NaCl

HgCl2 (saturated solution)

12.  References

Bradshaw, A. L. and P. G. Brewer.  1988.  High precision

measurements of alkalinity and total carbon dioxide in

seawater by potentiometric titration:  Presence of unknown

protolytes?  Marine Chemistry, 23, 69-86.

Dickson, A.  1981.  An exact definition of total alkalinity

and a procedure for the estimation of alkalinity and total

inorganic carbon dioxide from titration data.  Deep-Sea

Research, 28A, 609-623.

Dickson, A.  1992.  pH buffers for seawater media based on

the total hydrogen ion concentration scale.  Deep-Sea Research,

submitted.

Edmond, J. M.  1970.  High precision determination of titration

alkalinity and total carbon dioxide content of sea water by

potentiometric titration.  Deep-Sea Research, 17, 737-750.

Harvey, H. W.  1966.  The Chemistry and Fertility of Sea Waters.

Cambridge Univ. Press, 240 pp.

Vogel, A. I.  1978.  Textbook of Quantitative Inorganic Analysis,

4th Ed., pp. 300-301. Longman, New York.

### Particulate S82

Particulate elemental sulfur (S8) is analyzed by ion chromatography after gravity filtering 200 ml samples from a Niskin bottle through a 0.2 m polycarbonate filter.  Filters are kept frozen (-20oC) until extracted in 5 mM sulfite solution for 2.5 hours to react all elemental sulfur with sulfite to thiosulfate (Luther III et al., 1991).

The method of (Rozan and Luther 2001) can be adapted for thiosulfate analysis using anion chromatography with subsequent spectroscopic detection at a wavelength of 215 nm.  The elutent is 8 mM perchlorate, at a flow rate of 2 ml/min.  Thiosulfate retention time is 7.3 minutes.

### Salinity, pH, & electrochemistry2

- Refractometer

- pH Electrode

- Blunt glass microelectrode

- Reference electrode (Ag/AgCl)

- Counter electrode (Pt)

- DLK-100A + laptop

- Faraday cage

- Buffers (4, 7, and 10) in NaCl 0.54 M

- Thermocouple probe

- 0.5 ml glass cell

#### Electrochemistry (O2, H2S, FeS, Fe2+, Mn2+, S0) and pH

- Before the sub comes back: Both the voltammetric and pH electrodes must be calibrated (O2 and Mn(II) for voltammetric electrode and two pH buffers in NaCl 0.54 M for pH electrode).

- Approximately 0.5 ml is directly filtered in the glass cell (minimize the mixing of samples to avoid degassing H2S)

- Electrochemical measurements are performed with a blunt Au/Hg glass electrode.

- pH is then recorded with the DLK-100A.

- Add 5-10 l of 0.5 N HCl, degas H2S with nitrogen and measure S(0).

### Iron: Fe(II), total dissolved Fed, total FeT 2

- UV-vis. Spectrophotometer + Mr. Sipper

- Ferrozine (0.01 M)

- Ammonium Acetate (5 M)

- Hydroxylamine (0.2 M in 0.1 M HCl)

- 36 x 5 ml Falcon tubes (samples) + 5 ml tube racks

- 7 x 14 ml Falcon tubes (standards) + 14 ml tube racks

- 100 ml graduate cylinder + 150 ml PP bottle for Ferrozine mix solution

#### Fe2+, total dissolved Fe (Fed), and total Fe (FeT)

- Before the sub comes back: 100 ml of a ferrozine-acetate reagent is prepared by adding 50 ml of 0.01 M Ferrozine to 50 ml 5 M Ammonium Acetate.

- 100 ml of a 0.2 M hydroxylamine in 0.1 M HCl is prepared.

- 10 ml of a stock solution of 10-3 M Fe2+ is prepared in 0.01 M HCl (degassed milli-Q water to which 0.2 ml of 0.5 M HCl is added).

- Five standards are prepared from 0 to 20 M Fe2+: 2.5 ml degassed milli- Q water is mixed with 2.5 ml of ferrozine-acetate mix in 14 ml Falcon tubes. Aliquots of the Fe2+ stock solution are added to make up the appropriate concentration. The color development takes 5 min. The standards can be used for a couple of hours. Measure the absorbance at 562 nm.

- 100 l of filtered sample is injected in 5 ml Falcon tubes containing 1 ml of degassed milli-Q water and 1 ml of ferrozine-acetate mix.

- 100 l of filtered sample is injected in 5 ml Falcon tubes containing 1 ml of milli-Q water and 100 l of hydroxylamine mix. These samples are sealed and stored at overnight at ambient temperature. They should be analyzed the next day for Fe(II) by the Ferrozine method.

- 100 l of unfiltered sample is injected in 5 ml Falcon tubes containing 1 ml of milli-Q water and 100 l of hydroxylamine mix. These samples are sealed and stored overnight at ambient temperature. They should be analyzed the next day for Fe(II) by the Ferrozine method.

### H2S by UV-vis 2

- N,N-dimethyl-p-phenylenediamine sulfate

- Solid FeCl3

- HCl (50%)

- 24 x 5 ml Falcon tubes spiked with reagents for H2S

- 7 x 14 ml Falcon tubes for H2S standards

- 500 ml solution of the diamine reagent is prepared by diluting 2 g. of N,N-dimethyl-p-phenylenediamine sulfate and 3 g. of FeCl3 in a solution of 50% HCl. The solution is kept in a dark bottle and must be stored in a refrigerator.

- A stock solution of 10-2 M H2S is prepared in a 100 ml volummetric flask by adding the washed and dried crystals to degassed milli-Q water. From this stock solution, a standard of 1 mM H2S is prepared in a 14 ml Falcon tube containing 10 ml of degassed water. These stock solutions are prepared freshly and used immediately.

- Five standards are prepared from 0 to 40 M H2S: 5 ml of degassed milli- Q water is mixed with 0.4 ml of the diamine reagent in 14 ml Falcon tubes. Aliquots of the H2S stock solution are added to make up the appropriate concentration. The color development takes 20 min. The standards can be used for a couple of hours. Measure absorbance at 670 nm.

- Prepare 12 Falcon tubes (5 ml) for H2S analysis by UV-vis. Add 2 ml degassed milli-Q water and 160 l of diamine reagent to the tubes.

- Place 100 l of the unfiltered sample in the H2S Falcon tubes and mix. The color development takes 20 min. The samples should be analyzed within two hours.

- Place 100 l of the filtered sample in the H2S Falcon tubes and mix. The color development takes 20 min. The samples should be analyzed within two hours.

### FeS & FeS22

#### FeS and FeS2

- Zinc acetate 0.2 M solution

- 24 x 1.5 ml Eppendorf tubes

- 0.5 M NaOH

#### FeS and FeS2

- Before the sub comes back: prepare 24 x 1.5 ml Eppendorf tubes and add 100 l of 0.2 M zinc acetate and 10 l of 0.5 M NaOH.

- Add 1 ml of the 12 unfiltered samples to the Eppendorf tubes. Shake and store in the refrigerator.

- Add 1 ml of the 12 filtered samples to the Eppendorf tubes. Shake and store in the freezer.

### Bimane (thiols, thiosulfate, polysulfides)2

- 12 x 0.6 ml amber tubes

- Heatblock

- 50 mM HEPES with 5 mM EDTA buffer

- 10 mM monobromobimane

- 50% acetonitrile

- 65 mM methane sulfonic acid

- timer

- Before the sub comes back: make a pH 8 buffer with 50 mM HEPES and 5 mM EDTA in 50% acetonitrile.

- Add 100 l of the HEPES and EDTA buffer to the 0.6 ml amber tubes.

- Add 10 l of 10 mM bimane stock solution (for samples with less than 500 M thiols).

- Add 50 l unfiltered sample to the 0.6 ml amber tubes.

- Let the samples derivatize for 30 min in the dark.

- Put the samples in a 60oC heatblock for 10 min.

- Add 100 l of 65 mM methane sulfonic acid.

- Store frozen at -80 or in liquid nitrogen until analysis.

### Sulfate, nitrate, nitrite, ammonium2

#### Sulfate, nitrate, nitrite, ammonium

- HCl (0.05 M)

- 12 x 0.5 ml Eppendorf tubes + tube rack

- Before the sub comes back: a 0.05 M HCl stock solution is prepared by diluting 1 ml of 0.5 M HCl in a total of 10 ml milli-Q water.

- 0.5 ml Eppendorf tubes are spiked with 50 l of 0.05 M HCl (pH = 2).

- 400 l of filtered sample is added to the 0.5 ml Eppendorf tube.

- Purge the samples for 2 minutes with nitrogen (use a Pasteur pipette).

- Samples are frozen until analysis.

### Chloride & phosphate2

- 12 x 1.5 ml Eppendorf tubes + tube rack

- Add 1 ml of filtered sample to 1.5 ml Eppendorf tubes (No acid).

- Samples are frozen until analysis.

### Metals by ICP (Mg, Mn, Fe, Co, Ni, Zn)2

- Trace metal grade concentrated HNO3

- Trace metal grade concentrated HCl

- 24 x 5 ml Falcon tubes + 5 ml tube rack

- Before the sub comes back: add 50 l concentrated trace metal free HNO3 to twelve 5 ml Falcon tubes. Add 50 l concentrated trace metal free HCl to the other twelve 5 ml Falcon tubes.

- Add 2.5 ml of filtered samples in Falcon tube with HCl and 2.5 ml to Falcon tubes with HNO3.

- Add remaining of unfiltered samples in Falcon tubes. Add appropriate acid to have a total of 0.1 M HNO3.

- Samples are refrigerated until analysis.

# Field & Laboratory Protocols Sources

1 – HOTS - http://hahana.soest.hawaii.edu/hot/hot_jgofs.html

2 – Luther, III