Identification_Information:

Citation:

Citation_Information:

Originator: NOAA Pacific Islands Fisheries Science Center, Coral Reef Ecosystem Division, Pacific Islands Benthic Habitat Mapping Center

Publication_Date: December 2005

Title: Optical Validation Data at the Sarigan Island in the Commonwealth of the Northern Mariana Islands to Support Benthic Habitat Mapping

Geospatial_Data_Presentation_Form: raster digital data

Online_Linkage: http://www.pifsc.noaa.gov/cred/hmapping/

Description:

Abstract: Optical validation data were collected using a Tethered Optical Assessment Device (TOAD), an underwater sled equipped with an underwater digital video camera and lights. Data were collected in the Commonwealth of the Northern Mariana Islands (CNMI), around 18 different islands and banks, to support Benthic Habitat Mapping efforts during NOAA Ship Oscar Elton Sette cruise OES0307, from August 22 through September 21, 2003.

Purpose: These data provide optical observations that will be correlated with bathymetry and acoustic backscatter imagery to develop a benthic habitat map of Maug Island. Refer to supplemental information for description of instrument and survey.

Supplemental_Information:

Sarigan Island is an uninhabited volcanic cone with a land area of 4.9 square kilometers. The island has experienced no terrestrial impacts since the removal of feral animals, although fishery resources continue to be harvested. Since the elimination of feral animals on this island (1997-1998), the vegetation has dramatically recovered, presumably reducing sediment runoff. According to the NOAA MARAMP towed-diver surveys, continuous reef areas along the east and south shores contained roughly 50% live coral cover. The MARAMP dive at Sarigan revealed a layer of sediment that had been deposited from the 2003 Anatahan eruption.

Optical validation data were collected using the Tethered Optical Assessment Device (TOAD), a sled equipped with underwater video camera and lights. These data are used to provide ground-truth validation for benthic habitat maps based on multibeam echosounder surveys. Camera sled deployments were conducted at night, usually between 1800 and midnight. The duration of each tow varied but averaged about 40 minutes of bottom time at a given location. The camera sled was deployed from the port J-frame mounted amidships on the NOAA Ship Oscar Elton Sette. At each station the ship was positioned with the wind on the port side and drifted downwind; occasional light turns were applied to the ship’s screws if necessary to reduce the ship’s motion.  The TOAD was lowered slowly to the bottom by the deck crew using a capstan.  The operator monitored a live video feed from the camera and began recording data on a video tape recorder.  When the camera reached bottom the deck crew was notified by radio to stop lowering.  The operator continued to monitor the vehicle and provided commands to raise or lower it to keep the camera just above the bottom.
     
Equipment Description: The TOAD was deployed from the lower arm of the 
vessel’s port side J-frame.   The TOAD is a camera sled based on the Guildline MiniBat model 8820 tow body.  The frame has been extensively modified from its original configuration and was equipped with an ROS model 54-00100-13 color underwater video camera as the primary data collection instrument. The ROS camera was mounted to point at approximately a 45 degree angle toward the seafloor. A Deep Sea Power and Light model 2050 MultiSeaCam low-light color video camera was also mounted on the sled and aimed straight ahead. The signal from this camera was fed to a second video monitor to provide warning of underwater obstructions the sled might be headed for. Illumination was provided by two 500 W DeepSea Power & Light Multi-SeaLite model 1050 underwater lights mounted on the original sled frame. The lights were located near the base and each side of the sled to provide the maximum possible horizontal distance from the ROS camera. Cable between the sled and the surface consisted of a underwater electrical cable (cable 2, black in color) with an internal kevlar strength member to support the sled frame.  The cable was led from the camera sled over a 22-inch diameter sheave hung from the J-frame, and from there around a gypsy head mounted on the alternate CTD winch, amidships on the vessel's port side. All TOAD surface components were located in the Dry Lab in an equipment rack on the after bulkhead.

Name & address of person collecting data:

John Rooney

NOAA PIFSC

1125B Ala Moana Blvd

Honolulu, HI 98614

Data Files: Video data were recorded on a video tape recorder.  The position of the camera sled was recorded in WGS-84 using Guildline MiniBat In-Tow data acquisition software and a data feed from a shipboard Northstar Chartplotter.

File naming convention: Each tow is given a name consisting of a 3-letter designator for the island area followed by a tow number, which increments by one for each new tow around that island.  For Sarigan Island the designator is "SAR."During OES0307 (NOAA Ship Oscar Elton Sette’s 7th cruise in calendar year 2003) the consecutive numbers started at 100. Video tape labels, the navigation files (*.glo) and paper log forms are annotated with the tow name and number, e.g., SAR102. If the navigation file is edited during processing the file name has a suffix ‘a’ added.  For example, for a navigation data file named “SAR102a.glo,” the ‘a’ would indicate that metadata were extracted from the navigation data and recorded to a file with the same name as the navigation file except that a file type of ‘.met’ was appended; for example, ‘SAR102a.glo.met.

Time Correlation: All times are based on UTC.  Two clocks were manually synchronized prior to starting data collection; the clock in the video character generator that was used to annotate the video tape, and the clock in the TOAD data acquisition computer. These clocks were set to UTC at the beginning of each evening’s operations.

Time_Period_of_Content:

Time_Period_Information:

Range_of_Dates/Times:

Beginning_Date: 20030824

Ending_Date: 20030824

Currentness_Reference: ground condition

Status:

Progress: Complete

Maintenance_and_Update_Frequency: None planned

Spatial_Domain:

Bounding_Coordinates:

West_Bounding_Coordinate: 145.75
     East_Bounding_Coordinate: 145.78
     North_Bounding_Coordinate: 16.68
     South_Bounding_Coordinate: 16.72

Keywords:

Theme:

Theme_Keyword_Thesaurus: None

Theme_Keyword: Benthic habitat

Theme_Keyword: Underwater video

Place:

Place_Keyword_Thesaurus: None

Place_Keyword: Sarigan Island
     Place_Keyword: Commonwealth of the Northern Mariana Islands (CNMI)
     Place_Keyword: Mariana Archipelago

Access_Constraints: None

Use_Constraints: Please acknowledge the NOAA Coral Reef Ecosystem Division, Pacific Islands Fisheries Science Center as the source of this information.

Point_of_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Pacific Islands Benthic Habitat Mapping Center, CRED, PIFSC, NOAA

Contact_Person: John Rooney

Contact_Address:

Address_Type: mailing and physical address

Address: 1125B Ala Moana Blvd

City: Honolulu

State_or_Province: HI

Postal_Code: 96814

Country: USA

Contact_Voice_Telephone: 808-956-9729

Contact_Electronic_Mail_Address: john.rooney@noaa.gov

Data_Set_Credit: Pacific Islands Benthic Habitat Mapping Center, Coral Reef Ecosystem Division (CRED), Pacific Islands Fisheries Science Center (PIFSC), NOAA

Data_Quality_Information:

Logical_Consistency_Report: Unspecified

Completeness_Report: Complete

Positional_Accuracy:

Horizontal_Positional_Accuracy:

Horizontal_Positional_Accuracy_Report: The horizontal position accuracy for the camera sled position is estimated at ±100 meters.  There are three primary sources of this error.  The ship’s positioning is based on GPS SPS, often called standalone or non-differential GPS positioning.  SPS has a measured accuracy of under 5 meters.  The position of the GPS antenna is used; no attempt is made to translocate this position to that of the tow block from which the camera is deployed. The difference between the tow block and the antenna is about 14 meters. The largest source of error is uncertainty associated with the sled’s position relative to the tow block.

Lineage:

Process_Step:

Process_Description: After a tow was completed the video tape data and still photograpshs were reviewed by spot-checking to verify that data were recorded. Navigation data were copied from the acquisition computer to the data archive.  A metadata extractor (gloMeta.py, version 9/28/04) was run to summarize the navigation data and test for errors.  If errors were detected they were corrected or flagged as appropriate and a new data file was made.  This file's header was updated to document what type of processing occurred.  A metadata file was recorded in the data archive in the same location as the navigation data.  Processed navigation data were then read into ArcView 3.2 shapefiles using an Avenue script named `toadimport.playback.ave', version 9/25/01, written by Ronald Hoeke.

Process_Dates: 20051017

Distribution_Information:

Distributor:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization:

Pacific Islands Benthic Habitat Mapping Center, CRED, PIFSC, NOAA

Contact_Person: John Rooney

Contact_Position: Coastal Geomorphologist

Contact_Address:

Address_Type: mailing and physical address

Address: 1125 'B' Ala Moana Blvd

City: Honolulu

State_or_Province: Hawaii

Postal_Code: 96814

Country: USA

Contact_Voice_Telephone: 808-956-9729

Contact_Electronic_Mail_Address: john.rooney@noaa.gov

Resource_Description:

Digital video imagery that is geo-referenced to navigation files.

Distribution_Liability: These data are not to be used for navigational purposes. NOAA makes no warranty regarding these data, expressed or implied, nor does the fact of distribution constitute such a warranty. NOAA cannot assume liability for any damages caused by any errors or omissions in these data, nor as a result of the failure of these data to function on a particular system.

Standard_Order_Process:

Digital_Form:

Digital_Transfer_Information:

Format_Name: ASCII

Format_Information_Content: Navigation information are recorded as successive lines in a comma-delimited ascii text file, with a .glo extension. Each line of a GLO file includes the following data: the year, day of year, time, latitude, longitude, water column depth, vehicle depth, layback, and wing angle. The field for wing angle does not contain useful data. A total of 4 navigation files were collected, including:

Sar100.glo

Sar101.glo

Sar102.glo

Digital_Transfer_Option:

Offline_Option:

Offline_Media:  CD-ROM

Recording_Format:  ISO 9660

Digital_Form:

Digital_Transfer_Information:

Format_Name: mini digital video

Format_Information_Content:

These data were recorded on 2 video tapes, include 2 hours and 32 min of

video data covering 4.1 linear km of seabed.

List of video tapes:

SAR100-101

SAR102

Digital_Transfer_Option:

Offline_Option:

Offline_Media:  Video tape

Recording_Format: Mini DV

Fees: None

Metadata_Reference_Information:

Metadata_Date: 200512

Metadata_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization:

Pacific Islands Benthic Habitat Mapping Center, CRED, PIFSC, NOAA

Contact_Person: Dr. Michael Parke

Contact_Position: Research Biologist

Contact_Address:

Address_Type: mailing and physical address

Address: 1125 'B' Ala Moana Blvd

City: Honolulu

State_or_Province: Hawaii

Postal_Code: 96814

Country: USA

Contact_Voice_Telephone: 808-592-7025

Contact_Electronic_Mail_Address: michael.parke@noaa.gov

Metadata_Standard_Name: FGDC Content Standards for Digital Geospatial Metadata

Metadata_Standard_Version: FGDC-STD-001-1998

Metadata_Time_Convention: local time

Metadata_Access_Constraints: None

Metadata_Use_Constraints: None