# Tide Tool Manual

V7.0

June 2009

## Tide Tool Version 7.0, Client Version 1.1

Abstract

Tide Tool is an operational tool developed and supported by the Pacific Tsunami Warning Center for the decoding, display, and manipulation of sea level data transmitted over the WMO Global Telecommunications System (GTS). Tide Tool continuously decodes GTS sea level messages in real-time and displays the time series on a computer screen; users are able to interactively manipulate the time series to measure wave amplitude and period, and to remove the tidal signal. Non-operational versions also exist for data transmissions through the internet (ftp) and for archived data.

# 1. Introduction

Tide Tool is created by the Tide.tcl script, which is based on the Tcl/Tk extension called “BLT”. Tcl/Tk is an open source, platform-independent software package. Tcl/Tk at its heart is a robust shell programming language (tclsh) with a powerful, graphical toolkit (wish) (Appendix I). getTide is an additional BLT script that can be used to read and analyze old sea-level data log files. Tide.tcl provides end users with the ability to decode, display, and manipulate sea level data broadcast over the Global Telecommunications System of the World Meteorological Organization (WMO).  Data are continuously downloaded directly from the GTS to a data file, which is then decoded and displayed.  A version is also available using a non-GTS ‘ftp’ data transmission protocol in Linux and Windows XP/Vista for tsunami centers without GTS links (Appendices II and III).  The ‘ftp’ method should be considered non-operational if commercial internet service is utilized since connection reliability and timeliness, especially during tsunami emergencies, cannot be guaranteed.

The software application was originally developed by the US NOAA NWS Pacific Tsunami Warning Center to provide an operational tool for the real-time continuous tsunami monitoring in the Indian Ocean.  Its primary users would be National Meteorological and Hydrological Service (NMHS), or other agencies with a down link from the GTS or to a data file containing those data formatted in a similar manner.  It has been tested under UNIX, LINUX, Windows 2000, XP, and Vista environments in Indonesia, Malaysia, the IODE (Belgium), ITIC and at the PTWC.

The software is easily modified for changes in sea-level message formatting and the addition of sea-level stations.  The primary use of this software is as an operational program run by tsunami warning centers, or other agencies, that need to continuously monitor sea levels.  Mouse-clickable functions include the expansion of the time series, and measurement of the arrival time, wave height and wave period from the incoming signal.

Tide Tool V7.0 comes with dynamic map “clients” which allow users to select individual stations for display. The clients have a “zoom” feature that allows users to select an area on the map client and enlarge it. Socket handlers were added to the Tide.tcl script, such that Tide.tcl becomes a server listening for requests on a designated socket. The clients connect to Tide.tcl over this socket.

## Requirements:

In order to decode, run the dynamic map clients and display the data, the following are required:

- Computer running Tcl/Tk software with BLT extension

- Sea level data that are continuously archived into a data file

- Tide.tcl software

Tide Tool can be downloaded from http://ilikai.soest.hawaii.edu/ptwc/. Look for the latest version (versions numbered sequentially) and download the accompanying TideTool_MetaData file and the Tide_Coeffs directory. A complete metadata inventory of PTWC’s sea-level station list can also be found at this link in the COMP_META file. This will allow you to customize the stations your TideTool will decode.

In a related project, PTWC, in collaboration with colleagues at NOAA’s IDEA Center, is standing up a metadata server. We are encouraging maintainers of sea-level gauges and sea-level gauge networks to participate in this project. The goal is to establish a web service that provides up-to-date metadata for all sea-level stations. Sea-level station network managers will be able to maintain the metadata for their networks using this service. In the future, TideTool would be able to directly access the service to ensure the metadata it is using is up to date. If you are interested in being a user, supplier, or both, of sea-level station metadata, please contact Stuart Weinstein, PTWC.

Currently, the NOAA Idea Center is producing a kml file that, when imported into Google Earth, will give a global view of the sea-level stations and their metadata content. The kml file is developed from the COMP_META file. You can find the kml file at:

http://www.sealevelstations.org /stations.kml

## Computer and Software Requirements:

The software requires the installation of the Tcl/Tk software package and the BLT extension, both of which are freely available for download and easy to install.  The software has been installed and tested under Unix, Linux, and Windows  2000, XP, and Vista operating systems (OS).  Use on other platforms is possible as it only depends on the Tcl/Tk and BLT software being available.  The software does not require substantial computing power, and can thus run easily on a Pentium III or higher PC system.

Some users download the sea-level messages on a PC running a flavor of Windows and then transfer the data to a UNIX/LINUX system or vice versa. You must make sure that the scripts, metadata and sea-level message files are consistent with the platform Tide Tool is running on. Tide Tool checks what OS it is running under and expects the files to have the end-of-line and end-of-file terminators appropriate for that OS. Use the dos2unix or unix2dos utilities to ensure consistency with the OS.

It is not our intention to give a Tcl/Tk tutorial in this manual, as there are several good books and websites on the subject.  Please refer to a short list of them in Appendix I. There is a large Tcl/Tk user community, and there is a Google group (comp.lang.tcl) devoted to Tcl/Tk issues.

# Configuration

For Decoder

To execute Tide.tcl, you must run the bltwish.exe program. Note that bltwish24.exe is an alias to bltwish.exe, so either can be used.  UNIX users can turn the Tide.tcl script into an executable by making the first line in the script:

#!/full_path/bltwish  and then making the script executable (chmod u+x Tide.tcl).

When the bltwish console appears, just type source Tide.tcl and the program should start. Alternatively, you may type bltwish Tide.tcl on a command line to start the program.

For Windows PC users, another easy way to start Tide.tcl and the Map clients without having to open a console to type the command is to simply drop the script onto a bltwish24.exe icon.  To do this, create a short cut/alias of bltwish24.exe and place it as an icon on the desktop.  To run Tide.tcl and the map clients, highlight the tcl script from the directory where they reside and move/drop it onto the bltwish24.exe icon to start the script.

There are 13 customizable parameters. You can assign their values at the top of the Tide.tcl script. getTide also uses the same parameters. The assignment statements are already placed near the top of the script. They have the form:

set parameter “”

If nothing is placed between the “”, default values will be used or some features will be disabled. You must however specify the filename parameter.

The 13 parameters are:

1. filename: Tide.tcl assumes the log files which contain the GTS (WMO) data are in the form rootJJJYY.log where JJJ is a three digit Julian day and YY is the two digit year. The filename includes the path and filename suffix or root, and should be specified as path/root. Note that the ‘\’ is a special character in the Tcl world and must be escaped.  Therefore if you use the backslash in the path specification, it must be written as ‘\\’ otherwise the path will be garbled. The filename parameter must be set otherwise Tide Tool will not work.

An example from PTWC’s implementation of Tide Tool is:

set filename “/data/logs/nmc/sr”

where /data/logs/nmc/ is the path and sr is the root. Hence the filename of a log file for day 116, 2006 is /data/logs/nmc/sr11606.log .

2. dirname: Tide.tcl also has the ability to write the decoded tide data to files. The decoded data files have the syntax: station.gauge.JJJ where JJJ is Julian day. The data is arranged in a two-column ASCII format: time in fractional Julian day and sea-level height in meters. To enable this feature you must define the “dirname” parameter in the script. This parameter specifies where the decoded tide data is written. If this parameter is not specified decoded data will not be written to your system.

An example from PTWC’s implementation is:

set dirname  “/home/stuart/Tide/2009/”

Note: This directory must be created before running Tide.tcl; it will not be created for you.

3. hardirname: Tide.tcl will now de-tide the decoded sea-level messages. The files that contain the harmonic coefficients have the naming convention station_code.tide.coeffs. They are to be placed in the directory specified by the hardirname variable.

An example from PTWC’s implementation is:

set hardirname “/home/stuart/Tide/Tide_Coeffs/”

4. metadata_filename: The full pathname for the metadata file which determines which stations to decode and display. If not specified, “./TideTool_MetaData is assumed.

An example from PTWC’s implementation is:

set metadata_filename “./COMP_META”

5. mapdirname:  Tide Tool can now display maps showing the geographical context in which the sea-level station lies. These maps contain a lot of information and they will be described later in this document.

An example from PTWC’s implementations:

set mapdirname "/data/ttt_dbase/geoware/maps/"

6. tilelistfile: This specifies the name of a file which contains station-sensor code pairs. After Tide Tool is finished with its initial read of the logfile after startup it will automatically tile the screen with displays corresponding to those stations.

An example from PTWC’s implementation is:

set tilelistfile “/home/stuart/Tide/Tile_List”

7. record_filename: You can now record measurements of sea-level heights and those measurements will be written to the file specified by record_filename.

An example from PTWC’s implementation is:

set record_filename “../observations”

8. error_log: Messages with bad time stamps, improper formats and unrecognized headers will be written to the file specified by error_log.

An example from PTWC’s implementation is :

set error_log “Error_log”

The actual logfiles will be error_log.jd  where jd is Julian Day.

9. printer_command: Tide Tool stores the latest 100 messages from each tide station. These messages are viewable and can be made printable if the variable printer_command is set to the local system’s printer command.

An example from my implementation is:

set printer_command “enscript –fCourier-Bold8 2> /dev/null”

10. TWC_DART_FORMAT: If this is set to “1”, Tide Tool will write the decoded DART data files using the same format as does PTWC. An example of that format is given below:

File: 55015_nha_2008.235

The filename is composed of the DART buoy number, a three letter code and the year and Julian day. The three letter code can be one of nha, tra, tha, nhb, trb, thb, which indicate normal transmission mode (nha, nhb), trigger transmission mode (tra, trb) and triggered hourly messages (tha, thb). The contents of file 55015_nha_2008.235 look like:

# DART_Tasman_Sea_55015 dtas nha 55015 NDBC NORMAL_MODE -46.9222 160.5622 2008234

# DART m UTC 15 min Water_Depth PTWC Unfiltered

# DCP_PLATFORM_ID 55015P

# Data Format: SampleTIme(seconds from midnight 1/1/1970) WaterLevel SampleTime(yyyymmddhhmmss)

1219276800  04944.306  20080821000000

1219277700  04944.378  20080821001500

1219278600  04944.442  20080821003000

1219279500  04944.504  20080821004500

1219280400  04944.556  20080821010000

1219281300  04944.602  20080821011500

1219282200  04944.643  20080821013000

1219283100  04944.670  20080821014500

1219284000  04944.694  20080821020000

1219284900  04944.709  20080821021500

1219285800  04944.708  20080821023000

1219286700  04944.700  20080821024500

1219287600  04944.684  20080821030000

1219288500  04944.659  20080821031500

1219289400  04944.625  20080821033000

The first column is the epochal time (seconds since midnight 1/1/1970), the second is the water column thickness in meters and the right column is the human readable time at the time of measurement.

11. num_buttons_row:  This parameter sets the number of buttons per row on the main decoder GUI.

12. plot_width and 13. plot_height: These parameters set the dimensions of the basic Tide Tool sea level displays like that shown below for Benoa. An example value would be 3.0i where ‘i’ stands for inches. Likewise, the dimensions can be specified as 3.0c where ‘c’ is for cm. The defaults are 7.0i and 3.0i respectively.

For Clients

The clients have 10 required parameters however, you should be able to use the clients with little modification.  The only parameters that you need to configure are the metadata_filename and Map_File_Name parameters. The parameter descriptions are as follows:

1. left_lon  and 2. rht_lon : These parameters represent the longitude limits of the map.

3. top_lat  and  4. bot_lat : These parameters represent the latitude limits of the map.

5. Image_Width_Pixels  and  6. Image_Height_Pixels: The width and height of the map in pixels.

7. Map_File_Name : The name of the map file. As an example, for the Pacific client we specify the map file using:

set Map_File_Name  "./pacific_ocean_map.gif"

8. metadata_filename : This is the name of the metadata file (full pathname).

9. client_title : The name of the client that appears at the top of the client.

10. scale : The amount by which the selected area will expand when creating a client zoom widget. A scale value of two seems to give good results. A scale value greater than three produces somewhat fuzzy maps. You can use non-integers, but then the generation of the client zoom widget may take several tens of seconds depending on your computer.

You can certainly generate your own map files (in the GIF format) and if you change the above parameters in the client Tcl/Tk scripts you can create your own clients. For now, the clients can only do the linear/cylindrical projection. However, I expect to add the math needed to do some other common projections like the Mercator in the future.

## The COMP_META File

The COMP_META file is a flat file database containing a list of all stations decoded by Tide Tool. The COMP_META file has 18 columns and the metadata it contains includes the geographic coordinates for each station, and a number of parameters that facilitates the decoding of the sea-level messages. From left to right, the 18 columns are: station code, geographic name, platform ID number, WMO header, sensor type, sample rate for that sensor, number of samples for that sensor per message, observation units (F for .01ft and M for mm), a number that indicates the number of non-data fields in the data payload, time ordering (1 for chronological and -1 for reverse chronological order), a scale factor, transmission time interval, hour:minute of first transmission of the day, latitude, longitude, availability of tide coefficients for tide prediction (1 for available, 0 for unavailable), owner of the sea-level station and GLOSS ID (added at the request of GLOSS). An example is given below for the Benoa sea-level station pressure sensor.

beno  Benoa_ID  22856700  SXXX32 prs  1  30  M  3  -1   1.00   015  0007   -8.7500   115.2166  1  UHSLC  049

Any line in the metadata file that begins with “#” will be ignored. When Tide Tool starts it performs a “quality” check on the metadata file and will stop if the metadata file is not configured properly.

One can construct a customized metadata file using just the stations of interest. An example of this is the TideTool_Metadata file that is also available at the ftp site. This metadata file is customized for the Indian Ocean. The metadata can be organized into regions of your choice by placing a region name like “SUMATRA_JAVA” on a line by itself. The region name is your choice, however, white space is not allowed, hence underscores “_” must be used instead of blank space. The presence of region names will cause the main GUI to be segmented into different regions making it easier to locate stations on the GUI. An example is given in section 3.

To be consistent, the same Metadata file (parameter ‘metadata_filename’) should be used in both Tide.tcl and the map clients (Pacific_Client.tcl, Indian_Client.tcl, Caribbean_Client.tcl).  This will ensure that the Tide.tcl station GUI (showing decoded stations) and the map client (showing dots for each station with color-coded station health) are correctly shown and updated.

# Operational Details

Please note that Tide Tool enforces GMT (Greenwich Mean Time) when decoding. Therefore, you must make sure that your system gives the correct GMT time. This requires that you set the time zone on your computer to “0” offset from GMT. Do not simply change the clock on your system; change the time zone. Changing the clock will cause Tide Tool to assume that the change of day occurs either too late or too early. The use of GMT is necessary to remove any ambiguities with respect to time, as GTS data also uses GMT. If your system does not have the correct GMT time, Tide Tool’s behavior will be erratic.

Tide Tool version 7.0 consists of two basic parts, the decoder (created by the Tide.tcl script) with the familiar main GUI containing buttons for each sea-level station. The other part is the dynamic map client. There are currently three map clients; one each for the Indian Ocean, Pacific Ocean and the Caribbean. Tide.tcl listens for requests on port 15500. Communication between the decoder and clients is two-way. The clients and decoder can send instructions back and forth. The decoder can support any number of clients, so if you choose, you can run multiple instances of the same client, or all three clients.

When Tide.tcl is started it will read data from the current day data log. Tide.tcl will keep up to ~48 hours of data. As more data arrives beyond what Tide.tcl can hold, it will discard the older data to make room for the new. We have left some active debug code in Tide.tcl, so if you see a lot of output, do not worry. If Tide.tcl fails, please send PTWC’s Stuart Weinstein (stuart.weinstein@noaa.gov) as much of the debug output as you can, up to the last statement  before Tide.tcl failed. If possible, send the log file Tide Tool was reading at the time. Also send the error_log if you have one.

## The Decoder

When Tide.tcl is started, the current day log file is read and decoded, a simple GUI will be displayed that has buttons for each tide station in the metadata file. After the initial scan of the current day log file Tide.tcl goes into the background. Tide Tool sleeps and wakes up every 20 seconds to see if any new data has arrived. If so, it is decoded. Once started, Tide Tool  runs continuously.

The station name is listed as a four-letter code. The stations codes are obtained from the metadata file. The gauge code is only three letters; prs stands for pressure sensor, bub indicates bubbler, enc indicates encoder, rad indicated radar, etc. More information on the gauge types can be found in Appendix IV, which is extracted from the PTWC Operations, Systems, and Procedures Manual (Version 1.0, Dec 2008).

With Version 2.19, there were changes made to the Tide Tool GUI. Prior to 2.19, the GUI would not appear until Tide Tool was finished reading the log file. Now, the GUI appears first, but in a deactivated mode while the Tide Tool is first reading the log file. After the initial read is done, the GUI is activated. On the bottom of the GUI is a message widget showing how many bytes have been read from the current log file.

Note the lettering on the station buttons is dark violet indicating they are active. When Tide.tcl is first started, the lettering on all buttons is grey (inactive) except for the “EXIT” button. The station button section is placed on a scrollable, expandable canvas. Depending on the size and resolution of your screen, you may or may not need to scroll. Region

names like “SUMATRA_JAVA” are yours to create and are to be placed in the metadata file as described previously

Clicking on a button in the GUI will display a menu showing the gauges available:

In this snap shot, the Benoa, Indonesia station was selected and the menu showing three gauges (a pressure, radar and encoder gauge ) plus two other items “Info” and “Mess” are displayed. Those will be discussed later. Four buttons, EXIT, RETILE, REPORT and UPDATE STATUS appear on the bottom. If you make changes to the file referred to by the parameter tilelistfile, clicking RETILE will retile the screen. Clicking on “UPDATE STATUS” will send station

status information to the clients. Clicking “REPORT” will generate a table showing the number of messages received for each station. The last two features will be described in greater detail later

## The Monitor and Zoom Widgets

Selecting one of the sensors (for example, prs, rad, enc, bub) will create a monitor widget. This shows the sea-level curve as a function of time. Clicking on the rad  (acoustic radar sensor) for the Benoa (beno) station produces a monitor widget for Benoa station sensor:

Two curves are in the display. The green curve is the sea-level curve resulting from decoding the sea-level messages. The white curve is the de-tided sea-level curve using a tide prediction model. The tide coefficients were computed by Dr. Dailin Wang at PTWC. In some cases the available time series for the newer stations are too short to allow rigorous de-tiding. However, the bulk of the tide signal is usually eliminated. In the above figure the “TIDE” menu button has been clicked and it shows 4 options. The default is to plot both the observed and corrected tides. Other options are to show the observed or corrected separately or both with the predicted tide. On the bottom of the display, the time of the next expected transmission (Next XMT) for this station is given.

Clicking the “MENU” button produces a menu showing other sensors for this station:

This way, you won’t have to go to the main GUI to display curves for the other sensors.

Another feature is the “DESPIKE” button. Clicking the “DESPIKE” button will run the sea-level curve through a three point median filter.  Basically, at each point, the value is replaced with the median value amongst that point and its nearest neighbors. This gets rid of solitary bad points. For spikes/glitches lasting more than one sample interval, it may not eliminate the spike/glitch. In our experience, however, this simple technique has proven to be effective in the majority of cases.

Consider the mareogram obtained at Red Dog, Alaska. Clearly this station is having some sort of software or telemetry problem:

Clicking the “DESPIKE” button results in:

As you can see, this is quite an improvement. Also note that the “DESPIKE” button now reads “UNDESPIKE”. Clicking this button now will undo the despiking.

As new data is decoded by Tide.tcl, the monitor widget will be automatically updated to reflect the new data. The widget will only show up to ~24-48 hours of tide record. If new data is decoded for a station while a display widget for that station exists, the background of that display widget will change from black to blue for 0.6 seconds (This is lowered from 3 seconds in previous Tide Tool versions).

What operations can you do? By using the right button or number three button on your mouse, you can draw an expanding rectangle to zoom in on a section of the sea-level curve.  Hold the right button down and move the mouse diagonally, and you will draw a thin red rectangle. When the section of curve you wish to examine is in the rectangle, release the right button, and a zoom widget will pop-up with the selected portion of time serious expanded. This was done in the earlier figure showing the monitor widget for Benoa. The resulting zoom widget in the figure below:

You can also further zoom on the zoom widget itself. When the right button is released, the appropriate section of curve will be displayed in the zoom widget. The “TIDE” button at the top left has the same function as with the display widget.

On the zoom widget you can conduct simple measurements of amplitude and period. Using the left or number one mouse button, simply click on the curve in the zoom widget, and a summary inset will appear in the top left corner showing the coordinates of the point you just clicked on. A cross will mark the spot you picked. After a second point is clicked on, the coordinates of the second point will be displayed. In addition the difference in height and time between the two points will be displayed on the summary inset. Time difference is in minutes:seconds, and height differences are in meters.

A summary inset can be seen on the Benoa zoom widget shown above. Note the two crosshairs. If you wish to clear the summary display, click the middle or number two mouse button any where on the zoom widget canvas.

The record button “REC” will allow you to record sea-level observations in the file referenced by the record_filename parameter.

By clicking the point where tsunami waves first arrive and then clicking the “First Arrival” menu item, you can record the arrival of the tsunami at a sea-level station. By using two picks you can record either the “Zero to Peak” or “Peak to Peak” variation on the sea-level curve. Entries in the “record_filename” file are in this form:

kota  prs  First Arrival  111/07  08:43 2007111 11:48

kota  prs  Peak to Peak   111/07  08:43 H 00.208  111/07  09:08 H 00.155  Per 00:24 Amp 00.052 2007111 11:48

The observation records start with the station and gauge followed by the type of measurement and then the measurement(s). In the case of “First Arrival”, we have the day time stamp of the arrival in the format JJJ/YY HH:MM followed by the time this observation was recorded in format YYYYJJJ HH:MM. In the “Peak to Peak” or “Zero to Peak” cases, both picks are recorded with a time stamp JJJ/YY HH:MM followed by “H” (sea-level height flag) followed by the sea-level in meters. After the picks we have the time difference (after the “Per” flag) and sea-level difference (after the “Amp” flag) between the two picks followed by the time the observation was recorded.

The “Info” and “Mess” items allow you to display basic station data and recent messages for a given station. This is an example of an information widget. It shows basic information about the sea-level station.

Clicking on the “Mess” item will create a widget that shows the last 100 messages for the station that were recognizable to Tide Tool. An example of the messages widget is shown below:

Note that this widget is scrollable and has a print button. However, as different systems utilize different commands for printing text, the print button requires some configuration by the user. In particular the PRINTER_COMMAND variable must be set to a string that is the printer command. If this variable is not set, the “PRINT” button will not appear. It may require more work in the future to make this option a bit more user-friendly. If you cannot get it to work, contact Stuart Weinstein and we will try to develop a customized solution.

Clicking the “REPORT” button on the main GUI will give an account of the number of messages received and the number of messages expected for each station in the metadata file over a specified time period. You can select 3, 6, 12 or up to 48 hours. You can also print the report. The report is approximate in terms of the number of messages expected, however it is accurate enough so that you can keep track of stations that are having telemetry problems or other issues. Note that receipt of a message as indicated on the report does not mean the message was successfully decoded; only that it was received.

Another feature included is the “MAP” feature. Clicking on the MAP button will display a map (GIF file imported into a tk widget) showing the geographic context of the stations. There are three views showing the station at Benoa in a global, regional and local context. The location of the station is centered on the map. Contour lines showing tsunami travel times at one-minute intervals are drawn using the station is the starting point. These maps are useful when it comes to assessing how the station may be impacted by a tsunami. For example if the station is relatively sheltered or is facing away from the likely direction of tsunami propagation, it may not be ideal or useful for evaluating the destructiveness of the tsunami.

## Tide Tool Clients

Tide Tool has three basic clients, one each for the Indian Ocean, Pacific Ocean and Caribbean Sea. These clients, shown below, are interactive map interfaces.

Indian Ocean Client				  Caribbean Client

Pacific Ocean Client

In each interface display window, the small dots show the location of coastal sea-level stations and the larger dots show the position of the DART buoys. The colors of the dots indicate “status” and the meaning of the colors is given by the legend just below the map. Green indicates that at least one message was decoded for that station within the last 7 hours. Yellow indicates that at least one message was decoded within the last 24 hours, and Red indicates no messages received in over 24 hours. The reason these criteria were chosen is that the DART buoys in their normal mode (as opposed to their trigger mode) only transmit data 4 times a day.

Station status can be updated at any time by clicking the “STATUS” button on the decoder (Tide.tcl) GUI. These clients do not require any special configuration. Place the client scripts in the same folder as the decoder and start them running the same way as the decoder. Like the decoder, clients expect the metadata file to be in the same folder. When the clients connect, the message “CONNECTED” will appear on the bottom left of the client GUI as shown above. If the decoder is not running or the clients have not connected yet, the message will be “NOT CONNECTED”. If the decoder is shut down after the clients connected the message will be “DISCONNECTED’. Note that after the decoder (Tide.tcl) is started, it will not accept clients until the initial read of the current log file is completed.

A single left button click on a dot will display the station code for that station. A double left button click will create a menu button, just like the one on the decoder GUI, and you can choose to display a sensor.

The Exit selection will delete the button. Only one button at a time will appear, so if you request another button, any pre-existing button will be deleted. You will notice that some stations have a yellow halo; this is to inform you that the station has been moved, graphically, from its actual location so that it can be located and clicked on the map. When you use the zoom feature, most of these halos will disappear as there is more resolution and less overlap among the station dots.

The “DISP/FIND STATIONS” menu button on the bottom can be used to find stations on the map. Clicking this button produces the menu:

The first two items if checked will cause the station codes and/or DART NDBC numbers to be displayed on the map. The “FIND” items are cascading menus that allow you to locate a station by its station code or DART NDBC number:

To zoom in a section of the Client map, use the right mouse button. Click and traverse from one corner of the region you wish to enlarge, to the opposite corner and release the button. This will create a “zoom” client

The zoom client has the same functionality as above. It also has “EXIT” and “TILE” buttons on the bottom. Clicking “EXIT” will destroy the zoom widget.

Clicking “TILE” will tile the screen with all stations that have data, present in the zoom region. The “TILE” button will then switch to “UNTILE”. Clicking “UNTILE” will remove the tiled widgets. Currently, you can only have one zoom widget per client. If a new zoom widget is created, the old one will disappear. If you click “TILE” on the new zoom widget, the old tiling will be destroyed and a new tiling using stations in the current zoom will be created. If you wish to tile using stations from two separate regions, you will need to create another instance of the client, and create a zoom widget from each client.

## 4. Future Directions

Among the new features under consideration for future releases of Tide Tool are :

1. The use of environment variables in the scripts. This way, no modification will be necessary if you define the environment variables on your system. Tide.tcl will query

environment variables to obtain the parameter values.

2. Strip Chart Widget. In addition to tiling the screen with selected stations, the user will also be given the option to have those stations arranged in a strip chart widget. This is what is typically done in displays of seismic data.

3. Real Time tide modeling. In some cases, not enough data is available to create a good set of tide harmonic coefficients. In other cases the tide coefficients are no longer adequate or meteorological effects are skewing the de-tiding. In these cases, it is useful to compute tide coefficients on the “fly” using the 48 hours of data held by Tide Tool.

4. Dynamic reconfiguration. Changing station metadata or adding new stations will not require a restart of Tide Tool.

getTide

The getTide program has all of the above features (with the exception of the auto-tiling feature), but is designed to read old logfiles. There is one important caveat to be aware of in using getTide. You must to be sure that the metadata used by getTide is consistent with the date of the logfile you wish to examine. Sensors and data units do change through time. This is another important reason to consider joining the metadata web service effort described earlier. Such a service will create a metadata file consistent with any time frame and release you from having to keep track of metadata changes yourself. The use of getTide is very simple. Upon starting getTide a simple entry widget will appear:

This widget requires three entries, Julian day, Year and SPAN which is the number of days of data you wish to read starting at the Julian day and year entered. After clicking the “DONE” button, this widget disappears and the Tide Tool GUI then appears. After this, everything is basically the same as Tide Tool.

Author:

Dr. Stuart Weinstein

Asst Director, Richard H. Hagemeyer Pacific Tsunami Warning Center

Please send all questions, bugs, requests etc. to:

stuart.weinstein@noaa.gov

For other related questions or assistance, please contact Dr. Laura Kong, Director, IOC International Tsunami Information Centre, l.kong@unesco.org

Acknowledgments

S. Weinstein is indebted to several colleagues for their help and ideas in creating and improving Tide Tool. The genesis of Tide Tool was a request from Dr. Laura Kong in late 2005 to develop a quick and “dirty” tool that can be used to decipher GTS sea-level data, display the results and allow basic measurements of amplitude and period to be made from the sea-level record. Tide Tool 1.0 was released in Nov. 2005. The earliest users (and perhaps “victims”) of Tide Tool were the Met. Office in Malaysia and BMG in Indonesia. Tide Tool has proven so capable that it is now part of the operational software suite at PTWC.

In Malaysia, Namran Hussein provided valuable assistance in debugging Tide Tool. Dr. Yap Kok Seng, Low Kong Chiew, and Saw Bun Liong also provided suggestions and tested Tide Tool on Windows platforms.

Dr. Fauzi (BMG) provided the first detailed set of instructions for obtaining everything needed to get Tide Tool running on LINUX. Indra Gunawan was also very helpful in trouble shooting problems with Tide Tool.

Dr. Yohko Igarashi (JMA/ITIC) provided valuable assistance in improving the port of Tide Tool to both LINUX and Windows and developing the scripts/processes used to obtain GTS data from the Web.

Dr. Dailin Wang (PTWC) created and maintains the suite of tide coefficients used in de-tiding and is currently helping me to develop an “on the fly” de-tiding algorithm in Tcl.

Dr. Nathan Becker (PTWC) designed and created the station maps and the maps for the Tide Tool clients.

Peter Pissierssens (IODE) offered several valuable suggestions during my visit to IODE in 2006. Mark Van Crombrugge (IODE) helped get Tide Tool running on Windows.

Last but not least, I thank Chip McCreery and the PTWC staff for putting up with my mood swings while working on Tide Tool.

# Appendix I.

Reading Materials:

Practical Programming in Tcl and TK by Brent Welch. Prentice Hall.

A super book and introduction to Tcl/Tk. You can find it in many bookstores or obtain it from an online source.

Effective Tcl/Tk Programming by Harrison and McLennan

This covers fundamentals of event driven programming. For those of you who do mostly serial programming in Fortran or C, event driven programming is a bit a of a trip.

Tcl and the Tk Toolkit by John Ousterhout

Ousterhout invented Tcl/Tk. So I feel obliged to mention his book.

Websites:

http://www.activestate.com/Products/ActiveTcl/

You can get binaries here. This site has something for everyone.

http://tcl.sourceforge.net/

http://sourceforge.net/projects/blt/

You can get binaries here and keep tabs on the latest FAQs, versions, bugs, and advances in the Tcl world.

http://phaseit.net/claird/comp.lang.tcl/HowToC.html

Howto, on how to integrate the power of C programming with Tcl/Tk. I use combinations like this at PTWC. Cameron is one of the great Tcl/Tk experts and frequently answers Tcl/Tk questions on the comp.lang.tcl Google news group. You may find me there on occasions as well.

http://wiki.tcl.tk/199

A starting point for BLT

# Appendix II.

# Tide Tool using Internet Data Download

# Windows XP and Vista

Developed and maintained by the Pacific Tsunami Warning Center

Windows internet version by International Tsunami Information Center

Distributed by International Tsunami Information Center

For more information, contact

Stuart Weinstein (stuart.weinstein@noaa.gov)

Laura Kong (l.kong@unesco.org, laura.kong@noaa.gov)

Version September 2009

Tide Tool is an operational tool developed and supported by the Pacific Tsunami Warning Center for the decode, display, and manipulation of sea level data transmitted over the WMO Global Telecommunications System (GTS).  The tool continuously decodes sea level in real-time and displays the time series on a computer screen; users are able to interactively manipulate the time series to measure wave amplitude and period, and to remove the tidal signal.   A separate process archives GTS-downloaded data continuously through a dedicated line (recommended), or alternatively by commercial internet wget (not recommended for operations requiring 100% reliability).  An offline tool (getTide) is available for archived data.

The following are instructions for the installation and running of Tide Tool on a PC under a Windows XP or Vista operating system and with data collection through internet through “wget”

Instruction for Tide Tool (Windows version)

- If you already have an old version of Tide Tool, delete the files.

- Download the latest TideTool_ftp_Win_7.x.zip.  When unzipped, the directory has the files:

SLdecode_display_summary4.pdf   (general summary)

Tide Tool 7.0 Manual.doc  (User manual – these install instructions are in Appendix II)

PC_install_dirinfo.txt   (contains directory structure specific for this installation)

Setup.exe  (installs software)

- Run Setup.exe to place files in the correct directories and create desktop shortcuts.

Setup.exe will:

- copy all files in C folder to C:\

- create Shortcut for bltwish.exe on desktop and name it: RunBltwish

- create Shortcut for tclsh.exe on desktop and name it: RunTclsh

- create Shortcut on desktop to go to C:Tcl\bin and name it TideTool_bin

- create Shortcut on desktop to go to C:Tcl\TideTool and name it TideTool_data

- create C:Tcl\uninst_TideTool.exe (Uninstall program) in case there is a need to uninstall this package.  Unist.exe will delete all installed files except the collected data files in the folder SR_LOG (srJJJYY.log) and Decoded_Data (decoded station data by Julian Day)

Setup,exe in Tide_Tool_ftp_Windows.zip places the following files / folders under C:\

FILE/Folder NAME

----------------------------------------------

Tcl  <Folder> -->	bin        <Folder> --> 	Tide.tcl(V7.0)

get_data.tcl

wget.exe

chikan.exe

Pacific/Indian/Caribbean_Client.tcl

pacific/indian/caribbean_ocean_map.gif

Tile_List (stations to display at start up)

getTide.tcl

TclTK & BLT files

MetaData      <Folder>--->	COMP_META and

TideTool_MetaData region files

doc        <Folder>

include    <Folder>

lib        <Folder>

licenses   <Folder>

INSTALL.LOG

license-at8.4-thread.terms

MANIFEST_at8.4.txt

README-8.4-thread.txt

uninst.exe

TideTool   <Folder> --> 	BOX           <Folder>--->	data downloaded from ftp site

Decoded_Data  <Folder>--->	decoded data by stations

Error         <Folder>--->	Error_LOG file

MAPS          <Folder>--->	maps for stations

SR_LOG        <Folder>--->	srJJJYY.log files of GTS

transmissions

Tide_Coeffs   <Folder>--->	(station_name).tide.coeff

past_metadata <Folder>--->	past metadata files for

getTide.tcl

README

SLmap-AO.jpg (station maps)

SLmap-IO.jpg

SLmap-PO.jpg

Tide Tool 7.0 Manual.doc

Instr_TideTool_Windows.pdf

README

- TclTk (V8.4.19.1) and BLT (V2.4) are included so there is no need to separately download

- wget.exe is used to download data files (e.g., files from NWS telecommunications gateway)

- chikan.exe is used to convert from Unix (\n) to MS-DOS (\r\n).

- Set your PC to use UTC (GMT, Greenwich Mean Time).

- Customize Tide.tcl and map client scripts to specify directories where files to be found or created, and which files are to be used.  For this Windows package, the customization has already been done using the information in PC_install_dirinfo.txt.

Some variables that you may want to additionally change are:

  - ‘metadata_filename’ variable, if you want only a subset of the data to be decoded and displayed.  Pre-selected files available are:

    - COMP_META (default) will decode all stations and list in alphabetical order

    - COMP_META_region will decode all stations and list by region

    - TideTool_MetaData_IO is for Indian Ocean

    - TideTool_MetaData_NEPO is for Northeast Pacific Ocean

    - TideTool_MetaData_NWPO is for Northwest Pacific Ocean

    - TideTool_MetaData_SEPO is for Southeast Pacific Ocean

    - TideTool_MetaData_SWPO is for Southwest Pacific Ocean

    - TideTool_MetaData_NWAO_CAR is for Northwest Atlantic Ocean, Caribbean, Gulf of Mexico

    - TideTool_MetaData is to specify user-selected stations to decode/display.

  - ‘tilelistfile’, which specifies which sea level station monitors to bring up at Tide.tcl startup.  An example Tile_List is provided.  You can set it not to bring up any monitors at startup by leaving this file unspecified.  For example, if you need to monitor the 3 stations “yapi”, “mani” and “davo” for prs sensor, then Tile_List would look like

yapi prs

mani prs

davo prs

After Tide.tcl is running, you can edit the file Tile_List to designate other stations, and then hit the button “RETILE” to show these station monitors.

  - ‘printer_command’, which specifies the command for printing.  Presently, the default is set for a linux/unix command.

- Start get_data.tcl (drop icon onto RunTclsh icon)

This short cut has already been created by the Package to provide an easy way to start get_data.tcl without having to open a console to type the command.  This is to simply drop the script onto a tclsh.exe icon.  To run get_data.tcl, highlight the tcl script and move/drop it onto the tclsh.exe icon to start the script.

get_data.tcl runs wget.exe twice, first to obtain the filename of the most current data file and second to download that data file.  The downloaded data file is then concatenated to the existing srxxx.log file of transmissions, which is used by Tide.tcl.

- Start Tide.tcl (drop icon onto RunBltwish icon)

This short cut has already been created by the Package to provide an easy way to start Tide.tcl and the Map clients without having to open a console to type the command.  This is to simply drop the script onto a bltwish.exe icon.  To run Tide.tcl and the map clients, highlight the tcl script and move/drop it onto the bltwish.exe icon to start the script.

Tide.tcl reads and decodes the existing data in the current day’s SRLOG file (srxxx.log), and then reads Tile_List and automatically displays small same-sized windows for each station data.  Note that at the beginning, when srJJJYY.log is small and/or little station data are obtained, only a small number of small windows will be displayed.  If you rerun the Tide.tcl later on the first day of your usage, it will display more station data since srJJJYY.log is larger.

IMPORTANT NOTE:

get_data.tcl must always be running in order to collect the sea level data.  If not, the data gaps will display for the times when data are not collected!

- Start map clients (drop each map client icon onto RunBltwish)

Each client opens a map of a different ocean showing the stations (Pacific_Client.tcl, Indian_Client.tcl, Caribbean_Client.tcl).   More than 1 map client can be run at the same time.   Note that Tide.tcl must always be running since the maps are only clients of Tide.tcl.

You may also start Tide.tcl and the map clients manually by:

- Double click on the C:\Tcl\bin\bltwish.exe (or C:\Tcl\bin\bltwish24.exe) to display console for each script.

- In the console window, type (example is for Indian Ocean) % bltwish Tide.tcl &

- In the console window, type (example is for Pacific Ocean map)

% bltwish Pacific_Client.tcl &

getTide.tcl

TideTool v7.0, which has many new features, but these features have not yet been fully implemented in the accompanying getTide.tcl script.  This is planned in the future.

To run getTide.tcl, double click C:\Tcl\bin\bltwish.exe (or C:\Tcl\bin\bltwish24.exe) and type

% bltwish getTide.tcl   or   % bltwish getTide.tcl > tmp

where tmp is a log file to see what has happened in case of error.  The use of tmp to save the error output is useful for troubleshooting since older versions of Tide Tool use old-format metadata file formats.  See the Tide Tool Manual for more information.

# Appendix III.

# Tide Tool using Internet Data Download

# Linux / Unix

Developed and maintained by the Pacific Tsunami Warning Center

Distributed by International Tsunami Information Center

For more information, contact

Stuart Weinstein (stuart.weinstein@noaa.gov) or Laura Kong (l.kong@unesco.org)

Version March 2009

Tide Tool is an operational tool developed and supported by the Pacific Tsunami Warning Center for the decode, display, and manipulation of sea level data transmitted over the WMO Global Telecommunications System (GTS).  The tool continuously decodes sea level in real-time and displays the time series on a computer screen; users are able to interactively manipulate the time series to measure wave amplitude and period, and to remove the tidal signal.   Non-operational versions also exist for data transmissions through the internet (wget) and for archived data.

Instructions for the installation and running of Tide Tool on a PC under a Linux operating system and with data collection through the internet ftp are provided below.

Instruction for Tide Tool (Linux version)

1. Put Tide_Tool.tar.gz on the Desktop and extract the files;

> gunzip Tide_Tool.tar.gz

> tar xvf Tide_Tool.tar

Check if

- directory named “Tide_Tool” is created on the Desktop

- directories named “Decoded_Data”, “MAPS”, “past_metadata”, “SR_LOG”, “Tide_Coeffs” and

“V7.0” are under “Tide_Tool”

- files named “Tide Tool 7.0 Manual.doc” and “SLmap-AO/IO/PO.jpg” are under “Tide_Tool”

Create a directory named “BOX” on the Desktop.

2. Check if the following files are in “V7.0”;

-- COMP_META

-- TideTool_MetaData, TideTool_MetaData_IO/PO/NEPO/NWPO/SEPO/SWPO/NWAO_CAR

-- URL_FILE

-- Tile_List (sample)

-- Tide.tcl ………………… script to monitor the sea level data (near real-time)

-- get_data.tcl …………….. script to continuously download sea level from NWS gateway

-- getTide.tcl ………………script to show the past sea level data

-- Pacific_Client.tcl, Indian_Client.tcl, Caribbean_Client.tcl …… scripts to display a map

-- pacific_ocean_map.gif, indian_ocean_map.gif, caribbean_map.gif…..maps for each ocean

3. Install TclTk & BLT and set the path.

Download software of TclTk (8.4.14 or upper, lower than 8.5) and BLT and install on your machine.

http://www.tcl.tk/software/tcltk/  for TclTk

http://sourceforge.net/project/showfiles.php?group_id=18616  for BLT

How to install is mentioned in the instruction of each source, so please follow the instruction.

How to set the path (commands, filenames, file locations, etc.) depends on your machine.

Here is an example;

> whereis tclsh8.4 ….Find the location of tclsh8.4

> whereis wish8.4 ….Find the location of wish8.4

> whereis bltwish  ….Find the location of BLT

> vi .bash_profie……Open the setting file to edit.

File name might be different if yours is not using bash but tcsh and so on.

With the command “ls –al”, you can search which file is for the setting.

Add the path for tclsh8.4, wish8.4 and BLT in the setting file.

For example, when all of the three are in /usr/local/bin, add the following lines;

PATH=$PATH:/usr/local/bin

Export PATH

4.  Edit Tide.tcl to specify different filenames or directory paths than currently specific.

Some variables that you may want to change are:

a. ‘metadata_filename’ variable, if you may want only a subset of the data to be decoded and displayed,   Some files available are:

COMP_META (default) will decode all stations

TideTool_MetaData_IO is for Indian Ocean

TideTool_MetaData_NEPO is for Northeast Pacific Ocean

TideTool_MetaData_NWPO is for Northwest Pacific Ocean

TideTool_MetaData_SEPO is for Southeast Pacific Ocean

TideTool_MetaData_SWPO is for Southwest Pacific Ocean

TideTool_MetaData_NWAO_CAR is for Northwest Atlantic Ocean,

Caribbean, Gulf of Mexico

TideTool_MetaData to select user-selected stations

b. ‘tilelistfile’, which specifies which sea level station monitors to bring up on Tide.tcl startup.  An example Tile_List is provided.  Presently, the default is not to bring up any monitors at startup.  For example, if you need to monitor the 3 stations “yapi”, “mani” and “davo” for prs sensor, then Tile_List would look like

yapi prs

mani prs

davo prs

c. ‘printer_command’, which specifies the command for printing.  Presently, the default is set for a linux/unix command.

5. Set your machine to use UTC

Tide.tcl works following UTC so set your machine to use UTC, or your machine will not create a

new srJJJYY.log when the UTC date changed and shows errors.

7. Run the scripts

Type as follows under Desktop/Tide_Tool/V7.0, and get_data.tcl starts.

> /usr/local/bin/tclsh ./get_data.tcl &   (if you’ve set the path, just type ./get_data.tcl &)

Wait until the 1st file is successfully downloaded and then start the decoding / displaying

Type as follows under Desktop/Tide_Tool/V7.0, and Tide.tcl starts.

> /usr/local/bin/bltwish ./Tide.tcl &   (if you’ve set the path, just type ./Tide.tcl &)

When you need a map of Pacific Ocean, type

> /usr/local/bin/bltwish ./Pacific_Client.tcl &

You can add more maps if you need, just by adding lines in the same way.

Tide.tcl reads srJJJYY.log, which get_data.tcl creates.

Tide.tcl reads Tile_List and automatically displays small data windows for each station

Note that at the beginning, srJJJYY.log is small and not all the station data are obtained,

so only a limited number of small windows will be displayed.

If you rerun the Tide.tcl later on the first day of your usage, it will display more stations

since srJJJYY.log is larger. Then, you can keep those windows forever and need not rerun again.

When you run getTide.tcl, type

> /usr/local/bin/bltwish ./getTide.tcl                          (if you have set the path, just type  ./getTide.tcl)

Then, put the day, year and span in a small window. # As mentioned above, use UTC to count days.

Desktop       Tide_Tool        V7.0                      Tide.tcl

Pacific/Indian/Caribbean_Client.tcl

Pacific/Indian/Caribbean_map.gif

get_data.tcl

wget.exe

getTide.tcl

URL_FILE

TideTool_MetaData

BOX                                               TideTool_MetaData_IO/PO/NEPP/NWPO/SEPO/SWPO/NWAO_CAR

COMP_META

Tile_List

Error_LOG (to be created)

Error_LOG_past (to be created)

Tide_Coeffs -- coefficient files for the sea level stations

Decoded_Data -- decoded data are archived in this directory

SR_LOG -- srJJJYY.log files are archived in this directory

MAPS -- maps for each station

past_metadata -- past COMP_META & TideTool_MetaData

Tide Tool 7.0 Manual.doc

SLmap_AO/IO/PO.jpg

Tide_Tool_LOG (to be created)

Tile_Tool_LOG_past (to be created)

Directory Structure

APPENDIX IV

SEA-LEVEL GAUGE INSTRUMENTATION

Extracted from

PTWC OPERATIONS, SYSTEMS AND PROCEDURES MANUAL, Edition 1.0, December 2008

### CHAPTER 6.  SEA-LEVEL DATA PROCESSING

### C.  SEA-LEVEL GAUGE INSTRUMENTATION

### a.  Coastal Gauges

The sea-level networks of the world’s oceans utilize a combination of NOAA/GOES, JMA/GMS or EUROSAT satellite transmission communications. These data arrive at PTWC via the NESDIS/NWS communications routing through the NWSTG onto line 58.  The sea-level data is collected and transmitted from the sea-level station using a Data Collection Platform, (DCP) that are procured from various manufacturers. The DCPs provide a suite of programmable parameters for data (water-level) monitoring and data transmission via satellite. Other meteorological sensors can also be utilized in conjunction with these systems. The water-level data is sampled at a 2 second rate averaged over a 2 minute period and transmitted via GOES or GMS on an hourly basis to PTWC. However, NOS (US NOAA National Ocean Service) is upgrading sea-level stations to transmit 6 one-minute samples every 6 minutes. Already a number of NOS stations in Hawaii and Alaska region have been upgraded to the higher transmission rate. Supported by the IOC with funds from NOAA, the UHSLC is upgrading the GLOSS stations in the Indian Ocean. The upgraded gauges in the Indian Ocean transmit every 12 or 15 minutes, with 12 to 30 one-minute samples for the sensors with the highest sampling rate.

Each sea level site may use one or more of 6 types of sensors:

- SPS – (Submersible Pressure Sensor) measures water-level sea pressure from 0 – 45 feet. Backup pressure sensor code is SPB.

- PR1,PR2 – Pressure sensors.

- BUB -  (Bubbler Pressure Sensor) measures water-level sea pressure, from 0 – 45 feet (utilizing a nitrogen gas purging system) in a cavity submerged in the water. As sea-level changes, the pressure in the nitrogen gas cavity changes. Backup bubbler code is BBB.

- ENC – (Incremental Encoder Sensor) measures water changes developed in a stilling well system, incrementally changing a pulley using a float and weight.  As sea-level changes a float moves up and down. The operation is similar to the float in your toilet. Backup encoder code is ENB.

- PAQ – (Primary Aquatrak) measures sea-level with acoustics. The transmitter and receiver are above the water. Sea-level is determined by the round trip time for the acoustic signal. Backup aquatrak code is BAQ. PAQ designation is used for NTF gauges.

- PWL – This is the designation for NOS aquatraks. BWL is the backup NOS sea-level gauge designation, usually a Paros pressure sensor.

- RAD  – (Radar) measures sea-level using radar. The RAD sensor is placed above the sea-level and bounces radar signals off the sea-surface. Timing the round trip time yields the sea-level.

The sea-level stations systems are generally located in ports or bay areas in secure areas with facilities. However, they can be located in remote locations on most any shoreline where there is a suitable area for anchoring the sensor and the Electronic Systems are safe from the elements. The photos below (courtesy Richard Nygard) show the basic elements of the Installation at Hiva Oa, Marquesas. The Hiva Oa station primary sensor is a pressure sensor (SPS) while the secondary gauge is an encoder (ENB).

Hiva Oa Installation                                		Hiva Oa Stilling Well

Vaisala DCP, wiring, 12v battery, and solar regulator.

## DART Buoys

The DART buoy concept is illustrated here:

The DART sensor is a pressure sensor that sits on the sea floor in very deep water and measures sea-level fluctuations averaged over 15 sec to a precision of 1 mm. The data is relayed by acoustic telemetry to the buoy and is then transmitted to satellite. The data is transported to PTWC via line 58.

Why DART buoys?  The Achilles heel of the warning system is the unnecessary evacuations. An evacuation of the State of Hawaii will easily cost the State tens of millions of dollars in lost business revenue and the costs associated with executing emergency procedures. Frequent unnecessary evacuations will undermine public confidence in the warning system.  If people stop paying attention to the warning system, then we have failed the public.

Why do we have unnecessary evacuations? Because forecasting how large a tsunami generated on the Pacific Rim will be when it arrives on distant shores is still very much in its nascent stages. Coastal tide gauges provide an imperfect guide as to how destructive a tsunami will be as it propagates away from the source. This is because the near shore bathymetry and geometry of the harbor where the tide gauge is situated have as much effect on the wave-height and inundation as the fundamental characteristics of the tsunami itself.  Along the same coast, it is not unusual to find harbors that resonate strongly with tsunami and those which show much less tsunami activity. Therefore the picture given by coastal tide gauges is somewhat murky; and forecasts with respect to damage on distant coasts fraught with peril. What was needed was a way to distinguish between tsunamis that will cross oceans with destructive power and tsunamis that may produce measurable, but non-damaging waves on distant coasts.

By measuring the properties of a tsunami far from the influences of near-shore bathymetry and geometry, the DART buoys give us a look at the tsunami in deep water unfettered by resonant effects. Furthermore, while modeling tsunami propagation in the near shore environment is difficult, modeling tsunamis as they make their way into deep water, is a much simpler problem and can be simulated with more robustness.

Therefore both Paul Whitmore of the ATWC (EARTHVU) and Vasily Titov of PMEL (SIFT) have produced a tsunami wave-height forecast database of pre-computed tsunami scenarios. These models will make predictions based on basic seismic data as to what size tsunami should be recorded by the DART and coastal buoys. By comparing the predicted results to the observed, the watchstander should be able to gauge whether or not the tsunami will be destructive when it arrives on distant shores.

- TSUNAMI TRAVEL-TIME AND WAVE-HEIGHT FORECASTING

Whenever a Watch/Warning message is issued, the watchstander needs to run software to compute the travel times to the various warning points supplied by our clients. This information is then relayed to our clients in our message products.

As mentioned earlier, travel-time calculations are relatively simple calculations; they can be done quickly in real-time. However, even this calculation can be speeded up by compiling a database of pre-computed scenarios. So we have two approaches for computing tsunami travel-times. The approach to be used during an event is to use the travel-time database because it is faster. However, this approach (eta.tcl) does not produce any graphical products. After bulletins are sent the watchstanders can work with travel-time tool (tttool) to produce a travel-time map. tttool will also compute travel-times, and can be used as a backup to eta.tcl.

Among the most challenging numerical simulation problems is that of the tsunami. It isn’t the fluid that’s difficult; it’s the boundary conditions that are a pain. In order to model wave-heights, the details of near shore bathymetry and coastline must be taken into account. This requires very fine numerical resolution. Yet, applying such resolution to the whole of an ocean basin would require vast computational resources. Therefore such models involve multiple numerical grids on many scales. Wave-height forecasting is still in its nascent stages although we can see the light at the end of the tunnel. Coupled with a network of DART gauges, these current models should enable the watchstander to distinguish between destructive and non-destructive tele-tsunamis.  Two tools that are currently being used are SIFT (still being developed and tested) and EARTHVU (WC/ATWC)