FOLDER CONTENTS:

August 2010, lkong@unesco.org

The folder 27feb10_TideTool_data contains the folder SRLOG_Chile_27feb10 with the raw data files collected by Tide Tool for the 27 February 2010 Chile tsunami.

Tide Tool can be run in ‘archive’ mode to view past data.  In order to run, use the following:

Use the correct station metadata file to view archived data.  For 27 Feb 2010, this is the file in in the folder 27feb10_COMP_META.  Replace the current COMP_META file with this (but be sure to save the current COMP_META file to another filename so you will have it for the real-time version)

- Open command window in C:/Tcl/bin

- Type “bltwish.exe Tide.tcl H”

- Enter JD, YR (2 digit), SPAN (1 or 2 corresponding to number of days of SRLOG files)

Tide.tcl will decode and then display the SRLOG files specified

Pacific_Client.tcl (and other map clients) can be run